3GPP TSG RAN WG1 #106bis-e		R1-2108897

e-Meeting, October 11th – 19th, 2021

Agenda Item:     8.1.2.2

Source:	Spreadtrum Communications

Title:	Discussion on enhancements on multi-TRP inter-cell operation

Document for:	Discussion and decision

Introduction

According to R17 FeMIMO WID [1], the following objective is included in the scope:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception

In RAN1#106e meeting, after extensive meeting discussions, the following agreements have been achieved [2]:

Agreement Introduce a new RRC indicator/signalling (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with, where the new indicator/signaling is not the exact PCI value - Detailed signalling design is up to RAN2 Agreement Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS : Which values to support other than 1. - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States Agreement - For inter-cell mTRP , one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex - FFS : The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP Agreement For a CSI-RS QCLed with a neighboring cell SSB, the CSI-RS EPRE is calculated based on powerControlOffsetSS and the SSB transmission power in the neighboring cell information.

In this paper, we will present our views on these issues.

Discussion

Regarding the value of X reported by UE, although both Alt 1 and Alt 3 could reflect UE’s capability, but we slightly prefer Alt 1 since it is a very simple solution.

Proposal 1:  Regarding the value of X reported by UE, support Alt 1, i.e., a single value of X is reported as UE capability for any possible SSB time domain position and periodicity.

In Rel-15/Rel-16, for serving cell, all other signals and channels except for SSB could not be mapped into the PRBs occupied by SSB, and only PDSCH or CSI-RS for beam management which is QCLed with a SSB can be transmitted in the same symbol as the SSB.  The motivation is straightforward and also important to ensure access performance for the cell. However, there is no any resource mapping restriction when referring to non-serving cell SSB. Thus, in our opinion, it seems there is no need to be rate-matched around non-serving cell SSB for all the signals/channels in the serving cell. Likewise, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info also should not be rate matched around serving cell SSB.

Proposal 2:  For inter-cell multi-TRP operation, PDSCH/PDCCH from the serving cell should not be rate-matched around non-serving cell SSB.

Proposal 3: For inter-cell multi-TRP operation, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is not rate matched around serving cell SSB.

Conclusion

In this paper, we provided our views on Multi-TRP inter-cell operation, and have the following proposals and observations.

Proposal 1:  Regarding the value of X reported by UE, support Alt 1, i.e., a single value of X is reported as UE capability for any possible SSB time domain position and periodicity.

Proposal 2:  For inter-cell multi-TRP operation, PDSCH/PDCCH from the serving cell should not be rate-matched around non-serving cell SSB.

Proposal 3: For inter-cell multi-TRP operation, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is not rate matched around serving cell SSB.

Reference

- RP-202024, Further enhancements on MIMO for NR

- Draft Report of 3GPP TSG RAN WG1 #106e