3GPP TSG RAN WG1 #106bis-e                                              R1-2108915

e-Meeting,  October 11th – 19th, 2021

Agenda Item:	8.6.2

Source:	Spreadtrum Communications

Title:	Discussion on early indication for RedCap UE

Document for:	Discussion and decision

Introduction

In the RAN1#106-e meeting, early indication for RedCap UEs was discussed thoroughly and the following agreement was made [1]:

Agreement Confirm the following working assumption with the modifications in red: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled via SIB - FFS how to support enable/disable the early indication - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported) - separate PRACH resource - PRACH preamble partitioning - FFS: whether/how to address RA-RNTI overlapping issue - FFS the possibility of supporting Msg3 for the early indication Whether/how to support early indication of RedCap UEs in Msg3 in Rel-17 is up to RAN2.

RAN2#115-e meeting has achieved an agreement on early indication for RedCap UEs in Msg3 [2]:

Agreements online: 1. A Msg3 early identification based on dedicated LCID is supported (if SA3 confirms there is no problem)

In the RAN1#105e meeting, the following agreement was made for 2-step RACH [3]:

Agreement: - Support 2-step RACH for RedCap UEs as an optional feature - FFS details of early indication in MsgA, e.g.: - Separation of 2-step RACH resources or MsgA preambles - Separation of initial UL BWP - Using a new indication in MsgA PUSCH part Note: Discussion on 4-step RACH for early indication should be prioritised

In this contribution, we provide our views on early indication for RedCap UEs.

Discussion

According to the agreements of RAN1#106e and RAN2#115e, early indication in Msg1 as well as early indication in Msg3 will be specified. Therefore, RedCap UEs can be explicitly identifiable to networks through early indication in Msg1 and/or Msg3. In our view, if early indications in Msg1 and Msg3 are both configured and just indicate that a UE is RedCap, early indication in Msg3 only works as a duplicated function. It is kind of a waste. We prefer that in addition to “RedCap UE”additional information should be carried by early indications in both Msg1 and Msg3.

Proposal 1: In case of early indications are simultaneously configured in Msg1 and in Msg3，in addition to “RedCap UE” indication, additional information can be carried.

For early indication for 2-step RACH, it is better to follow the agreement on 4-step RACH, considering UE may fall back to 4-step RACH. That is, network can identify RedCap UEs through early indication in MsgA preamble part via separate PRACH resource or PRACH preamble partitioning, or/and in MsgA PUSCH part. We think it is reasonable that the configuration of early indication in 2-step RACH is in line with 4-step RACH, i.e,

    - if early indication in Msg1 is configured, for 2-step RACH early indication in MsgA preamble part is also configured.

    - if early indication in Msg3 is configured, for 2-step RACH early indication in MsgA PUSCH part is also configured.

    - if early indication in both Msg1 and Msg3 is configured, for 2-step RACH early indication in both MsgA preamble part and MsgA PUSCH part is also configured.

Proposal 2: For 2-step RACH, specify early indication in MsgA preamble part via separate PRACH resource or PRACH preamble partitioning, as well as early indication in MsgA PUSCH part.

Proposal 3: The configuration of early indication in 2-step RACH should be in line with 4-step RACH.

Conclusion

Based on the analyses and discussions, we have the following proposals:

Proposal 1: In case of early indications are simultaneously configured in Msg1 and in Msg3，in addition to “RedCap UE” indication, additional information can be carried.

Proposal 2: For 2-step RACH, specify early indication in MsgA preamble part via separate PRACH resource or PRACH preamble partitioning, as well as early indication in MsgA PUSCH part.

Proposal 3: The configuration of early indication in 2-step RACH should be in line with 4-step RACH.

Reference

- 3GPP RAN1#106-e Chairman’s Notes, August 16th -27th, 2021.

- 3GPP RAN2#115-e Meeting Report, August 9th -27th, 2021.

- 3GPP RAN1#105-e Chairman’s Notes, May 10th - 27th, 2021.