3GPP TSG RAN WG1 #106bis		R1-2108963

e-Meeting, October 11th – 20th, 2021

Source:	vivo

Title:	Discussions on PDSCH PUSCH enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.5

Document for:	Discussion and Decision

- Introduction

In RAN1#106-e, the following agreements and proposals were reached for processing timeline, PTRS, DMRS related design [1]:

- Timeline

Agreement:

For NR operation with 480 kHz and/or 960 kHz SCS, value(s) for PDSCH processing time (N1) for PDSCH processing capability 1 and PUSCH preparation time (N2) are to be defined for PDSCH/PUSCH timing capability 1 only.

Agreement:

For NR operation with 480 kHz and/or 960 kHz SCS, only value(s) for CSI computation delay requirement 2 are to be defined.

FFS: The specific values

Agreement:

When defining value ranges and/or default values for k0/k1/k2 for NR operation with 480 and 960 kHz SCS, RAN1 assumes the following definitions (this agreement does not define the following and these definitions may be updated later)

- The value of k0 indicates the slot offset between DCI and its scheduled PDSCH in number of slots

- The value of k1 indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs in number of slots

- The value of k2 indicates the slot offset between DCI and its scheduled PUSCH in number of slots

- Note: Default values are indicated by DCI format 1_0 and 0_0

Agreement:

For NR operation with 480 and 960 kHz SCS, adopt at least the values of N1, N2 and N3 as in the following tables for single and multi-PDSCH/PUSCH scheduling.

- Note: N1/N2 applies to any PDSCH/PUSCH for multi-PDSCH/PUSCH scheduling

- RAN1 to study (until RAN1#106b-e) and possibly introduce smaller values considering at least the following factors

  - PDCCH monitoring capability

  - Mix numerology scheduling

  - Multi-PDSCH/PUSCH scheduling

  - Cross-carrier scheduling

- Note: The decision for the number of HARQ processes should take this agreement into account.

Table 2-2.1 PDSCH processing time arrange for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 3 (120 kHz) | 20 | 24 |
| 5 (480 kHz) | 80 | 96 |
| 6 (960 kHz) | 160 | 192 |

Table 2-2.2 PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 3 (120 kHz) | 36 |
| 5 (480 kHz) | 144 |
| 6 (960 kHz) | 288 |

Table 2-2.3 Minimum gap between the second detected DCI and the beginning of the first PUCCH resources

|  | HARQ-ACK multiplexing timeline N3 [symbols] |
| --- | --- |
| 3 (120 kHz) | 20 |
| 5 (480 kHz) | 80 |
| 6 (960 kHz) | 160 |

Agreement:

For NR operation with 480 and 960 kHz SCS, adopt at least the values of Z1, Z2 and Z3 as in the following table for single and multi-PDSCH/PUSCH scheduling to maintain the same absolute time duration as that of 120 kHz SCS in FR2.

- Note: Xµ is UE reported capability beamReportTiming; KB3 and KB4 is UE reported capability beamSwitchTiming for 480 and 960 kHz SCS respectively.

- RAN1 to study (until RAN1#106b-e) and possibly introduce smaller values for CSI computation delay requirement

Table 2-4.  CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  | Z3 [symbols] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 | Z3 | Z'3 |
| 3 | 97 | 85 | 152 | 140 | min(97, X3+ KB2) | X3 |
| 5 | 388 | 340 | 608 | 560 | [min(388, X5+ KB3)] | [X5] |
| 6 | 776 | 680 | 1216 | 1120 | [min(776, X6+ KB4)] | [X6] |

- PTRS pattern for CP-OFDM

Agreement:

Further study and conclude on whether to introduce any PTRS enhancement for CP-OFDM by RAN1#106b.

- Note: details of specification impact for any proposed PTRS enhancement shall be provided to facilitate drawing conclusion in RAN1#106b

- PTRS density in frequency for CP-OFDM

Agreement:

Further study and conclude on whether to introduce K=1 for Rel-15 PTRS pattern for CP-OFDM with small (< =32) RB allocation by RAN1#106b.

- PTRS design for DFT-s-OFDM related agreements.

Agreement:

Further study and conclude on whether to introduce (Ng = 16, Ns = 2, L = 1) and/or (Ng = 16, Ns = 4, L = 1) for DFT-s-OFDM by RAN1#106b.

- Note: Ng number of PT-RS groups, Ns number of samples per PT-RS group, and PTRS every L number of DFT-s-OFDM symbols

- FFS applicable to which RB allocation(s) if agreed to introduce (Ng = 16, Ns = 2, L = 1) and/or (Ng = 16, Ns = 4, L = 1)

- DMRS related design.

Agreement:

- For 480 kHz and/or 960 kHz SCS, for rank 1 PDSCH at least with DMRS type-1, support a configuration of DMRS where the UE is able to assume that FD-OCC is not applied.

  - Note: “FD-OCC is not applied” refers to the UE may assume that a set of remaining orthogonal antenna ports are not associated with the PDSCH to another UE, wherein the set of remaining orthogonal antenna ports are within the same CDM group and have different FD-OCC

  - FFS whether applies to DMRS type-2

  - Down select between the following options for the indication to UE

    - RRC configuration

    - antenna port(s) field in DCI scheduling the rank 1 PDSCH

Conclusion:

In Rel-17, for NR operation with 480 kHz and/or 960 kHz SCS, new DMRS pattern with increased frequency domain density is not supported.

- Basic design for multi-PDSCH/PUSCH scheduling.

Working assumption:

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

Agreement:

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz.

- FFS: Whether UE capability is introduced for restricting the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

Agreement:

If a scheduled PDSCH/PUSCH is dropped due to collision with UL/DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, HARQ process number increment is skipped for the PDSCH/PUSCH and applied only for valid PDSCH(s)/PUSCH(s).

- FFS: HARQ process number determination for the case where a scheduled PDSCH/PUSCH collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated) if the UE is configured to monitor DCI format 2_0.

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - Priority indicator and open loop power control parameter set indication fields are applied to all of scheduled PUSCHs.

- For a DCI that can schedule multiple PDSCHs,

  - Priority indicator field is applied to all of scheduled PDSCHs.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, by configuring {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH (or PUSCH) in the row of TDRA table.

- Note: Whether and how to reduce RRC overhead is left to RAN2.

Agreement:

For a DCI that can schedule multiple PDSCHs,

- Each of VRB-to-PRB mapping, PRB bundling size indicator, ZP-CSI-RS trigger, and rate matching indicator fields appears only once in the DCI.

- VRB-to-PRB mapping and PRB bundling size indicator fields are applied to all the PDSCHs scheduled by the DCI.

- For ZP-CSI-RS trigger field, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) in which the PDSCH(s) scheduled by the DCI are contained.

- When receiving a PDSCH scheduled by the DCI, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 (according to indication of rate matching indicator field) are not available for the scheduled PDSCH.

Working assumption:

For NR FR2-2, two codeword transmission is supported, subject to UE capability.

- RRC parameter configures whether two codeword transmission is enabled or disabled.

  - FFS: Details on signaling of MCS/NDI/RV for the second TB in a DCI that can schedule multiple PDSCHs when two codeword transmission is enabled

  - FFS: Whether unified or separate parameter to enable/disable 2-TB for single and for multiple PDSCH scheduling

  - Strive to minimize the increase in the number of bits in the DCI needed to support this feature

Agreement:

For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

- Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI.

- Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

- HARQ-ACK feedback for multi-PDSCH scheduling.

Agreement:

Adopt Alt 1 (C-DAI/T-DAI is counted per DCI) for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs.

Agreement:

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

- Note: The optimization of HARQ codebook size for Type 1 or Type 2 codebook design is considered as a low priority in Rel-17 (this does not preclude HARQ ACK bundling in time domain).

- The agreement made in RAN1#105-e is revised as follows.

Agreement:

Consider the following options to construct type-2 HARQ-ACK codebook when CBG operation is configured, and down-select to one of the following options in RAN1#106bis-e.

- Option 1: HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook.

- Option 2: HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

- Option 3: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group.

- Note: Multi-PDSCH reception refers to the case where multiple PDSCHs are scheduled by a DCI that is configured with TDRA table containing at least one row with multiple SLIVs.

In this contribution, we first discuss the design for PTRS and DMRS, then the processing timeline, finally, the multi-PDSCH/PUSCH scheduling and feedback related design.

- PTRS and DMRS enhancements

  - PTRS related design

2.1.1 CP-OFDM

- Higher PTRS density in frequency: K_PTRS = 1

Higher PTRS density in frequency (K_PTRS = 1) for Rel-15 PTRS pattern was discussed during RAN1-104-bis meeting and RAN1-106 meeting.

Here we perform more simulations and focus on the cases with allocated number of RB <= 32.

Below are the simulation assumptions on PTRS density and RB numbers.

  - CPE and ICI PN compensation

    - Note: Results for CPE compensation-only are to be reported for reference

  - (K = 1, L = 1), (K = 2, L = 1),

    - Note: PTRS per K number of PRBs, and PTRS every L number of OFDM symbols

  - Number of RBs: 8, 16, 32

  - Other values of K and number of RBs are not precluded

Link-level simulations were performed to compare the performance of de-ICI (3 taps) and CPE only with different number of PDSCH RB allocation and different MCS for SCS of 120/480/960 KHz respectively. The simulation parameters are listed in Table 12, while the detailed simulation results include Figure 2, Figure 3, Figure 4 and Figure 5.

The SNR points corresponding to 10%/1% BLER performance are summarized in Table 1, Table 2 and Table 3 for SCS of 120 kHz, 480 kHz and 960 kHz respectively.

Table 1 SNR in dB achieving PDSCH BLER of 10% and 1%: 120kHz SCS, Rank 1, TDL-A 5ns DS.

| PDSCH PRB number | MCS | Algorithm for PN compensation | SNR in dB to achieve 10%/1% BLER |  |
| --- | --- | --- | --- | --- |
|  |  |  | K_PTRS |  |
|  |  |  | 1 | 2 |
| 8 | 7 | CPE-only | 4.93/9.24 | 5.26/9.65 |
|  |  | direct de-ICI | 7.49/- | -/- |
|  | 16 | CPE-only | 12.70/- | 12.55/- |
|  |  | direct de-ICI | 15.08/- | -/- |
|  | 22 | CPE-only | 19.96/- | 19.53/- |
|  |  | direct de-ICI | 22.44/- | -/- |
|  | 26 | CPE-only | -/- | -/- |
|  |  | direct de-ICI | -/- | -/- |
| 16 | 7 | CPE-only | 3.29/6.55 | 3.38/6.67 |
|  |  | direct de-ICI | 4.60/7.61 | 5.98/9.11 |
|  | 16 | CPE-only | 11.52/14.95 | 11.22/14.72 |
|  |  | direct de-ICI | 12.68/15.92 | 13.79/- |
|  | 22 | CPE-only | 19.42/26.02 | 18.56/23.85 |
|  |  | direct de-ICI | 19.14/23.17 | 20.80/- |
|  | 26 | CPE-only | -/- | -/- |
|  |  | direct de-ICI | -/- | -/- |
| 32 | 7 | CPE-only | 2.96/5.47 | 2.81/5.30 |
|  |  | direct de-ICI | 3.58/6.02 | 4.04/6.46 |
|  | 16 | CPE-only | 11.39/14.15 | 10.90/13.64 |
|  |  | direct de-ICI | 11.98/14.54 | 12.10/14.68 |
|  | 22 | CPE-only | 19.63/- | 18.50/23.49 |
|  |  | direct de-ICI | 18.50/21.85 | 18.55/22.30 |
|  | 26 | CPE-only | -/- | -/- |
|  |  | direct de-ICI | -/- | -/- |

Table 2 SNR in dB achieving PDSCH BLER of 10% and 1%: 480kHz SCS, Rank 1, TDL-A 5ns DS.

| PDSCH PRB number | MCS | Algorithm for PN compensation | SNR in dB to achieve 10%/1% BLER |  |
| --- | --- | --- | --- | --- |
|  |  |  | K_PTRS |  |
|  |  |  | 1 | 2 |
| 8 | 7 | CPE-only | 4.7/7.69 | 5.13/8.28 |
|  |  | direct de-ICI | 7.28/- | -/- |
|  | 16 | CPE-only | 12.19/- | 12.23/15.32 |
|  |  | direct de-ICI | 15.21/- | -/- |
|  | 22 | CPE-only | 18.17/21.64 | 17.94/21.51 |
|  |  | direct de-ICI | 20.06/23.55 | -/- |
|  | 26 | CPE-only | -/- | -/- |
|  |  | direct de-ICI | -/- | -/- |
| 16 | 7 | CPE-only | 3.14/5.44 | 3.35/5.70 |
|  |  | direct de-ICI | 4.45/6.69 | 5.88/8.36 |
|  | 16 | CPE-only | 11.04/13.30 | 10.77/13.14 |
|  |  | direct de-ICI | 12.04/14.44 | 13.44/15.88 |
|  | 22 | CPE-only | 17.12/19.66 | 16.69/19.19 |
|  |  | direct de-ICI | 17.49/19.84 | 18.91/21.43 |
|  | 26 | CPE-only | -/- | -/- |
|  |  | direct de-ICI | 26.70/- | 27.19/- |
| 32 | 7 | CPE-only | 2.66/4.49 | 2.59/4.47 |
|  |  | direct de-ICI | 3.31/5.10 | 3.97/5.69 |
|  | 16 | CPE-only | 10.64/12.52 | 10.26/12.14 |
|  |  | direct de-ICI | 11.48/13.24 | 11.74/13.56 |
|  | 22 | CPE-only | 17.08/19.54 | 16.40/18.52 |
|  |  | direct de-ICI | 16.71/18.57 | 16.93/18.86 |
|  | 26 | CPE-only | -/- | -/- |
|  |  | direct de-ICI | 25.77/- | 23.71/- |

Table 3 SNR in dB achieving PDSCH BLER of 10% and 1%: 960kHz SCS, Rank 1, TDL-A 5ns DS.

| PDSCH PRB number | MCS | Algorithm for PN compensation | SNR in dB to achieve 10%/1% BLER |  |
| --- | --- | --- | --- | --- |
|  |  |  | K_PTRS |  |
|  |  |  | 1 | 2 |
| 8 | 7 | CPE-only | 4.74/7.14 | 5.33/8.01 |
|  |  | direct de-ICI | 7.19/9.75 | -/- |
|  | 16 | CPE-only | 12.33/14.65 | 12.28/14.83 |
|  |  | direct de-ICI | 14.66/- | -/- |
|  | 22 | CPE-only | 17.89/20.30 | 17.75/20.39 |
|  |  | direct de-ICI | 20.13/22.64 | -/- |
|  | 26 | CPE-only | 26.72/- | 24.28/- |
|  |  | direct de-ICI | 27.11/- | -/- |
| 16 | 7 | CPE-only | 3.25/5.30 | 3.54/5.82 |
|  |  | direct de-ICI | 4.48/6.44 | 6.33/8.29 |
|  | 16 | CPE-only | 11.04/13.00 | 10.93/13.00 |
|  |  | direct de-ICI | 12.35/14.24 | 13.69/- |
|  | 22 | CPE-only | 16.91/18.91 | 16.56/18.61 |
|  |  | direct de-ICI | 17.62/19.51 | 18.91/21.09 |
|  | 26 | CPE-only | -/- | 24.62/- |
|  |  | direct de-ICI | 25.07/- | 25.64/- |
| 32 | 7 | CPE-only | 2.68/4.26 | 2.83/4.45 |
|  |  | direct de-ICI | 3.22/4.77 | 4.02/5.42 |
|  | 16 | CPE-only | 10.66/12.21 | 10.47/12.04 |
|  |  | direct de-ICI | 11.39/12.88 | 11.95/13.24 |
|  | 22 | CPE-only | 16.56/18.14 | 16.21/17.66 |
|  |  | direct de-ICI | 16.71/18.23 | 16.97/18.52 |
|  | 26 | CPE-only | -/- | 24.53/- |
|  |  | direct de-ICI | 24.18/- | 22.70/- |

From the simulation results above, we have the observation below.

Observation 1:

- The performance of de-ICI (3-tap filter) with K_PTRS = 2 is worse than K_PTRS = 1 when PDSCH RB number <= 16;

- When PDSCH RB number <= 16, CPE only with K_PTRS = 2 has much better performance than de-ICI with K_PTRS = 1.

- For all evaluated cases, the preferred method for PN compensation (with the best performance) is shown in Table 4. The best performance is obtained with legacy PTRS density in frequency, i.e. K_PTRS = 2. There’s no motivation to justify higher PTRS density in frequency domain for small RB allocation.

Table 4 Preferred PN compensation method when number of RB <=32

| SCS (kHz)/MCS | 7 | 16 | 22 | 26 |
| --- | --- | --- | --- | --- |
| 120 | CPE only (K_PTRS=2) | CPE only (K_PTRS=2) | CPE only (K_PTRS=2) | no method to achieve 10% BLER. |
| 480 | CPE only (K_PTRS=2) | CPE only (K_PTRS=2) | CPE only (K_PTRS=2) | If PDSCH RB num <= 16, use CPE only (K_PTRS=2); else, use de-ICI (K_PTRS=2). |
| 960 | CPE only (K_PTRS=2) | CPE only (K_PTRS=2) | CPE only (K_PTRS=2) | If PDSCH RB num <= 16, use CPE only (K_PTRS=2); else, use de-ICI (K_PTRS=2). |

Proposal 1: For FR2-2, there is no need to introduce higher PTRS frequency density as K_PTRS = 1 in Rel-17.

- PTRS pattern: block-based PTRS pattern

Evaluation on block-based PTRS pattern with cyclic ZC sequence

In this sub-section, link-level simulations were performed to compare the performance among the following options:

- CPE only;

- de-ICI with Rel-15 PTRS pattern (5-tap filters);

- ICI filter approximation (3 PTRS blocks, 21 PTRS tones/ Block, 5-tap filters)

- De-ICI with cyclic ZC sequence (5-tap filters).

- PN-spectrum based ICI filter approximation (3 PTRS blocks, 21 PTRS tones/ Block, 5-tap filters)

T-doc [3] proposes option d) and it mentions that de-ICI has better performance than ‘ICI filter approximation’, thus only de-ICI is evaluated here.

Length of base sequence and circular sequence for block PTRS in option d) are as below.

Table 5 Length of base sequence and circular sequence for block PTRS in option d)

| Block size M | 64 |
| --- | --- |
| ICI order estimated | 5 |
| Length of base sequence L | 59 |
| Length of head circular sequence q | 2 |
| Length of tail circular sequence p | 3 |

T-doc [6] proposes option e) which includes block-PTRS pattern and the algorithm of PN-spectrum based ICI filter approximation.

Simulation parameters are listed in Table 13, while the simulation results include Figure 6, Figure 7 and Figure 8.

The SNR points corresponding to 10% BLER performance are summarized in Table 6.

Table 6 SNR in dB achieving PDSCH BLER of 10%: 120kHz SCS, Rank 1, MCS-22.

| DS (ns) | SNR in dB to achieve 10% BLER for SCS-120kHz and MCS-22 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CPE only | de-ICI with Rel-15 PTRS pattern | ICI filter approximation | de-ICI with cyclic ZC sequence | PN-spectrum based ICI filter approximation |
| 5 | - | 16.84 | 21.46 | 17.83 | 17.27 |
| 10 | - | 16.73 | 21.34 | 18.00 | 16.96 |
| 20 | - | 17.00 | 20.43 | 17.84 | 17.05 |

From the simulation results above, we have the observation below.

Observation 2: With current simulation parameters, the performance of de-ICI with Rel-15 PTRS pattern outperforms the performance of block-based PTRS pattern with cyclic ZC sequence.

Evaluation on block-based PTRS pattern with zero-power REs

Contribution [2] proposes block-based PTRS pattern with zero-power RE, and these 2 points were mentioned in meeting summary [4]:

- Block PTRS pattern of 7 clusters each with 9 tones (8 ZP and 1 NZP) can achieve good performance;

- 6dB power boost can recover most of the performance loss due to the power difference of the PTRS tones, which should not cause issues from PAPR/CM perspective.

In this sub-section, link-level simulations were performed on this PTRS pattern to compare performance between ‘6dB power boost’ and ‘full power boost’ with conditions below, while other simulation assumptions are listed in  Table 13.

- De-ICI compensation method;

- 7 clusters each with 9 tones (8 ZP and 1 NZP).

The simulation results include Figure 9, Figure 10 and Figure 11.

The SNR points corresponding to 10%/1% BLER performance are summarized in Table 7.

Table 7 SNR in dB achieving PDSCH BLER of 10%/1%: 120kHz SCS, Rank 1, MCS-22.

| DS (ns) | SNR in dB to achieve 10%/1% BLER for SCS-120kHz and MCS-22 |  |
| --- | --- | --- |
|  | Full power boosting | 6dB power boosting |
| 5 | 17.18/20.76 | 19.63/24.42 |
| 10 | 16.93/19.76 | 19.62/23.32 |
| 20 | 17.44/20.03 | 19.76/23.07 |

From the simulation results above, we have the observation below.

Observation 3: For block-based PTRS pattern with zero-power RE, the performance of ‘6dB power boosting’ is around 2.5 dB worse than that of ‘full power boosting’ for SNR achieving 10% BLER.

Based on simulation results in Table 6 and Table 7, the following observation can be derived.

Observation 4: For block-based PTRS pattern with zero-power RE, the performance of ‘full power boosting’ doesn’t outperform the performance of ‘de-ICI with Rel-15 PTRS pattern’.

Based on the observations above, the proposal below is derived.

Proposal 2: For FR2-2, do not support block-based PTRS pattern in Rel-17.

2.1.2 DFT-s-OFDM

For DFT-s-OFDM, it was proposed to further evaluate the PTRS pattern of (Ng = 16, Ns = 2, L = 1) and (Ng = 16, Ns = 4, L = 1) [1].

As the length of one DFT-s-OFDM symbol for SCS of 120 kHz is 4 times and 8 times as that for SCS of 480kHz and SCS of 960kHz, this topic should focus on SCS-120kHz, high code rate transmission and large number of PRB allocation.

The following CN (chunk number, NgroupPT-RS) and CS (chunk size, Nsamplegroup) combinations are evaluated, where (CN, CS) = (8, 4) is the legacy configuration with largest chunk number as the baseline for comparison:

- (CN, CS) = (8, 4);

- (CN, CS) = (16, 2);

- (CN, CS) = (16, 4);

- (CN, CS) = (8, 8);

- (CN, CS) = (12, 4).

The simulation parameters are listed in Table 14, while the simulation results include Figure 15 - Figure 22.

The SNR points corresponding to 10%/1% BLER performance are summarized in Table 8 and Table 9 for DS of 10 ns and 20 ns respectively.

Table 8 SNR in dB achieving PUSCH BLER of 10% and 1%: DS = 10ns.

Table 9 SNR in dB achieving PUSCH BLER of 10% and 1%: DS = 20ns.

Observation 5: The performance gap is not obvious between (CN, CS) = (8, 4) and the configuration with the best performance for most of the cases on MCS-7 and MCS-16.

Observation 6: For MCS-22/MCS-26 and PUSCH RB number as 256, configuration (CN, CS) = (16, 4) achieves best performance.

As DFT-s-OFDM is mainly used for UL coverage, it is unlikely that very high MCS, such as MCS-26, is scheduled to the UE configured with this waveform. On the other hand, considering that UE’s high RF impairment in FR2-2, it is unlikely to achieve very high SNR for UL. Therefore, the focus should not be on high MCS or high SNR.

Based on these observations, the necessity to introduce more PTRS chunk number needs further discussion to find if there is other benefit. If there is valid reason to introduce more PTRS chunks, configuration (CN, CS) = (16, 4) can be considered, and the PTRS should be placed at the middle of each segment defined by PTRS group to avoid the PTRS wrapping-around to the head of the symbol caused by Rx timing shift [5].

Proposal 3: The necessity to introduce more PTRS chunk number needs further discussion as there is no significant performance benefit. If there is valid reason to introduce more PTRS chunks, configuration (CN, CS) = (16, 4) can be considered.

  - DMRS related design

2.2.1 FD-CDM invalidation for DMRS type-2

It was agreed that FD-CDM can be invalidated for rank-1 PDSCH for DMRS type-1 for SCS of 480/960 kHz, and it needs further study on whether this approach should be applied to DMRS type-2.

For this purpose, the link-level simulation is performed for DMRS type-2 while the TB size and coderate are fixed, i.e. no multiplexing between data and DMRS on the same OFDM symbol.

The candidate DMRS patterns include:

a) Type-1 with FD-OCC, 3-dB power boosting;

b) Type-1 no FD-OCC, 3-dB power boosting;

c) Type-2 with FD-OCC, 4.77-dB power boosting;

d) Type-2 no FD-OCC, 4.77-dB power boosting.

The simulation parameters are listed in Table 15, while the simulation results include Figure 23 - Figure 28.

Below are the performance comparisons for SCS as 960 KHz and 480 KHz respectively.

The SNR points corresponding to 10%/1% BLER performance are summarized in Table 10 and Table 11 for SCS of 960 kHz and 480 kHz respectively.

Table 10 SNR in dB achieving PDSCH BLER of 10% and 1%: 960 kHz SCS, Rank 1.

| DS (ns) | MCS | SNR in dB to achieve 10%/1% BLER |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | DMRS type-1 |  | DMRS type-2 |  |
|  |  | with FD-OCC | no FD-OCC | with FD-OCC | no FD-OCC |
| 10 | 22 | 16.51/18.29 | 16.32/17.92 | 18.72/20.37 | 16.94/18.62 |
|  | 26 | 24.00/- | 23.58/- | 28.21/- | 25.19/- |
| 20 | 22 | 18.75/20.86 | 17.75/19.29 | 22.72/25.35 | 18.94/20.71 |
|  | 26 | -/- | 26.18/- | -/- | -/- |
| 40 | 22 | -/- | 19.82/23.19 | -/- | 28.54/- |
|  | 26 | -/- | -/- | -/- | -/- |

Table 11 SNR in dB achieving PDSCH BLER of 10% and 1%: 480 kHz SCS, Rank 1.

| DS (ns) | MCS | SNR in dB to achieve 10%/1% BLER |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DMRS type-1 |  | DMRS type-2 |  |
|  |  | with FD-OCC | no FD-OCC | with FD-OCC | no FD-OCC |
| 10 | 22 | 16.56/19.39 | 16.50/19.35 | 17.44/20.33 | 16.78/19.66 |
|  | 26 | -/- | -/- | -/- | -/- |
| 20 | 22 | 17.66/20.37 | 17.36/20.00 | 20.64/23.90 | 18.55/21.58 |
|  | 26 | -/- | -/- | -/- | -/- |
| 40 | 22 | 20.34/23.65 | 19.00/21.82 | 25.25/- | 20.69/25.38 |
|  | 26 | -/- | -/- | -/- | -/- |

From these simulation results, we have the observation below.

Observation 7: ‘Type-2 no FD-OCC’ has better performance than ‘Type-2 with FD-OCC’ by 1~4 dB, and the gain on SCS-960KHz is higher than the gain on SCS-480KHz.

Based on this observation, the proposal below is derived.

Proposal 4: For FR2-2, support ‘type-2 no FD-OCC’ for SCS of 480/960 kHz in Rel-17.

2.2.2 Signalling for FD-CDM invalidation

It was agreed that FD-CDM can be invalidated for rank-1 PDSCH for DMRS type-1 for SCS of 480/960 kHz, and it needs further study on the signaling solution, which was down-selected between the following options for the indication to UE: RRC configuration, or antenna port(s) field in DCI scheduling the rank 1 PDSCH.

The DCI based approach has 2 options:

- Keep existing bit-width of antenna port(s) field and reuse the values with ‘reserved’ state.

This option has good compatibility with existing standard and the latency for effectiveness is low. However, there is limitation on the case when there are very limited entries with ‘reserved’ state. For example, dmrs-Type=1, maxLength=2 and one codeword, in this case, there is only one value (31) that corresponds to ‘reserved’ state. This restricts the flexibility of scheduling.

Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=2 （From TS 38.212）

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4-31 | reserved | reserved | reserved |
| 5 | 2 | 2 | 1 |  |  |  |  |
| 6 | 2 | 3 | 1 |  |  |  |  |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | Reserved | Reserved | Reserved |  |  |  |  |

- Extend antenna port(s) field by one bit and this new bit to indicate whether UE can assume no FD-CDM from other UE

This option provides flexible scheduling and very low latency for effectiveness, at the cost of higher DCI payload. As DCI is a frequent physical signalling, thus it is very sensitive for the payload increment.

The RRC signalling based approach also has 2 options:

- Add one bit to indicate whether UE can assume no FD-CDM from other UE

This option provides flexible scheduling, and the RRC signalling is in-frequent thus it is insensitive to the payload increment. However, the latency for effectiveness is high. FD-CDM invalidation can relate to the MCS and channel variation, thus swift switching between FD-CDM on/off is necessary, but this option can’t support this requirement.

- Add one field including a set of values, and only when antenna port(s) field in DCI belongs to this set, UE can assume no FD-CDM from other UE.

In existing standard, there is statement below.

For DM-RS configuration type 1,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

If the set highlighted can be configured by RRC signalling, it can provide good flexibility, and FD-CDM by other UE can be turned on if gNB sets the antenna port(s) field in DCI with the value that is not belonging to the set， which means the switching between FD-CDM on/off is very swift.

This option is to configure the set which includes part of candidate values for antenna port(s) field in DCI. Only when antenna port(s) field in DCI belongs to this set, UE can assume no FD-CDM from other UE.

This option can provide good flexibility for DMRS port scheduling, and low latency for effectiveness. In terms of payload, it just increases the payload of RRC signalling which is insensitive because of infrequent signalling.

Based on the analysis above, the proposal below is derived.

Proposal 5: For 480/960 kHz SCS in FR2-2, for rank 1 PDSCH, FD-CDM invalidation should be indicated by RRC signalling where one new field to indicate a set of values, and only when antenna port(s) field in DCI belongs to this set, UE can assume no FD-CDM from other UE.

- Processing timeline aspects

  - UE PDSCH/PUSCH processing time related timeline constants

For FR2-2, some values for timeline related constants N1, N2, N3, Z1, Z2, Z3 were agreed and RAN1 was encouraged to further study on potential smaller values for better performance. On the other hand, the value ranges for other timeline constants, including d1,1, d2, Text, d2,1, Tswitch, d2,2, need to be determined so that PDSCH processing time Tproc,1 and PUSCH processing time Tproc,2 can be derived. The definitions of Tproc,1 and Tproc,2 are referenced as below for convenience.

UE PDSCH processing time:

UE PUSCH processing time:

Some parameters in the above equations are explained as below.

- Parameter d1,1 is defined in the unit of OFDM symbol and depends on PDSCH length;

- Parameter d2 is defined in the unit of OFDM symbol and depends on collision with UL transmission with lower priority;

- Parameter Text is used in CP extension for operation with shared spectrum channel access;

- Parameter d2,1 is defined in the unit of OFDM symbol and depends on whether there is data in the first symbol of PUSCH or not;

- Parameter Tswitch is defined for UL switching gap between uplink carriers (for intra-RAT or inter-RAT);

- Parameter d2,2 is defined for BWP switching and the duration is defined in the unit of slots with the value dependent on the SCS.

Figure 1 Definition of d2,2 (From TS 38.133-g30 Chapter 8.6.2)

Based on the design on these parameters, it is reasonable to give the proposal below.

Proposal 6: Among parameters d1,1, d2, Text, d2,1, Tswitch, d2,2, only parameter d2,2 for 480/960 kHz SCS should use the absolute time duration for 120 kHz SCS as the upper bound for FR2-2 in Rel-17.

  - Parameter k0, k1, k2

For FR2-2, it was agreed that the units of k0, k1, k2 were defined as slot, and values of N1, N2, N3 for 480/960 kHz SCS were proposed based on the absolute time duration for 120 kHz SCS.

According to the achieved agreement below, legacy range for k0, i.e. 0~32, should be enough to provide good flexibility for PDSCH scheduling.

Agreement:

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz.

- FFS: Whether UE capability is introduced for restricting the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

Proposal 7: For FR2-2, the range of k0 for 480/960 kHz SCS can be kept as 0~32.

Based on N1 value, i.e. 20/80/160 symbols for 120/480/960 kHz SCS respectively, it means that after the last symbol of a PDSCH reception, HARQ-ACK for the PDSCH reception can be transmitted later than 1.4/5.7/11.4 slots for 120/480/960 kHz SCS respectively. While the value range for k1 is 0~15, the smaller values can’t be applied due to not accommodating N1 symbols, and the rest applicable values may not be sufficient and large enough to provide desirable flexibility.

To deal with this issue, it is feasible to adjust the range of k1 for 480/960 kHz SCS based on corresponding SCS specific offset. When an original k1 is selected from the range 0~15, the effective k1 is determined as the original k1 plus the SCS specific offset. For example, offset for 480/960 kHz SCS can be configured by high layer signaling, or defined as 6/12 which is the next integer larger than 5.7/11.4.

Proposal 8: For FR2-2, the range of k1 for 480/960 kHz SCS should be defined as offset ~ offset+15, where the offset is dependent on SCS, for example, offset = 6/12 for 480/960 kHz SCS respectively.

For the same reason, based on N2 value, i.e. 36/144/288 symbols for 120/480/960 kHz SCS respectively, after the last symbol of PDCCH reception, PUSCH transmission can be started later than 2.6/10.3/20.6 slots for 120/480/960 kHz SCS respectively. It’s feasible to define the range of k2 for 480/960 kHz SCS based on an SCS specific offset.

Proposal 9: For FR2-2, the range of k2 for 480/960 kHz SCS should be defined as offset ~ offset+32, where the offset is dependent on SCS, for example, offset = 11/21 for 480/960 kHz SCS respectively.

- Multi-PDSCH/PUSCH scheduling

  - DCI design for multi-PDSCH/PUSCH scheduling

In the following sections, some issues which may be common for DL and UL are discussed firstly, then followed by UL specific issue(s).

4.1.1 Common issues for DL and UL

4.1.1.1 Time domain scheduling

During RAN1#105-e meeting, there was discussion on whether to limit the number of scheduled PDSCHs/PUSCHs within a single slot to 1. The reason behind the discussion lies in that a slot is very short for 480/960 kHz SCS and thus it is unnecessary to schedule more than one PDSCH/PUSCH within a slot.

During RAN1#106-e meeting, it was agreed that for 120 kHz SCS and single TRP operation, one or more PDSCHs/PUSCHs can be scheduled within a single slot for a UE, subject to UE capability. But for 480/960 kHz SCS and single TRP operation, after a lot of discussion no consensus was achieved in the end. The relevant part of the agreement is referenced as below for convenience.

Agreement:

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

…

From our perspective, since for 120 kHz SCS more than one PDSCH/PUSCH scheduled within a slot is supported and it is up to UE capability reporting, it is unnecessary to introduce any different operation among applicable SCSs for the new band from 52.6 GHz to 71 GHz, if there is no strong motivation to justify the difference. Besides, allowing more than one PDSCH/PUSCH in a slot may be beneficial for LBT operations when deployed in unlicensed band and LBT mechanism is required, to enable more LBT chances and reduce LBT failures. In addition, it may also benefit repetition transmission, e.g. in URLLC scenarios where high reliability and low latency are required.

Proposal 10: Support more than one PDSCH/PUSCH scheduled within a slot by a single or multiple DCIs for 480/960 kHz SCS and single TRP operation.

4.1.1.2 Frequency domain scheduling

During RAN1#104-e meeting, whether/how to enhance frequency domain scheduling for multi-PUSCH scheduling has been discussed with no consensus. During RAN1#105-e and RAN1#106-e meetings, this issue was deprioritized due to limited input and time.

It can be observed that the maximum number of PRBs is not changed based on the maximum channel bandwidths agreed for NR operation in 52.6 GHz to 71 GHz, so there is no motivation to enhance the frequency domain scheduling since legacy solution(s) can work well. This applies to multi-PDSCH scheduling as well.

Proposal 11: Legacy frequency domain scheduling in NR Rel-15/16 is reused for multi-PUSCH/PDSCH scheduling.

4.1.1.3 CBG based scheduling

During RAN1#105-e meeting, the following agreement regarding CBG based scheduling for multi-PDSCH/PUSCH scheduling has been achieved.

Agreement:

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

- FFS:

  - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs

  - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

During RAN1#106-e meeting, this issue was discussed from the beginning, but the discussion was suspended in later period to wait for the outcome of discussion on which option Type-2 codebook will adopt for codebook construction.

According to the above agreement, at least for 120 kHz SCS for multi-PUSCH scheduling, the mechanism agreed in NR-U Rel-16 is reused, but there is an FFS for 480/960 kHz SCS for multi-PUSCH scheduling whether to apply the same behavior with 120 kHz SCS or not. In our opinion, it is undesirable to introduce different behavior for different SCS, if there is no strong motivation to justify the difference. Regarding this issue, the same behavior with 120 kHz SCS for multi-PUSCH scheduling can be applied to 480/960 kHz SCS as well.

Proposal 12: For CBG based scheduling, the same behaviour for multi-PUSCH scheduling with 120 kHz SCS is applied to 480/960 kHz SCS as well, i.e., CBG based scheduling is supported only when a DCI schedules a single PUSCH.

For multi-PDSCH scheduling, the same mechanism adopted for multi-PUSCH scheduling can be reused for simplicity. Regarding CBGFI field, if configured, it can be handled in the same way as CBGTI field. In other words, CBGTI/CBGFI fields are included in a DCI only when the DCI schedules a single PDSCH, when CBG based transmission is configured.

4.1.2 UL specific issues

4.1.2.1 Frequency hopping

During RAN1#105-e meeting, frequency hopping issue for multi-PUSCH scheduling was deprioritized due to limited input and time. But based on some brief discussions on it, it seems that companies have different views on which frequency hopping mode(s) is/are supported for multi-PUSCH scheduling in NR-U Rel-16.

During RAN1#106-e meeting, frequency hopping for multi-PUSCH scheduling was discussed under NR-U agenda, ended with no consensus, due to channel access issues identified during the meeting, i.e. two-hop transmission may span multiple RB sets subject to different LBT procedures. But based on the discussion, most companies have the same view on frequency hopping for multi-PUSCH scheduling in general, i.e., only intra-slot frequency hopping is applicable to multi-PUSCH scheduling when no channel access issue is identified.

However, for multi-PUSCH scheduling when operated in 52.6 GHz to 71 GHz, there is no channel access issue (i.e. two-hop transmission span multiple RB sets subject to different LBT procedures) anymore since LBT bandwidth is always the whole UL BWP and sub-band based LBT is not supported according to the agreement(s) achieved so far under channel access agenda. Therefore, it is safe to apply the common understanding for multi-PUSCH scheduling in NR-U discussions, i.e. only intra-slot frequency hopping is applicable and inter-slot frequency hopping is inapplicable for multiple PUSCHs scheduled by a DCI.

Proposal 13: For frequency hopping for multi-PUSCH scheduling, only intra-slot frequency hopping is applicable, and is applied to each scheduled PUSCH when configured and enabled, while inter-slot frequency hopping is inapplicable.

  - HARQ-ACK feedback for multi-PDSCH scheduling

4.2.1 Type-1 HARQ-ACK codebook

Time domain bundling is an efficient way to reduce the HARQ-ACK codebook size, and when it could be combined with Type-1 codebook, the codebook size can be controlled based on configurable bundling granularity while keeping semi-static. So it is beneficial to study Type-1 codebook in conjunction with time domain bundling.

Regarding time domain bundling for Type-1 codebook when multi-PDSCH scheduling is configured, two alternatives can be identified.

- Alt. 1: HARQ-ACK information corresponding to all valid PDSCH(s) scheduled by a DCI, each of which does not collide with any semi-static UL symbol, by indicating a row in the TDRA table can be bundled into a HARQ-ACK bit (or two HARQ-ACK bits if two codewords are enabled). When determining the set of occasions for constructing the Type-1 codebook, the last SLIV configured in each row, or the last valid SLIV without collision with any semi-static UL symbol in each row, can be considered for SLIV grouping. Thus the legacy pseudo code for SLIV grouping can be reused, where there is only a SLIV considered for each row, and each row will be mapped to one and only one occasion in the set of occasions. If one or more rows corresponding to an occasion are scheduled actually, the HARQ-ACK bit(s) corresponding to the occasion will be set according to decoding results of each valid PDSCH of the scheduled one or more rows, where binary AND operation can be performed based on these decoding results.

- Alt. 2: A set of occasions can be determined with the same way as the case where time domain bundling is not configured, i.e., the set of occasions is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots, then time domain bundling is performed across one or more neighbouring occasions, which can be regarded as a subset of occasions from the set of occasions. Regarding how to determine each subset of occasions for time domain bundling, two sub-alternatives can be identified further as follows.

  - Alt. 2-1: A subset of occasions are obtained from one or more occasions determined for a DL slot. For example, all occasions determined based on the set of SLIVs corresponding to a DL slot can be divided into a single subset of occasions, thus each DL slot is mapped to a subset. Alternatively, a granularity of N can be configured, and if the number of occasions determined based on the set of SLIVs corresponding to a DL slot is more than N, each N neighbouring occasions for the DL slot can be divided into a subset of occasions, otherwise, all occasions for the DL slot can be divided into a single subset of occasions.

  - Alt. 2-2: The set of occasions can be divided into one or more subsets of occasions based on a configured granularity of M. If the number of occasions in the set is more than M, each M neighbouring occasions in the set can be divided into a subset of occasions, otherwise, all occasions in the set can be divided into a single subset of occasions.

Alt. 1 may result in different procedures or pseudo codes for the two cases when time domain bundling is configured and not configured, but it can guarantee that time domain bundling is performed across a whole row of the TDRA table, or a scheduling DCI in other words.

With respect to Alt. 2, the same procedure or pseudo code for determining the set of occasions can be maintained when time domain bundling is configured or not, thus less complication can be introduced. But the time domain bundling is not performed in the granularity of a whole row, or a scheduling DCI in other words. Therefore, all or part of valid PDSCH(s) from one or more DCIs/rows are divided into a subset of occasions for time domain bundling. The time domain bundling may be performed as that binary AND operation is performed across the HARQ-ACK bit(s) corresponding to each occasion in the subset of occasions, resulting in one HARQ-ACK bit (or two HARQ-ACK bits if two codewords are enabled) for the subset.

Proposal 14: Regarding time domain bundling for Type-1 codebook when multi-PDSCH scheduling is configured, consider the following two alternatives:

- Alt. 1: A set of occasions is determined based on the last (valid) SLIV in each row of the TDRA table, and time domain bundling is performed across all valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table.

- Alt. 2: A set of occasions is determined based on all (valid) SLIVs in each row of the TDRA table, in the same way as the case when time domain bundling is not configured, and time domain bundling is performed for each subset of occasions divided from the set of occasions.

4.2.2 Type-2 HARQ-ACK codebook

During RAN1#106-e meeting, the following agreement has been achieved regarding constructing Type-2 HARQ-ACK codebook when CBG operation is configured.

Agreement:

Consider the following options to construct type-2 HARQ-ACK codebook when CBG operation is configured, and down-select to one of the following options in RAN1#106bis-e.

- Option 1: HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook.

- Option 2: HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

- Option 3: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group.

- Note: Multi-PDSCH reception refers to the case where multiple PDSCHs are scheduled by a DCI that is configured with TDRA table containing at least one row with multiple SLIVs.

Regarding Option 1, HARQ-ACK bits corresponding to PDSCH receptions with different scheduling characteristics are merged into a same sub-codebook, which is not a straightforward solution and may require more specification efforts. Furthermore, the maximum number of CBGs per TB may not match the maximum number of PDSCHs scheduled by a single DCI exactly, thus additional NACK padding in the sub-codebook is required to align them, resulting in additional overhead and larger codebook size.

Regarding Option 2, it is a simple and straightforward way to address the issue, and no additional overhead in the HARQ-ACK codebook will be involved. Nevertheless, it may require additional T-DAI field in UL non-fallback DCI format to improve reliability.

Regarding Option 3, it is too restrictive to disallow simultaneous configurations of CBG transmission and multi-PDSCH scheduling in a same PUCCH cell group. For example, a UE may be configured with CA between FR1/2-1 cell(s) and FR2-2 cell(s), where FR1/2-1 cell(s) may be configured with single PDSCH scheduling and CBG based transmission, and FR2-2 cell(s) may be configured with multi-PDSCH scheduling. If Option 3 is adopted, these cells can not be configured in a same PUCCH cell group, resulting in more overhead and complexity.

Based on the above discussion, Option 2 is preferred for simplicity and flexibility.

Proposal 15: For constructing Type-2 HARQ-ACK codebook when CBG operation is configured, support Option 2, i.e. HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

To control or reduce the codebook size, time domain bundling can be introduced for Type-2 codebook as well. E.g., the bundling granularity can be configurable, to enable the tradeoff between the feedback overhead and retransmission efficiency.

Regarding time domain bundling for Type-2 codebook when multi-PDSCH scheduling is configured, two alternatives can be identified.

- Alt. 1: HARQ-ACK information corresponding to all valid PDSCH(s) scheduled by a DCI, each of which does not collide with any semi-static UL symbol, by indicating a row in the TDRA table can be bundled into a HARQ-ACK bit (or two HARQ-ACK bits if two codewords are enabled). Since it was agreed during RAN1#106-e meeting that C-DAI/T-DAI is counted per DCI, legacy pseudo code for constructing a Type-2 codebook can be reused, where each DAI counting corresponds to one or two HARQ-ACK bits. The HARQ-ACK bit(s) corresponding to a detected DCI can be set by performing binary AND operation across decoding results of each valid PDSCH scheduled by the DCI, while the HARQ-ACK bit(s) corresponding to a miss-detected DCI can be set to NACK directly.

- Alt. 2: Valid PDSCH(s) scheduled by a DCI, each of which does not collide with any semi-static UL symbol, can be divided into N subsets of valid PDSCHs. The division can be as even as possible, e.g. 5 valid PDSCHs divided into 3 subsets results in 2 valid PDSCHs, 2 valid PDSCHs and 1 valid PDSCH in the first subset, second subset and third subset, respectively. Each subset may contain zero, one or more neighbouring valid PDSCHs, and corresponds to one HARQ-ACK bit (or two HARQ-ACK bits if two codewords are enabled) in the HARQ-ACK codebook. The HARQ-ACK bit(s) corresponding to a subset can be set by performing binary AND operation across decoding results of each valid PDSCH contained in the subset if the subset contains at least one valid PDSCH, otherwise the HARQ-ACK bit(s) corresponding to the subset can be set to NACK directly. Regarding N, it can be configured by gNB directly. Alternatively, gNB can configure the size or maximum size of a subset, named M, and N can be determined by ceiling (maximum number of PDSCHs scheduled by a DCI based on TDRA table configuration / M).

Alt. 1 leads to simpler operation, less specification efforts, and less overhead, but the granularity for time domain bundling can not be controlled as required by gNB. On the contrary, Alt. 2 is more flexible, allowing trade-off as required between overhead and retransmission efficiency.

Proposal 16: Regarding time domain bundling for Type-2 codebook when multi-PDSCH scheduling is configured, consider the following two alternatives:

- Alt. 1: Time domain bundling is performed across all valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table.

- Alt. 2: Time domain bundling is performed across a subset of valid PDSCHs, which is divided from the set of valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table.

- Conclusion

In this contribution, we first discuss the design for PTRS and DMRS, then the processing timeline, finally, the multi-PDSCH/PUSCH scheduling and feedback related design. We have the following observations and proposals.

- PTRS density in frequency for CP-OFDM: K_PTRS = 1 or K_PTRS = 0.5

Observation 1:

- The performance of de-ICI (3-tap filter) with K_PTRS = 2 is worse than K_PTRS = 1 when PDSCH RB number <= 16;

- When PDSCH RB number <= 16, CPE only with K_PTRS = 2 has much better performance than de-ICI with K_PTRS = 1.

- For all evaluated cases, the preferred method for PN compensation (with the best performance) is shown in Table 4. The best performance is obtained with legacy PTRS density in frequency, i.e. K_PTRS = 2. There’s no motivation to justify higher PTRS density in frequency domain for small RB allocation.

Proposal 1: For FR2-2, there is no need to introduce higher PTRS frequency density as K_PTRS = 1 in Rel-17.

- PTRS pattern for CP-OFDM: block-based PTRS pattern

Observation 2: With current simulation parameters, the performance of de-ICI with Rel-15 PTRS pattern outperforms the performance of block-based PTRS pattern with cyclic ZC sequence.

Observation 3: For block-based PTRS pattern with zero-power RE, the performance of ‘6dB power boosting’ is around 2.5 dB worse than that of ‘full power boosting’ for SNR achieving 10% BLER.

Observation 4: For block-based PTRS pattern with zero-power RE, the performance of ‘full power boosting’ doesn’t outperform the performance of ‘de-ICI with Rel-15 PTRS pattern’.

Proposal 2: For FR2-2, do not support block-based PTRS pattern in Rel-17.

- PTRS density in time for DFT-s-OFDM

Observation 5: The performance gap is not obvious between (CN, CS) = (8, 4) and the configuration with the best performance for most of the cases on MCS-7 and MCS-16.

Observation 6: For MCS-22/MCS-26 and PUSCH RB number as 256, configuration (CN, CS) = (16, 4) achieves best performance.

Proposal 3: The necessity to introduce more PTRS chunk number needs further discussion as there is no significant performance benefit. If there is valid reason to introduce more PTRS chunks, configuration (CN, CS) = (16, 4) can be considered.

- DMRS related design

Observation 7: ‘Type-2 no FD-OCC’ has better performance than ‘Type-2 with FD-OCC’ by 1~4 dB, and the gain on SCS-960KHz is higher than the gain on SCS-480KHz.

Proposal 4: For FR2-2, support ‘type-2 no FD-OCC’ for SCS of 480/960 kHz in Rel-17.

Proposal 5: For 480/960 kHz SCS in FR2-2, for rank 1 PDSCH, FD-CDM invalidation should be indicated by RRC signalling where one new field to indicate a set of values, and only when antenna port(s) field in DCI belongs to this set, UE can assume no FD-CDM from other UE.

- Processing timeline aspects

Proposal 6: Among parameters d1,1, d2, Text, d2,1, Tswitch, d2,2, only parameter d2,2 for 480/960 kHz SCS should use the absolute time duration for 120 kHz SCS as the upper bound for FR2-2 in Rel-17.

Proposal 7: For FR2-2, the range of k0 for 480/960 kHz SCS can be kept as 0~32.

Proposal 8: For FR2-2, the range of k1 for 480/960 kHz SCS should be defined as offset ~ offset+15, where the offset is dependent on SCS, for example, offset = 6/12 for 480/960 kHz SCS respectively.

Proposal 9: For FR2-2, the range of k2 for 480/960 kHz SCS should be defined as offset ~ offset+32, where the offset is dependent on SCS, for example, offset = 11/21 for 480/960 kHz SCS respectively.

- DCI design for multi-PDSCH/PUSCH scheduling

Proposal 10: Support more than one PDSCH/PUSCH scheduled within a slot by a single or multiple DCIs for 480/960 kHz SCS and single TRP operation.

Proposal 11: Legacy frequency domain scheduling in NR Rel-15/16 is reused for multi-PUSCH/PDSCH scheduling.

Proposal 12: For CBG based scheduling, the same behaviour for multi-PUSCH scheduling with 120 kHz SCS is applied to 480/960 kHz SCS as well, i.e., CBG based scheduling is supported only when a DCI schedules a single PUSCH.

Proposal 13: For frequency hopping for multi-PUSCH scheduling, only intra-slot frequency hopping is applicable, and is applied to each scheduled PUSCH when configured and enabled, while inter-slot frequency hopping is inapplicable.

- HARQ-ACK feedback for multi-PDSCH scheduling

Proposal 14: Regarding time domain bundling for Type-1 codebook when multi-PDSCH scheduling is configured, consider the following two alternatives:

- Alt. 1: A set of occasions is determined based on the last (valid) SLIV in each row of the TDRA table, and time domain bundling is performed across all valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table.

- Alt. 2: A set of occasions is determined based on all (valid) SLIVs in each row of the TDRA table, in the same way as the case when time domain bundling is not configured, and time domain bundling is performed for each subset of occasions divided from the set of occasions.

Proposal 15: For constructing Type-2 HARQ-ACK codebook when CBG operation is configured, support Option 2, i.e. HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

Proposal 16: Regarding time domain bundling for Type-2 codebook when multi-PDSCH scheduling is configured, consider the following two alternatives:

- Alt. 1: Time domain bundling is performed across all valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table.

- Alt. 2: Time domain bundling is performed across a subset of valid PDSCHs, which is divided from the set of valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table.

- Appendix:

  - Evaluations of PTRS density in frequency for CP-OFDM

6.1.1 Simulation parameters

Table 12 Simulation parameters for PTRS density in frequency (K_PTRS = 1 or 0.5) for CP-OFDM

| Parameters | Value |
| --- | --- |
| Carrier frequency (GHz) | 60 |
| PUSCH Bandwidth (MHz) | 400 |
| Waveform | CP-OFDM |
| Antenna configuration | TDL-A: 2T2R |
| MIMO Layer | 1 |
| SCS (KHz) | 120/480/960 |
| Phase noise model | TR38.803 Example 2 |
| MCS | NR Rel-15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A, 3km/h. |
| DS (ns) | 10 |
| FFT size | 4096/1024/512 |
| PTRS frequency density (1/RB) | 2, 1, 0.5. |
| PDSCH allocation (RB) | 8, 16, 32 for SCS of 120 KHz; 8, 16, 32 for SCS of 480 KHz; 8, 16, 32 for SCS of 960 KHz. |
| Modulation (Coderate) | QPSK: 526/1024, i.e. MCS 7. 16QAM: 658/1024, i.e. MCS 16. 64QAM: 666/1024, i.e. MCS 22. 64QAM: 873/1024, i.e. MCS 26. Note: Assume NohPRB = 0 for MCS calculations. |
| Phase noise compensation methods | a) CPE only; b) Direct de-ICI filtering approach (Rel-15 PTRS structure, 3 taps). |

6.1.2 Simulation results

Figure 2 Performance for SCS of 120kHz, DS of 10ns and MCS of 7

Figure 3 Performance for SCS of 120kHz, DS of 10ns and MCS of 16

Figure 4 Performance for SCS of 120kHz, DS of 10ns and MCS of 22

Figure 5 Performance for SCS of 120kHz, DS of 10ns and MCS of 26

  - Evaluations of PTRS pattern for CP-OFDM

6.2.1 Simulation parameters

Table 13 Simulation parameters for block-based PTRS pattern for CP-OFDM

| Parameters | Value |
| --- | --- |
| Carrier frequency (GHz) | 60 |
| PUSCH Bandwidth (MHz) | 400 |
| Waveform | CP-OFDM |
| Antenna configuration | TDL-A: 2T2R |
| MIMO Layer | 1 |
| SCS (KHz) | 120 |
| Phase noise model | TR38.803 Example 2 |
| MCS | NR Rel-15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A, 3km/h. |
| DS (ns) | 5, 10, 20 |
| FFT size | 4096 |
| PTRS density in frequency: K_PTRS | 4 |
| PN compensation methods | - CPE only; - De-ICI with Rel-15 PTRS pattern; - ICI filter approximation; - De-ICI with cyclic ZC sequence; - PN-spectrum based ICI filter approximation; - De-ICI with ZP RE; |
| Tap number for de-ICI and ICI filter approximation | 5 |
| PDSCH allocation (RB) | 256 |
| Modulation (Coderate) | 64QAM: 666/1024, i.e. MCS 22. Note: Assume NohPRB = 0 for MCS calculations. |
| Power boosting | For method a, b, c , d and e, 3 dB power boosting is applied to both block PTRS and distributed PTRS. For method f, 6 dB power boosting is applied to the NZP RE in the block PTRS. For method f, full power boosting where 12.6 dB power boosting is applied to NZP RE in the block PTRS. |

6.2.2 Simulation results

Evaluation on block-based PTRS pattern with cyclic ZC sequence

Figure 6 SCS=120kHz, PDSCH BW = 400MHz, DS = 5ns

Figure 7 SCS=120kHz, PDSCH BW = 400MHz, DS = 10ns

Figure 8 SCS=120kHz, PDSCH BW = 400MHz, DS = 20ns

Evaluation on block-based PTRS pattern with zero-power REs

Figure 9 Performance comparison between different power boosting (SCS=120kHz, PDSCH BW = 400MHz, DS = 5ns)

Figure 10 Performance comparison between different power boosting (SCS=120kHz, PDSCH BW = 400MHz, DS = 10ns)

Figure 11 Performance comparison between different power boosting (SCS=120kHz, PDSCH BW = 400MHz, DS = 20ns)

Figure 12 Performance comparison between ICI compensation methods (SCS=120kHz, PDSCH BW = 400MHz, DS = 5ns)

Figure 13 Performance comparison between ICI compensation methods (SCS=120kHz, PDSCH BW = 400MHz, DS = 10ns)

Figure 14 Performance comparison between ICI compensation methods (SCS=120kHz, PDSCH BW = 400MHz, DS = 20ns)

  - Evaluations of PTRS density in time for DFT-s-OFDM

6.3.1 Simulation parameters

Table 14 Simulation parameters for PTRS density in time for DFT-s-OFDM

| Parameters | Value |
| --- | --- |
| Carrier frequency (GHz) | 60 |
| PUSCH Bandwidth (MHz) | 400 |
| Waveform | DFT-s-OFDM |
| Antenna configuration | TDL-A: 2T2R |
| MIMO Layer | 1 |
| SCS (KHz) | 120 |
| Phase noise model | TR38.803 Example 2 |
| MCS | NR Rel-15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A, 3km/h. |
| DS (ns) | 10, 20 |
| FFT size | 4096 |
| PTRS chunk number and chunk size (CN, CS) | - (CN, CS) = (8, 4); - (CN, CS) = (16, 2); - (CN, CS) = (16, 4); - (CN, CS) = (8, 8); - (CN, CS) = (12, 4). |
| PUSCH allocation (RB) | 256, 128, 64. |
| Modulation (Coderate) | QPSK: 526/1024, i.e. MCS 7. 16QAM: 658/1024, i.e. MCS 16. 64QAM: 666/1024, i.e. MCS 22. 64QAM: 873/1024, i.e. MCS 26. Note: Assume NohPRB = 0 for MCS calculations. |

6.3.2 Simulation results

Figure 15 DS = 10ns, MCS = 7

Figure 16 DS = 10ns, MCS = 16

Figure 17 DS = 10ns, MCS = 22

Figure 18 DS = 10ns, MCS = 26

Figure 19 DS = 20ns, MCS = 7

Figure 20 DS = 20ns, MCS = 16

Figure 21 DS = 20ns, MCS = 22

Figure 22 DS = 20ns, MCS = 26

  - Evaluations of DMRS density in frequency and FD-OCC

6.4.1 Simulation parameters

Table 15 Simulation parameters for DMRS for CP-OFDM

| Parameters | Value |
| --- | --- |
| Carrier frequency (GHz) | 60 |
| PDSCH Bandwidth (MHz) | 400 |
| Waveform | CP-OFDM |
| Antenna configuration | TDL-A: 2T2R |
| MIMO Layer | 1 |
| SCS (KHz) | 480/960 |
| Phase noise model | TR38.803 Example 2 |
| Phase noise compensation | CPE only |
| PTRS density in frequency: K_PTRS | 4 |
| MCS | NR Rel-15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A, 3km/h. |
| DS (ns) | 10/20/40 |
| FFT size | 1024/512 for 480/960 KHz respectively. |
| DMRS pattern | a) Type-1 with FD-OCC, 3-dB power boosting; b) Type-1 no FD-OCC, 3-dB power boosting; c) DMRS on every RE with FD-OCC, no power boosting; d) DMRS on every RE no FD-OCC, no power boosting; e) Type-2 with FD-OCC, 4.77-dB power boosting; f) Type-2 no FD-OCC, 4.77-dB power boosting. |
| PDSCH allocation (RB) | 64/32 for SCS of 480/960 KHz respectively. |
| Modulation (coderate) | 64QAM: 666/1024, i.e. MCS 22. 64QAM: 873/1024, i.e. MCS 26. Note: Assume NohPRB = 0 for MCS calculations. |

6.4.2 Simulation results

Figure 23 DS = 10ns, MCS = 22

Figure 24 DS = 10ns, MCS = 26

Figure 25 DS = 20ns, MCS = 22

Figure 26 DS = 20ns, MCS = 26

Figure 27 DS = 40ns, MCS = 22

Figure 28 DS = 40ns, MCS = 26

References

- Chair's Notes RAN1#106-e v24, e-Meeting, August 16th – 27th, 2021.

- R1-2103161, PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band, Qualcomm Incorporated, RAN WG1 #104bis-e, e-Meeting, April 12th – 20th, 2021.

- R1-2102331, PDSCH/PUSCH enhancements for 52-71GHz spectrum, Huawei, HiSilicon, RAN WG1 #104bis-e, e-Meeting, April 12th – 20th, 2021.

- R1-2104041, Discussion summary #3 of [104b-e-NR-52-71GHz-05], Moderator (vivo), RAN WG1 #104bis-e, e-Meeting, April 12th – 20th, 2021.

- R1-2106446, PDSCH/PUSCH enhancements for 52-71GHz spectrum, Huawei, HiSilicon, RAN WG1 #106-e, e-Meeting, August 16th – 27th, 2021.

- R1-2107581, Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz, Intel Corporation, RAN WG1 #106-e, e-Meeting, August 16th – 27th, 2021.