3GPP TSG RAN WG1 #106bis-e		R1-2109011

e-Meeting, October 11th – October 19th, 2021

Source:	vivo

Title:	Discussion on time and frequency synchronization enhancements on NB-IoT/eMTC for NTN

Agenda Item:	8.15.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #106 e-Meeting [1], the following agreements on the time and frequency synchronization for NB-IoT/eMTC over NTN were achieved:

Agreement:

Duration of UL transmission segment for UE pre-compensation for PUSCH transmission is a number of PUSCH repetition units configured by the network

- For NB-IoT, repetition unit is MidenticalNPUSCH×NslotUL×Tslot

- For eMTC, repetition unit is  NslotUL×Tslot for sub-PRB allocation, where Tslot = 0.5 ms. For full-PRB allocation, repetition unit is one subframe.

- NOTE1: MidenticalNPUSCH, NslotUL, Tslot are defined in TS 36.211 10.1.2.3 and 10.1.3.6 for NB-IoT

- NOTE2: MsymbUL, MslotUL NslotUL is defined in TS 36.211, 5.2.3A for eMTC

- FFS: RAN1 to further discuss valid and invalid subframes

- FFS: Configuration details

Agreement:

- For NB-IoT/eMTC NTN, the network configures one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH in a k-bit field.

  - For NB-IoT, maximum 3-bit field with a maximum number of K=8 candidate values 2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms

- FFS: Down scoping of K candidate values, size of k-bit field

Agreement:

- For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format.

- Format 0 and format 1: 3-bit field, K=6 candidate values 2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ)

- Format 2:  2-bit field, K=4 candidate values 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ)

- FFS: Down scoping of K candidate values, size of k-bit field

- FFS: Whether the same segment duration can be used for all preambles within a preamble format

Agreement:

For eMTC, the network configures one of K values for the UL transmission segment duration of PRACH in a k-bit field.

- FFS: K candidate values, size of k-bit field

Agreement:

The UL transmission segment duration is configured by the network

- FFS: Details of the configuration signalling.

In this contribution, long UL transmission and new UL gap are discussed to support NB-IoT and eMTC over NTN.

- UL Segment Transmission

UL segment transmission was discussed during previous RAN1 meetings. There is good consensus that UL transmission segment duration may be configured based on satellite deployment, elevation angle, and the numerology. According to the achieved agreements, the PUSCH repetition unit for NB-IoT and eMTC are MidenticalNPUSCH×NslotUL×Tslot and NslotUL×Tslot, respectively.

In current specification, the transmit timing error Te is 80*Ts = 2.6 us for NB-IoT, and Te is 24*Ts = 0.78 us for eMTC. When considering IoT over NTN scenarios, e.g., the case of LEO-600, the maximum TA drift rate including both feeder link and service link is 93 us/s. In order to meet the requirement of the transmit timing error Te, the maximum continuous transmission cannot exceed 32 ms for NB-IoT over NTN, and 8 ms for eMTC over NTN. Then candidate segment transmission duration can be determined based on the repetition unit duration and the limitation of maximum continuous transmission. The repetition number N can be determined as (UL transmission segment duration / PUSCH repetition unit), as shown in table 1.

Table 1: Segment transmission duration for PUSCH transmission in LEO scenario

|  |  | ∆f [kHz] | NslotsUL | MidenticalNPUSCH | Tslot | repetition unit duration | candidate segment transmission duration | repetition number N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB-IoT |  | 15 | 2 | 4 | 0.5 ms | 4ms | {4, 8, 16, 32}ms | {1, 2, 4, 8} |
|  |  | 15 | 4 | 4 | 0.5 ms | 8ms | {8, 16, 32}ms | {1, 2, 4} |
|  |  | 15 | 8 | 4 | 0.5 ms | 16ms | {16, 32}ms | {1, 2} |
|  |  | 15 | 16 | 1 | 0.5 ms | 8ms | {8, 16, 32}ms | {1, 2, 4} |
|  |  | 3.75 | 16 | 1 | 2 ms | 32ms | {32}ms | {1} |
| eMTC | sub-PRB allocation | 15 | 4 |  | 0.5 ms | 2ms | {2, 4, 8}ms | {1, 2, 4} |
|  |  | 15 | 8 |  | 0.5 ms | 4ms | {4, 8}ms | {1, 2} |
|  |  | 15 | 16 |  | 0.5 ms | 8ms | {8}ms | {1} |
|  | full-PRB allocation | 15 | 4 |  | 0.5 ms | 1ms | {2, 4, 8}ms | {2, 4, 8} |
|  |  | 15 | 8 |  | 0.5 ms | 1ms | {2, 4, 8}ms | {2, 4, 8} |
|  |  | 15 | 16 |  | 0.5 ms | 1ms | {2, 4, 8}ms | {2, 4, 8} |

Hence, for different numerologies of NB-IoT, network can use 2-bit field with at most 4 candidate values {4ms, 8ms, 16ms, 32ms} for PUSCH transmission segment duration. And the segment duration can be provided by UE-specific RRC signalling. For eMTC, network can use 2-bit field with 3 candidate values {2ms, 4ms, 8ms}.

Proposal 1: For PUSCH transmission segment duration in LEO scenario, use

2-bit field with 4 candidate values {4ms, 8ms, 16ms, 32ms} for NB-IoT,

2-bit field with 3 candidate values {2ms, 4ms, 8ms} for eMTC.

For case of GEO, the maximum continuous transmission cannot exceed 256ms. Network can use 3-bit field with 7 candidate values {4ms, 8ms, 16ms, 32ms, 64ms, 128ms, 256ms} for NB-IoT, 3-bit field with 8 candidate values {2ms, 4ms, 8ms, 16ms, 32ms, 64ms, 128ms, 256ms} for eMTC.

Proposal 2: For PUSCH transmission segment duration in GEO scenario, use

3-bit field with 7 candidate values {4ms, 8ms, 16ms, 32ms, 64ms, 128ms, 256ms} for NB-IoT,

3-bit field with 8 candidate values {2ms, 4ms, 8ms, 16ms, 32ms, 64ms, 128ms, 256ms} for eMTC.

For PRACH transmission, the repetition unit for NB-IoT and eMTC is P symbol groups and one preamble, respectively. In order to meet the requirement of the transmit timing error Te, 1-bit field with 2 candidate values {2*4*(TCP + TSEQ), 4*4*(TCP + TSEQ)} can be applied for format 0 and 1 of NB-IoT, 1-bit field with 1 candidate values {1*6*(TCP + TSEQ)} for format 2 of NB-IoT, as shown in table 2.

For eMTC, 2-bit field with 3 candidate values {2*(TCP + TSEQ), 4*(TCP + TSEQ), 8*(TCP + TSEQ)} can be applied for all formats.

Table 2: Segment transmission duration for PRACH transmission in LEO scenario

|  | Format | repetition unit duration | candidate segment transmission duration | repetition number N |
| --- | --- | --- | --- | --- |
| NB-IoT | 0 | 4*(TCP + TSEQ) = 5.6ms | {2*4*(TCP + TSEQ) = 11.2ms, 4*4*(TCP + TSEQ) = 22.4ms} | {2, 4} |
|  | 1 | 4*(TCP + TSEQ) = 6.4ms | {2*4*(TCP + TSEQ) = 12.8ms, 4*4*(TCP + TSEQ) = 25.6ms} | {2, 4} |
|  | 2 | 6*(TCP + TSEQ) = 19.2ms | {1*6*(TCP + TSEQ) = 19.2ms} | {1} |
| eMTC | 0 | (TCP + TSEQ) = 1ms | {2*(TCP + TSEQ) = 2ms, 4*(TCP + TSEQ) = 4ms, 8*(TCP + TSEQ) = 8ms} | {2, 4, 8} |
|  | 1 | (TCP + TSEQ) = 2ms / 3ms | {2*(TCP + TSEQ) = 4ms, 4*(TCP + TSEQ) = 8ms} / {2*(TCP + TSEQ) = 6ms} | {2, 4} / {2} |
|  | 2 | (TCP + TSEQ) = 2ms | {2*(TCP + TSEQ) = 4ms, 4*(TCP + TSEQ) = 8ms} | {2, 4} |

Proposal 3: For PRACH transmission segment duration in LEO scenario, use

1-bit field with 2 candidate values {2*4*(TCP + TSEQ), 4*4*(TCP + TSEQ)} for format 0,1 of NB-IoT,

1-bit field with 1 candidate values {1*6*(TCP + TSEQ)} for format 2 of NB-IoT,

2-bit field with 3 candidate values {2*(TCP + TSEQ), 4*(TCP + TSEQ), 8*(TCP + TSEQ)} for eMTC.

For case of GEO, the maximum continuous transmission cannot exceed 256ms. Network can use 3-bit field with 5 candidate values {2*4*(TCP + TSEQ), 4*4*(TCP + TSEQ), 8*4*(TCP + TSEQ), 16*4*(TCP + TSEQ), 32*4*(TCP + TSEQ)} for format 0 and 1 of NB-IoT, 2-bit field with 3 candidate values {2*6*(TCP + TSEQ), 4*6*(TCP + TSEQ), 8*6*(TCP + TSEQ)} for format 2 of NB-IoT, and 3-bit field with 8 candidate values {2*(TCP + TSEQ), 4*(TCP + TSEQ), 8*(TCP + TSEQ), 16*(TCP + TSEQ), 32*(TCP + TSEQ), 64*(TCP + TSEQ), 128*(TCP + TSEQ), 256*(TCP + TSEQ)} for eMTC.

Proposal 4: For PRACH transmission segment duration in GEO scenario, use

3-bit field with 5 candidate values {2*4*(TCP + TSEQ), 4*4*(TCP + TSEQ), 8*4*(TCP + TSEQ), 16*4*(TCP + TSEQ), 32*4*(TCP + TSEQ)} for format 0 and 1 of NB-IoT,

2-bit field with 3 candidate values {2*6*(TCP + TSEQ), 4*6*(TCP + TSEQ), 8*6*(TCP + TSEQ)} for format 2 of NB-IoT,

3-bit field with 8 candidate values {2*(TCP + TSEQ), 4*(TCP + TSEQ), 8*(TCP + TSEQ), 16*(TCP + TSEQ), 32*(TCP + TSEQ), 64*(TCP + TSEQ), 128*(TCP + TSEQ), 256*(TCP + TSEQ)} for eMTC.

- New UL Gap

In legacy mechanism, an UL gap with 40ms is added after long UL transmission exceeding 256ms, TA value and doppler offset are re-calculated and compensated during the UL gap. While in IoT over NTN, if reuse the legacy UL gaps, the updated TA value and doppler offset will be out of date and cause larger error for next continuous transmission. Hence, legacy UL gap mechanism needs to be enhanced.

To support segmented pre-compensation mechanism, new enhanced UL gap can be adaptively configured by network. For example, in LEO case, satellite moves fast and time drift changes rapidly, more segments and more UL gaps are needed. The details of gaps, including gap number, gap length and start time, can be flexibly configured by network, and informed to UE in advance. While in MEO case, time drift changes slow, less segments and less UL gaps can be configured.

During this UL gap, UE updates UE-specific TA and common TA value and applies them before next segmented transmission. Besides, the estimated UE-specific TA can be reported to the network by UE through this gap to achieve the alignment on TA adjustment between UE and network. Moreover, the issue of the overlapping between two adjacent segments with different TA values needs to be considered. With this UL gap, the overlapping problem can be solved, shown in Figure 1.

Figure 1: Segment NPUSCH transmissions

Similarly, frequency synchronization can be pre-compensated during UL gaps. In addition, utilizing the enhanced UL gap mechanism, UE can not only perform timing and frequency synchronization, but also perform other operations, e.g., cell re-selection, beam switch, and satellite ephemeris information update.

Proposal 5: Support to enhance UL gaps for time and frequency segment pre-compensation during UL transmission.

- Conclusion

In this contribution, we discussed the issues about long UL transmission and new UL gap with the following observations and proposals:

Proposal 1: For PUSCH transmission segment duration in LEO scenario, use

2-bit field with 4 candidate values {4ms, 8ms, 16ms, 32ms} for NB-IoT,

2-bit field with 3 candidate values {2ms, 4ms, 8ms} for eMTC.

Proposal 2: For PUSCH transmission segment duration in GEO scenario, use

3-bit field with 7 candidate values {4ms, 8ms, 16ms, 32ms, 64ms, 128ms, 256ms} for NB-IoT,

3-bit field with 8 candidate values {2ms, 4ms, 8ms, 16ms, 32ms, 64ms, 128ms, 256ms} for eMTC.

Proposal 3: For PRACH transmission segment duration in LEO scenario, use

1-bit field with 2 candidate values {2*4*(TCP + TSEQ), 4*4*(TCP + TSEQ)} for format 0,1 of NB-IoT,

1-bit field with 1 candidate values {1*6*(TCP + TSEQ)} for format 2 of NB-IoT,

2-bit field with 3 candidate values {2*(TCP + TSEQ), 4*(TCP + TSEQ), 8*(TCP + TSEQ)} for eMTC.

Proposal 4: For PRACH transmission segment duration in GEO scenario, use

3-bit field with 5 candidate values {2*4*(TCP + TSEQ), 4*4*(TCP + TSEQ), 8*4*(TCP + TSEQ), 16*4*(TCP + TSEQ), 32*4*(TCP + TSEQ)} for format 0 and 1 of NB-IoT,

2-bit field with 3 candidate values {2*6*(TCP + TSEQ), 4*6*(TCP + TSEQ), 8*6*(TCP + TSEQ)} for format 2 of NB-IoT,

3-bit field with 8 candidate values {2*(TCP + TSEQ), 4*(TCP + TSEQ), 8*(TCP + TSEQ), 16*(TCP + TSEQ), 32*(TCP + TSEQ), 64*(TCP + TSEQ), 128*(TCP + TSEQ), 256*(TCP + TSEQ)} for eMTC.

Proposal 5: Support the enhanced UL gaps mechanism for time and frequency segmented pre-compensation during UL transmission.

References

- RAN WG1 #106 e-meeting Chairman’s Notes.

- RP-193235, MediaTek, “New Study WID on NB-IoT/eMTC support for NTN”, 3GPP RAN #86, 2019