3GPP TSG RAN WG1 #106bis-e                                                    R1-2109015

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE features for IIoT and URLLC

Agenda Item:	8.17.3

Document for:	Discussion and Decision

Introductions

In this contribution, we share our view on UE features for IIoT and URLLC[1][2].

URLLC/IIOT UE features

FG 25-1

In Rel-15, downlinkSPS is configured per UE corresponding to FG 5-18 since only one SPS configuration is supported per cell group. In Rel-16, sps-r16 is defined per band i.e., FG12-2 to indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. So, we think prerequisite feature groups should include FG 5-18 or FG12-2.

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Identify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot | 5-18or12-2 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2.

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Identify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot | 5-18or12-2 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-3 & FG 25-16 & FG 25-17

Since prerequisite feature groups of FG 25-3 are FG 11-3/FG 4-23 and the type of FG 11-3 is Per FeatureSetUplink, we think the type of FG 25-3 should be per FS.

For FG 25-16, the prerequisite feature groups are FG 11-3 which is per FeatureSetUplink. Thus, the type of FG 25-16 should be Per FS.

For FG 25-17, the prerequisite feature groups are FG 11-3/FG 12-1. The type of both FG 11-3/FG 12-1 are per FeatureSetUplink. Thus, the type of FG 25-17 should be Per FS.

| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23, 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] | 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. | 11-3 12-1 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |

Proposal 2: For FG 25-3, FG 25-16 and FG 25-17, the type should be Per FS.

| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23, 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] | 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. | 11-3 12-1 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |

25-3a

Since prerequisite feature groups of FG 25-3a are FG 25-3/FG 30-X and the type of FG 25-3 should be Per FS as discussed above, the type of FG 25-3a should also be per FS.

On the other hand, the dynamic repetition indication for sub-slot-based PUCCH repetition is based on the agreement on dynamic repetition indication for slot-based PUCCH repetition, which is specified in Coverage Enhancement WI in Rel-17. This can be captured in the “Note” column.

| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Yes | N/A |  | Per UE Per FS | No | No | N/A | Dynamic repetition indication for slot-based PUCCH from Cov. Enh. WI | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: For 25-3a, the dynamic repetition indication for slot-based PUCCH from Coverage Enhancement WI is captured in the “Note” column. The type should be per FS.

| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Yes | N/A |  | Per UE Per FS | No | No | N/A | Dynamic repetition indication for slot-based PUCCH from Cov. Enh. WI | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

25-7

For first bullet in components column of FG 25-7, we think ‘sub-slot’ should be added.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: For first bullet in components column of FG 25-7, ‘sub-slot’ should be added.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

25-9

For FG 25-9, semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI. This should be captured in the corresponding components column.

| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | - Semi-static PUCCH cell (FFS or PUCCH carrier) switching using configured time-domain domain pattern of applicable PUCCH cell / carrier - semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:  For FG 25-9, the following agreement should be captured in components column.

- Semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI.

| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | - Semi-static PUCCH cell (FFS or PUCCH carrier) switching using configured time-domain domain pattern of applicable PUCCH cell / carrier - semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

25-10

According to the agreement, in addition to HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to:

- HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

- HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

- triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI

For FG 25-10, these applicable cases should be explicitly described in the components:

| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication | - PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication in the DCI scheduling the PUCCH PDSCH and SPS release - PUCCH cell switching based on dynamic indication is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: For FG 25-10, the description for applicable cases should be captured in the components.

| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication | - PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication in the DCI scheduling the PUCCH PDSCH and SPS release - PUCCH cell switching based on dynamic indication is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion

In the contribution, we have some investigations on UE feature for URLLC/IIOT, and propose that,

Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2.

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Identify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot | 5-18or12-2 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: For FG 25-3, FG 25-16 and FG 25-17, the type should be Per FS.

| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23, 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] | 11-3 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. | 11-3 12-1 | Yes | N/A |  | Per UE Per FS | No | No | N/A |  | Optional with capability signaling |

Proposal 3: For 25-3a, the dynamic repetition indication for slot-based PUCCH from Coverage Enhancement WI is captured in the “Note” column. The type should be per FS.

| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Yes | N/A |  | Per UE Per FS | No | No | N/A | Dynamic repetition indication for slot-based PUCCH from Cov. Enh. WI | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: For first bullet in components column of FG 25-7, ‘sub-slot’ should be added.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:  For FG 25-9, the following agreement should be captured in components column.

- Semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI.

| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | - Semi-static PUCCH cell (FFS or PUCCH carrier) switching using configured time-domain domain pattern of applicable PUCCH cell / carrier - semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: For FG 25-10, the description for applicable cases should be captured in the components.

| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication | - PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication in the DCI scheduling the PUCCH PDSCH and SPS release - PUCCH cell switching based on dynamic indication is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

3GPP TR 38.822 V16.0.0

R1-2108679 Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.) RAN WG1 #106-e