3GPP TSG-RAN WG1 Meeting #106bis-e		R1-2109021

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	UE features for NR sidelink enhancement

Agenda Item:	8.17.11

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our view on the Rel-17 NR sidelink enhancement UE features, based on the preliminary RAN1 UE features lists in [1][2]. The preliminary UE features of Rel-17 NR sidelink enhancement are listed in A1 and A2 for NR and LTE, respectively.

- Discussion

  - R16 basic feature group for Rel-17 pedestrian UE

In Rel-16, the following UE features are defined as basic UE feature group for NR sidelink, i.e., should be indicated as “support” by UE if it supports NR sidelink:

| 15-1 | Receiving NR sidelink | 1) UE can receive NR PSCCH/PSSCH. Up to a total of A sidelink HARQ processes across all links are supported. 2) UE can receive X PSCCH in a slot. 3) UE can attempt to decode Y= NRB non-overlapping RBs per slot 4) UE supports reception of PSSCH according to the 64QAM MCS table 5) UE supports PT-RS reception in FR2. 8) UE can receive using the subcarrier spacing and CP length defined for a given band in RAN4 10) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 12) UE can receive using 30 kHz subcarrier spacing with normal CP in FR1, 120 kHz subcarrier spacing with normal CP FR2 |
| --- | --- | --- |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to 8 configured grants can be configured for a UE. Up to C sidelink HARQ processes are supported including those for configured grants 2) UE can transmit PSSCH according to the normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2 on the same carrier as sidelink. 6) UE can transmit using the subcarrier spacing and CP length it reports. 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 9) Support downlink pathloss based open loop power control 11) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1 |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform mode 2 sensing and resource allocation operations 6) UE can transmit using the subcarrier spacing and CP length it reports for FG 15-1 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu |
| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive up to N PSFCH(s) resources in a slot. 3) UE can transmit up to M PSFCH(s) resources in a slot |
| 15-23 | Support of open loop SL power control and RSRP report | - Support sidelink pathloss based open loop power control and RSRP report in case of unicast |

The first question should be clarified is that, whether these Rel-16 basic UE features are still considered as mandatory features for Rel-17 sidelink UE, especially for pedestrian UE. The problem is that some of these features (or sub-features), such as full sensing or PSCCH/PSSCH reception as highlighted above, may not be supported by pedestrian UE. To address the problem, there are several approaches to design the UE features for pedestrian UE in Rel-17:

- Alt-1: The Rel-16 basic FG that is not supported for PUE is not mandatory in Rel-17.

- Alt-2: Introduce Rel-17 UE feature to indicate which Rel-16 basic FG is not supported by the PUE.

- Alt-3: Introduce a new basic FG that overrides Rel-16 basic FG.

Alt-1 may resolve this problem for some of the FG (e.g., 15-11), but may not help for the FG where only some component(s) is not applicable (e.g., 15-3). Moreover, it may have some backward compatibility issues, e.g., the Rel-16 network may consider it as an error if a Rel-17 UE does not report some Rel-16 “basic FG”.

Alt-2 seems to have less spec impact, but it seems to be not aligned with the following RAN2 guidance [3]:

1 Avoid defining “incapability” bits as they may cause interpretation issues The definition of the capability should not say that “a UE setting the bit does not support Rel-16 feature X”. Such statements caused a lot of problems in Rel-15. One example was the pucch-F0-2WithoutFH that indicates that “the UE does not support PUCCH formats 0 and 2 without frequency hopping”.

Alt-3 may have large specification impacts, and may even have impacts on Rel-16 UE implementation.

It seems none of the alternatives is the perfect solution.

On the other hand, the current preliminary features related to power saving enhancements (e.g., 32-1/2/3/4) are somewhat confusing or problematic. For example, the FG 32-3 indicates that UE supports Rel-16 full sensing, which is duplicated with the Rel-16 FG 15-3. Moreover, the absence of FG 32-3 means that the UE only supports random selection. Consequently, it seems to imply that every Rel-16 UE should indicate FG 32-3, which has backward compatible issue. Considering that the Rel-17 UE features related to power saving enhancements highly depend on how to handle the Rel-16 basic features and how to define UE feature for pedestrian UE, RAN1 should first investigate how the PUE handles the Rel-16 basic FG.

Proposal 1: RAN1 should first investigate how the Rel-17 Pedestrian UE (e.g., does not support full sensing) handles the Rel-16 NR sidelink basic UE features.

  - Inter-UE coordination

There are two schemes (and probably several sub-schemes) are being discussed for inter-UE coordination [4]. It is preferable to have separate UE features for different inter-UE coordination (sub-)schemes, considering that these (sub-)schemes are targeting different scenarios.

Proposal 2: Separate UE features are defined for different inter-UE coordination schemes.

  - LTE UE feature

The Rel-17 NR sidelink enhancement only targets NR-specific enhancements. There is no impact on LTE sidelink, nor on LTE/NR sidelink interaction. Consequently, there should not be any impact to a Rel-17 UE supporting LTE sidelink only. For a Rel-17 UE supporting both LTE sidelink and enhanced NR sidelink, it should report the Rel-17 related UE features in NR sidelink interface only. Therefore, in our view, the Rel-17 NR sidelink enhancement should have no impact on LTE UE feature.

Observation 1: The Rel-17 NR sidelink enhancement WI has no impact on LTE UE feature.

- Conclusion

In the contribution, we provide our view on UE features for NR sidelink enhancement WI, with the following proposals and observation:

Proposal 1: RAN1 should first investigate how the Rel-17 Pedestrian UE (e.g., does not support full sensing) handles the Rel-16 NR sidelink basic UE features.

Proposal 2: Separate UE features are defined for different inter-UE coordination schemes.

Observation 1: The Rel-17 NR sidelink enhancement WI has no impact on LTE UE feature.

- Reference

- R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, RAN1#106-e.

- R1-2108678, “Preliminary RAN1 UE features list for Rel-17 LTE”, RAN1#106-e.

- R2-2002378, “Guidelines for UE capability definitions”, RAN2, RAN2#109-e.

- R1-2108999, “Discussion on mode-2 enhancements”, vivo, RAN1#106bis-e.

Annex.

A1 NR feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by NR Uu or preconfiguration. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [32-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [32-1], [32-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [32-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |

A2 LTE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. [NR_SL_enh] | 4-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by LTE Uu. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [4-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by LTE Uu. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [4-1], [4-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [4-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |

Only for licensed band