3GPP TSG RAN WG1 #106-bis-e,		 R1-2109098

e-Meeting, Oct. 11th -19th, 2021

Source:	OPPO

Title:	Discussion on cross-carrier scheduling from SCell to PCell

Agenda Item:	8.13.1

Document for:	Discussion and Decision

# Introduction

The following was captured in RAN1 Chairman notes in RAN1 #105-e meeting for cross-carrier scheduling from SCell to PCell/PSCell:

Agreement - At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1), and at least when UE is not provided monitoringCapabilityConfig for any cell, down select one from [based on Option A/C] or [based Option C] below - [based on Option A/C] - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] PDCCH BD candidates per sSCell slot (Note: this is assumed per Rel16) - UE is additionally not required to monitor more than β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - 0≤α≤1 and0≤β≤1 are based on RRC configuration and at least cases of α+β≤1 are supported - FFS the following for [based on Option A/C] - Distribution of PDCCH BD candidates between multiple sSCell slots overlapping a P(S)Cell slot including whether the above additional BD limitation is defined per sSCell slot or per P(S)Cell slot. - Discuss further using following alternatives as starting point (other alternatives/further refinement of alternatives not precluded) - Alt1 - The additional BD limitation is per sSCell slot with further limitation that UE is not required to monitor more than 2µ-µ1*β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per sSCell slot - Alt 2 - The additional BD limitation is per P(S)Cell slot and no further restrictions - Alt 3 - The additional BD limitation is per P(S)SCell slot with below further limitation - All search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell are within a single span of 3 consecutive OFDM symbols within a duration spanning P(S)Cell slot - Whether/how the definition of MPDCCHtotal,slot,μ orMPDCCHtotal,slot,μ1 is modified compared to Rel16 when UE is configured with CCS from sSCell to P(S)Cell - Whether separate α and β are configured by RRC or if β=1-α and only α is configured - [based on Option C] - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than mins1*MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than mins2*MPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per sSCell slot - When determining MPDCCHtotal,slot,μ and MPDCCHtotal,slot,μ1 - P(S)Cell self-scheduling is counted by applying scaling factor s1, - sSCell to PCell scheduling is counted additionally (assuming SCS of sSCell) by applying scaling factor s2 - 0≤s1≤1 and 0≤s2≤1 - FFS the following - Allowed combinations of s1 and s2 , and whether they are fixed or configured via RRC - Whether/how the definition of MPDCCHtotal,slot,μ orMPDCCHtotal,slot,μ1 is modified compared to Rel16 when UE is configured with CCS from sSCell to P(S)Cell - FFS the following - Multi-TRP handling - PDCCH BD handling when monitoringCapabilityConfig = r16monitoringcapability is configured for any cell

In this contribution, we provide our further views on cross-carrier scheduling scheme.

# Discussion

For Rel-15/16 cross-carrier scheduling, one SCell is scheduled by PCell or another scheduling SCell, and the PCell or the scheduling SCell has to be scheduled by itself. In Rel-17, to support cross-carrier scheduling from a SCell to PCell/PSCell, RAN1 agrees that PCell/PSCell can be scheduled by both itself and the sSCell. The PDCCH monitoring capability for PCell and the search spaces configuration /linkage need further discussion.

## Monitoring restriction of CSS type 0/0A/1/2 for type A UE

In RAN1 #106e meeting, the following proposal was discussed. For type A UE, whether Type-0/0A/1/2 CSS configured on PCell can be monitored simultaneously with USS configured on sSCell need to be clarified.

### Proposal 2v3 - For Type A UE, there is no restriction on Type-0/0A/1/2-CSS sets configurations - FFS the following for Type A UE - Alt1: - In overlapping [symbol/slot] of P(S)Cell and sSCell where a UE is configured to monitor Type-0/0A/1/2/-CSS sets on P(S)Cell and configured to monitor search space sets on sSCell (for scheduling P(S)Cell), the UE does not expect to decode DCI format(s) with CRC scrambled C-RNTI/MCS-C-RNTI/CS-RNTI on Type-0/0A/1/2-CSS sets on P(S)Cell - Alt2: - In overlapping [symbol/slot] of P(S)Cell and sSCell where a UE is configured to monitor Type-0/0A/1/2/-CSS sets on P(S)Cell and configured to monitor search space sets on sSCell (for scheduling P(S)Cell), UE drops the overlapped USS sets on sSCell - Alt 3: - In overlapping [symbol/slot] of P(S)Cell and sSCell where a UE is configured to monitor Type-0/0A/1/2/-CSS sets on P(S)Cell and configured to monitor search space sets on sSCell (for scheduling P(S)Cell), UE decodes DCI format(s) on Type-0/0A/1/2-CSS sets on P(S)Cell and the overlapped USS sets on sSCell.

The type A UE will not monitor USS configured on PCell and sSCell in a same slot(or several symbols). The complexity of implementation for USS monitoring for cross-carrier scheduling from SCell to PCell is similar with cross-carrier scheduling defined in Rel-15/16. It needs new capability to support monitoring PDCCH configured on two scheduling cells for one scheduled cell simultaneously. As the discussion and agreements in previous meetings, type B UE needs to define the new capability. If Alt.2 is selected, the capability for type A UE is similar with Rel-15/16. If Alt.1 or Alt.3 is selected, the new capability should also be defined for type A UE. In this way, it has similar complexity for PDCCH monitoring when cross-carrier scheduling from sCell to PCell is configured between type A and type B UEs. Group common DCIs are transmitted in CSS, so monitoring for CSS should be prioritized.

Proposal 1: Alt.2 is preferred for type A UE in order to simplify the complexity of implementation.

## MPDCCHtotal,slot,μ determination

According to the spec 38.213, the MPDCCHtotal,slot,μ is MPDCCHmax,slot,μ or γ∙MPDCCHmax,slot,μ when μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, or is Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  when μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap. In order to simplify the discussion, NcellsDL,μ is used to instead Ncells,0DL,μ+γ∙Ncells,1DL,μ, assuming there is no M-TR. The definition of NcellsDL,μ is ‘the number of configured DL-CCs whose scheduling cell is with active DL BWP having SCS configuration ’, and one cell can be scheduled by only one cell. If CCS is configured, PCell may have two scheduling cells, so the main point is how to count PCell, as one scheduled cell of PCell and SCell(Alt. 1), or as one scheduled cell of PCell and one scheduled cell of sSCell(Alt. 2), or as γμ1 of one scheduled cell of PCell and γμ2 of one scheduling cell of sSCell. And in RAN1 #105e meeting, three candidates calculation scheme is mentioned for ‘CA scaling calculation’ by FL.

- Alt1 - P(S)Cell is counted once - sSCell is counted once - Alt 2 - P(S)Cell is counted once - sSCell is counted twice (with same SCS as sSCell) - Alt 3 - Use scaling factors γμ1 and γμ2 with SCS configuration μ1 and μ2 respectively for P(S)Cell and sScell (the scaling factors may be less than 1)

When PCell and sSCell are configured with same SCS, Alt.1 can be used. The monitoring capability for PCell on PCell and sSCell are limited by the total capability with the configured SCS. And Alt.1 will not impact the capability for other scheduling cells configured for the UE. When PCell and sSCell are configured with different SCS, Alt.1, Alt.2 and Alt.3 shows different limitation for monitoring capability for PCell and Alt.2 may increase the total number of scheduling cells when calculate MPDCCHtotal,slot,μ.

Figure 1 An example for different SCS case

The details for different SCS case are showed in an example as Figure 1. There are 5 cells configured for a UE. The SCS configuration is μ1=15kHz and μ2=30kHz. In baseline case, PCell is only scheduled by itself. In case 1, PCell can be scheduled by PCell and sSCell simultaneous. The NcellsDL,μ and MPDCCHtotal,slot,μ are showed in Table 1 for each case. NcellsDL,μ1 is the number of scheduled cells with scheduling cell configured with SCS μ1. NcellsDL,μ2 is the number of scheduled cells with scheduling cell configured with SCS μ2. ‘a1’ and ‘a2’ are PDCCH monitoring times for PCell, ‘a1’ is monitored on PCell and ‘a2’ is monitored on sSCell. ‘a3’ is PDCCH monitoring times on sSCell for self-scheduling. ‘a4’, ‘a5’ and ‘a6’ are PDCCH monitoring times for SCell 1, SCell 2 and SCell 3.

Table 1 NcellsDL,μ and monitoring capability limitation

|  | Baseline | Alt. 1 | Alt. 2 | Alt. 3 |
| --- | --- | --- | --- | --- |
| NcellsDL,μ1 | 1 | 1 | 1 | γμ1 |
| NcellsDL,μ2 | 3 | 3 | 4 | γμ2+3 |
| NcellsDL,μ3 | 1 | 1 | 1 | 1 |
| MPDCCHtotal,slot,μ1 | 4*44*1/(1+3+1)=35 | 4*44*1/(1+3+1)=35 | 4*44*1/(1+4+1)=29 | 4*44*γμ1/(γμ1+γμ2+3+1) |
|  | Upper limit for a1 | Upper limit for a1+a2 | Upper limit for a1 | Upper limit for a1 |
| MPDCCHtotal,slot,μ2 | 4*36*3/(1+3+1)=86 | 4*36*3/(1+3+1)=86 | 4*36*4/(1+4+1)=96 | 4*44*(γμ2+3)/(γμ1+γμ2+3+1) |
|  | Upper limit for a3+a4+a5 | Upper limit for a3+a4+a5 | Upper limit for a2+a3+a4+a5 | Upper limit for a2+a3+a4+a5 |
| MPDCCHtotal,slot,μ3 | 4*22*1/(1+3+1)=17 | 4*22*1/(1+3+1)=17 | 4*22*1/(1+4+1)=14 | 4*22*1/(γμ1+γμ2+3+1)=17 |
|  | Upper limit for a6 | Upper limit for a6 | Upper limit for a6 | Upper limit for a6 |

For Alt.1

The PCell is calculated as one scheduled cell for PCell and sSCell. And the reference SCS is the SCS configuration of PCell. As PCell is counted based on reference SCS, monitoring capability of PDCCH on PCell and sSCell are also counted in the total capability with reference SCS. The total capability determination procedure is similar with Rel-15/16, but there is additional capability ‘a2’ is considered because of sSCell scheduling PCell. In case 1, ‘a1’ and ‘a2’ are limited by the capability calculated with SCS μ1, where ‘a1’ is PDCCH monitoring times on PCell and ‘a2’ is PDCCH monitoring times on sSCell.

For Alt.2

The PCell is counted as one scheduled for PCell and one scheduled cell for sSCell. In the examples, the total number of scheduled cells used for capability calculation is 6, larger than the real number of configured cells 5. ‘a1’ is limited by the total capability with SCS μ1, and ‘a2’ is limited by the total capability with SCS μ2. And the total capacity for scheduling cells with SCS μ3 is decreased because of the total number of cells is increased.

For Alt.3

The PCell is counted as a scaling value γμ1 for PCell and γμ2 for sSCell. In the examples, the total number of scheduled cells used for capability calculation is 5, same with the number of configured cells. Similar with Alt.2, the monitoring capability for PCell on PCell and sSCell are respectively counted in the total capability for SCS μ1 and μ2.

Table 1 shows monitoring capability limitation for difference cases. In Alt.1, the capability for PCell is limited by only one MPDCCHtotal,slot,μ , and the results are same with baseline case. In Alt.2 and Alt.3, the capabilities for PCell are limited respectively with different SCS. The capability is defined independently for PCell and sSCell. If the capability for PCell is defined with a total capability on PCell and sSCell, Alt.1 is preferred because the limitation for PCell and sSCell comes from only one MPDCCHtotal,slot,μ.

Proposal 2: If the capability for PCell is defiend with a total capability on PCell and sSCell, Alt.1 is preferred. If the capability for PCell is defined independently on PCell and sSCell, Alt.2 and Alt.3 are preferred.

## BD/CCE limit handling

In RAN1 #106e meeting, there were 2 options of BD/CCE limit handling mentioned, option A/C and option C. In both option A/C and option C, two explicit capabilities are defined respectively monitoring on PCell and monitoring on sSCell used for scheduling PCell, and they are fixed for each slot(s). The monitoring for PCell can be independently on PCell and sSCell. The main difference between these two options is the SCS configuration used for monitoring capability determination.

In option A/C, only one SCS configuration is used for two scheduling cells during capability determination. The SCS is the smallest one between PCell and sSCell. And the time interval used to define PDCCH capability are same for PCell and sSCell according to the selected SCS configuration. As one SCS is selected, Alt.1 in section 2.2 should be used.

In option C, PCell and sSCell use their own SCS to determine PDCCH monitoring capability. The time interval used to define PDCCH capability are independently for PCell and sSCell. As PCell and sSCell use their own SCS for PDCCH capability, Alt.2 or Alt.3 are more suitable.

Proposal 3: Option C is preferred for PDCCH capability determination for type B UE.

## Numerology for PCell and sSCell

In DSS scenario, PCell shares spectrum with LTE cell. And the SCS configuration should be same for PCell and LTE cell (15kHz). The case SCS of PCell is higher than sSCell could not be considered.

Proposal 4: The DSS WI only considers the case where PCell has no larger SCS than sSCell when SCell-to-PCell CCS is configured.

## Scheduling restriction when sSCell is de-activated or domant

When CCS is configured, sSCell can schedule PCell, sSCell and/or other sCell if configured. According to the agreement in RAN1 #104 and RAN1 #106 meetings, CA activation/deactivation and dormant operation are supported for sSCell. It means sSCell can be deactivated or configured as dormancy state. In this case, there is no PDCCH transmitted on sSCell. And the cross-carrier scheduling relationship need be clarified. The cells scheduled by sSCell cannot be scheduled by sSCell if sSCell is deactivated or dormant. How to schedule these cells need be clarified. According to current spec, the new scheduling relationship is not specified. Figure 2 shows the probable scheduling relation when sSCell is de-activated or configured as dormancy state.

Figure 2 An example for sSCell de-activated/dormant

When sSCell is configured to dormant, the cells scheduled by sSCell, except for PCell, can be re-configured by RRC to achieve new scheduling relationship, or the scheduled cell is not scheduled any more, also in dormant state, until sSCell go out of dormancy state. It can be regards as the scheduled cells except for PCell are configured as dormancy state together with sSCell unless re-configuration by RRC. When sSCell is de-activated, the cells scheduled by sScell, except for PCell, will not be scheduled any more unless new scheduling cell is configured.

For PCell, no matter the sSCell is de-activated or dormant, self-scheduling can be used to maintain scheduling for PCell. Also, re-configuration by RRC can also be used.

Proposal 5: No additional spec modification is needed to support de-activated/dormant for sSCell.

# Conclusion

In this contribution, we show our views on cross-carrier scheduling with following proposals:

Proposal 1: Alt.2 is preferred for type A UE in order to simplify the complexity of implementation.

Proposal 2: If the capability for PCell is defiend with a total capability on PCell and sSCell, Alt.1 is preferred. If the capability for PCell is defined independently on PCell and sSCell, Alt.2 and Alt.3 are preferred.

Proposal 3: Option C is preferred for PDCCH capability determination for type B UE.

Proposal 4: The DSS WI only considers the case where PCell has no larger SCS than sSCell when SCell-to-PCell CCS is configured.

Proposal 5: No additional spec modification is needed to support de-activated/dormant for sSCell.

# Reference

- R1-2108661 Summary#5 of Email discussion [106-e-NR-DSS-01]

- R1-2102416 Discussion on cross-carrier scheduling for Scell to Pcell

- R1-2104806 Discussion on cross-carrier scheduling

- R1-2107277 Discussion on cross-carrier scheduling