3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109157

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.15

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for LTE-based 5G terrestrial broadcast

Document for:	Discussion and Decision

Introduction

RAN1 related part of the work item “New bands and bandwidth allocation for 5G terrestrial broadcast” [1] was discussed in RAN1#106-e meeting and the agreements reached are recorded in the chair notes [2]. With this, the WI is essentially completed in RAN1 from a technical perspective.

The preliminary UE feature related is provided in [3] for information in last RAN1 meeting. This contribution discusses some components of the UE feature based on the input from rapporteur in [3].

UE feature related discussion

Components of the feature

It was noted in [3] regarding the basic components of the feature as follows:

[TBD: whether separate components are needed for different bandwidths]

The supported system bandwidth indicated in MIB is set to 5 MHz or 3 MHz, for which PMCH can be configured with 6/7/8 MHz. A single UE capability supporting flexible bandwidth of 6/7/8 MHz is sufficient. The component of the feature could be written to support the feature of flexible bandwidth for PMCH with system bandwidth limited to 5 or 3 MHz.

Proposal 1: The component of the feature can be written as:

- Support flexible PMCH bandwidth (6/7/8 MHz) allocation for the system bandwidth indicated in MIB set to 5 MHz or 3 MHz.

Type for reporting granularity

This is a feature regarding bandwidth of PMCH allocation for specific system bandwidths, hence the report should be per band.

Proposal 2: The capability of the feature is per band.

Mandatory/Optional

On one hand, eNB doesn’t need to know whether the feature is supported by UE per [3]. On the other hand, such feature targets UE receiving broadcast on dedicated cells for all RRC states. Hence, for UE receiving broadcast especially for RRC_IDLE state, such feature should be optional without capability signalling.

Proposal 3: The feature is optional without capability signaling.

Conclusions

This contribution presents our views on UE feature regarding supporting flexible PMCH bandwidth based on the preliminary input and summarizes the proposals as follows:

Proposal 1: The component of the feature can be written as:

- Support flexible PMCH bandwidth (6/7/8 MHz) allocation for the system bandwidth indicated in MIB set to 5 MHz or 3 MHz.

Proposal 2: The capability of the feature is per band.

Proposal 3: The feature is optional without capability signaling.

- References

RP-211144, Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 1, EBU, Qualcomm, RAN#92-e, June 14-18, 2021.

RAN1#106e Chair notes. RAN1#106-e, Aug. 16-27, 2021.

R1-2108678, Preliminary RAN1 UE features list for Rel-17 LTE, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106-e, Aug. 16-27, 2021.