3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109158

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.16

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR DL 1024-QAM

Document for:	Discussion and Decision

Introduction

In RAN1#106 meeting, the preliminary UE features of Rel-17 was provided in [1] for information in last RAN1 meeting. This contribution provides views on the UE feature for NR DL 1024-QAM based on the input.

Discussion

As the MCS and CQI tables introduced for 1024QAM contains entries of 256QAM, the 1024QAM-capable UE should support 256QAM as a prerequisite feature group. Therefore, we have the following proposal:

Proposal 1: The prerequisite feature group for NR DL 1024QAM is the support of 256QAM for PDSCH in FR1.

As instructed by WID, the capability for 1024QAM is a per-band UE capability. Since FDD and TDD are in different bands, then it should be “Yes” for FDD/TDD differentiation.

Proposal 2: The need of FDD/TDD differentiation is “Yes”.

The 1024QAM for downlink in FR1 should be an optional feature, and it should be reported by UE to gNB so that gNB can perform proper scheduling.

Proposal 3: The 1024QAM for PDSCH for FR1 is optional with capability signaling.

Then, we have the following proposal on the UE features list for NR DL 1024-QAM.

Proposal 4: Adopt the following in the UE feature table for NR DL 1024-QAM.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | 256QAM for PDSCH for FR1 | Yes |  | No support of 1024 QAM for PDSCH | Per Band | Yes | No |  | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional with capability signaling |

Conclusion

In this contribution, we discussed the UE feature for 1024QAM for PDSCH for FR1, and have following proposals:

Proposal 1: The prerequisite feature group for NR DL 1024QAM is the support of 256QAM for PDSCH in FR1.

Proposal 2: The need of FDD/TDD differentiation is “Yes”.

Proposal 3: The 1024QAM for PDSCH for FR1 is optional with capability signaling.

Proposal 4: Adopt the following in the UE feature table for NR DL 1024-QAM.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | 256QAM for PDSCH for FR1 | Yes |  | No support of 1024 QAM for PDSCH | Per Band | Yes | No |  | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional with capability signaling |

- References

R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.