3GPP TSG RAN WG1 Meeting #106bis-e  	R1-2109169

e-Meeting, October 11th – 19th, 2021

Agenda Item: 8.4.2

Source: MediaTek

Title: UE Time and frequency Synchronisation for NR-NTN

Document for: Discussion and Decision

# Introduction

A RAN2-led Rel-17 Working Item on Solutions for NR to support non-terrestrial networks (NTN) was revised in RAN Plenary #88e [1].  The study item phase identified issues and made recommendations on NR UL synchronization in TR 38.821 [2]. In this contribution, we make observations and proposals to address issues discussed in RAN1#106-e and summarized in FL summary  for UL synchronization in [3].

# UL Timing synchronization aspects

## Introduction

RAN2#113bis-e sent LS to RAN1 on TA pre-compensation:

- RAN2 would like to ask RAN1 to prioritize the TA pre-compensation work on: (i) whether and/or what parameters to broadcast for TA pre-compensation, and (ii) when broadcasted, how often the broadcasted parameters are expected to change over time..

- RAN2 has discussed TA pre-compensation reporting and agreed that at least for uplink scheduling adaptation, the UE may report information about the UE specific TA pre-compensation. RAN2 would like to ask RAN1 to provide input on the exact content and frequency of UE reporting of information about the UE specific TA pre-compensation at least for uplink scheduling adaptation.

RAN1#104bis-e made agreement on NTN UE Timing Advance formula:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

UE implementation can adjust TA on a slot basis in NR as illustrated in Figure 1. The UE reads NTN SIB  Common TA parameters at time t = n. UE calculate TA(n+1), TA(n+2), …, TA(n+K) and apply TA pre-compensation at time t=n+1, n+2, .., n+K using TA(n+1), TA(n+2), …, TA(n+K). Then, at time t=n+K+1, UE reads NTN SIB  Common TA parameters, calculate new TA values TA(n+K+2), TA(n+K+3), …, TA(n+2*K) and apply TA pre-compensation at time t=n+K+1, n+K+2, .., n+2*K using TA(n+K+2), TA(n+K+3), …, TA(n+2*K). And so on.

Figure 1: UE pre-compensation of common TA on feeder link

UE pre-compensation on TA and Doppler shift over service link is illustrated in Figure 2. The accuracy of service link prediction of TA calculation and Doppler shift calculation was shown to be very accurate in Section 3. NR device has no restriction on using its GNSS module, which means it can be assumed that the device has an accurate GNSS-acquired location. Hence, GNSS acquired position in UE is not significant factor for time alignment validity timer.

Figure 2: UE pre-compensation on TA and Doppler shift over service link

## Indication of common TA drift parameters

This section is related to Issue #2 Indication of common TA drift parameters in FL summary in [3].

RAN1#106-e agreed the following Working assumption:

Common TA may include parameter(s) indicating timing drift.

- The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

In RAN1#106-e, the Moderator made the following recommendation:

- RAN1 to discuss Issue#8 “Broadcasting the position of a reference point”after SA1 and SA3 response to the LS on broadcast of NTN GW or gNB position.

- Companies are encouraged to provide more inputs on the suitable Common TA parameters that need to be indicated by the Network to allow accurate prediction of the common TA

Figure 3 shows the plots of Common TA, NTA,common, Common TA drift rate NTA,common,drift,rate, and Common TA drift rate variation NTA,common,drift,rate,variation over feeder link for LEO=600 km Set 1 parameters.

Figure 3: common TA, Common TA drift rate, and Common TA drift rate variation

Observation 1: In case the reference point is at the gNB, broadcast common TA with a SIB periodicity as high as 1 second is not sufficient to maintain accurate UL-UL subframe alignment, which exceeds the cyclic prefix for PUSCH and PUCCH transmission

Figure 4 shows that the maximum delay error over the feeder link to less than 0.1 us assuming the device reads the NTN SIB carrying common TA parameters  NTA,common,drift,rate , NTA,common,drift,rate,variation and  NTA,common,third,order,derivative once every 30 seconds. The figures in Table 1 would suggest that prediction time of 10 seconds would be reasonable with indication of drift rate and drift rate variation with a maximum common delay error in the order of 1.Ts; prediction time of 30 seconds would be reasonable with additional indication of third derivative with a maximum common delay error in the order of 1.Ts.

Figure 4: Maximum common delay error over the feeder link

| Prediction time [s] | 2.2 us | 5.5 s | 10.9 s | 16.4 s | 21.8 s | 27.3 us | 32.8 s |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Delay error with drift rate | 0.11 us / 4.Ts | 0.68 us / 21.Ts | 2.81 us / 89.Ts | 6.36 us / 196.Ts | 11.3 us / 348.Ts | 17.3 us / 533.Ts | - |
| Delay error with drift rate and drift rate variation | 0.006 us / 1.Ts | 0.009 us / 1.Ts | 0.03 us / 1.Ts | 0.10 us / 3.Ts | 0.25 us / 8.Ts | 0.49 us / 16.Ts | 0.84 us / 26.Ts |
| Delay error with third derivative | 0.002 us / 1.Ts | 0.007 us / 1.Ts | 0.007 us / 1.Ts | 0.009 us / 1.Ts | 0.02 us / 1.Ts | 0.03 us / 1.Ts | 0.07 us / 2.Ts |

Table 1: Maximum common delay error over the feeder link in µs and rounded up in Ts

To be consistent with the legacy transmit timing error Te specified in TS 38.133 Section 7.1.2 copied and pasted in Table 2, the maximum common delay error should be small (Only entries for FR1 are considered in the Table 2 based on RAN4#100-e discussions). It should also be smaller than the  Te_NTN Timing Error Limit agreed in RAN4 copied in Table 3 (Note that Ts = 64.Tc). In order to derive Te_NTN, RAN4 assumed Te_NTN = Te + Te_GNSS + Te_SAT, where,

- Te is the legacy timing error

- Te_GNSS is the GNSS accuracy

- Te_SAT is the serving-satellite position estimation error

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 2: TS 38.133 Section 7.1.2 Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 15 | [X1]*64*Tc |
|  |  | 30 | [X2]*64*Tc |
|  |  | 60 | [X3]*64*Tc |
|  | 30 | 15 | [Y1]*64*Tc |
|  |  | 30 | [Y2]*64*Tc |
|  |  | 60 | [Y3]*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |

Table 3: Te_NTN Timing Error Limit (R4-2115347)

Assuming FR1,  SCS of SSB signal is 15 kHz and SCS of UL signal is 60 kHz

- SCS=15 kHz of SSB, SCS=60 kHz of UL signal, Te = ±640.Tc = ±10.Ts = ±0.325 µs

- SCS=30 kHz of SSB, SCS=60 kHz of UL signal, Te = ±448.Tc = ±7.Ts = ±0.227 µs

RAN4 still discussing the values X1-3 and Y1-3. It can be observed from the figures in Table 1 that prediction times in the order of 10 seconds with indication of drift rate and drift rate variation or 30 seconds with additional indication of third derivative targeting a maximum common delay error of 1.Ts would be consistent with RAN4 discussions.

Proposal 1: If feeder link timing drift is to be compensated by UE using common TA parameters, the Network may periodically broadcast:

- Common delay

- Common delay drift rate

- Common delay drift rate variation

- Common third order derivative

Where

- Common delay  is NTA,common  / 2

- Common delay drift rate is  NTA,common,drift, rate / 2

- Common delay drift rate variation is  NTA,common,drift,rate,variation  / 2

- Common delay third order derivative is NTA,common,drift,thrird_order,derivative  / 2

## Granularity and signalling of Common TA Parameters

This section is related to Issue#3 Granularity and signalling of Common TA in FL summary in [3].

The moderator made updated proposal in RAN1#105e and encouraged companies to further comment and contribute in next meeting.

In case of LEO/MEO based non-terrestrial access network:

- The granularity of Common TA is set to be 64⁄2^μ ∙T_c

In case of GEO based non-terrestrial access network:

- The granularity of Common TA is set to the same as the granularity of N_TA, i.e., (16∙64)⁄2^μ ∙T_c.

μ is the highest allowed numerology for the given Frequency Range

In TS 38.213 Section 4.2, in case of random access response and  = 0, 1, 2, ..., 3846. If the granularity of common TA, NTA,common  is set to the same granularity of NTA, this gives a common TA error of 0.52 us with numerology μ=0 for GEO. This exceeds the legacy UE timing error (Te) in TS 38.133 as shown in Table 1 in Section 2.4.

Observation 2: If   (16∙64)⁄2^μ ∙T_c is used for GEO, this gives a common TA error of 0.52 us, which exceeds the legacy UE timing error (Te) in TS 38.133.

For LEO/MEO/GEO, using granularity 64⁄2^μ ∙T_c gives a common TA error of 32.55 ns for numerology μ=0 and 2.03 ns for numerology μ=4. Assuming max RTD of feeder link in GEO of 270.73 ms as in TS 38.811, this gives a maximum of

- 27 bits for numerology μ=4 (i.e. 270.73 ms / (64 * 1/(480000*4096) / 24) = 132710478 ≤ 227)

- 23 bits for numerology μ=0 (i.e. 270.73 ms / (64 * 1/(480000*4096) / 20) = 8294410 ≤ 223)

Observation 3: The maximum range of common TA is

Nmax TA,common=maximum RTD on the feeder (ms) 64.Tc/2μ

It seems reasonable if the delay error due to common TA granularity is not higher for GEO than it is for LEO/MEO. This allows same bit-length is used for the indication without need for additional indication on how to distinguish GEO and LEO to determine the corresponding granularity and reduces impact on the specifications.

Proposal 2: The granularity of Common TA is set to be 64⁄2μ ∙Tc for LEO/MEO/GEO based non-terrestrial access network.

## TA update in RRC connected

This section addresses Issue#6 in FL summary in [3]. RAN1#104e made the following agreement:

- An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

  - FFS: Operation of closed loop and open loop TA control

- For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

  - FFS: Details of the combination of open and closed loop TA control

In RAN1#106-e, the following agreement was made:

- in NR NTN, NTA update based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction as follows:

- When TAC (TA) in msg2/msgB is received,  UE receives the first adjustment and NTA is updated as follows:

NTA=NTA_old+TA. 16.642μ  , FFS: the value of NTA_old,

where, TA is the TAC field in msg2/msgB

- When TACs (TA) provided within the MAC CE is received, NTA is updated as follows:

NTA_new=NTA_old+TA-31.16.642μ ,

Where, TA is the TAC field receivd in MAC CE  command

For RAN1#106bis-e, the moderator encouraged companies to provide more inputs on:

- The update/maintenance of  NTA,UE-specific and +NTA,common

- And provide details on the combination of open and closed loop TA control

The UE-specific TA NTA,UE-specific is calculated by UE based on its GNSS-acquired location and the satellite ephemeris broadcast on NTN SIB. The need for additional timing indication in DL between satellite denoted by ds,DL and device and in UL between device and satellite denoted by ds,UL should be clarified.  Assume the device read the satellite ephemeris broadcast at time t. The device implementation can determine ds,DL  and ds,UL autonomously at any given time t+t1+K and t1+t2 +K respectively and use the correct calculated NTA,UE-specific at time t+t1+t2+K. Alternatively, the UE can determine t3=t1+t2 and then use the correct calculated NTA,UE-specific = t+t3+K. Note that K is the prediction time from time t where UE reads the satellite ephemeris on NTN SIB and time t+K where the UE pre-calculate the amount of delay pre-compensation. Knowing the UE-satellite distance at time t and also knowing the UE-satellite distance at time t1, the UE can determine the  ds,DL  and ds,UL autonomously in straightforward way.

Observation 4: Inclusion in NTA,UE-specific formula of additional timing in DL between satellite denoted by ds,DL and device and in UL between device and satellite denoted by ds,UL is not necessary.

The common TA, NTA,common, can be determined by the device based on the common TA parameters broadcast on NTN SIB. Similarly to the analysis above for NTA,UE-specific,  the device implementation can determine t3=t1+t2 and then use the correct calculated NTA,common = t+t3+K.

Proposal 3: How the UE calculates/update the NTA,common can be postponed discussion until the issue of what common TA parameters are indicated on NTN SIB is concluded.

## NTN UE Time Alignment Timers

This section addresses Issue#7 in FL summary in [3]. In RAN1#106-e, the following agreement was made:

- A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris.

  - FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration.

- FFS: Whether the same validity duration can be applied for Common TA.

The GateWay propagates the satellite ephemeris with some error that depends on the prediction time in the NTN Control Center (NCC) [4]. This error on the satellite ephemeris broadcast by the eNB is unknown to the UE.

- Position error due to NTN prediction < 30 m

- Velocity error due to NTC prediction < 30 mm/s

There is a factor of 1000 between the position error (in m) and the velocity error (in m/s)

Adding the NCC position to the prediction error of the UE-specific calculated TA could break the transmit timing error or cyclic prefix. For example, assuming the NCC position error of 30 m could lead to a one-way delay error of 0.1 us.

Assuming NCC position error is 90 m, then the one-way delay error could be 0.3 us.

Assuming the satellite ephemeris and the common TA parameters are broadcast on NTN SIB with same periodity, it seems sufficient if the network configures only a single UL timing alignment validity timer. This could be achieved within a prediction time of 30 seconds with  maximum common delay error in the order of 1.Ts (as shown in Table 1 in Section 2.2) and a prediction time of 30 seconds with  maximum delay error over the service link in the order of 1.Ts (as shown in Table 5 in Section 4). Hence, it seems reasonable to only use a single time alignment timer that defines the maximum time during which the UE can apply the satellite ephemeris and common TA parameters without having acquired new satellite ephemeris or common TA parameters.

Observation 5: A prediction time of 30 seconds with  maximum common delay error in the order of 1.Ts  and a prediction time of 30 seconds with  maximum delay error over the service link in the order of 1.Ts can be achieved.

Proposal 4: A single time alignment validity timer is configured by the network defines the maximum time during which the UE can apply the satellite ephemeris and common TA parameters without having acquired new satellite ephemeris or common TA parameters.

# Indication of common frequency pre-compensation offset on DL service link

This section addresses Issue#9 in FL summary in [3]. In RAN1#106-e, the following recommendations were made by the moderator:

- For the next RAN1’s meeting, proponents are encouraged to provide more details on the solution, signalling design …if common DL frequency compensation is supported.

- If common DL frequency compensation is not supported, as pointed out by some companies the main concern is that the frequency hypotheses tests during initial SSB search could be increased and it could be at low frequencies the UE may not know what the absolute value of ARFCN is if it synchronized in the neighbor channel raster grid. Companies are encouraged to discuss other solutions to be considered in this case .e.g. the feasibility to adapt the channel raster to the much enlarged space search.

It seems reasonable to deprioritize support of DL common frequency compensation for the service link Doppler in Release 17. The main issues are:

- Impact on DL Synchronization and DL demodulation performance if DL common frequency pre-compensation offset is applied by the network:

Before receiving the NTN SIB with the indication of the DL common Doppler shift pre-compensation, the UE cannot track the delay drift over the service link if the received frequency has been pre-compensated by the DL common Doppler shift. This is because the DL frequency is shifted by the differential Doppler shift based on DL common Doppler shift pre-compensation, while the delay drift is based on the satellite movement regardless of whether the DL common Doppler shift is applied or not. In such case the UE cannot derive the right sampling rate for the DL signal using the DL frequency as a reference resulting in a delay drift rate of ±23 ppm. We include some analysis below to further describe this issue.

Satellite links impacts on DL signals transmitted by the gNB is illustrated in Figure 5:

  - Doppler shift on the service link (dsf0) which is a result of the service delay drift impact to the carrier frequency

  - Satellite Common Doppler shift pre-compensation on the service link (-dcf0) if applied; zero otherwise.

  - Feeder link delay τf and service link delay τs

  - Feeder link delay drift df and service link delay drift ds

- Baseband signal:  a(t)

- Signal at the satellite: xt=ej2πf01-dcta(t+dft-τf)

- Received signal UE: zt=α xt+dst-τs  =β ej2πf01+ds-dcta1+ds1+dft-τf-τs

Figure 5: Impact of moving satellite on UE signals transmitted by the UE

After Initial cell search UE is synchronized to the DL reference frequency fRx = fc(1+ds-dc)

The received signal in baseband is  yt= e-j2πf01+ds-dctzt=β a1+ds1+dft-τf-τs

Since the UE does not know yet about the feeder link delay drift, the sampling frequency is generated based on  fRx but assuming the nominal f0

  - fs=fRx/f0 x 1/Tc= (1+ds-dc)/Tc

  - Sampled signal: yn=β a1+ds1+dfn Tc/(1+ds-dc)-τf-τs

DL common Doppler shift pre-compensation over service link is not applied by the network:

With dc = 0, we have ideal sampling result at UE with yn= β an Tc+df n Tc-τf-τs

There is some marginal degradation of DL performance for PDSCH before SIB decoding with common TA parameters is indicated due to the delay drift term df.

The distortion will lead to Demod SINR degradation, where

I=1KLl=0L-1k=K2K2-1ej157π. df.k.l-12

Demod SINR = S/(I+Noise/Nrx)

Demod SNR degradation is small for low SNR  (~0.2dB degradation @ 0dB) as can be observed in Figure 6.

Observation 6: The degradation due to Feeder Link delay drift is not marginal for initial access.

Observation 7: For connected mode, the feeder link delay drift has to be known to the UE to avoid performance degradation for DL demodulation.

Figure 6: Impact of moving satellite on gNB signals transmitted by the UE with df.

DL common Doppler shift pre-compensation over service link is applied by the network:

With dc ≠ 0, we have non ideal sampling result at UE with yn≈β an Tc+(df+dc) n Tc-τf-τs

In case DL common Doppler shift pre-compensation is applied, during initial stage of DL synchronization before DL Common Doppler shift pre-compensation are indicated, there can be issue with DL synchronization due to the delay to the delay drift term dc. It is preferable if dc = 0, which can be achieved simply by not supporting the DL common Doppler shift pre-compensation. If applied still, then it is better if indication of related parameters are provided as early as can be during initial cell search and DL synchronization

Note that there is similar issue with the delay drift term over the feeder link df. In case reference point is at the satellite, df can be assumed to be zero. In that case we get yn= β an Tc-τf-τs.

With 48ppm drift, the degradation is more significant due to the delay drift term over the feeder link, df, and the delay drift term due to the DL common Doppler shift compensation dc as shown in Figures 7 and 8. It is NRB=48 : 0.9dB@SNR=0dB and NRB=96 : 2.6dB@SNR=0dB

Figure 7: Impact of moving satellite on UE signals transmitted by the UE with df and dc, NRB=48.

Figure 8: Impact of moving satellite on UE signals transmitted by the UE with df and dc, NRB=96.

Proposal 5: For connected mode, the feeder link delay drift is indicated.

- Impact on Doppler shift discontinuity at the beam boundary if DL common frequency pre-compensation offset is applied by the network:

During RRC_CONNECTED, to avoid Doppler Frequency Shift discontinuity when switching the beam is avoided if DL common frequency compensation is not applied as illustrated in Figure 9, which greatly simplifies AFC implementation for frequency tracking and cell search.

- If pre-compensation is used in LEO’600 km, the Doppler shift at centre of the beam is 0 Hz.

- The residual Doppler shift with a satellite beam of diameter 200 km is in range [-7.2 KHz, +7.2 KHz].

- The Doppler discontinuity can be expected to be 14.4 kHz 2*7.2 kHz (=2*544 Hz /s * 200 km / 7.5622 km/s / 2).

Figure 9: Satellite Doppler shift discontinuity with DL common frequency compensation

|  | fc<3GHz (FR1) | 3GHz<fc<6GHz (FR1) | fc>24.25GHz (FR2) |
| --- | --- | --- | --- |
| Raster spacing/2 | 50 kHz | 720 kHz | 8.64 MHz |
| Crystal error @ ±10 ppm | ±20 kHz @ 2 GHz | ±50 kHz @ 5 GHz | ±200 kHz @ 20 GHz |
| Max Doppler shift | ±48 kHz @ 2 GHz | ±120 kHz @ 5 GHz | ±480 kHz @ 2 GHz |
| Max residual Doppler shift @ 200 km beam diameter | ±7.2 kHz @ 2 GHz | ±18 kHz @ 5 GHz | ±72 kHz @ 2 GHz |
| Doppler discontinuity at beam boundary | ±14.4 kHz @ 2 GHz | ±36 kHz @ 5 GHz | ±144 kHz @ 2 GHz |

Table 4: Maximum Doppler drift for LEO @ 600 km @ 200 km beam diameter

- Impact on signaling if DL common frequency pre-compensation offset is applied by the network:

The UE is indicated the common Doppler shift pre-compensation if applied by the gNB and subtracts it from the UE-specific Doppler shift for pre-compensation. A smaller max residual Doppler shift of ±10 ppm or ±20 kHz assuming cell radius of up to 250 km compare to max Doppler shift of  ±24 ppm or ±48 kHz can be experienced by the UE. We consider the following to allow UE to calculate the common Doppler shift accurately in case of earth-fixed beam:

- The common Doppler shift  w.r.t. beam centre can be broadcast and read  on the NTN SIB.

- The beam-specific ECEF co-ordinates of a fixed Reference Point can be indicated and read on NTN SIB to allow UE to determine the amount of common Doppler shift compensation applied by the network at the fixed reference point within a beam.

Vt=VSat,USatBeamCentreUSatBeamCentre

fCommon(t)=fc.Vtc

Conclusion on DL common frequency pre-compensation offset:

Observation 8: There are significant benefits if DL frequency compensation for the service link Doppler is not supported:

- High update rate of DL common frequency parameters NTN SIB by UE is not needed.

- Doppler Frequency shift discontinuity when switching the beam is avoided if DL common frequency compensation is not applied, which greatly simplifies AFC implementation for frequency tracking, cell search and cell measurement.

- Signalling overhead and complexity for indication of common Doppler shift pre-compensation for earth-moving bemas and earth-fixed beams.

It is not necessary for a working satellite system that the gNB applies the DL common frequency pre-compensation offset. This may result in the maximum Doppler shift of ±48 kHz, which is an issue that needs solution in device. During initial cell search to resolve the ambiguity due to the satellite Doppler shift and crystal inaccuracy in the DL synchronization stage for DL carrier frequency < 3 GHz where the Carrier Frequency Offset (CFO) can be greater than half the raster spacing 100 kHz/2 = 50 kHz. This may result in the UE synchronizing to the neighbor raster spacing, which gives ambiguity on the value of the DL carrier frequency. During initial access, the UE synchronizes to a DL carrier frequency and calculates the Doppler Frequency Shift (DFS) corresponding to the satellite Doppler shift using its GNSS-acquired location and satellite ephemeris broadcast on NTN SIB. The UE can determine accurately the DL ARFCN by subtracting the DFS from the DL carrier frequency.

Observation 9: The UE can determine the DL ARFCN by using its GNSS-acquired location and satellite ephemeris to calculate the Doppler Frequency Shift (DFS) corresponding to the satellite Doppler shift and subtracting it from the DL carrier frequency it synchronized to during initial access.

Proposal 6: DL frequency compensation for the service link Doppler is not supported.

# Serving satellite ephemeris format

This section addresses Issue#12 in FL summary in [3]. RAN1#104bise made the following agreements:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

In RAN1#106-e, the moderator proposed the following working assumption:

- Support serving satellite ephemeris format bit allocations:

- Position and velocity state vector ephemeris format [17 bytes payload].

  - The field size for position [m]  is [78 bits]

    - Position range is driven by GEO : +/- 42 200 km

    - The quantization step is [1.3m] for position

  - The field size for velocity [m/s] is [54 bits]

    - Velocity range is driven by LEO@600 km: +/- 8000 m/s

    - The quantization step is [0.06 m/s] for Velocity

- Orbital parameter ephemeris format [18 byte payload]

  - Semi-major axis α [m] is [33 bits]

    - Range: [6500, 43000]km

  - Eccentricity e is [19 bits]

    - Range: ≤ 0.015

  - Argument of periapsis ω [rad] is [24 bits]

    - Range: [0, 2π]

  - Longitude of ascending node Ω [rad] is [21 bits]

    - Range: [-180o , +180o]

  - Inclination i [rad] is [20 bits]

    - Range: [-90o  , +90o ]

  - Mean anomaly M [rad] at epoch time to is [24 bits]

    - Range: [0, 2π]

FFS: Additional enhancement to optimize the signalling overhead.

And made the following recommendation:

- RAN1 to discuss serving satellite ephemeris bit allocations in the next RAN1 meeting, and hopefully agree on WA on the field size for the different ephemeris parameters that can be revisited/updated after RAN4 response to the LS on timing synchronization requirements.

The satellite position and velocity state vectors and satellite orbital parameters are discussed in Annex B. The prediction error with satellite position and velocity state vectors and satellite orbital parameters are shown in Figure 9 and Table 5.

Figure 9: Prediction of delay error and frequency error over service link

| Periodicity | Radial Position error | Delay error | Radial velocity error | Frequency error |
| --- | --- | --- | --- | --- |
| 30 s | 11.2 m | 0.037 us / 1.Ts | 0.33 m/s | 2.2 Hz |
| 60 s | 34.6 m | 0.076 us / 2.Ts | 0.72 m/s | 4.8 Hz |
| 120 s | 327.6 m | 0.247 us / 8.Ts | 1.61 m/s | 10.7 Hz |

Table 5: Maximum delay error over the service link in µs and rounded up in Ts , maximum Doppler shift error

Payload of Position and velocity state vector ephemeris format:

It was observed via simulations that a SIB overhead for satellite position {SX, SY, SZ} and velocity {VX, VY, VZ} ephemeris of 16 bytes are sufficient to allow accurate prediction of satellite delay and Doppler shift as shown in Table 3.

| Position and Velocity Ephemeris | Resolution | #bits | Resolution | #bits |
| --- | --- | --- | --- | --- |
| Satellite Location (Range ≤ ±43000 km) | 0.33m | 3*28=84 | 1.3m | 3*26=78 |
| Satellite Velocity (Range ≤ ±8 km/s) | 0.015 m/s | 3*20=60 | 0.06 m/s | 3*18=54 |
| Payload | 18 bytes |  | 16 bytes |  |

Table 6: Payload of Position and velocity state vector ephemeris format.

Payload of Orbital parameter ephemeris format:

It was observed via simulations that a SIB overhead for orbital parameters of 16 bytes are sufficient to allow accurate prediction of satellite delay and Doppler shift as shown in Table 4.

| Orbital ephemeris | α | e | ω | Ω | I | Mo |
| --- | --- | --- | --- | --- | --- | --- |
| Bit allocation | 26 bits | 19 bits | 24 bits | 18 bits | 17 bits | 24 bits |
| Range | ±43000 km | ≤ 0.015 | [0, 2π] | [-180o , +180o] | [-90o , +90o ] | [0, 2π] |
| Payload | ~18 bytes |  |  |  |  |  |

Table 7: Payload of orbital parameter ephemeris format.

Proposal 7: Support satellite ephemeris format bit allocations

- Position and velocity state vector ephemeris format (16 bytes payload).

  - The field size for position [m]  is 78 bits

  - The field size for velocity [m/s] is 54 bits

- Orbital parameter ephemeris format (18 byte payload)

  - Semi-major axis α [m] is 33 bits

  - Eccentricity e is 19 bits

  - Argument of periapsis ω [rad] is 24 bits

  - Longitude of ascending node Ω [rad] is 21 bits

  - Inclination i [rad] is 20 bits

  - Mean anomaly M [rad] at epoch time to is 24 bits

# Serving satellite ephemeris Epoch time

This section addresses Issue#13 in FL summary in [3]. RAN1#106-e made the following agreements:

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

Time reference provisioning for PV ephemeris and orbital ephemeris formats:

Figure 7 shows that prediction 120 s ahead for UE pre-compensation is accurate within 0.247 us for delay error and within 10.7 Hz for Doppler error for position and velocity ephemeris format; and accurate within 0.523 us for delay error and within 17.6 Hz for Doppler error for position and velocity ephemeris format. Although the prediction can be done ahead over several minutes, it is necessary that the ephemeris is broadcast with low latency and high accuracy for access. A UE first coming into coverage of a satellite needs to immediately access if it is paged or if it needs to transmit data. The UE must be able to receive the satellite ephemeris on NTN SIB broadcast with periodicity 0.5s or 1 s. A longer SIB periodicity is not desirable due to short satellite dwell time (~10 minutes).

For both ephemeris formats Set 1 and Set 2, the epoch time can be implicitly known as a reference time linked to DL subframe index and System Frame Number where NTN SIB is broadcast.

Proposal 8: Epoch time is implicitly known as a reference time linked to DL subframe index and System Frame Number where NTN SIB is broadcast.

# Conclusion

In this contribution, we summarize issues and discuss impact on specifications for solutions for UL time and frequency synchronization.

Indication of common TA drift parameters:

Observation 1: In case the reference point is at the gNB, broadcast common TA with a SIB periodicity as high as 1 second is not sufficient to maintain accurate UL-UL subframe alignment, which exceeds the cyclic prefix for PUSCH and PUCCH transmission

Proposal 1: If feeder link timing drift is to be compensated by UE using common TA parameters, the Network may periodically broadcast:

- Common delay

- Common delay drift rate

- Common delay drift rate variation

- Common third order derivative

Where

- Common delay  is NTA,common  / 2

- Common delay drift rate is  NTA,common,drift, rate / 2

- Common delay drift rate variation is  NTA,common,drift,rate,variation  / 2

- Common delay third order derivative is NTA,common,drift,thrird_order,derivative  / 2

Granularity and signalling of Common TA Parameters:

Observation 2: If   (16∙64)⁄2^μ ∙T_c is used for GEO, this gives a common delay error of 0.52 us, which exceeds the legacy UE timing error (Te) in TS 38.133.

Observation 3: The maximum range of common TA is

Nmax TA,common=maximum RTD on the feeder (ms) 64.Tc/2μ

Proposal 2: The granularity of Common TA is set to be 64⁄2^μ ∙T_c for LEO/MEO/GEO based non-terrestrial access network.

TA update in connected mode

Observation 4: Inclusion in NTA,UE-specific formula of additional timing in DL between satellite denoted by ds,DL and device and in UL between device and satellite denoted by ds,UL is not necessary.

Proposal 3: How the UE calculates/update the NTA,common can be postponed discussion until the issue of what common TA parameters are indicated on NTN SIB is concluded.

NTN UE Time Alignment Timers:

Observation 5: A prediction time of 30 seconds with  maximum common delay error in the order of 1.Ts  and a prediction time of 30 seconds with  maximum delay error over the service link in the order of 1.Ts can be achieved.

Proposal 4: A single time alignment validity timer is configured by the network defines the maximum time during which the UE can apply the satellite ephemeris and common TA parameters without having acquired new satellite ephemeris or common TA parameters.

Indication of common frequency pre-compensation offset on DL service link

Observation 6: The degradation due to Feeder Link delay drift is not marginal for initial access.

Observation 7: For connected mode, the feeder link delay drift has to be known to the UE to avoid performance degradation for DL demodulation.

Proposal 5: For connected mode, the feeder link delay drift is indicated.

Observation 8: There are significant benefits if DL frequency compensation for the service link Doppler is not supported:

- High update rate of DL common frequency parameters NTN SIB by UE is not needed.

- Doppler Frequency shift discontinuity when switching the beam is avoided if DL common frequency compensation is not applied, which greatly simplifies AFC implementation for frequency tracking, cell search and cell measurement.

- Signalling overhead and complexity for indication of common Doppler shift pre-compensation for earth-moving bemas and earth-fixed beams.

Observation 9: The UE can determine the DL ARFCN by using its GNSS-acquired location and satellite ephemeris to calculate the Doppler Frequency Shift (DFS) corresponding to the satellite Doppler shift and subtracting it from the DL carrier frequency it synchronized to during initial access.

Proposal 6: DL frequency compensation for the service link Doppler is not supported.

Serving satellite ephemeris format

Proposal 7: Support satellite ephemeris format bit allocations

- Position and velocity state vector ephemeris format (16 bytes payload).

  - The field size for position [m]  is 78 bits

  - The field size for velocity [m/s] is 54 bits

- Orbital parameter ephemeris format (18 byte payload)

  - Semi-major axis α [m] is 33 bits

  - Eccentricity e is 19 bits

  - Argument of periapsis ω [rad] is 24 bits

  - Longitude of ascending node Ω [rad] is 21 bits

  - Inclination i [rad] is 20 bits

  - Mean anomaly M [rad] at epoch time to is 24 bits

Proposal 8: Epoch time is implicitly known as a reference time linked to DL subframe index and System Frame Number where NTN SIB is broadcast.

# ANNEX A

Serving Cell ephemeris

- Satellite Position vector S =(Sx, Sy, Sz)

- Satellite Velocity vector 𝑉 ⃗=(Vx, Vy, Vz)

- Time reference for real-time Satellite position and Velocity is the end of frame where SIB was transmitted at the satellite side

- UE propagate satellite position and velocity between SIB transmissions to track satellite delay drift and Doppler drift

UE acquires its position PUE using its GNSS receiver

Calculated satellite delay:

  - USat-UE(t)=PUE -St ,   dt=USat-UE(t)

  - Td(t)=dt/c

Calculated satellite Doppler:

  - fdt=<USat-UE(t),Vt>dt  fc/c

The UE Propagate the satellite position and velocity in an inertial referential using canonical laws of mechanics:

  - ∂S∂t=V

  - ∂V∂t=g

  - g= -GMS S 3   is the gravity force vector with Gravity constant GM = 3.986004418e14

# ANNEX B

The orbital parameters can be transformed to state vector position and velocity at epoch time t0 followed by propagation of satellite position and velocity to time t; or the orbital parameters can be propagated to time t followed by transform to state vectors position and velocity at time t. The satellite orbital parameters are illustrated in Figure 7

Figure 7: Satellite ephemeris Orbital parameters

An example of serving satellite ephemeris with orbital parameters broadcast on NTN SIB is shown below. The orbital elements are osculating elements (instantaneous ones to give the same accuracy as the position/velocity), and not the average/mean ones. The orbital parameters eccentricity, periapsis, semi-major axis vary significantly over the duration. The mean anomaly varies significantly as this reflects the difference between the assumption that the satellite orbit is a perfect ellipse/circle assuming the earth is a perfect sphere, and the reality that the orbit does not follow a Keplerian orbit of a perfect ellipse/circle due to the non-spherical earth shape. This is explained by the perturbations that affect the orbital propagation, mainly the non-sphericity of the Earth shape (Earth’s oblateness). Earth radius at pole is 21 km smaller than Earth radius at equator. For a nearly polar orbit (and very close to the Earth), it impacts significantly the propagation of the orbital elements.

Epoch time to                              α (km)                  e                     I (deg)              Ω (deg)               ω (deg)                M (deg)

------------------------------------------------------------------------------------------------------------------------------------------------------------

2021/01/01-00:00:00.000     6919.955143    0.001650732      97.574021193    36.126537772    64.902440674      295.175021671

2021/01/01-00:00:01.000     6919.951705    0.001651935      97.574022789    36.126537826    64.940253179      295.200230115

2021/01/01-00:00:02.000     6919.948274    0.001653137      97.574024401    36.126537882    64.978022032      295.225482138

The corresponding satellite position {SX, SY, SZ} and satellite velocity {VX, VY, VZ} are shown below for the example

Epoch time                           SX (km)             SY (km)                SZ (km)        VX (km/s)          VY (km/s)          VZ (km/s)

---------------------------------------------------------------------------------------------------------------------------------------------------

2021/01/01-00:00:00.000     5584.564377     4078.146732      -11.231645     0.591075179    -0.807929297     7.528623249

2021/01/01-00:00:01.000     5585.151982     4077.336215       -3.702637      0.584333173    -0.812851898     7.528630518

2021/01/01-00:00:02.000     5585.732846     4076.520777        3.826369      0.577590439    -0.817773537     7.528628699

# References

- RP-201256, Thales, Solutions for NR to support non-terrestrial networks (NTN), RAN#88e, June 2021

- TR 38.821 “Solutions for NR to support non-terrestrial networks”

- R1-2108587, FL summary for UL synchronization, Moderator (Thales), RAN1#106e, August 2021

- R1-2104301, Considerations on UL timing and frequency synchronization in NTN, Thales, RAN1#105e, May 2021

- R1-2009092, Ericsson, On UL time and frequency synchronization enhancements for NTN, RAN1#103e, November 2020

- MediaTek MT3333 All-in-One GNSS Datasheet, https://labs.mediatek.com/en/chipset/MT3333

- https://www.gps.gov/systems/gps/performance/accuracy/

- René Schwarz,  Keplerian Orbit Elements to Cartesian State Vectors, https://downloads.rene-schwarz.com/download/M002-Cartesian_State_Vectors_to_Keplerian_Orbit_Elements.pdf

- R1-2008809, MediaTek, UE Time and frequency Synchronisation for NR-NTN, RAN1#103e, November 2020.