# 3GPP TSG RAN WG1 #106bis-e							R1-2109177

# e-Meeting, October 11th – October 22nd, 2021

Agenda item:	8.15.2

Source: 	Qualcomm Incorporated

Title: 	Timing relationship enhancements

Document for:	Discussion and Decision

# Background

In RAN1#106-e, the following agreements for eMTC/NB-IoT over NTN were made with regards to this agenda item:

Agreement:

For NB-IoT, on receiving UL grant on DCI format N0 in subframe n, NPUSCH Format 1 is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For NB-IoT, on receiving a NPDSCH with a RAR message that ends in subframe n, the corresponding Msg3 is transmitted on NPUSCH format 1, with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For NB-IoT, a UE upon detection of a NPDSCH transmission for which it should provide an ACK/NACK feedback, shall transmit the HARQ ACK/NACK with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For NB-IoT, on receiving a timing advance command ending in DL subframe n, the corresponding adjustment of the uplink transmission timing by the received time advance shall be delayed by Koffset as compared to current specification.

Agreement:

For eMTC, on receiving an UL grant via MPDCCH that ends in DL subframe n, PUSCH is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, on receiving a RAR in a PDSCH that ends in subframe n, PUSCH for Msg3 is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, when an MPDCCH ending in subframe n activates UL SPS, the time of the first subframe in which the UE is allowed to transmit SPS-PUSCH is delayed by Koffset as compared to transmission per current specification.

Agreement:

For eMTC, on reception of a PDSCH ending in subframe n, the corresponding HARQ-ACK feedback on PUCCH is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, the ending time for DL physical resources forming a CSI reference resource set is advanced by Koffset as compared to current specification.

Agreement:

For eMTC, for an MPDCCH received in subframe n that triggers aperiodic SRS transmission, SRS is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, on receiving a timing advance command ending in subframe n, the corresponding adjustment of the uplink transmission timing by the received time advance shall be delayed by Koffset as compared to current specification.

Agreement:

For IoT NTN, support cell-specific Koffset configuration for use during initial access.

Agreement:

For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode.

Agreement:

UE-specific TA reporting is supported in IoT-NTN

- FFS: Detailed contents of report

Conclusion:

In IoT NTN the initialisation of generators for scrambling codes for UL channels and DM-RS shall use the subframe number of the UL channel or UL signal that is indicated by the Koffset-modified timing relationship.

NOTE: In the view of RAN1, this does not necessarily involve a specification change.

Conclusion:

For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.

In this contribution, we provide our views on the remaining aspects related to timing relationship enhancements for IoT-NTN.

# Aspects related to the reporting of UE-specific TA

In terms of triggering conditions, a natural option is for the UE to report a new value of UE-specific TA when it determines that the last reported value differs from the current value by a certain threshold. This mechanism ensures that the TA is tracked accurately by the base-station, in an on-demand basis. The network can use this value to update the UE-specific K_offset to maintain causality and avoid collisions in half-duplex scheduling.

Proposal 1: A UE reports the UE-specific TA to the network when the previously reported value differs from a current value by a pre-determined threshold.

In terms of reporting the TA, a solution that is attractive for NTN (since it avoids round-trip delays in case an uplink resource is to be requested) is to use an uplink semi-persistent scheduling (SPS) message (such as the one specified for NB-IoT to transmit a buffer status report (BSR)) on which the UE can transmit this TA report when required.

Proposal 2: The UE-specific TA is reported in an uplink semi-persistent scheduling (UL-SPS) message, such as the one used for reporting buffer status reports (BSRs) in NB-IoT.

# Definition of interrupted downlink subframes

Due to large, UE-specific TAs, DL subframes interrupted due to an UL transmission are UE-specific. Current specs assume a small TA (e.g., up to 1 ms), and do not incorporate a “TA term” in defining these subframes. However, as illustrated in Fig. 1, for NTN, the interrupted subframes need to be defined using a term that captures the impact of this very large TA. In the figure, we want X_DL to Y_DL to be the interrupted DL SFs—i.e., the subframes where the UE is not required to monitor another PDCCH, since it must get ready to transmit the PUSCH starting at Y_UL. Without the “-TA” term for Y_DL, the interrupted SFs would be from X_DL +TA to Y_DL +TA, while it would (problematically) be expected to monitor PDCCH from X_DL to Y_DL!

Figure 1: Definition of DL interrupted subframes in the presence of large NTN-specific TAs.

Examples (for NB-IoT) where this needs to be reflected include DL subframes before and after a NPUSCH or NPRACH transmission, DL subframes corresponding to half-duplex guard periods, etc.

Proposal 3: The definition of downlink interrupted subframes (e.g., those before and after a PUSCH, PRACH, PUCCH, and half-duplex guard periods), where a half-duplex UE is not expected to monitor PDCCH, is modified, in accordance with the large UE-specific TAs in NTN.

# Conclusion

In this contribution we presented our views on enhancements for NB-IoT/eMTC over NTN as it relates to timing relationship enhancements. We summarize our proposals below.

Proposal 1: A UE reports the UE-specific TA to the network when the previously reported value differs from a current value by a pre-determined threshold.

Proposal 2: The UE-specific TA is reported in an uplink semi-persistent scheduling (UL-SPS) message, such as the one used for reporting buffer status reports (BSRs) in NB-IoT.

Proposal 3: The definition of downlink interrupted subframes (e.g., those before and after a PUSCH, PRACH, PUCCH, and half-duplex guard periods), where a half-duplex UE is not expected to monitor PDCCH, is modified, in accordance with the large UE-specific TAs in NTN.