3GPP TSG RAN WG1 #106b-e		    R1-2109213

e-Meeting, October 11th – 19th, 2021

Source:	CATT

Title:	Channel access mechanism for up to 71GHz operation

Agenda Item:	8.2.6

Document for:	Discussion and Decision

- Introduction

In RAN#90-e meeting, a new Rel-17 WI on extending current NR operation to 71GHz was approved [1]. This WI extends NR operation up to 71GHz according to the outcome of the study item [2], with the following objectives on channel access mechanism for up to 71GHz operations.

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

In this document, we share our views on channel access mechanism for up to 71GHz operation, focusing on the channel access with LBT and No-LBT mode, LBT related procedure, multi-beam operation, receiver assisted channel access mechanism and short control signaling.

- Chanel access with LBT and No-LBT mode`

It has been agreed that gNB should indicate to the UE if this gNB-UE connection is operating in LBT mode (LBT or no-LBT) for regions where LBT is not mandated. For this purpose, it has been agreed that both cell-specific gNB indication and UE-specific gNB indication [3] are supported. One remaining issue is how to indicate LBT mode of gNB-UE connection to the UE before the RRC connection. There are 4 alternatives as follows.

- Alt.1: MIB

The LBT mode indication is transmitted by means of MIB. The UE can obtain LBT mode after reading the MIB. In the Rel-16 NR-U, there are 2 additional bits in DCI format 1_0 to indicate combinations of channel access type and CP extension for the share spectrum channel access. If the UE can obtain the LBT mode of the gNB-UE connection before monitoring the DCI format1_0 of SIB1, the size of DCI 1_0 can be determined according to the LBT mode of the gNB-UE connection. When the gNB-UE connection is operating in LBT mode, there are 2 additional bits in DCI 1_0 for indicating combinations of channel access type and CP extension. When the gNB-UE connection is operating in No-LBT mode, these 2 additional bits can be omitted just like in the licensed spectrum operation.

- Alt.2: Reserved bits in DCI format 1_0 scrambled by SI-RNTI

The LBT mode indication is transmitted by means of reserved bits in the DCI format 1_0 scrambled by SI-RNTI. The UE can obtain the LBT mode of gNB-UE connection after decoding the DCI format 1_0 scrambled by SI-RNTI. The additional 2 bits is always in the DCI 1_0 regardless of LBT mode. This scheme makes use of the reserved bits in DCI 1_0 without additional signaling overhead.

- Alt 3: SIB 1

The LBT mode indication is transmitted by means of SIB1. The UE can obtain the channel access mode of gNB-UE connection after reading SIB1. The additional 2 bits are always in the DCI 1_0 regardless of the LBT mode. A new parameter is required to be added in SIB1to indicate LBT mode information.

- Alt.4: Implicit indication via the size of DCI format 1_0 scrambled by SI-RNTI

The UE needs to make two assumptions of the DCI size during PDCCH decoding while reading SIB1, and perform two blind decoding. If the UE can decode DCI format 1_0 with additional 2 bits, the UE knows gNB-UE connection is operating in LBT mode. Otherwise, the gNB-UE connection is operating in No-LBT mode. The additional DCI size will introduce new issue on alignment of DCI size procedure.

For Alt.1 and Alt.4, the size of the DCI 1_0 for 60GHz NR-U can be determined by the LBT mode of gNB-UE connection. It is noted that Alt.4 requires the UE to perform two blind detections based on the assumption of different DCI sizes, which will have impact on the DCI size alignment. For Alt.2 and Alt.3, the size of DCI 1_0 for 60GHz NR-U is same as the size of DCI 1_0 for NR-U in Rel-16. Additionally, Alt.2 will not increase signaling overhead compared to Alt.3. It is recommended that the LBT mode indication can be transmitted by MIB or reserved bits in DCI format 1_0 scrambled by SI-RNTI before the RRC connection.

Proposal 1: During the initial access procedure before RRC_CONNETED state, the LBT mode indication can be transmitted by MIB or reserved bits in DCI format 1_0 scrambled by SI-RNTI.

- LBT related procedure

3.2 Cat 2 LBT

In Rel-16 NR-U [6], there are two types of LBT, Cat 4 LBT and Cat 2 LBT. For Cat 4 LBT, the number of sensing slots that are sensed to be idle before a transmission is random. And for Cat 2 LBT, the number of sensing slots that are sensed to be idle before a transmission is deterministic. The LBT mechanism in EN 302,567 is similar to Cat 4 LBT and this has been introduced for 60GHz NR-U. The remaining issue is whether the Cat 2 LBT is required to be introduced for 60GHz NR-U.

The advantage of Cat 2 LBT is that the duration of LBT is determined and the measurement procedure is simpler than Cat 4 LBT. A short time Cat 2 LBT should be introduced for 60GHz NR-U to ensure the channel is not occupied within the COT which has been defined as 5ms.

Proposal 2: Cat 2 LBT should be introduced for 60GHz NR-U.

3.4 CAPC and CWS

The CAPC (Channel Access Priority Class) have been specified in Rel-16 NR-U to align with IEEE 802.11 medium access protocol where the range of the contention window is traffic class dependent. However, there is no concept of traffic class for 60GHz band in the EN 302.567. Therefore, it is recommended that both CAPC and CWS procedure not to be introduced in 60 GHz NR-U. This will align with EN 302.567 and avoid increasing the unnecessary design complexity.

Proposal 3: There is no need to introduce CAPC and CWS.

3.5 LBT bandwidth

Agreement: (RAN1#106-e) - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements)

In the last meeting, it has reached an agreement that the gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) for single carrier transmission and performs multiple LBT that one for each channel bandwidth separately for multi-carrier transmission in intra-band CA. The remaining issue is that whether additional support of performing single LBT over all CCs. The EDT is related to LBT bandwidth, the larger the LBT bandwidth, the higher the EDT. If performing single LBT over all CCs is supported, the EDT should be computed based on the smallest channel bandwidth among the all CCs. Only then it will be fair to the gNB/UE which performs LBT for each channel bandwidth.

Proposal 4: If performing single LBT over all CCs is supported, the EDT should be computed based on the smallest channel bandwidth among the all CCs

3.6 ED threshold base on beamforming gain

In RAN1#104-e meeting, it has been agreed that the baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, and they also satisfy Pout≤Pmax. The definition of RF output power is result of Pt and Gt multiplication, where Pt represents the transmit power of transmitter and Gt is the beamforming gain of transmission beam. The baseline EDT implicitly assumes that the sensing beam at the receiver uses the same beamforming gain as transmission beam. However, the actual transmission beam might be different from the sensing beam. Thus, an adjustment value based on baseline EDT should be considered.

Proposal 5: Adjustment value should be considered for the baseline ED threshold.

If sensing beam with a wider beam width covers the transmission beam, the actual energy detection origination from transmission beam may be lower. To compensate for the beamforming gain difference between the transmission beam and sensing beam, the EDT should be reduced by an adjustment value. The adjustment value A could be computed as in following formula：

A=-10log10GmaxGmaxsensing

Where Gmax is the beamforming gain of transmission beam and the Gmaxsensing is the beamforming gain of sensing beam.

Proposal 6: For adjustment value on baseline EDT, at least beamforming gain difference between the transmission beam and sensing beam should be considered.

- Multi-beam operation

Regarding to multi-beam LBT operation, the LBT for multiples beams (TDM/SDM) sharing one COT needs to be defined. There are two alternatives for performing LBT at the start of COT.

- Alt 1: Perform a single LBT sensing with wide beam ‘cover’ all beams to be used within the COT.

- Alt 2: Perform independent per-beam LBT sensing for all beams to be used within the COT.

Alt 1 is suitable for the scenarios where the transmission beams to be used in the COT are spatially continuous, and the width of LBT beam should cover the detection of multiple transmission beams. There may be overlapping areas between adjacent transmission beams and a single LBT with wide beam can avoid unnecessary repeated measurement of overlapping areas. Therefore Alt 1 should be supported.

Alt 2 is suitable for the scenarios where the transmission beams in the COT are spatially dispersive and per-beam LBT sensing will be performed for each transmission beam. There would be some gap area between non-adjacent transmission beams. If Alt 1 is applied to this scenario, the LBT result also reflects the interference situation in the gap area, which may block the transmission. Therefore, Alt 2 should be support as well.

Proposal 7：Consider supporting both of single LBT sensing with wide beam and independent per-beam LBT sensing for all beams to be used within the COT at the start of the COT.

For “independent per-beam LBT sensing” in Alt 2, the following agreement has been reached agreement in RAN1#104b-e:

Agreement: (RAN1#104b-e) For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Both Alt A and Alt B should be supported for “independent per-beam LBT sensing”. For the node that doesn’t have the capability to simultaneously sense in different beams, performing per-beam LBT in TDM fashion (Alt A) requires further clarification.

For the Alt A-1, the node performs eCCA check on one beam and then performs eCCA check on the remaining beams one after another. For the Alt A-2, the node performs eCCA check on one beam and starts transmission on this beam, then performs eCCA check and transmits data on the remaining beams one after another. The eCCA checks on different beams are in “serial” method and maintaining one LBT process. The eCCA check failure on the previous beams will affect the eCCA check on the subsequent beam, and this “blocking issue” as shown in Figure 3 exists in both Alt A-1 and Alt A-2. Because if the channel is busy for an observation slot, the node will perform energy detection on additional deferral period until the channel is detected to be idle in the eCCA check procedure. If the channel on one beam is busy all the time, the device will perform CCA check on this beam for a long time according to the CCA check definition in EN 302.567 [4]. This will block the CCA check and transmission on the subsequence beams. Therefore, if supporting Alt A-1 or Alt A-2, the ‘blocking issue’ (failure on previous beam LBT causes subsequent beams unable to perform LBT) should be addressed.

Figure 1: “Blocking issue” in Alt A-1 and Alt A-2

Proposal 8: If supporting Alt A-1 or Alt A-2, the ‘blocking issue’ (failure of previous beam LBT causes subsequent beams unable to perform LBT) should be addressed.

For Alt A-3, the node performs eCCA check round robin between different beams as shown in Figure 3. Multiple LBT processes are maintained by one node in “parallel” method. Alt A-3 can avoid “blocking issue” and improve the efficiency of the multi-beam LBT. However, the node requires performing energy detection in each 5us observation slot in TDM fashion for different beams, thus it should be noted that the total number of beams supported for Alt A-3 is limited.

Figure 2: Performing eCCA check round robin between different beams

Proposal 9: Alt A-3 of which node performs eCCA round robin between different beams should be supported to increase the multi-beam LBT efficiency.

When LBT mode is used, time domain multiplexing of DL/UL transmissions in different beams within the COT could be supported without cross-interference to each other. After performing LBT sensing at the start of COT, one remaining issue is whether performing Cat 2 LBT before beam switch in the middle of the COT as shown in Figure 4.

Figure 3：One example for performing additional directional LBT within the COT

For the case where the beams to be transmitted within the COT are spatially continuous, when one of the beams is transmitting data, the adjacent beams are less likely to be interfered by other nodes. It’s not necessary to perform an additional LBT. But, if the beams to be transmitted within the COT are spatially dispersive, the beam direction that is not transmitting data may be occupied by other nodes. In this case, an additional LBT before beam switching is required for gNB. Performing Cat 2 LBT before beam switching within the COT could be supported. Therefore, we suggest that whether performing a Cat 2 LBT before beam switch within the COT could be decided by gNB.

Proposal 10: Performing Cat 2 LBT before beam switching within the COT could be supported, and it can be decided by gNB.

- Receiver assisted channel access mechanism

Agreement: (RAN1#106-e) For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e - Scheme 1: L1-RSSI based receiver assistance - Resource used for RSSI measurement - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS - FFS: any enhancement needed for ZP-CSI-RS for this purpose (eg., ZP-CSI-RS over all REs in BWP over one or more symbols). - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval - L1-RSSI is reported in an AP-CSI report - L1-RSSI trigger in UL grant - FFS if L1-RSSI trigger can also be carried in DL grant - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed - FFS: How to indicate the measurement beam for L1-RSSI - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc

In the RAN1#106-e meeting, channel sensing and reporting have been discussed, there are four schemes for further studied. For the scheme 1, the receiver assistance is based on L1-RSSI measurement and the reporting scheme is based on the AP-CSI report mechanism. There are still some remaining issues about scheme1:

- Resource used for RSSI measurement

If the RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS, the UE can only perform the RSSI measurement on the ZP-CSI-RS. The original introduction of ZP-CSI-RS is for cross link interference which is a time-slowly interference. However, the interference in unlicensed band is bustier. The RSSI measurement on ZP-CSI-RS can’t satisfy the level of interference accuracy. In additional, the RSSI measurement on ZP-CSI-RS will need extra time for DFT/FFT processing. If the RSSI measurement is based on the operating BW over indicated or specified number of symbols, the receiver will perform L1-RSSI over the channel bandwidth, which is similar with LBT. In our view, energy measurement on operating BW over specified number of symbols is preferred, which can reflect interference level at the receiver.

Proposal 11: For receiver assistance based on L1-RSSI measurement, Alt 2 (energy measurement on operating BW over specified number of symbols) is preferred.

- L1-RSSI trigger and L1-RSSI report

When L1-RSSI is carried by DL grant, the gNB can transmit PDSCH and L1-RSSI trigger at the same time. From the gNB perspective, the COT has been occupied, and the gNB can use this opportunity to transmit data to the receiver，which can improve the efficiency of transmission in 60GHz NR-U band. For the L1-RSSI report, two possible methods can be considered. One simple method is to define the value of the L1-RSSI measurement as L1-RSSI report. However, the drawback of this approach is lots of bits need to be reported by the receiver. Therefore this will increase the overhead of L1-RSSI report. Another method is to define the comparision between the value of RSSI measurement and Energy Detection threshold, similar to CCA check, as L1-RSSI report. In this method, the L1-RSSI report can be 1 bit, which causes less signaling overhead. Therefore, it is recommended that the L1-RSSI report can be 1 bit comparison outcome of the value of RSSI measurement and Energy Detection threshold.

Proposal 12: For receiver assistance based on L1-RSSI measurement, L1-RSSI can be triggered by DL grant.

Proposal 13: For receiver assistance based on L1-RSSI measurement, the L1-RSSI report can be 1 bit which is the outcome of the value of RSSI measurement and Energy Detection threshold.

- The indicated measurement beam for L1-RSSI,

Considering the characteristics of 71GHz operation and the high path loss, beam forming is used on both gNB and UE side to achieve robust measurement result and avoid the hidden/ exposed node issue. The receiver requires performing L1-RSSI on the specified direction. It is recommended to add the QCL source information for the L1-RSSI measurement.

Proposal 14: For receiver assistance based on L1-RSSI measurement, it is recommended to add the QCL source information for the L1-RSSI measurement.

- Short control signaling

Agreement: (RAN1#105-e) - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

It was agreed that the Contention Exempt Short Control Signaling (CESCS) rules can be applied to the transmission of msg1 for the 4 step PRACH and MsgA for the 2-step RACH for all supported SCS. The remaining issue is that whether the 10% over any 100ms interval restriction should be counted from the cell perspective or UE perspective. If the 10% over any 100ms interval restriction is counted from UE perspective, the total amount of UL signals which applies to Contention Exempt Short Control Signaling rule may be too large to interfere with other systems. Therefore, it is more reasonable to count the 10% limit from the perspective of the cell, which can control the amount of the Contention Exempt Short Control Signals in the cell.

Proposal 15: The 10% over any 100ms interval restriction should be applicable to all Contention Exempt Short Control Signals from cell perspective.

In additional, the total time resource of msg1/msgA in the cell may exceed 10% limit over 100ms interval. And other UL signals /channel, such as msg3, SRS, PUCCH, etc., can be also supported to transmission with CESCS rules. In this case, even if the total time resource for msg1/msgA does not exceed 10ms over 100ms interval, the total time of msg1/msgA and other UL signals/channel may exceed the limit. In order to meet 10ms limit over 100ms, it should be supported to apply the Contention Exempt Short Signaling rules to sub-set of PRACH slots for msg1/msg3.

Figure 4: An example for applying CESCS rules to sub-set of PRACH slots for msg1/msgA

One possible approach is to use SIB1/RRC signaling to configure which PRACH slots in a frame can be applied with CESCS rules. As shown in Figure 4, there are 3 PRACH slots for msg1/msgA transmission within one frame (10ms). A bitmap is used to configure sub-set of PRACH slots applied CESCS rules. The ‘1’ in the bitmap represents a certain PRACH slot applies CESCS rules, and the ‘0’ in the bitmap represents a certain PRACH slot does not apply CESCS rules. With the method mentioned above, the gNB can take control of the total time resource of msg1/msgA applied CESCS rules in the cell to meet 10ms limit over 100ms.

Proposal 16: In order to meet 10ms limit over 100ms, it should be supported to apply the Contention Exempt Short Signaling rules to sub-set of PRACH slots for msg1/msg3.

Furthermore, the short control signaling transmission has been defined in EN 302.567 [4] as follow:

Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames without sensing the channel for the presence of other signals.

Thus, the Short Control Signaling Transmission can be used for transmission management and control signaling without LBT. There is no specific definition of management and control signaling in EN 302.567. While a test method about Short Control Signaling Transmission was described in EN 302.567 as follow:

b) Apart from transmission of the frames for short control signalling (such as, for example, ACK/NACK signals, beacon frames, other time synchronization frames and frames for beamforming) no frame shall be initiated.

From above text, it can be inferred that the short control signaling in EN 302.567 includes ACK/NACK, beacon frame, time synchronization frame and frames. Therefore, in order to be consistent with EN 302.567, the PUCCH and PUSCH without user-plane data can be applied to Contention Exempt Short Signaling rules, when the restriction of less than 10% duty cycle within 100msec can be met.

Proposal 17: For UL signal, the Contention Exempt Short Control Signaling rules can be applied to the PUCCH and PUSCH without user-plane data.

When the Contention Exempt Short Control Signaling rule is applied to SS/PBCH block transmission, any signaling without user-plane data multiplexed with SS/PBCH block could be included in Short Control Signaling and can be transmitted without LBT. This does not violate the Contention Exempt Short Control Signaling rule, and will eliminate the uncertainty transmission caused by LBT in 60 GHz NR-U.

Proposal 18: The Contention Exempt Short Control Signaling can be applied to any signaling without user-plane data multiplexed with SS/PBCH block transmission.

For the 120 kHz SS/PBCH block transmission, assuming the periodicity of SS/PBCH block is 20 msec and the number of SSB beams is 64, the total duration of SS/PBCH block transmission is 11.4msec which exceeds 10ms limit over 100ms. In RAN1#104b-e, the agreement that Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH includes a condition that the 10% duty cycle rule restriction still applies. In order to meet 10ms limit over 100ms, we suggest that the Contention Exempt Short Signaling rules may be applied to sub-set of SSB beams for 120kHz SCS when the up to 64 SSBs transmission is supported.

Observation 1: When the periodicity of SS/PBCH block is 20 msec and the number of SSB beams is 64, the total duration of SSB transmission is more than 10ms over 100ms.

Proposal 19: In order to meet 10ms limit over 100ms, the Contention Exempt Short Signaling rules may be applied to sub-set of SSB beams for 120 kHz SCS when the up to 64 SSBs transmission is supported.

- Conclusions

In this contribution, we provide our view on channel access mechanism for up to 71GHz operation. The proposals and observation are summarized as follows:

Proposal 1: During the initial access procedure before RRC_CONNETED state, the LBT mode indication can be transmitted by MIB or reserved bits in DCI format 1_0 scrambled by SI-RNTI.

Proposal 2: Cat 2 LBT should be introduced for 60GHz NR-U.

Proposal 3: There is no need to introduce CAPC and CWS.

Proposal 4: If performing single LBT over all CCs is supported, the EDT should be computed based on the smallest channel bandwidth among the all CCs.

Proposal 5: Adjustment value should be considered for the baseline ED threshold.

Proposal 6: For adjustment value on baseline EDT, at least beamforming gain difference between the transmission beam and sensing beam should be considered.

Proposal 7：Consider supporting both of single LBT sensing with wide beam and independent per-beam LBT sensing for all beams to be used within the COT at the start of the COT.

Proposal 8: If supporting Alt A-1 or Alt A-2, the ‘blocking issue’ (failure of previous beam LBT causes subsequent beams unable to perform LBT) should be addressed.

Proposal 9: Alt A-3 of which node performs eCCA round robin between different beams should be supported to increase the multi-beam LBT efficiency.

Proposal 10: Performing Cat 2 LBT before beam switching within the COT could be supported, and it can be decided by gNB.

Proposal 11: For receiver assistance based on L1-RSSI measurement, Alt 2 (energy measurement on operating BW over specified number of symbols) is preferred.

Proposal 12: For receiver assistance based on L1-RSSI measurement, L1-RSSI can be triggered by DL grant.

Proposal 13: For receiver assistance based on L1-RSSI measurement, the L1-RSSI report can be 1 bit which is the outcome of the value of RSSI measurement and Energy Detection threshold.

Proposal 14: For receiver assistance based on L1-RSSI measurement, it is recommended to add the QCL source information for the L1-RSSI measurement.

Proposal 15: The 10% over any 100ms interval restriction should be applicable to all Contention Exempt Short Control Signals from cell perspective.

Proposal 16: In order to meet 10ms limit over 100ms, it should be supported to apply the Contention Exempt Short Signaling rules to sub-set of PRACH slots for msg1/msg3.

Proposal 17: For UL signal, the Contention Exempt Short Control Signaling rules can be applied to the PUCCH and PUSCH without user-plane data.

Proposal 18: The Contention Exempt Short Control Signaling can be applied to any signaling without user-plane data multiplexed with SS/PBCH block transmission.

Observation 1: When the periodicity of SS/PBCH block is 20 msec and the number of SSB beams is 64, the total duration of SSB transmission is more than 10ms over 100ms.

Proposal 19: In order to meet 10ms limit over 100ms, the Contention Exempt Short Signaling rules may be applied to sub-set of SSB beams for 120 kHz SCS when the up to 64 SSBs transmission is supported.

- Reference

RP-202925, Revised WID: Extending current NR operation to 71 GHz, CMCC, RAN#90-e meeting

3GPP TR 38.808 v1.0.0: Study on supporting NR from 52.6 GHz to 71 GHz

RAN1 Chair’s Notes, 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 10th – 27th, 2021

ETSI EN 302.567, Multiple-Gigabit/s radio equipment operating in the 60 GHz band, Harmonised Standard for access to radio spectrum V2.2.0, Dec., 2020.

3GPP TR 37.213 v16.2.0: Physical layer procedures for shared spectrum channel access (Release 16).