3GPP TSG RAN WG1 #106b-e		                                                      R1-2109216

e-Meeting, October 11th – 19th, 2021

Source:	CATT

Title:	CSI feedback enhancements

Agenda Item:	8.3.1.2

Document for:	Discussion and Decision

Introduction

At the RAN#106-e meeting, the following agreements for Rel-17 URLLC were reached [1].

Agreement For subband CQI reporting with more than 2 bits per subband - Support 4-bits CQI only Agreement For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration. - This feature is subject to UE capability - FFS: Whether wideband CQI report can be omitted Conclusion There is no consensus in RAN1 on the support of delta-MCS in Rel-17.

In this contribution, we discuss the remaining issue on 4-bits subband CQI reporting.

Discussion

At the RAN#106-e meeting, the 4-bits subband CQI was agreed to be supported with an FFS on whether wideband CQI can be omitted.

The 2-bits subband CQI report is a differential report, which is the difference between subband CQI and wideband CQI, hence the 2-bits subband CQI needs to be reported together with wideband CQI. However, the 4-bits subband CQI directly indicates the subband CQI information, which does not need to be reported together with wideband CQI. In addition, omitting the wideband CQI could save overhead. For example, if a bandwidth part is configured with 72 PRBs and the subband size is 8 PRBs, omitting the wideband CQI could save about 1/10 reporting overhead. In case wideband CQI information is needed at gNB side, UE can be configured with another CSI report to report wideband CQI separately.

Hence, it is proposed that the 4-bits subband CQI should be reported without wideband CQI.

Proposal 1: For 4-bits subband CQI reporting, the wideband CQI is omitted.

Conclusion

In this contribution, we discuss the remaining issue on 4-bits subband CQI reporting with the following proposal.

Proposal 1: For 4-bits subband CQI reporting, the wideband CQI is omitted.

References

- RAN1 Chair’s Notes 3GPP TSG RAN WG1 #106-e, e-Meeting, August 16th – 27th, 2021.