3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109264

e-Meeting, October 11th – 19th, 2021

Source: 	ZTE, Sanechips

Title: 	Discussion on NR Rel-17 IAB MT features

Agenda item:	8.17.10

Document for:	Discussion and Decision

Introduction

A preliminary RAN1 UE features lists for Rel-17 NR have been uploaded for RAN1#106bis-e discussions in [1], and the IAB MT feature list is also updated. This contribution provides some analysis and our further views for Rel-17 IAB MT features.

Analysis for IAB MT features

According to the WID of Rel-17 IAB and the discussion in previous RAN1 meetings, we agree that all the listed Rel-17 IAB-MT FGs are some enhancements of Rel-16 IAB MT, and are not necessarily mandatory even for device supporting IAB backhaul.

Observation 1: All the listed Rel-17 IAB-MT FGs are some enhancements of Rel-16 IAB MT, and are optionally supported with capability signalling.

And according to the following agreement in RAN1#106-e[2], as an enhancement feature, an IAB MT may report its capability regarding simultaneous transmission and reception on multiple serving cells in a frequency band to its IAB-donor-CU, which is missing in the table.

Agreement: - The IAB-donor-CU can be made aware of the IAB-MT’s capability regarding simultaneous transmission and reception on multiple serving cells in a frequency band, configured by the two parent nodes in intra-donor DC scenarios.

Proposal 1: Add the followings which relate simultaneous transmission and reception on multiple serving cells in a frequency band to IAB MT features table.

| ... | .... |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31-x | Simultaneous Rx Tx Intra-Band | Support [Simultaneous Rx Tx Intra-Band] reporting |  | Yes | N/A | Reporting of the IAB-MT’s capability regarding simultaneous transmission and reception on multiple serving cells in a frequency band is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Conclusion

In this contribution, observation and proposal on NR Rel-17 IAB features are introduced.

Observation 1: All the listed Rel-17 IAB-MT FGs are some enhancements of Rel-16 IAB MT, and are optionally supported with capability signalling.

Proposal 1: Add the followings which relate simultaneous transmission and reception on multiple serving cells in a frequency band to IAB MT features table.

| ... | ... |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31-x | Simultaneous Rx Tx Intra-Band | Support [Simultaneous Rx Tx Intra-Band] reporting |  | Yes | N/A | Reporting of the IAB-MT’s capability regarding simultaneous transmission and reception on multiple serving cells in a frequency band is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Reference

R1-2108679 Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.).

Chair’s note RAN1#106-e.