3GPP TSG RAN WG1 #106bis-e		R1-2109269

e-Meeting, October 11th – 19th, 2021

Agenda item:	5.1

Title:	Discussion on Rel-17 UL Tx switching

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN1#106-e meeting, the switching mechanism between two bands were agreed, leaving a few remaining issues: Tx state ambiguity, downgraded MIMO setting, DCI 0_1 for CA option 2, number of switching in 14 symbols. We would like to further discuss the Tx state ambiguity issue in this contribution.

- Tx state ambiguity

An agreement on this issue was reached in the last meeting:

Agreement Down select one of the following options in RAN1#106b-e: - Option 1: For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, then - 1Tx on carrier 1 and 1Tx on carrier 2 is assumed if the carrier is configured with uplinkTxSwitchingPeriodLocation as true. - the state of Tx chains supporting 2Tx transmission is assumed on the carrier if the carrier is configured with uplinkTxSwitchingPeriodLocation as false. - Option 2: For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, then the state of Tx chains supporting 2Tx transmission on the carrier is assumed. - Option 3: For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, then 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

The options above can be simplified (to some extend) as:

- Option 1: 1Tx / 2Tx by configuration (uplinkTxSwitchingPeriodLocation)

  - “True” for 1Tx

  - “False” for 2Tx

- Option 2: always 2Tx when ambiguous

- Option 3: always 1Tx when ambiguous

We would like to share a way to look at it: when 1p + 0p is configured for current slot, how much is the likelihood that it becomes 2p + 0p for the next slot, and how much is the likelihood to maintain 1p+1p? If it is very likely to get 2p + 0p transmission immediately after, then going option 2 is better. And vice versa.

CMCC’s configuration is taken as an example in Fig 1 below.

Fig 1: CMCC’s deployment illustration on frame structure and SRS configuration

Notes:

- SRS is configured to special slots, so that PUSCH can take full advantage of UL slots

- The SRS period is usually configured as 20 or 40ms, i.e., not for every frame

Observations:

- When SRS is to be sent, it is always followed by UL slots where 2 ports of PUSCH is possible and preferred.

- Option 2 is better: In such configuration, the best Tx switching strategy is to always have 2Tx on each carrier. That is, option 2 (always 2Tx when ambiguous).

- When Option 3 is better: Inspired by Fig 1, Option 3 is more favorable if, SRS or PUCCH occasions on different carriers are aligned or consecutive, as illustrated in Fig 2 below.

Fig 2: A possible frame structure and SRS configuration that favors Option 3

Conclusion: it is up to the concerned situation, whether 2Tx+0Tx is better or 1Tx+1Tx is better. Therefore the “signaling” solution looks like the best one but it is already ruled out for Rel-17. Then the second-best solution is to exam the actual Tx switching application scenarios on major operators’ network and take decision accordingly. If for most of the time/scenarios, 2Tx is the righteous choice, then assuming 2Tx will be most efficient.

Proposal 1: Exam the actual Tx switching application scenarios on major operators’ network, to decide the solution for Tx state ambiguity issue.

Proposal 2: Prefer Option 2 as the solution, considering the scenario for CMCC.

- Conclusions

Proposal 1: Exam the actual Tx switching application scenarios on major operators’ network, to decide the solution for Tx state ambiguity issue.

Proposal 2: Prefer Option 2 as the solution, considering the scenario for CMCC.

- References

- RP‑202799, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1), RAN#90e.

- R1-2108643, Summary of email discussion on Rel-17 uplink Tx switching, RAN1#106-e