3GPP TSG RAN WG1 #106bis-e		R1-2109324

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.4.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on NTN uplink time synchronization

Document for:	Discussion

# Introduction

In RAN1#106e, the agreements related to uplink synchronization are as following:

In this contribution, we discuss the issues related to uplink time synchronization.

# Discussion

## Common TA granularity

The formula to determine TA in RRC connected state is agreed in previous meeting, and the granularity of the common timing offset NTA,common is still FFS.

There is discussion in RAN1#106e on  the granularity, and the FL’s proposal is as following:

If feeder link timing drift is compensated by UE using Common TA parameters, the granularity of Common TA is set to be:

  - Option 1:  The same as the granularity of NTA, i.e., 16∙642μ∙Tc.

  - Option 2 : 642μ∙Tc

μ is the highest allowed numerology for the given Frequency Range

From our perspective, the indicated NTA,common is corresponding to the RTT between the satellite and the reference point. The reference point is where the DL and UL frame are aligned. The selection of TA command unit in legacy release is to address UE moving within the cell coverage area. The NTA,commonindication is due to moving of satellite. Although RTT for GEO and LEO/MEO are different, we prefer same granularity for different scenarios as the requirement on TA error are same. And the granularity for common TA is better to be comparable with the granularity of legacy TA command, as both common TA and legacy TA command are combined together to determine the final TA value. So we prefer same granularity as legacy TA command for both GEO and LEO/MEO.

Proposal 1: Support the granularity of NTA,common to be same as granularity of TA command for both GEO and LEO/MEO scenario: 16∙642μ∙Tc.

## TA update in RRC connected state

In RAN1#106e, the FL’s recommendation is as following:

FL Recommendation:

For next RAN1 meeting: Companies are encouraged to provide more inputs on:

- The update/maintenance of  NTA,UE-specific and NTA,common

- And provide details on the combination of open and closed loop TA control

From our perspective, the issue related to contradiction between close loop and open loop TA should be resolved. For UE specific TA estimation, the UE may estimate NTA,UE-specific-1 based on satellite’s position  and UE’s positions in a first time instance, and as the satellite is moving, the UE may estimate an updated NTA,UE-specific-2 based on satellite’s position  and UE’s positions in a later second time instance. Between the first and the second time instance, uplink signal may be transmitted from UE to gNB to determine TAC in MAC CE. In this case, the impact of satellite moving are counted twice by both the UE specific TA calculation and the TAC determination, so the total TA value by adding these two values is not correct.

In legacy releases, there is close loop TA control by MAC CE, and there is also open loop TA control by SSB or DL signal reception. Actually there is also contradiction between close-loop and open-loop TA control, and there is a sentence in TS 38.213 to resolve the contradiction: “If the received downlink timing changes and is not compensated or only partly compensated by the uplink timing adjustment without timing advance command as described in TS 38.133, the UE changes N_TA accordingly”. Whether there is contradiction between open loop and close loop TA control is determined by the time domain order of the nearest SSB reception when applying the MAC CE and uplink signal reception associated with MAC CE. If the SSB reception is after the uplink signal reception, then the received downlink timing change is not fully compensated, otherwise, it is compensated. As the cell coverage area in TN is relatively small, it is not necessary to differentiate the time instance between transmission and reception, so at gNB side, it can be determined by the time domain order of SSB transmission and uplink signal reception, and at UE side, it can be determined by time domain order of SSB reception and uplink signal transmission. However, there is large propagation delay in NTN, so it is necessary to differentiate the time instance for DL/UL transmission and reception. Additionally, open loop TA in NTN is performed autonomously by UE and in NR SSB transmission/reception is known by both gNB and UE, so there should be some mechanisms to maintain common understanding between gNB and UE.

There can be several alternatives to resolve the contradiction issue:

- Alt 1: Predefine a rule to determine the time instance to determine TAC in MAC CE;

  - e.g. Suppose the MAC CE is received at T_mac, and a predefined duration is T_duration, then the time instance can be determined to be the latest UL reception time instance at satellite side before T_mac – T_duration. T_duration is used to address the propagation delay between gNB and UE, and it can be configured or same as K-offset for DL reception and UL transmission timing relationship.

  - At UE side, the UL reception time instance can be determined based on the UE’s position and satellite’s position.

- Alt 2: Explicit indicate a time instance to determine TAC in MAC CE;

  - The explicit indication can be a time domain info. And the time domain info can be carried together with the MAC CE;

  - The explicit indication can be also a DL signaling. The DL signaling indicates UE to determine a time instance based on reception of the DL signaling.

With either Alt 1 or Alt 2, the time instance to determine TAC in MAC CE can be determined, so that UE can calculate the NTA,UE-specific for the determined time instance, and apply TAC to the determined NTA,UE-specific, so there is no contradiction. Meanwhile, common understanding between gNB and UE is also achieved.

Proposal 2: Study mechanisms to resolve the contradiction between open loop and close loop TA control. The mechanism can be to define a time instance to determine TAC in the MAC CE.

## Common TA draft rate

There are also discussions on whether a drift rate of common timing offset indication is necessary. The common timing offset is corresponding to RTT between satellite and the reference point. As the satellite moves, the common timing offset is also changed. If there is no drift rate indication, the change of common timing offset can only be maintained by frequent common timing offset indications, which imposes large signaling overhead. Therefore, we propose to support indication of the drift rate of common timing offset.

In addition, the drift rate as a linear function of time can have significant variations, hence demanding indication of drift rate variations. If the drift rate variation is described by a constant, the result is TA indication as a quadratic function of time, which provides further degree of freedom and, as a result, a further reduction of the signaling overhead.

Proposal 3: Support indication of common timing offset drift rate and indication of drift rate variation.

## TA margin indication

There are four options to avoid UE over pre-compensation of TA during PRACH transmission. For option 4 by UE implementation, we don’t think this issue can be handled. For option 1 by delaying PRACH transmission, we think this may impact the tolerance of PRACH compared to regular case. We prefer TA margin is indicated, and we prefer option 3 for signaling reduction.

Proposal 4: Support TA margin to be included in common TA to avoid over pre-compensation.

# Conclusion

In this contribution, we discussed the issues related to TA in NTN network, and our proposals are as following:

Proposal 1: Support the granularity of NTA,common to be same as granularity of TA command for both GEO and LEO/MEO scenario: 16∙642μ∙Tc.

Proposal 2: Study mechanisms to resolve the contradiction between open loop and close loop TA control. The mechanism can be to define a time instance to determine TAC in the MAC CE.

Proposal 3: Support indication of common timing offset drift rate and indication of drift rate variation.

Proposal 4: Support TA margin to be included in common TA to avoid over pre-compensation.

# References

RP-201256, RANP#88e, “Solutions for NR to support non-terrestrial networks (NTN)”

TR 38.821 v16.0.0

RAN1#106e chairman notes

R1-2108587, Thales, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”