3GPP TSG RAN WG1 #106-bis-e	R1- 2109364

e-Meeting, October 11th – 19th, 2021

Agenda item:		8.5.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 		Views on enhancing UL AoA

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#90 a new WI was approved and updated at RAN#91 on enhancements in Rel-17 to positioning [1]. This contribution discusses our views on the enhancements related to UL AoA. Our companion contributions discuss our others views [2-6]. The objective from the WID is to

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

# Discussion

## UL AoA Measurement Enhancement

UL RX beams are beamformed with specific angles in a gNB and RX beamforming information itself is useful to estimate AoA. Rel-16 NR positioning has introduced a framework for angle-based positioning methods, and we consider AoA measurement improvements in this WI phase.

As a first consideration, angle-based method has high performance dependency on LoS detection. If the channel is not LoS for the angle estimation, it requires more complex steps to collect information on wave reflection circumstances. Therefore, we prefer to focus first on AoA estimation under LoS channel status.

If the channel link is LoS, one straightforward method for AoA estimation can be designed with the best RX beam selection, its estimation is derived from accurate UL RSRP measurements per beam at the gNB/TRP. This method would be practical and implementable, but it still has a few problems. Firstly, although the LoS path is guaranteed, the AoA is not always the same across all receiving antennas since the TRP cannot assume a single planar wave for the overall receiving antennas depending on the distance of the UE. Secondly, UL signal input angle from a target UE is not always located at the peak angle of a beam lobe, and it can be an in-between angle of two discrete beam directions. Moreover, a beam codebook in practical gNB implementation has non-uniform beam width and angle grid, so some beams may have high angle accuracy while the accuracy can be degraded in other beams. As a solution, a gNB must be able to estimate such an in-between angle with reasonable accuracy regardless of beam directions. The thrid problem occurs regarding multiple UEs AoA estimation. A gNB is designed to serve always multiple UEs, so AoA estimations needs to be served for multiple UE targets. These issues set some restrictions to AoA estimation algorithms in practical gNB implementation.

Observation 1: LoS path estimation is required for accurate AoA estimation in most cases.

Observation 2: UL signal input angle from a target UE is not always located at the peak angle of a beam lobe, and it can be an in-between angle of two discrete beam directions.

Observation 3: A gNB serves multiple UEs, so AoA estimation also needs to be designed for multiple UE positioning targets.

## AoA measurement for Near-field

In this section, we provide our views the necessity of UL-AoA measurement association with ARP information.

Agreement:

Further study and conclude whether association of UL-AOA, UL-TDOA, Multi-RTT measurements with ARP (Antenna Reference Point) information is supported at RAN1#106bis-e.

Mostly, 3GPP RAN1 has discussed AoA measurement improvements based on the far-field assumption. In this assumption, the propagated signal from a UE can be seen as a planar wave for overall receiving antennas or antenna elements, so the TRP estimates an single incident angle using the antenna elements. However, depending on the distance between the UE and the TRP, a single planar wave assumption across all antennas could be invalid. Simply put, if the UE is close to the TRP, the TRP cannot assume the same incident angle θ for all M antennas of Figure 1. In particular, this is relevant for IIoT use cases like the InF model where the UE may be close to the TRP. In addition, for 3D based positioning it was shown by multiple sources in the SI that variable gNB heights are needed and this may involve some TRPs being even closer to the UEs near the ground.

Figure 1. Illustrative example of AoA measurements

More specifically, in Figure 1, if the distance between the UE and TRP is large enough compared to d, but not large enough compared to (M-1)d, a single planar wave assumption would only be possible for a part of the M reception antennas. For accurate AoA measurements for a target UE, the TRP needs to separate reception antennas or antenna elements into multiple antenna groups where those antennas within a group could assume a single planar wave for a propagated signal from the UE. It is necessary to identify the threshold distance to assume far-field or near-field to figure out in which case the far-field assumption is not valid for all antennas.There is a well-known threshold distance called Frounhofer distance to discriminate if it is far-field or near-field, which is denoted by Rth=2D2λ, where λ is the wavelength of the radio wave and D is the dimension of the radiator. This threshold has also been used in 3GPP such as TR 38.810 [7] to study test methods. Based on the definition of section 5.2.1.2 of [7], we can consider D a diameter of the sphere that encloses the radiating parts of a device. In case that the carrier frequency is 28 GHz and D is 30 cm, the threshold distance is Rth=16.8 meters, which is a given example in Table 5.2.1.2-1 of TR 38.810[7]. This table is focusing on the mobile device size but the TRP size is normally bigger than the mobile device, so Rth would not be small value.

Figure 2. Illustrative example of different value of D (D1, D3) of the antennas of a TRP

Observation 4: The far-field assumption for a planar wave across all antennas is not always valid according to the distance between UE and TRP.

Thus, for the received signal from a target UE, the number of reception antennas or antenna elements may be different to assume a single planar wave, so the TRP needs to group one or more receiving antenna elements that may assume to have the same incident angle. In Figure 2, for a UE located in between 2D12λ and 2D32λ, the TRP may need separation of the multiple antennas into 4 antenna groups as shown in this figure so that the planar wave assumption can be respectively valid for each group of antenna elements, and then the TRP may estimate each AoA measurement by using each group of antennas assuming a single incident angle. Then the gNB needs to report the AoA measurement for each group of antennas to the LMF. For this purpose, when the gNB reports AoA measurement, it also needs to report coordinate information of each group of antennas associated with the AoA measurement.

One of the possible ways in the current system is to use ARP information. In the current spec, the gNB can provide geometric location information of the antennas in detail to the LMF and also LMF provides the UE with it using ReferencePoint-r16 and RelativeLocation-r16. If the gNB reports antenna location information for every reporting instance, it might be an unnecessary overhead since antenna location information is static, so it might be reasonable that the network assigns an ARP ID for a location of a group/all of the antenna elements and then the gNB can report AoA measurement and its associated ARP ID.

Proposal 1: Support the gNB to report an AoA measurement and its associated ARP ID, where a group of antenna elements of the same ARP ID is assumed to see the same incident angle for a planar wave.

As an additional issue, technical discussion may be needed on how to determine the group of antenna elements from the gNB that can see a planar wave for a target UE of which distance is unknown to the gNB/LMF.

## SRS RSRP of the first path

RAN1 reached the following agreements in the previous meeting:

Agreement:

Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

- Note: The use of SRS for MIMO resource is transparent to the UE

- FFS: Reporting of gNB Rx-Tx

Agreement:

Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

In the previous meeting, for a specific positioning SRS resource, it was agreed to support UL-RTOA, gNB Rx-Tx time difference, AoA measurement(s) reporting for the first path, but whether or not to report RSRP of the first path is a remaining issue. Reporting of the first path RSRP along with the timing measurements might be helpful for positioning accuracy improvement, but before introducing reporting it we first may need to conclude what the first path RSRP is. This issue has also been discussed in AI 8.5.3 and the definition should be aligned. In our understanding, the first path detection is depending on the TRP implementation, so it might be reasonable that standard leave the definition of the first path more like an implementation point of view as follows: the first path to define the RSRP of the first path is the channel tap where TRP measures ToA for reporting of the RSRP of the first path. We have the following proposal to define the SRS RSRP of the first path.

Proposal 2: The measured first-path SRS RSRP corresponds to the power of the channel impulse response, at the first path delay, over which the SRS is received.

- Note: the first path delay is independent of sampling grid.

- Note: the first path delay is the channel tap where gNB measures ToA for reporting of the RSRP of the first path

- FFS: window/time-duration around the first path (e.g., size of 1 Ts) to calculate power

Regarding this issue, we have an aligned view in our companion contribution [3].

## UL beam based AoA estimation algorithm

As we discussed above, the AoA estimation algorithms are related with the additional path measurements. We provide further details of AoA estimation algorithms and properties in this section. In addition, we suggest a way for the AoA measurement improvement based on UL beam based AoA estimation.

With these observations, we consider practical AoA estimation approaches. Since the gNB RX beam angle can be interpretated as RX signal input angle, it can be reported as the AoA measurement. The RX beam selection can be made based on UL RSRP measurement, which is an important metric for the AoA estimation. The first gNB requirement of AoA measurement is accurate UL RSRP measurement on the received SRS-P signal, so we need to look into how to obtain accurate UL-RSRP measurements. In the presence of Tx and Rx beamforming in a target UE and a gNB respectively, UL RSRP can be measurable per an antenna port, and the received SRS-P signal for Tx beam l, assuming for a single layer transmission, can be written as

H=i,jhijarθijatHθij #1

In particular, if the Tx adopts the DFT codebook, we have wtl=1, e-j 2πlOtNt , …, e-j 2πNt-1lOtNt Twhere Ot is a oversampling factor and Nt is a number of TX antenna port, also  wr can be defined as well. The propagation channel especially in mm-wave communications tends to be sparse in the angular domain, i.e., to be characterized by a relatively small number of rays, typically concentrated around the direction. And  arθ and at(θ) are the steering vectors, respectively for the receiver and the transmitter sides, depending on the antenna array geometry. For example, for a ULA, the steering vector is aθ=1, e-j 2πdλcosθ , …, e-j 2πdλN-1cosθ T where d is the antenna array spacing, λ is the signal wavelength, and N is the number of antennas of the array (i.e., Nt for the transmitter or Nr for the receiver).

Then, the UL RSRP equation (4) is obtained as

rθi,l=σPRS 2 iE hij2    wrHlarθiatHθiwt(l)2 #3

We note that the UL RSRP value is expressed as a function of θi and w beamforming codebook selection. The steering angle is not at exact LoS direction (θ≠θLOS), then multiple beam interpolation can induce the in-between target angle. For example, of two beam RSRP measurement case, the θ estimation is expressed as

θ=r1'r1'+r2'acosθ1+r2'r1'+r2'acosθ2.

The accuracy is guaranteed when TX and RX steering vectors are aligned toward a direction, and then selection of a beamforming codebook affects accuracy of measured UL-RSRP values, therefore the accuracy of UL-RSRP measurements are also important to estimate accurate AoA measurement.

Observation 5: UL RSRP accuracy function is shown as a function of beamforming codebook selection and steering angle.

- TX and RX steering vectors must be aligned toward a steering direction.

- Configuration of a beamforming codebook affects accuracy of measured UL-RSRP values.

A gNB conducts multiple tasks for data communications and measurements including positioning services, and it is designed to make unified and fair behaviours to serve multiple users with multiple tasks. In this aspect, a beam interpolation method will be useful as a practical solution, although it may have some accuracy limitation due to interpolation resolution. For an enhanced solution, LMF may request to improve precise AoA measurements to address some positioning use cases. It could be possible that beam angle resolution factors such as beam resolution or oversampling rate can be controlled by LMF request. The proposed operation is presented at the high level in Figure 3.

In order to set up discrete beam based angle estimation, LMF collects analog beam codebook from gNBs. Since practical gNB has some options of beam design ( i.e. uniform beam, DFT beam, narrow/wide angle beam etc), LMF stores beam information in pre-stage. LMF can let a gNB choose an RX beam and collect RSRP with the RX beam information, or in the other way, LMF can request a gNB to utilize a specific beam set up. LMF can send a request signals about the number of antennas and beam oversampling factor, to be used by each gNB for generating positioning beams. Also the total number of beams measured may be decided either by the gNB/LMF for interpolation.

Proposal 3: Study beam resolution control/recommendation by LMF for more accurate AoA measurement by beam interporlation based AoA estimation method based on RSRP measurements.
   - Request to gNB : beam resolution recommendation, number of RSRP measurement

Figure 3. Operation of beam angle interpolation and resolution control/recommendation

# Conclusion

We made the following observations and proposals in this paper:

Observation 1: LoS path estimation is required for accurate AoA estimation in most cases.

Observation 2: UL signal input angle from a target UE is not always located at the peak angle of a beam lobe, and it can be an in-between angle of two discrete beam directions.

Observation 3: A gNB serves multiple UEs, so AoA estimation also needs to be designed for multiple UE positioning targets.

Observation 4: The far-field assumption for a planar wave across all antennas is not always valid according to the distance between UE and TRP.

Proposal 1: Support the gNB to report an AoA measurement and its associated ARP ID, where a group of antenna elements of the same ARP ID is assumed to see the same incident angle for a planar wave.

Proposal 2: The measured first-path SRS RSRP corresponds to the power of the channel impulse response, at the first path delay, over which the SRS is received.

- Note: the first path delay is independent of sampling grid.

- Note: the first path delay is the channel tap where gNB measures ToA for reporting of the RSRP of the first path

- FFS: window/time-duration around the first path (e.g., size of 1 Ts) to calculate power

Observation 5: UL RSRP accuracy function is shown as a function of beamforming codebook selection and steering angle.

- TX and RX steering vectors must be aligned toward a steering direction.

- Configuration of a beamforming codebook affects accuracy of measured UL-RSRP values.

Proposal 3: Study beam resolution control/recommendation by LMF for more accurate AoA measurement by beam interporlation based AoA estimation method based on RSRP measurements.

- Request to gNB : beam resolution recommendation, number of RSRP measurement

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

[2] R1-2109363, Views on on mitigating UE and gNB Rx/Tx timing errors, Nokia, Nokia Shanghai Bell.

[3] R1-2109365, Views on enhancing DL AoD, Nokia, Nokia Shanghai Bell.

[4] R1-2109366, Views on PHY Latency Reductions, Nokia, Nokia Shanghai Bell.

[5] R1-2109367, Views on LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

[6] R1-2109368, Additional views on Inactive Mode Positioning and on-demand PRS, Nokia, Nokia Shanghai Bell.

[7] 3GPP TR 38.810, Study on test methods, Sept. 2020.