3GPP TSG RAN WG1 #106-bis-e 																R1-2109445

e-Meeting, 11 – 19 October, 2021

Agenda item:		8.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Beam Management Aspects

Document for:		Discussion and Decision

# Introduction

The revised work item on supporting NR from 52.6 GHz to 71 GHz [1] was approved at RAN#90-e. Before that 3GPP carried out a study on required changes to NR using existing DL/UL waveform to support operation between 52.6 GHz and 71GHz [2]. This contribution deals with the following objectives of the WID:

- Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

  - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

  - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

TR [2] captures Physical layer impacts to beam management and CSI in the following way:

For the study item, it is recommended to study potential enhancements or alternatives to the scheduling request mechanism to reduce scheduling latency due to beam sweeping, if needed.

For the study item, it is recommended to consider at least the following aspects in system operations with beams:

-	study of BFR mechanism enhancements, if supported,

-	e.g., the use of aperiodic CSI-RS for BFR, increased number of RSs for monitoring/candidates and efficient utilization of the increased number of RSs, enhanced reliability to cope with narrower beamwidth,

-	study of UE capabilities on beam switch timing in beam management procedure,

-	study of enhancements for beam management and corresponding RS(s) in DL and UL are needed further considering at least the following aspects, if supported:

-	beam switching time, beam alignment delay (including initial access), LBT failure, and potential coverage loss (if large SCS is supported),

-	study of beam switching gap handling for signals/channels (e.g. CSI-RS, PDSCH, SRS, PUSCH) for higher subcarriers spacing, if supported.

It is recommended to further investigate potential enhancements, if needed, to beam management at least considering one or more of potentially narrower beamwidths, CP duration, multiple beam indications for multi-PUSCH/PDSCH scheduling, triggering of reference signals for beam management, enhancements to beam management for random access procedure, intra- and/or inter-cell mobility, and adaptation to LBT failures. Minimum requirement on beam switching delay in > 52.6 GHz spectrum should be further studied by RAN4 when specification is further developed.

It is recommended to investigate whether or not enhancements to CSI processing unit (CPU) availability check is needed when the UE is required to process CSI reports corresponding to multiple numerologies across active BWPs in different component carriers.

# Discussion

## Timing related

RAN1#106-e made the following agreements related to maxNumberRxTxBeamSwitchDL [3]:

Agreement: For maxNumberRxTxBeamSwitchDL, - Support at least 2 and 4 as candidate values for 480 kHz - FFS: 7 - Support at least 2 as a candidate value for 960 kHz - FFS: Support for additional candidate value(s) including 4 Agreement: For maxNumberRxTxBeamSwitchDL, - For 480 kHz, support 7 as a candidate value for 480 kHz in addition to the agreed candidate values 2 and 4 - For 960 kHz, support one of the following alternatives - Alt-1: Support 1, 4 and [7] as candidate values for 960 kHz in addition to the agreed candidate values 2 - Alt-2: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2 - No additional candidate values are supported

maxNumberRxTxBeamSwitchDL defines the number of Tx and Rx beam changes UE can perform on the band within a slot. UE shall report one value per each subcarrier spacing supported by the UE. Available values for 120 kHz and 240 kHz SCSs are as follows:

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

…

scs-120kHz                                  ENUMERATED {n4, n7, n14}                     OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                     OPTIONAL

}

For 480 kHz it was agreed to support values 2, 4 and 7. For 960 kHz only value 2 was agreed so far. Considering the UE’s reception of PDCCH and PDSCH within a slot in case different TCI states are used for those the UE would need be to able to perform at least two RX beam changes within a slot (first to receive PDCCH and second to receive PDSCH). Thus, the supported values should be ≥2. Correspondingly, we propose Alt-2, i.e. to support value 4 in addition to the agreed value 2.

Proposal 1: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2.

Regarding aperiodic CSI-RS triggering with PDCCH having different subcarrier spacing the following agreements were made in RAN1#106-e [3]:

Agreement: For the threshold values 48 or 48+ d∙2μCSIRS/2μPDCCH mentioned in Clauses 5.2.1.5.1 and 5.2.1.5.1a of 38.214, scale 48 to 4*48 for 480 kHz and 8*48 for 960 kHz. Agreement: - For additional beam switching time delay d of 120 kHz, support one of the following alternatives - Alt-1: 14 symbols - Alt-2: 28 symbols - FFS: value for 480 kHz

3GPP TS 38.214 defines the d values for the case of µPDCCH < µCSIRS as follows:

Table 5.2.1.5.1a-1: Additional beam switching timing delay d

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |

As already the main delay component is scaled with 4 or 8 for 480 and 960 kHz, respectively, compared to 120 kHz, the additional beam switching timing delay d does not need to scale similarly. We propose to select 14 symbols for the 120 kHz and 28 symbols for the 480 kHz.

Proposal 2: Additional beam switching time delay d of 120 kHz is 14 symbols.

Proposal 3: Additional beam switching time delay d of 480 kHz is 28 symbols.

## On the beam indication in multi-PDSCH transmission

RAN1#106-e made the following agreement and working assumption related to QCL assumption/TCI state indication for multi-PDSCH transmission [3]:

Agreement: For the single TRP case, For multi-PDSCHs scheduled by a single DCI with a single DCI field ‘Transmission Configuration Indication’ that indicates a single TCI state (if the DCI field is present), - Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL - Case 1-1: tci-PresentInDCI enabled - Single QCL assumption based on the indicated codepoint of the single DCI field ‘Transmission Configuration Indication’ is applied for all scheduled PDSCHs - Case 1-2: tci-PresentInDCI not present - Single QCL assumption of the single scheduling DCI scheduled multi-PDSCHs is applied for all scheduled PDSCHs - Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - Down select one of the following alternatives - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs - FFS: Details of single QCL assumption - Alt 2: multiple QCL assumptions are applied - FFS: Details of multiple QCL assumptions - FFS: When some of PDSCHs are collided with semi-static UL symbols and then skipped - FFS: The multi-TRP case Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states - FFS: Details of multiple TCI state association with multiple PDSCHs

When scheduling duration of multi-slot scheduling exceeds timeDurationForQCL, as shown in the figure 1, slots scheduled by single DCI can have different assumption for TCI states. ):

Figure 1 Impact of UE’s capability timeDurationForQCL to QCL assumptions for multi-PDSCH.

As can be noted from the figure QCL assumption for the first four PDSCHs would be different to the last four PDSCHs. For the first four PDSCHs UE would apply so called “default PDSCH QCL assumption” and for the rest of the PDSCHs either the QCL assumption according to the TCI state of the scheduling PDCCH or the TCI state provided in the DCI. However, it’s considered that the same transmit beam (and thus the same QCL assumption) would be used for all the PDSCHs, at least in single TRP scenario. Of course, the gNB would be able to guarantee by the configuration that the same QCL assumption is in practice used for all the PDSCHs, e.g. by not providing TCI state in DCI and having only the same active TCI state shared by configured CORESET(s). On the other hand, in general while SSB is QCL-TypeD source for CORESET#0 and CSI-RS may be used as QCL-TypeD source for CORESETs other than zero. Thus, allowing flexible configuration of CORESETs and QCL sources it may be beneficial to consider specification work to enable single QCL assumption for all the PDSCHs in multi-PDSCH scheduling when some of the PDSCHs are having lower scheduling offset than timeDurationForQCL.

It’s also to be noted that 3GPP TS 38.214, section 5.1.5, says that in case of scheduling offset is equal to or greater than timeDurationForQCL and multi-slot PDSCH is configured the UE can assume single TCI state for across the slots [3GPP TS 38.214, 5.1.5]:

“The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH.”

Thus, considering

- Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL

  - Down select one of the following alternatives

    - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs

      - FFS: Details of single QCL assumption

    - Alt 2: multiple QCL assumptions are applied

      - FFS: Details of multiple QCL assumptions

we prefer Alt 1. The single QCL assumption for all the PDSCHs could be the one used for the first PDSCH.

Proposal 4: Support single QCL assumption for the multi-PDSCH transmission in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL.

Proposal 5: QCL assumption for the all the PDSCHs is the same as the one used for the first PDSCH in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL. I.e. the same default QCL assumption for all scheduled PDSCHs as for the for the first PDSCH.

Regarding the multi-TRP case and FFS point:

- FFS: The multi-TRP case

the same principle as with a single-TRP should be taken in the single DCI multi-TRP case when scheduling multiple PDSCHs/PUSCHs. In other words, single QCL assumption/spatial relation assumption should be provided per TRP when scheduling multiple PDSCHs/PUSCHs using single DCI in multi-TRP scenario.

Proposal 6: In multi-TRP case, single QCL assumption is applied for the multiple scheduled PDSCHs per TRP.

Consequently, we propose to confirm the working assumption regarding the multi-PDSCH scheduling for multi-TRPs.

Proposal 7: Confirm working assumption:

Working assumption:

For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs

- The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism

- The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism

- Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states

- FFS: Details of multiple TCI state association with multiple PDSCHs

## LBT impact on Beam Management

### TRS

In certain unlicensed scenario regulated maximum allowed EIRP limits (e.g. CEPT scenarios c1 and c2 with 40 dBm max EIRP) are such that larger arrays compared to FR2 may not be needed. Thus, beam dimensioning based on FR2 would be enough, e.g. in terms of maximum number of supported SSBs, and beam management procedures developed in Rel. 15 and Rel. 16 are expected to provide a good baseline for unlicensed operation above 52.6 GHz.

On the other hand, channel access mechanism(s) may have impact on the beam management. Depending on the co-existence scheme, use of periodic reference signals in beam management (e.g. in beam failure detection) may need to be reconsidered. Beam management relies heavily on periodic signals, more specifically on periodic TRS (P-TRS) as QCL source for DL signals and channels. Furthermore, beam failure detection RS and candidate RSs for new beam identification in defined beam failure recovery procedure need to be periodic, and typically failure detection RSs are P-TRSs as being active QCL sources for the PDCCH monitoring in CORESETs. Based on QCL source(s) RS UE prepares channel estimation filters (time and frequency domain estimates like delay spread, doppler spread) and sets its RX beam for the reception of the downlink signals and channels. It can also be noted that in typical deployment the same periodic RSs are used as spatial source for uplink signals and channels, i.e. DL reference signals based on which the UE forms the transmit beam(s) for uplink transmissions. Typical QCL configuration for the downlink signals and channels is provided in Figure 2 (where the RS in the start of the arrow represents the source and the signal in the end of the arrow represents the target).

Figure 2 Typical QCL source configuration for the target signals (signals in the end of the arrows).

One problem is that considered channel access mechanism, e.g. LBT, may prevent transmission of P-TRS that is the main QCL source for different signals and channels. Thus, UE may not have up to date QCL source for coming signals/channels to be received (or to be transmitted) and that would impact negatively on the downlink performance but as well the uplink performance. Furthermore, P-TRS must be validated by gNB in sub-7 NR-U, which is inefficient. Thus, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure within a period. That could be achieved e.g. by defining a P-TRS burst structure that has multiple opportunities within a certain period.

Observation 1: For P-TRS transmissions in the cell, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure.

Potential solutions could be for instance:

- A beam specific (SSB specific) aperiodic TRS transmission that could be triggered one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source)

- Multiple transmission opportunities for the P-TRS within a time period

Proposal 8: Consider solutions to provide robustness for TRS transmission due to LBT failures, for instance:

- A beam specific (SSB specific) aperiodic TRS transmission that could be triggered for one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source)

- Multiple transmission opportunities for the P-TRS within a time period

### BFD-RS

There was a proposal for a method for guaranteeing transmission of BFD-RS against LBT failure. In fact, BFD-RSs are perdic CSI-RS resources configured explicitly or implicitly. For explicit configuration, BFD-RS is explicitly configured with a set of periodic CSI-RS resource. For implicit configuration, BFD-RS is determined by periodic CSI-RS resource indicated by TCI-state for respective CORESETs for UE monitoring.

Thus, we don’t need to discuss BFD-RS separately for LBT fauilure handling. In addition, generally, periodic TRS is likely to be used for BFD-RS, if mechanism of handling LBT failure for periodic TRS is supported, this is automatically supported.

If other types of CSI-RS such as CSI-RS for acquisition or CSI-RS for beam management are used for BFD-RS, the same mechanism as TRS can be used.

Observation 2: More transmission opportunities for the BFD-RS against LBT failures can be supported by the same mechanism used for peridic CSI-RS such as TRS.

## Beam Management in the COT

If directional LBT is applied e.g. for the shared COT by the gNB, it is assumed that the subsequent transmissions in the COT can take place using transmit beam(s) confined within the spatial directions/domain sensed by the LBT beam(s). That may have impact on the validity of the configured DL RSs for L1-RSRP measurement and reporting as well as on the beam switching (e.g. MAC CE based TCI state update/switch) during the COT.

E.g. beam switching during the COT may need to differentiate the following cases:

1)	The new beam is within (one of) the LBT beam(s) of the COT, and

2)	The new beam is not QCLed with the LBT beam of the COT meaning that it cannot be seen to be within the spatial directions/domain sensed by the LBT beam(s).

In 2) it would be important to the gNB be able switch the beam for the next COT while still be serving the UE with the old beam during the current COT. Thus, e.g. application time of the beam switching command could take into account whether or not the new beam is within the LBT beam of the COT.

Proposal 9: In case of directional LBT (if applied), consider impacts on beam management in the COT, e.g.

- impact on validity of the configured DL RSs for L1-RSRP measurement and reporting and

- impact on beam switching application time within the COT (e.g. the case when the new beam is or is not QCLed with the LBT beam of the COT).

## Multi-slot CSI-RS

When operating with higher SCS, like 960 kHz SCS, PDCCH monitoring periodicity may likely be in absolute time domain similar to operation at 120 kHz SCS in order to keep PDCCH processing requirements similar to FR2. To reflect that it has been agreed to support multi-slot PDSCH and PUSCH scheduling. The same logic can be considered e.g. for the CSI-RS as well, i.e. to enable multi-slot aperiodic CSI-RS transmission. That can be provided e.g. with one of the following alternatives:

- Alt1: Use slot offset (could reuse the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated

- Alt2: Enable a single triggering state to have multiple CSI-RS resource sets for channel measurements where the sets can be configured with different slot offsets relative to each other

From the above alternatives, the Alt1 would require less standardization work and would be more simple. Thus, if multi-slot CSI-RS is introduced, Alt1 would be preferred approach.

Proposal 10: If multi-slot CSI-RS is supported, use slot offset (by reusing the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated.

# Conclusion

Based on the discussion in the previous chapter we made the following observations and proposals:

Proposal 1: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2.

Proposal 2: Additional beam switching time delay d of 120 kHz is 14 symbols.

Proposal 3: Additional beam switching time delay d of 480 kHz is 28 symbols.

Proposal 4: Support single QCL assumption for the multi-PDSCH transmission in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL.

Proposal 5: QCL assumption for the all the PDSCHs is the same as the one used for the first PDSCH in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL. I.e. the same default QCL assumption for all scheduled PDSCHs as for the for the first PDSCH.

Proposal 6: In multi-TRP case, single QCL assumption is applied for the multiple scheduled PDSCHs per TRP.

Proposal 7: Confirm working assumption:

Working assumption:

For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs

- The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism

- The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism

- Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states

- FFS: Details of multiple TCI state association with multiple PDSCHs

Observation 1: For P-TRS transmissions in the cell, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure.

Proposal 8: Consider solutions to provide robustness for TRS transmission due to LBT failures, for instance:

- A beam specific (SSB specific) aperiodic TRS transmission that could be triggered for one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source)

- Multiple transmission opportunities for the P-TRS within a time period

Observation 2: More transmission opportunities for the BFD-RS against LBT failures can be supported by the same mechanism used for peridic CSI-RS such as TRS.

Proposal 9: In case of directional LBT (if applied), consider impacts on beam management in the COT, e.g.

- impact on validity of the configured DL RSs for L1-RSRP measurement and reporting and

- impact on beam switching application time within the COT (e.g. the case when the new beam is or is not QCLed with the LBT beam of the COT).

Proposal 10: If multi-slot CSI-RS is supported, use slot offset (by reusing the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated.

# References

- RP-210862, “Revised WID: Extending current NR operation to 71 GHz”, CMCC

- TR 38.808, “Study on supporting NR from 52.6GHz to 71 GHz", 3GPP

- RAN1#106-e Chairman Notes