3GPP TSG RAN WG1 #106bis             	                                       R1-2109462

e-Meeting, Oct 11th – 19th, 2021

Agenda Item:		5

Source:			Samsung

Title:			Draft Reply to RAN2 LS on UE Power Saving

Document for:		Discussion and decision

# Introduction

As requested in [1], RAN1 needs to inform RAN2 the progress for supporting CSI-RS/TRS occasions to idle/inactive UEs, including:

- List of the parameters and potential structure for TRS/CSI-RS information, and

- TRS/CSI-RS availability.

# 2. Overall Description:

RAN1 had discussion on supporting TRS/CSI-RS occasion(s) for idle/inactive UEs, and has reached the following agreements related to the signaling of the TRS/CSI-RS occasion(s) configuration”:

From RAN1#104-e: Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values From RAN1#104b-e: Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration From RAN1#105-e: Agreement: Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source

Also, RAN1 discussed potential configuration structure as summarized in Proposal 4.1-1 (v4) [2]: We expect to continue the discussion in RAN1#106-e. RAN1 should inform RAN2 any agreement/conclusion regarding the configuration structure of TRS resource set based on the outcome in RAN1#106bis-e.

Proposal 4.1-1 (v4) For RS resources from configured TRS/CSI-RS occasion(s) for idle/inactive UEs, a RS resource set can be configured to include a set of RS resources with one or more common configuration parameters. - FFS the common configuration parameters per RS resource set, e.g. resource set ID (if support), QCL reference, startingRB, nrofRBs, powerControlOffsetSS, number of slots (if support). - FFS whether allow the possibility for a RS resource in a set to override the common parameter if needed. - FFS how to indicate availability/unavailability information for RS resources from the configured RS resource set(s) in L1 based availability indication, e.g. bit/codepoint per RS resource, or per RS resource set(s) - FFS maximum number of RS resources per RS resource set - FFS maximum number of RS resource set

Lastly, RAN1 is still discussing the availability of the potential TRS/CSI-RS occasion(s) after the TRS/CSI-RS occasion(s) are configured by SIB based on the following agreement. So, RAN1 should inform RAN2 any further agreement based on the outcome in RAN1#106bis-e.

From RAN1#105-e Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

Proposal #1: Send an LS to reply RAN2 LS in R2-2108997 with information, including

- Agreements related to the signaling of the TRS/CSI-RS occasion(s) configuration,

- Configuration structure for TRS resources and TRS resource set based on outcome in RAN1#106bis-e, and

- Whether and how to support SIB based availability indication based on outcome in RAN1#106bis-e.

# 6. Reference

[1] R2-2108997, LS on UE Power Saving, 3GPP TSG-RAN WG2 Meeting #115 electronic

[2] R1-2108515, Final summary for TRS/CSI-RS occasion(s) for idle/inactive UEs, Moderator (Samsung)