3GPP TSG RAN WG1 #106bis-e	        		             R1-2109486

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.4.1

Source: 	Samsung

Title: 	Timing relationship enhancements for NTN

Document for:	Discussion and Decision

Introduction

In this contribution, we continue to discuss the timing relationship enhancements for NTN [1]. During RAN1#106-e, the following agreements were made.

Agreement: - The UE-specific K_offset can be provided and updated by network with MAC CE. - FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration - FFS: Details on whether and how the two solutions work together Agreement: For random access procedure initiated by a PDCCH order received in downlink slot n, UE determines the next available PRACH occasion after uplink slot n+Koffset to transmit the ordered PRACH. - Note: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc, where NTA=0 is assumed for PDCCH ordered PRACH. - FFS: Which value of Koffset should be applied - FFS: Whether the n+Koffset timing relationship is impacted by UE behavior within or after the validity duration. Agreement: The unit of K_offset is number of slots for a given subcarrier spacing. - FFS: one subcarrier spacing value or different subcarrier spacing values for different scenarios. Agreement: The information of K_mac is carried in system information. Agreement: The unit of K_mac is number of slots for a given subcarrier spacing. - FFS: one subcarrier spacing value or different subcarrier spacing values for different scenarios. Agreement: In the estimate of UE-gNB RTT, which is equal to the sum of UE’s TA and K_mac, for delaying the starts of ra-ResponseWindow and msgB-ResponseWindow, the UE’s TA is equal to TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc with NTA=0. Agreement: For defining value range(s) of K_offset, down-select one option from below: - Option 1: One value range of K_offset covering all scenarios. - Option 2: Different value ranges of K_offset for different scenarios.

Discussions

Koffset update

In RAN1#106-e, the following was agreed.

Agreement:

- The UE-specific K_offset can be provided and updated by network with MAC CE.

- FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration

  - FFS: Details on whether and how the two solutions work together

Agreement:

For defining value range(s) of K_offset, down-select one option from below:

- Option 1: One value range of K_offset covering all scenarios.

- Option 2: Different value ranges of K_offset for different scenarios.

In order to provide the flexibility to the network operation, different set could be configured for K_offset. One way to do this is that the values of the set are configured by RRC signaling and MAC CE indicates which values within the set.

Proposal 1: For K_offset, RRC signaling configures the values of the set and MAC CE indicates which value is used within the set.

The configuration of K_offset value with two values makes the configuration more complicated unnecessarily. Configuring only one value for K_offset is enough.

Proposal 2: Configure only a single value for K_offset.

Configuration of Koffset

For configuration of Koffset, there are following candidates:

- Cell specific Koffset, which can be derived based on the maximum full TA within one cell.

- Beam specific Koffset, which can be derived based on the maximum full TA within one beam footprint.

- Beam-group specific Koffset, which can be derived based on the maximum full TA within one group of neighboring beam footprints

- UE specific Koffset, which can be derived based on UE specific full TA

Considering different scenarios and deployments, more than one of above configurations can be supported, and using which one can be dependent on gNB configuration. For example, for a small cell range wherein the TA variation range is small, cell specific Koffset is preferred. For a large cell range consisting of multiple small beam footprints, beam specific Koffset or beam-group specific Koffset is preferred. For a large beam footprint wherein the TA variation range is large, UE specific Koffset is preferred. Therefore, above configurations are suitable for different scenarios, and more than one configurations can be specified.

For initial access it has been already agreed that at least a cell-specific K_offset configuration should be supported. The K_offset for initial access should be selected conservatively to fulfill the worst case scenario, i.e. greater than the largest RTT in a cell. This assures all UEs in the cell are covered properly by the worst case scenario. Having the beam-specific option could increase the flexibility of the initial access in terms of timing, however, it is achieved at the expense of additional signaling overhead.

Proposal 3: Only Cell-specific K_offset in initial access is supported.

Proposal 4: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

Timing for configured grant scheduled PUSCH

3GPP has specifies two categories of Configured Grant: 1) Configured Grant Type 1; and 2) Configured Grant Type 2.

Configured Grant Type 1

When using Configured Grant Type 1, a transmission occasion starts when the following equation is satisfied:

| [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN-r16 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | (1) |
| --- | --- |

where timeDomainOffset and periodicity are both specified within the ConfiguredGrantConfig parameter structure. S corresponds to the starting symbol deducted from the timeDomainAllocation IE. N ≥ 0 is an integer corresponding to the Nth transmission occasion. As seen in Figure 1, gNB should configure UE such that the first transmission occasion (N = 0) happens after arrival of slot n at UE, i.e. the configured symbol for N = 0 transmission in equation (1) should happen in time in an amount of greater than or equal to TA after the current slot n. This can be done using two options:

- Add Koffset to equation (1), where Koffset is greater than or equal TA.  Note that in the other timing relationships discussed earlier, it is expected that the value of Koffset there is selected equal to TA. So the Koffset for configured grant could be the same value.  Of course to introduce it to equation (1) it has to be converted to symbol duration, i.e. Koffset × numberOfSymbolsPerSlot should be added to equation (1).

- Since in Rel-16 a new optional parameter namely timeReferenceSFN-r16 was introduced in ConfiguredGrantConfig IE, the value of Koffset can be implicitly included in this parameter by gNB.

Figure 1. Configured grant Type 1 timing

For other timing relationships for NTN a value for Koffset is being defined and hence Option 1 above will be practical and feasible.  However, Option 2 offers more flexibility and it leaves it to the Network to manage the timing with more freedom.

Proposal 5: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

HARQ timing indicator field

In RAN1#104-e, the following was agreed.

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

For Rel-15/16, depending on how many values for k are configured, PDSCH-to-HARQ_feedback timing indicator field in a DCI format could have up to 3 bits. This can provide a flexibility of the timing indication to the gNB. Even though the range of the K1 value is extended, there is no reason to increase the size of PDSCH-to-HARQ_feedback timing indicator field in a DCI format which should result in the performance degradation of DCI decoding.

Proposal 6: Do not change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Conclusions

In this contribution, we discussed the timing relationship enhancements for NTN. Our proposals are summarized as below.

Proposal 1: For K_offset, RRC signaling configures the values of the set and MAC CE indicates which value is used within the set.

Proposal 2: Configure only a single value for K_offset.

Proposal 3: Only Cell-specific K_offset in initial access is supported.

Proposal 4: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

Proposal 5: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

Proposal 6: Do not change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- References

- RP‑210908, “Revised WID: Solutions for NR to support non-terrestrial networks (NTN).”