3GPP TSG RAN WG1 #106bis-e	                                                    R1-2109508

e-Meeting, Oct 11th – 19th , 2021

Agenda Item:		8.8.3

Source:				Samsung

Title:					Type A PUSCH repetitions for Msg3

Document for:	Discussion

# Introduction

The Rel-17 study item “Study on NR coverage enhancements” evaluated the baseline performance for both FR1 and FR2. PUSCH of Msg.3 has been identified as the potential bottleneck channels for both FR1 and FR2. As supported in the Rel-17 work item on NR coverage enhancements [1], the Msg.3 enhancement by using PUSCH repetition type A is approved.

This contribution discusses the aspects related to the enhancements for type A PUSCH repetitions for Msg3.

# Discussion

## UE capability differentiation

As discussed in last 2 meeting,  the following agreement are reached :

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

- FFS the determination of the RSRP threshold.

Agreement:

- For requesting Msg3 PUSCH repetition, support the following:

  - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

    - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

    - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

  - FFS whether or not to additionally support one (& only one) more option:

    - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

    - E.g., Option 3: Use separate RO, which include

      - the separate RO configured by a separate RACH configuration index from legacy UE, and

      - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreement from RAN1#106e

The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition.

Regarding the several left FFS issue in the agreement, for the RSRP threshold, like almost all the RSRP thresholds before, e.g., the RSRP for SSB selection (RACH selection), NUL/SUL selection, 2step RACH vs 4step RACH selection, it should be configured and explicitly indicated by gNB. Otherwise, gNB cannot have control capability to manage the UE who may try to coverage enhancements.

Proposal 1: the RSRP threshold should be explicitly configured and indicated by gNB.

For the issue on the subset sharing RO, it is very beneficial for the gNB to have the capability to only share only part of the ROs for one SSB to the purpose of msg3 repetition. Especially the RACH resource are widely portioned for so many purpose, and the preamble number in one RO is still limited to 64. Thus, subset sharing RO could sometimes avoid overlapping with partition for other purposes.

Proposal 2: a PRACH mask index is supported for subset sharing.

One details on the issue is that shared RO could be also shared for 2step RACH,  that means, such RO will be used for 4step RACH, 2step RACH as well as msg3 repetition, so it needs to ensure the preamble grouping could be clearly designed to avoid ambiguity. There could be two options:

- Explicitly indicates the preamble number and preamble starting index. To reduce the overhead, a limited number of starting index could be specified, e.g., 4 values with 2 bit indication.

- Only indicates the preamble number, the preamble starting index is from the end of the preamble for a SSB in a RO. Thus, the preambles for msg3 repetition for coverage enhancement are derived from the end of the preambles for a SSB in one RO backwards.

Proposal 3: for indication of preambles in case of shared RO, down select from following options:

- Explicitly indicates the preamble number and preamble starting index

- Only indicates the preamble number, the preamble starting index is from the end of the preamble group for a SSB in a RO

FFS details.

## Repetition number indication

During last meeting, the repetition number indication for msg3 initial transmission and re-transmission has been decided to use the UL grant in RAR and DCI with CRC scrambled by TC-RNTI, respectively.

Working Assumption

Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission.

- Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors.

  - The new TDRA table is configured by SIB1, with selecting one of the two options below.

    - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor.

    - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor.

  - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied.

    - K=1.

- Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows.

  - X MSB bits of the MCS information field are used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

- Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below.

  - Option 1: X LSB bits of the TPC information field are used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

  - Option 2: A predefined TPC command table with including repetition factor K is introduced.

    - FFS details.

The next step is to determine what the actual indication method to be used is. For indication using UL grant (in RAR) for msg3 initial transmission, in which the existing fields are given below.

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 0, for operation without shared spectrum channel access 2, for operation with shared spectrum channel access |

Among which several items are not likely to be touched, like FH flag, thus it leaves the choice of changing the rest:

- Option 1: TDRA indication with a new TDRA table containing the repetition number indication

This option seems easiest way without impact the structure the RAR UL grant, however, it indeed limit the repetition number could be configured, e.g., for a 4bit indication of a 16 row TDRA table, each row has one number of repetition.

- Option 2: using explicit bit indication of repetition number

This option could provide the flexible repetition number indication in combine with the TDRA row. The cost is that some flexibility on the MCS configuration needs to be limited. Currently MCS has 4bits indicating the 16 possible MCS, while for msg3 repetition necessary UE, these flexibility may not be needed, e.g., we can use only 2 bits to indicate 4 MCS settings, and use the 2 bits (e.g., 2 MSBs) to indicate a 4 choices of repetition number. Of course, the size of the bit needed for the repetition indication is dependent on how many repetition number we need to indicate. For example, if only 2 values for repetition number, then use CSI request bit seems also possible.

There are some discussion on repurposing the frequency resource allocation, TPC command. However, since the initial BWP size is not change, the FDRA indication should be kept. For TPC command, one argument to revise it that the power may not use the negative value so these values could be repurpose. Such statement may not be true for two reasons:

- The power of PRACH will be inherited for the msg3 PUSCH transmission  power determination. Since the detection/decoding requirement for sequence and modulated data symbol are different, a negative is still needed.

- Since we are using the measured DL path loss for the UL Tx power determination, there could quite difference between DL and UL path loss, e.g., 6dB, thus the negative value in the TPC could still be used adjust to power to a proper level for UL transmission.

Proposal 4: for indication number of repetition in msg3 initial transmission, support to use X MSB bits of the MCS information field

On the other hand, for the indication in the DCI format targeting for the msg3 retransmission, there could be two options as well:

- Option 1: Use the same mechanism as supported for Msg3 initial transmission.

This option is to follow up the options in initial msg3 transmission if the new TDRA table is adopted. But if initial transmission utilizes the MCS to indicate the repetition number, it might be ok to not follow the same mechanism since it’s anyway explicitly indication.

- Option 2: using explicit/reserved  bit indication of repetition number

In DCI 0_0 to schedule msg3 retransmission, the DCI field seems have enough reserved  bits e.g., reserved or HARQ process number – 4 bits, reserved; with this flexibility, it is totally feasible to indicate the repetition number separately.

Thus, we suggest deciding the indication of repetition number for both initial and retransmission in a more consistent manner, which is using same manner to indicate repetition number in both cases.

Agreement

Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions.

- Option 1:

- When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

- Option 2:

- When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB.

  - FFS details, e.g. implicit or explicit indication or predefined.

  - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

Proposal 5: for indication number of repetition in msg3 retransmission,  using explicit bit indication of repetition number (HARQ process ID).

One more issue to consider if new TDRA table is adopted, is that, which TDRA table is to be applied for the indicated 4bit TDRA. The reason is that, even if UE requests the msg3 repetition with the separate preambles, the gNB could still refuse to configure the msg3 repetition due to lack of the resource or load balancing etc. Thus, the gNB either need to indicate the row in new TDRA table with repetition number to be 1 or indicate the row in legacy TDRA table. If we only limit the usage of the row(s) in new TDRA table with repetition number to be 1, which is a very few rows if there will even be any, the choice for the TDRA will be quite conservative and not acceptable for gNB scheduling. Thus, there will be a need to indicate which TDRA table to be applied in case of UE request msg3 repetition but not configured by gNB.

Proposal 6: An indication of TDRA table to be applied is supported if new TDRA table is supported.

## Beam determination

In Rel-16, the msg3 spatial setting is left to UE implementation and refinement of spatial setting from msg1 tx to msg3 tx was not finalized. For 2-step RACH, the PRACH and msgA PUSCH are specified to use a same spatial setting. Thus, from practical consideration, after a UE successfully detects a RAR corresponding to the transmitted preamble, that implies the spatial setting used for the PRACH transmission is qualified. There is no strong/identifiable motivation to change the spatial setting for msg3 PUSCH. Also, the relation between msg1 and msg3 affects power control as the msg.3 power is inherited from the msg.1 power setting with some msg.3-specific modification. Therefore, if the UE changes the spatial setting for msg.3 transmission compared to the one for msg1 transmission, the power setting may not be accurate. Thus, it is reasonable to use same spatial setting for msg.3 and msg.1.

Proposal 7: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission.

Once UE send the first msg3, the contention resolution timer starts and it can last 64ms. Then for the msg3 retransmission scheduled by DCI 0-0 scrambled by TC-RNTI, the preamble beam may not be very stable due to a relatively long time that passed. Thus, it can be beneficial to allow the UE to select the beam to use for the msg3 re-transmissions.

Proposal 8: The UE can select the beam for msg3 re-transmissions.

## DCI Monitoring after msg3 transmission

In Rel-16, upon sending msg3, a UE starts the ra-ContentionResolutionTimer, during which it monitors for a DCI format 1_0 scheduling the PDSCH carrying msg4 or format 0_0 for msg3 retransmission. With msg3 repetitions allowed, there are different options for enabling such a DCI monitoring.

Option 1: The UE can start the monitoring after the end of the first msg3 transmission occasion among the set of scheduled msg3 repetitions. This option allows the UE to efficiently complete the RACH procedure once at least one msg3 transmission is correctly received by the gNB.

Option 2: The UE can start the monitoring after the end of the last msg3 repetition. This provides the easiest design of the procedure from a UE implementation. However, this comes at the expense of possible latency in the RACH procedure due to the transmission of possibly unneeded msg3 repetitions.

Proposal 9: Support starting	DCI Monitoring after the end of the first msg3 transmission.

## Frequency hopping

In last meeting, the inter-slot frequency hopping is agreed to be supported for msg.3 repetition.

Agreements:

Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission.

FFS details, e.g., signaling etc.

The left issue is whether we should additionally support intra-slot FH. In legacy msg3 transmission (since there is no repetition), the intra-slot FH is the only choice for FH operation. Considering inter-slot is already supported, we need carefully study whether intra-slot FH is still necessary.

For the purpose of obtaining frequency diversity gain, we think inter-slot FH can already provide such effect. One additional purpose from the proponent of intra-slot FH is that the better resource utilization as shown in following figure.

However, in order to achieve the nicely fitting of legacy UE (UE 1 and UE 2) and new type of UE (UE 3), there are several conditions to be satisfied:

- The time domain resource allocation of three UEs need to be nicely aligned, i.e., the time domain starting positioning, the length of the transmission;

- The frequency domain resource allocation of three UEs need to be nicely aligned, i.e., the f-domain starting positioning, the number of PRBs allocated;

Moreover, even we consider above conditions are fulfilled, for a given time period, there are N legacy UEs’ msg3 resource to be allocated, there could be only one UE left out for no legacy UE to be paired with when N is odd number. It is difficult to see the true benefits of supporting this feature.

In addition, if we think the above case is a beneficial case to be supported, then for the same benefits and same reason, we should also support enable both intra-slot FH and inter-slot FH. As shown in following figure, if the UE1 and UE2 originally allocated to different F-domain positioning, which is totally possible, to better fit the new UE (UE 3), we probably need to support enable intra-slot FH and inter-slot FH for UE3 at the same time.

Observation 1: the benefits of better resource utilization on intra-slot FH for msg3 repetition are not clear.

Proposal 10: intra-slot FH is not supported for msg3 repetition.

## Available slots for msg3 repetition

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI.

Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated.

Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon.

- A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols.

  - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition.

  - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

During last meeting, the further condition on determine the available slot is discussed. It is agreed that the available slot depends on the semi-statistic configuration, like  TDD-UL-DL-Configcommon. It’s natural the UL symbols can be used, but for F symbols, there might be some other consideration.  Since the TDD-UL-DL configuration common could be conservative so that more F symbols will be allocated for better flexible usage later on. So not allowing use it will be quite limit the number of symobls to be used. But freely use all the possible F symbol might be also create difficulty for scheduling, a middle ground is that allowing gNB to indicate a valid/invalid symbol pattern  for UE to timely determine the F symbols could be used or not. For example, in the PDDCH addressed to RA-RNTI, the gNB could indicate such pattern, and it could apply to the all the UL grants provided by the RAR from this PDCCH.

Proposal 11: a valid/invalid symbol pattern could be indicated for usage of the flexible symbols configured by TDD-UL-DL-Configcommon.

## RV application

In last meeting, the following agreement reached for RV determination:

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission.

- The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214.

- The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214.

- FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

In current operation of PUSCH repetition type A, the RV for each repletion is relatively fixed. This is because some unexpected situation, e.g., miss of SFI indication, there might be some ambiguity to understand the RV for each repetition between gNB and UE.

However, in msg3 repetition case, since the available slot depends only on the TDD-UL-DL-Configcommon, so the UE and gNB hold no ambiguity on such information. Besides, even if the valid/invalid pattern introduced as explained above, for a UE who conducts the msg3 transmission, it shall successfully decode the PDCCH and PDSCH of Msg2 already. Therefore, same understanding on the resource to be used should be achieved by both UE and gNB.

With the above discussion, it’s beneficial and feasible to apply the RV cycling for the transmission/repetition on the available slot.

Proposal 12: support applying the RV cycling for the transmission/repetition on the available slot

## Available slot determination

Agreement

- The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered.

- If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot.

One small detail is that whether the Ngap symbols after the SSB should be considered or not. This Ngap symbol is from the specification of RO validation and msgA PO validation, as showing in following text using msgA PO as illustration.

==================================38.213. section 8.1A==================================

if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols, or

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

==================================38.213. section 8.1A==================================

The movtivation of having such Ngap symbol was back to R15 discussion, it’s to allow gNB switching from DL tx to UL rx, thus we can see the value of Ngap is not large. And it’s not targeting for TA change or UE switching. By this purpose, we think the N gap symbol should be considered here.

In addition, we can see the requirement of validation is that only overlapped with SSB symbols, it has to avoid the symbols preceding a SSB and only the symbols after the last SSB + Ngap will be considered as valid. So here the similar rule should be applied.

Proposal 13: If a symbol for Msg3 repetition in a slot precedes any SSB in a slot or overlaps with last SSB transmission plus Ngap symbols after the SSB, where Ngap is provided in Table 8.1-2 from TS38.213, the slot is determined as not available during the counting of repetitions.

# Conclusion

This contribution discusses the potential enhancements on channels in msg3 repetition. Observations and proposals are summarized as follows:

Proposal 1: the RSRP threshold should be explicitly configured and indicated by gNB.

Proposal 2: a PRACH mask index is supported for subset sharing.

Proposal 3: for indication of preambles in case of shared RO, down select from following options:

- Explicitly indicates the preamble number and preamble starting index

- Only indicates the preamble number, the preamble starting index is from the end of the preamble group for a SSB in a RO

FFS details.

Proposal 4: for indication number of repetition in msg3 initial transmission, support to use X MSB bits of the MCS information field

Proposal 5: for indication number of repetition in msg3 retransmission,  using explicit bit indication of repetition number (HARQ process ID).

Proposal 6: An indication of TDRA table to be applied is supported if new TDRA table is supported.

Proposal 7: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission.

Proposal 8: The UE can select the beam for msg3 re-transmissions.

Proposal 9: Support starting    DCI Monitoring after the end of the first msg3 transmission.

Observation 1: the benefits of better resource utilization on intra-slot FH for msg3 repetition are not clear.

Proposal 10: intra-slot FH is not supported for msg3 repetition.

Proposal 11: a valid/invalid symbol pattern could be indicated for usage of the flexible symbols configured by TDD-UL-DL-Configcommon.

Proposal 12: support applying the RV cycling for the transmission/repetition on the available slot

Proposal 13: If a symbol for Msg3 repetition in a slot precedes any SSB in a slot or overlaps with last SSB transmission plus Ngap symbols after the SSB, where Ngap is provided in Table 8.1-2 from TS38.213, the slot is determined as not available during the counting of repetitions.

# References

- RP-202928, “New WID on NR coverage enhancements”, RAN meeting #90e, Dec 2020.