3GPP TSG RAN WG1 #106bis-e	R1-2109526

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.3

Source: 	Samsung

Title: 				     UE features for enhanced IIoT and URLLC

Document for:	Discussion and Decision

Introduction

This contribution discussed UE features for Rel-17 IIoT/URLLC.

Discussion

In [1], preliminary RAN1 UE features lists have been provided in below table. On top of that, our comments for each corresponding feature is included in the last column.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional | Samsung’s comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot | 5-18 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - “Need of FDD/TDD differentiation” should be “Yes” since it is only applicable to TDD as “the name” is saying. |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - It is preferable to add the range of repetition factor that can be indicated by dynamic indication, for example, K={2, 4 ,8} |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - “12-1a (UL priority indication in DCI with mixed DCI formats) should be included in Prerequisite feature groups |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | Semi-static PUCCH cell (FFS or PUCCH carrier) switching using configured time-domain domain pattern of applicable PUCCH cell / carrier |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication in the DCI scheduling the PUCCH |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling [This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode] |  |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling | - Why this feature should be “per Band”? “ |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling | - Why this feature should be “per Band”? |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] | 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - 25-16 and 25-17 should be merged together since there is no benefit to make them separately, and 12-1 (UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer) should be prerequisite feature for that. |
| 25. NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. | 11-3 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |  |

Conclusion

This contribution considered the UE features list for Rel-17 URLLC/IIoT. Proposals are included in above table.

Reference

[1] R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR