3GPP TSG RAN WG1 Meeting #106bis-e 	R1-2109563

e-Meeting, October 11th – 19th, 2021

Agenda Item: 8.17.2

Source: MediaTek Inc.

Title: 	Views on UE features for supporting NR from 52.6 GHz to 71 GHz

Document for: Discussion

# Introduction

This paper provides our views on Rel-17 UE features for supporting NR from 52.6 GHz to 71 GHz. Detailed proposals are described in the following sections and summarized in feature list table based on our proposals are presented in Section 3.

# Views on UE features for supporting NR from 52.6 GHz to 71 GHz

Based on the conclusion made in RAN #92, indication of the UE features for FR2-2 only should be included in UE capability reporting, which is not allowed based on the current UE feature list structure. One way to address the issue could be to include the notion of “FR2-2” in the naming or components of the FG applied for FR2-2 only. Another way could be to include a column in the UE feature list table to indicate “FR1-FR2-1-FR2-2 differentiation” for FGs to allow UE to support different functionalities between FR1, FR2-1, and FR2-2.

Proposal 1: How to report UE features applied to only FR2-1 or FR2-2 should be further discussed.

Introduce FR2-1 for 24.25 – 52.6 GHz, and FR2-2 for 52.6 – 71 GHz, - The above two ranges to be introduced under the FR2 common range. Option A. Frequency range designation; Corresponding frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-1: 24250 MHz – 52600 MHz. FR2-2: 52600 MHz – 71000 MHz. NOTE: Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered in this release, unless otherwise stated. NOTE: The designations FR2-1 and FR2-2 should only be used when needed.. - The related UE capabilities and their applicability to the frequency range 52.6 to 71 GHz will have to be analysed on a case by case basis - The application of any of the UE feature introduced for 52.6-71 GHz to existing FR1/FR2 should be discussed case by case. TSG RAN specifications shall make it very clear (to readers) that frequency bands in the 52.6-71GHz range are only Release-independent from Rel-17 onwards, to ensure that there is clear industry understanding about which FR2 features are applicable for operation in 52.6-71GHz range.

FG 24-1 General FR2-2 support

We view this FG as basic UE feature for operating in FR2-2 and we suggest to split into UL and DL basic features to enable operation in a cell not configured with uplink. For the components, we suggest to include specific types of channels/signals instead of transmission and reception in the description to avoid ambiguity. We also suggest to separate the multi-RB PUCCH feature and wideband PRACH feature into other FGs as optional enhancements.

Proposal 2: Modify FG 24-1 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 |  |  |
| 24. NR_ext_to_71GHz | 24-1-1 | General FR2-2 support for DL | - Support 120kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  | Optional with capability signaling This FG is a part of basic DL operation in FR2-2 |
| 24. NR_ext_to_71GHz | 24-1-2 | General FR2-2 support for UL | - Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 120KHz subcarrier spacing and length 139 in FR2-2 |  | Optional with capability signaling This FG is a part of basic UL operation in FR2-2 |

FG 24-4 480kHz SCS support

We view this optional FG as basic UE feature for operation in FR2-2 if UE supports 480kHz subcarrier spacing. Similar to FG 24-1, we suggest to split the components into UL and DL basic features to enable operation in a cell not configured with uplink. We also suggest to separate multi-slot PDCCH monitoring and wideband PRACH into other FGs as optional enhancements.

Proposal 3: Modify FG 24-4 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] |  |  |
| 24. NR_ext_to_71GHz | 24-4-1 | 480kHz SCS support for DL in FR2-2 | - Support 480kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4-2 | 480kHz SCS support for UL in FR2-2 | - Support 480kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 480KHz subcarrier spacing and length 139 in FR2-2 |  | Optional |

FG 24-5 960kHz SCS support

Similar to our comment for FG 24-4, our proposal for FG 24-5 is as follows

Proposal 4: Modify FG 24-5 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 |  |  |
| 24. NR_ext_to_71GHz | 24-5-1 | 960kHz SCS support for DL in FR2-2 | - Support 960kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5-2 | 960kHz SCS support for UL in FR2-2 | - Support 960kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 960KHz SCS and length 139 in FR2-2 |  | Optional |

  - Views on UE features for initial access aspects

FG 24-2 120KHz SSB based stand-alone support

For FG24-2, the component should be “support 120kHz SSB for initial access” and we suggest to add FR2-2 notion to differentiate the support of 120kHz SSB in FR2-1.

Proposal 5: Fix typo in FG 24-2 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support in FR2-2 | 1. Support 480120KHz SSB for initial access in FR2-2 |  | Optional |

FG 24-3 480KHz SSB based stand-alone support

In the note of FG 24-3, it captures that 480 kHz is an optional SSB numerology for initial access. We suggest to add following description in the note based on one related sub-bullet in WID:

- only 480kHz CORESET#0/Type0-PDCCH SCS is supported for 480 kHz SSB SCS.

Proposal 6: Modify FG 24-3 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support in FR2-2 | 1. Support 480 KHz SSB for initial access in FR2-2 | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. o Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) | Optional |

FG for wideband PRACH enhancement

For wideband PRACH enhancement, we suggest to introduce an FG to capture the agreement made in RAN1 #104e meeting
Agreement:

- For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.

Proposal 7: Add an FG for wideband PRACH enhancement for 120kHz SCS as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-X | Wideband PRACH for 120kHz SCS in FR2-2 unlicensed operation | 1. Enhanced PRACH design for operation with shared spectrum channel access by adopting a single long ZC sequence, with ZC sequence = 571/1151 for 120kHz |  | Optional |

  - Views on UE features for PDCCH monitoring enhancements

FG for multi-slot PDCCH monitoring

Although the multi-slot PDCCH monitoring feature details are still under discussion, we suggest to reserve FGs for the feature and revise the components with more details when the functionality is mature.

Proposal 8: Add FGs for multi-slot PDCCH monitoring as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-Y-1 | Multi-slot PDCCH monitoring for 480kHz | - Support multi-slot PDCCH monitoring for 480KHz with X=4 |  | Optional |
| 24. NR_ext_to_71GHz | 24-Y-2 | Multi-slot PDCCH monitoring for 960kHz | - Support multi-slot PDCCH monitoring for 960KHz with X=8 |  | Optional |

  - Views on UE features for Enhancements for PUCCH formats 0/1/4

FG for multi-RB PUCCH format 0/1/4

Based on our comments on FG 24-1, we suggest to introduce FGs for multi-RB PUCCH format 0/1/4 enhancements based on the following agreements:

Agreement:

The maximum configured number of RBs, N_RB, for enhanced PF 0/1/4 is given by 16 RBs for 120 kHz SCS.

Agreement:

The maximum configured number of RBs, N_RB, for enhanced PF 0/1/4 is given by 16 RBs for 480 and 960 kHz SCS (same as for 120 kHz SCS).

Proposal 9: Add FGs for multi-RB PUCCH format 0/1/4 as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-Z-1 | 120kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers |  | Optional |
| 24. NR_ext_to_71GHz | 24-Z-2 | 480kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers |  | Optional |
| 24. NR_ext_to_71GHz | 24-Z-3 | 960kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers |  | Optional |

  - Views on UE features for PDSCH/PUSCH enhancements

FG for multi-PDSCH DL grant

We suggest to add FGs for the enhancements of both multi-PDSCH and multi-PUSCH scheduled by single DCI. We also suggest to add the notion of FR2-2 in this FG such that it can be differentiated from the existing multi-PUSCH feature introduced for Rel-16 NR-U and for FR2-1.

Proposal 10: Add FGs for multi-PDSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-S-1 | multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-S-2 | multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-S-3 | multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 960kHz SCS in FR2-2 |  | Optional |

Proposal 11: Add FGs for multi-PUSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-T-1 | multi-PUSCH UL grant for 120 kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 120kHz in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-T-2 | multi-PUSCH UL grant for 480kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 480kHz in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-T-3 | multi-PUSCH UL grant for 960kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 960kHz in FR2-2 |  | Optional |

  - Views on UE features for channel access mechanism

FG for Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation and Cat 2 LBT support for FR2-2 unlicensed operation

Up to RAN1 #106e meeting, COT sharing is the only agreed scenario for Cat2 LBT application and it is desirable to support such feature in additional to the support of Cat3 to Cat 4 LBT.  Therefore, we suggest to include the support of Cat3 or Cat4 LBT as the prerequisite features for the Cat2 LBT support.

Proposal 12: Modify FG 24-? as follows

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | [Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation] |  | Optional |

Based on the WID, both LBT and No-LBT related procedures can be supported in FR2-2 unlicensed spectrum. Therefore, we suggest to introduce a FG for UE to indicate support of only No-LBT mode transmission.

Proposal 13: Add an FG for No-LBT mode transmission if FR2-2 unlicensed operation as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-W | No-LBT mode transmission for FR2-2 unlicensed operation | 1. Support only No-LBT mode transmission for FR2-2 unlicensed operation |  | Optional |

# Conclusion

In summary, we have the following proposals for revising RAN1 UE feature list and the updated UE feature list table is provided accordingly. Revised parts are written in red.

Proposal 1: How to report UE features applied to only FR2-1 or FR2-2 should be further discussed.

Proposal 2: Modify FG 24-1 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 |  |  |
| 24. NR_ext_to_71GHz | 24-1-1 | General FR2-2 support for DL | - Support 120kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  | Optional with capability signaling This FG is a part of basic DL operation in FR2-2 |
| 24. NR_ext_to_71GHz | 24-1-2 | General FR2-2 support for UL | - Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 120KHz subcarrier spacing and length 139 in FR2-2 |  | Optional with capability signaling This FG is a part of basic UL operation in FR2-2 |

Proposal 3: Modify FG 24-4 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] |  |  |
| 24. NR_ext_to_71GHz | 24-4-1 | 480kHz SCS support for DL in FR2-2 | - Support 480kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4-2 | 480kHz SCS support for UL in FR2-2 | - Support 480kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 480KHz subcarrier spacing and length 139 in FR2-2 |  | Optional |

Proposal 4: Modify FG 24-5 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 |  |  |
| 24. NR_ext_to_71GHz | 24-5-1 | 960kHz SCS support for DL in FR2-2 | - Support 960kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5-2 | 960kHz SCS support for UL in FR2-2 | - Support 960kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 960KHz SCS and length 139 in FR2-2 |  | Optional |

Proposal 5: Fix typo in FG 24-2 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support in FR2-2 | 1. Support 480120KHz SSB for initial access in FR2-2 |  | Optional |

Proposal 6: Modify FG 24-3 as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support in FR2-2 | 1. Support 480 KHz SSB for initial access in FR2-2 | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. o Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) | Optional |

Proposal 7: Add an FG for wideband PRACH enhancement for 120kHz SCS as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-X | Wideband PRACH for 120kHz SCS in FR2-2 unlicensed operation | 1. Enhanced PRACH design for operation with shared spectrum channel access by adopting a single long ZC sequence, with ZC sequence = 571/1151 for 120kHz |  | Optional |

Proposal 8: Add FGs for multi-slot PDCCH monitoring as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-Y-1 | Multi-slot PDCCH monitoring for 480kHz | - Support multi-slot PDCCH monitoring for 480KHz with X=4 |  | Optional |
| 24. NR_ext_to_71GHz | 24-Y-2 | Multi-slot PDCCH monitoring for 960kHz | - Support multi-slot PDCCH monitoring for 960KHz with X=8 |  | Optional |

Proposal 9: Add FGs for multi-RB PUCCH format 0/1/4 as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-Z-1 | 120kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers |  | Optional |
| 24. NR_ext_to_71GHz | 24-Z-2 | 480kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers |  | Optional |
| 24. NR_ext_to_71GHz | 24-Z-3 | 960kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers |  | Optional |

Proposal 10: Add FGs for multi-PDSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-S-1 | multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-S-2 | multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-S-3 | multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 960kHz SCS in FR2-2 |  | Optional |

Proposal 11: Add FGs for multi-PUSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-T-1 | multi-PUSCH UL grant for 120 kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 120kHz in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-T-2 | multi-PUSCH UL grant for 480kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 480kHz in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-T-3 | multi-PUSCH UL grant for 960kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 960kHz in FR2-2 |  | Optional |

Proposal 12: Modify FG 24-? as follows

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | [Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation] |  | Optional |

Proposal 13: Add an FG for No-LBT mode transmission if FR2-2 unlicensed operation as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-W | No-LBT mode transmission for FR2-2 unlicensed operation | 1. Support only No-LBT mode transmission for FR2-2 unlicensed operation |  | Optional |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-1-1 | General FR2-2 support for DL | - Support 120kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 |  |  |  |  |  |  |  |  |  | Optional This FG is a part of basic DL operation in FR2-2 |
| 24. NR_ext_to_71GHz | 24-1-2 | General FR2-2 support for UL | - Support 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 120KHz subcarrier spacing and length 139 in FR2-2 |  |  |  |  |  |  |  |  |  | Optional This FG is a part of basic UL operation in FR2-2 |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support in FR2-2 | 1. Support 480120KHz SSB for initial access in FR2-2 | 24-1-1 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support in FR2-2 | 1. Support 480 KHz SSB for initial access in FR2-2 | 24-1-1, 24-1-2, 24-2, 24-4-1 |  |  |  |  |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) | optional |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] |  |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-4-1 | 480kHz SCS support for DL in FR2-2 | - Support 480kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 | 24-1-1 |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. | optional |
| 24. NR_ext_to_71GHz | 24-4-2 | 480kHz SCS support for UL in FR2-2 | - Support 480kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 480KHz subcarrier spacing and length 139 in FR2-2 | 24-1-2 |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. | optional |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 |  |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-5-1 | 960kHz SCS support for DL in FR2-2 | - Support 960kHz subcarrier spacing for DL data and control channels and reference signals in FR2-2 | 24-1-1 |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. | optional |
| 24. NR_ext_to_71GHz | 24-5-2 | 960kHz SCS support for UL in FR2-2 | - Support 960kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 - Support PRACH with 960KHz SCS and length 139 in FR2-2 | 24-1-2 |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. | optional |
| 24. NR_ext_to_71GHz | 24-? | Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation | 1. Cat 3 or 4 LBT support (not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement) | 24-1-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1-2, 24-? |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-W | No-LBT mode transmission for FR2-2 unlicensed operation | 1. Support only No-LBT mode transmission for FR2-2 unlicensed operation |  |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-X | Wideband PRACH for 120kHz SCS in FR2-2 unlicensed operation | 1. Enhanced PRACH design for operation with shared spectrum channel access by adopting a single long ZC sequence, with ZC sequence = 571/1151 for 120kHz | 24-1-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-Y-1 | Multi-slot PDCCH monitoring for 480kHz | - Support multi-slot PDCCH monitoring for 480KHz with X=4 | 24-4-1 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-Y-2 | Multi-slot PDCCH monitoring for 960kHz | - Support multi-slot PDCCH monitoring for 960KHz with X=8 | 24-5-1 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-Z-1 | 120kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers | 24-1-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-Z-2 | 480kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers | 24-4-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-Z-3 | 960kHz multi-RB PUCCH format 0/1/4 for FR2-2 unlicensed operation | - Support multi-RB PUCCH format 0/1 enhancements with configured integer number of RB in the range [1,… ,16] - Support multi-RB PUCCH format 4 enhancements with configured integer number of RB in the range [1,… ,16] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers | 24-5-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-S-1 | multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 120kHz SCS in FR2-2 | 24-1-1 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-S-2 | multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 480kHz SCS in FR2-2 | 24-4-1 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-S-3 | multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of scheduling up to 8 PDSCHs with a single DCI 1_1 for 960kHz SCS in FR2-2 | 24-5-1 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-T-1 | multi-PUSCH UL grant for 120 kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 120kHz in FR2-2 | 24-1-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-T-2 | multi-PUSCH UL grant for 480kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 480kHz in FR2-2 | 24-4-2 |  |  |  |  |  |  |  |  | optional |
| 24. NR_ext_to_71GHz | 24-T-3 | multi-PUSCH UL grant for 960kHz SCS in FR2-2 | - Support of scheduling up to 8 PUSCHs with a single DCI 0_1 for 960kHz in FR2-2 | 24-5-2 |  |  |  |  |  |  |  |  | optional |