3GPP TSG RAN WG1 Meeting #106bis-e                                     R1-2109576

e-Meeting, October 11th – 19th, 2021

Agenda Item: 8.3.2

Source: MediaTek Inc.

Title: On the enhancements for unlicensed band URLLC/IIoT

Document for: Discussion and Decision

# Introduction

During RAN#88-e plenary [1], it was agreed to specify the required UL enhancements for URLLC to operate in unlicensed controlled environment. Specifying support for UE-initiated COT for FBE and harmonizing UL configured-grant enhancements in NR-U and URLLC were particularly emphasized.

# COT Initiator Determination

In RAN1#105e [5], the following agreement has been reached to determine the COT initiator for an UL configured transmission.

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- To determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

To support this agreement in practice, the UE processing time needs to be considered. The UE needs additional time to detect any gNB DL transmission at the start of the gNB FFP and confirm that the gNB has initiated the COT. Hence having the transmission confined within a gNB FFP is not enough as a statement and needs further clarification. The transmission needs to be confined within [gNB_FFP_start + Δ1, gNB_idle_period_start] where Δ1 is the time required to receive and detect the gNB DL transmission at the start of the FFP.

But if the UE fails to detect the gNB DL transmission, it also needs some extra time for the CCA before initiating its own COT. Therefore, an additional time Δ2 is also required for CCA and the UL CG transmission needs in that case to be confined within [gNB_FFP_start + Δ1 + Δ2, gNB_idle_period_start]

Figure 1: The need to consider the UE processing time

Figure 2: The need for extra time for CCA before the UE initiates a COT

- UE processing time needs to be considered in semi-static channel access mode for configured UL transmission.

# UE to gNB COT sharing

In RAN1#106 [6], the following agreement has been made:

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

In this agreement, when the gNB shares a UE initiated COT, it should at least include a scheduled DL transmission or a DCI intended to the UE that has initiated the COT. Other UEs however are not allowed to share the UE initiated COT. Hence, other UEs need to know whether the gNB DL transmission is sharing the UE COT or initiaing its own COT. Another UE receiving a DL transmission based on a UE intiated COT sharing of a UE FFP can attempt to send for example a HARQ-Feedback or CG-PUSCH by assuming the gNB has initiated a COT. Hence, the UE needs awarness on how to recognize a gNB’s COT for the following CG UL transmission.

To address this issue, we propose to include a new DCI field specifically to signal whether the gNB is initiating its own COT or sharing a COT initiated by the UE. This proposal will work if the gNB is transmitting a DCI but this is not always the case as the gNB could also be transmitting PDSCH. Unless the gNB has to transmit a GC-DCI to inform all UEs that it is sharing the UE-COT before starting any transmission in the shared the UE COT. And this should be interpreted that other UEs are not allowed to transmit back in this UE COT.

Also, if a dedicated DCI bit-field is used for this prupose, then Other UEs may need to know the parameters of the shared UE-COT to know when the gNB will finish sharing the UE COT

Hence we propose to use a different implicit scheme to avoid extra signalling overhead, for example different DMRS encoding when the gNB is sharing a UE COT.

- For a DL transmission based on sharing a UE intiated COT, other UEs can recognize that the gNB is sharing the UE COT through the gNB use of a different DMRS encoding.

# UE initiated COT for Wideband operation

It was suggested to support FFP configuration per channel for wideband operation. The motivation is that different LBT BWs can have different transmission characteristics. Hence, different FFP configurations are needed. Also, this may help in reducing the access delay as the latency in getting an opportunity to transmit could be reduced by having staggered FFP starting points and different periodicities.

We find that although the proposal can bring some flexibility and may offer some latency gain but on the other hand this will over-complicate the design and complicate the UE implementation and also requires a lot of specification effort. The UE will need to carry the CCA procedure for every LBT BW and needs to operate differently for each LBT BW. Also, interference between devices needs further study. Also this design may compromise the alignement between the gnB-FFP and the UE-FFP and hence requires extra computation at the UE side.

- . When operating on multiple LBT-BWs, FFP parameters and the assumptions regarding the COT initiator are aligned across all LBT-BWs

# Control of UE-initiated COT

The UE could be configured with UE’s FFP period and offset. Hence, UE-COT initiation could be viewed as semi-statically enabled by default if the UE is configured with the FFP parameters.

However, the gNB may need to disable the UE COT-initiating functionality temporarily and dynamically to give priority to the gNB-initiated COT or for UEs to fall-back to the gNB-initiated COT for a certain period, one of the main motivation is to reduce the collision and the blockage probabilities. Also, it allows the gNB to better control UEs access to the medium and the gNB will have the ability to temporarly cancel some UE initiated COTs to schedule other UEs with potentially high priority traffic faster and in a more reliable manner.

- In FBE mode, support enabling/disabling UE COT-initiating functionality dynamically.

# FFP parameters for UE-initiated COT

Regarding the signaling of the FFP parameters, the following agreement has been made in RAN1#102e [2]:

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

First, we think reducing contention and especially restricting UEs with low priority traffic occupying the channel and exploiting UE-initiated COT should be further discussed. As one potential option, the UE COT-initiating should be a functionality under gNB control and the gNB should favor UEs with high priority traffic.

- UE COT initiation enabling/disabling is determined based on the traffic priority.

Regarding the FFP parameters signaling, according to the agreement above the FFP parameters could be signaled through RRC but it is still FFS if the FFP parameters could be provided by SIB-1. We support FFP parameters signalling in SIB-1 since in Rel-16 FFP parameters can already be provided by SIB-1 or dedicated RRC.

- FFP parameters for UE-initiated COT could be provided by SIB-1.

The second bullet point in the agreement above is about the UE FFP periodicity and if it could be determined implicitly from other higher layer parameters.

We support the use of implicit signalling in case there is no explicit signalling of the FFP periodicity and in that case other higher layer parameters could be used. One possibility is the periodicity of the CG/PRACH resources. The use of higher layer parameters could be overridden by the explicit signalling though.

- UE FFP periodicity determined from higher layer parameters but overridden by explicit dedicated signalling.

# Conclusion

In this contribution, we have made the following proposals:

- UE processing time needs to be considered in semi-static channel access mode for configured UL transmission.

- For a DL transmission based on sharing a UE intiated COT, other UEs can recognize that the gNB is sharing the UE COT through the gNB use of a different DMRS encoding.

- . When operating on multiple LBT-BWs, FFP parameters and the assumptions regarding the COT initiator are aligned across all LBT-BWs

- In FBE mode, support enabling/disabling UE COT-initiating functionality dynamically.

- UE COT initiation enabling/disabling is determined based on the traffic priority.

- FFP parameters for UE-initiated COT could be provided by SIB-1.

- UE FFP periodicity determined from higher layer parameters but overridden by explicit dedicated signalling.

# References

- RP-201310 “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” Nokia, Nokia Shanghai Bell, 3GPP TSG RAN Meeting #88e.

- Chairman’s Notes RAN1#102e.

- Chairman’s Notes RAN1#103e.

- Chairman’s Notes RAN1#104e.

- Chairman’s Notes RAN1#105e.

- Chairman’s Notes RAN1#106e.