3GPP TSG RAN WG1 #106bis-e                                                              	        R1-2109593

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.1.2.2

Source: 	Intel Corporation

Title:	Multi-TRP enhancements for inter-cell operation

Document for: 	Discussion/Decision

# Introduction

The feMIMO WID has been revised in the last RAN plenary to clarify the scope of this AI:

- Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework

# PCI for CSI-RS

Agreement Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS : Which values to support other than 1. - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States

Up to 7 non-serving RRC-configured PCIs may be configured per CC, the exact number can be left to UE capability.

We are generally okay to have the UE report a single value X or a pair of values (X2, X2).

Proposal-1: In terms of the max number of additional RRC -configured PCIs per CC, support up to X=7.

# Rate matching

In terms of rate-matching, the simplest approach is to replicate the current rate-matching behavior of UE based on ssb-PositionsInBurst and half-frame index for the non-serving cell. In other words, UE should be provided PositionsInBurst and half-frame index corresponding to a non-serving cell and it can perform PDSCH rate-matching based on the SSB patterns in both the serving and the non-serving cell for all assigned PDSCH.

Proposal-2: Support indication of ssb-PositionsInBurst and half-frame index associated with the non-serving cell to the UE

Proposal-3: UE performs PDSCH rate-matching based on the union of ssb-PositionsInBurst and half-frame index associated with the serving cell and the non-serving cell.

# Uplink spatial relation info and PL-RS

RANP clarified that Rel-17 inter-cell mTRP is based on Rel-15/16 beam-management. It is, therefore, natural to associate a SSB Index with a non-serving cell PCID for following purposes:

- Measurements for PUCCH to the other cell: PUCCH-PathlossReferenceRS, PUCCH-SpatialRelationInfo

- Measurements for PUSCH to the other cell: PUSCH-PathlossReferenceRS

- Measurements for SRS to the other cell: pathlossReferenceRS, SRS-SpatialRelationInfo

Proposal-4: Support indication of ss-PBCH-BlockPower associated with the non-serving cell to the UE

Proposal-5: Support configuration of SSB with non-serving PCID as QCL source RS for SRS, PUCCH, and PUSCH transmission

# Conclusions

Proposal-1: In terms of the max number of additional RRC -configured PCIs per CC, support up to X=7.

Proposal-2: Support indication of ssb-PositionsInBurst and half-frame index associated with the non-serving cell to the UE

Proposal-3: UE performs PDSCH rate-matching based on the union of ssb-PositionsInBurst and half-frame index associated with the serving cell and the non-serving cell

Proposal-4: Support indication of ss-PBCH-BlockPower associated with the non-serving cell to the UE.

Proposal-5: Support configuration of SSB with non-serving PCID as QCL source RS for SRS, PUCCH, and PUSCH transmission

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86