3GPP TSG RAN WG1 Meeting #106-bis-e	R1-2109600

e-Meeting, October 11 – 19, 2021

Source:	Intel Corporation

Title:	Discussion on PUCCH Format 0/1/4 Enhancements for NR up to 71 GHz

Agenda item:	8.2.3

Document for:	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #86, a new SID on studying the support of NR from 52.6 GHz to 71 GHz (RP-193228) [1] was approved. Updated working item (WI) has been approved with the aim to extending NR up to 71 GHz [2] in the 3GPP TSG RAN Meeting #93-e. As part of the objectives of the WI, the following aspects were included:

- Physical layer aspects including [RAN1]: - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

In this context, during the prior meetings [3-6] the following agreements were made:

RAN#1 104e [3]: Agreement: Tables 1, 2, and 3 in Section 2.3 of R1-2102127 are agreed as a common set of assumptions for link level simulations and link budget calculations for evaluating enhancements to PUCCH formats 0/1/4 Note: Other parameters can be additionally considered in the evaluations Agreement: For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs - FFS: Values of N_RB for each SCS - For 480/960 kHz SCS, all REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - For 120 kHz SCS, further discuss the following two alternatives: - Alt-1: All REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping) Agreement: - The configured number of RBs for enhanced PF 0/1/4 is denoted NRB - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings - The maximum value of NRB depends on subcarrier spacing - FFS: maximum value for each SCS and each of PF0/1/4 - FFS: Allowed values of NRB within the [min/max] range - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4) - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs. - For PF4: - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers - Note: if frequency hopping is enabled, NRB is the number of RBs per hop - Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets based at least on the agreed evaluation assumptions Agreement: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Agreement: - For DMRS of enhanced PF4, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives for sequence construction: - Alt-1: A single sequence of length equal to the total number of mapped Res of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4. - Alt-2: A single sequence of length equal to the number of mapped Res per PRB of the PUCCH resource is used, and the sequence is repeated in each PRB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Agreement: - For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 as defined for Rel-16 PF4 - Further study the following and decide in RAN1#104-b: - Whether or not additional OCC lengths are supported - Down-select to one of the following alternatives for blockwise spreading - Alt-1: Blockwise spreading is performed across all allocated RBs - Alt-2: Blockwise spreading and DFT is performed per-RB followed by per-RB PAPR/CM reduction mechanism. - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact RAN#1 104b-e [4]: Agreement: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS - FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions Agreement: Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4: - Alt-1: - For enhanced PF0/1 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS - For enhanced PF4 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. - Alt-2: - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)] Agreement: For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 only, as in Rel-15/16. Agreement: For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e). Agreement: For UCI of enhanced PF4, support pre-DFT blockwise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e). Agreement: For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz. Agreement: User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion. RAN#1 105-e [5]: Agreement: - For 120 kHz SCS: - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following: - PUCCH resources before dedicated PUCCH resource configuration - PUCCH resources after dedicated PUCCH resource configuration - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details - Notes: - Alt-1 = all REs within each RB are mapped - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping) - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106 Note: No further enhancements on RB shortage issue and frequency hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration. RAN1 #106-e [6]: Conclusion: For enhanced (multi-RB) PF4, maintain the same maximum UCI payload limit as in Rel-15/16 (115 bits). Agreement: - For enhanced (multi-RB) PF4, the UCI payload is rate matched to the configured number of RBs, N_RB - Note: This is analogous to Rel-16 for PF2/3 when interlacing is configured when there is a fixed number of RBs for the configured interlace(s). Agreement: - Support an RRC parameter to configure the number of RBs for a PUCCH resource for each of enhanced PUCCH formats 0, 1, and 4 - The parameter is provided by dedicated signaling (per UE) per BWP Agreement: For PF0/1 for PUCCH resource sets prior to RRC configuration, Alt-2 (sub-PRB interlaced mapping) is not supported. Agreement: In the following, Alt-1 and Alt-2 refer to the RE mapping agreement for 120 kHz from RAN1#105-e: - For enhanced PF0/1, for PUCCH resources after RRC configuration, Alt-2 (sub-PRB interlaced mapping) is not supported. - For DMRS of enhanced PF4, only Alt-1 is supported (all REs within each RB are mapped). - Note: optimization of user multiplexing for enhanced PUCCH format 0/1/4 is not considered in Rel-17. Agreement: - For PUCCH resource sets prior to RRC configuration, support a parameter in SIB1 that indicates the number of RBs for enhanced (multi-RB) PUCCH format 0/1 Agreement: The maximum configured number of RBs, N_RB, for enhanced PF 0/1/4 is given by 16 RBs for 120 kHz SCS Agreement: For the agreed RRC parameter that configures the number of RBs for a PUCCH resource, the value range is given by the following, where N_RB_Max is the maximum number of RBs per SCS value - For enhanced PF0/1 - All integer values in the range [1 .. N_RB_Max] - For enhanced PF4 - All integer values in the range [1 .. N_RB_Max] that fulfil the requirement 2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers Agreement: The maximum configured number of RBs, N_RB, for enhanced PF 0/1/4 is given by 16 RBs for 480 and 960 kHz SCS (same as for 120 kHz SCS). Agreement: For enhanced PF0/1 support a single sequence of length equal to the total number of mapped Res of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Note: this is Alt-1 from the RAN1#104 agreement

In this contribution, the following aspects related to PUCCH enhancements for extending NR up to 71 GHz are discussed:

- Potential Enhancements on the PUCCH Common resource Sets

- Beamforming Indication

# Potential Enhancements on the PUCCH Common Resource Sets

In NR, a PUCCH resource set is provided before dedicated PUCCH resource configuration by the higher layer parameter pucch-ResourceCommon, which is indicated via a 4-bit remaining minimum system information (RMSI) index. The 4-bit RMSI indicates an entry into a 16-row table (Table 9.2.1-1 of TS 38.213, which is provided below in Table 10), where each row in the table configures a set of 16 cell-specific PUCCH resources. Each PUCCH resource is indicated by 5 parameters:

- a PUCCH format;

- a starting symbol;

- a duration;

- a PRB offset RBBWPoffset;

- a set of initial cyclic shift (CS) indices.

Some group of rows within the table are defined to have orthogonal PUCCH resources between different rows, in order to account for multiple sectorized cell sites. For instance, {index1, index2}, {index4, index5, index6}, {index8, index9, index10}, {index12, index13, index14, index 15} are defined so that orthogonal resources are assigned to different resource sets within the same group. Notice that these resource sets support only format 0 and 1: i) for PUCCH format 0, only starting symbol #12 (and additionally for shared spectrum operation starting symbol #9), and symbol duration 2 is supported; ii) for PUCCH format 1, starting symbol #10, #4, and #0, and symbol duration 4/10/14 are supported.

Table 10 - PUCCH resource sets before dedicated PUCCH resource configuration (Table 9.2.1-1 from TS 38.213)

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

During RAN1#104-e meeting [3], it was concluded that due to the regulatory restrictions imposed globally, and specifically in Europe/CEPT, USA and South Korea, which highly limit the maximum conducted power, the three PUCCH formats with single-PRB frequency domain span (i.e., PUCCH formats 0/1/4) would need to be enhanced over NRB contiguous RBs. Furthermore, in prior RAN#1 meeting [6], it has been agreed that the maximum number of PRBs required to be able to transmit using the maximum allowed conducted power would be equal to 16 RBs regardless of the subcarrier spacing (SCS), and the specific granularity within which NRB could be configured in each of the enhanced PUCCH format. However, the details on how to signal NRB for each common resources set before RRC connection has been still left for further study. In this matter, it has been already agreed that the number of PRBs NRB after RRC connection is signaled in SIB-1, and this procedure could be also applied before RRC connection, so that the configured value would be applied to all the resource set indexes.

Proposal 1: Independently of the PUCCH resource set the number of RBs used for each PUCCH resource is the same as that configured through SIB1.

Since both PUCCH format 0 and 1 are enhanced to span over a number of PRBs, which could be larger than 1, the calculation of the lowest-indexed RBs for each PUCCH resource should be modified to reflect the dependency on the number of PRBs so that to ensure no overlapping across the resource sets. In this matter, the Section 9.2.1 in TS 38.213 could be updated as follows, where in red is highlighted the updated text:

--------------------------------------------TP#1 for Section 9.2.1 in TS 38.213 -----------------------------------------------

If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the first PRB index of the PUCCH transmission in the first hop as RBBWPoffset+rPUCCHNCS∙NRB and the first PRB index of the PUCCH transmission in the second hop as NBWPsize-1+RBBWPoffset+rPUCCHNCS∙NRB, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes and NRB is the number of PRBs for the PUCCH transmission. - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the first PRB index of the PUCCH transmission in the first hop as NBWPsize-1+RBBWPoffset+rPUCCH-8NCS∙NRB and the first PRB index of the PUCCH transmission in the second hop as RBBWPoffset+rPUCCH-8NCS∙NRB - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

Proposal 2: The first PRB index for each PUCCH resource is function of the offset RBBWPoffset  provided in Table 9.2.1-1, but also the number of RBs, NRB, over which the PUCCH transmission spans.

Proposal 3: TP1 is supported.

It is also important to note that given the enhancements described above and based on the specific configured number of NRB, for some numerologies (i.e., subcarrier spacing) and bandwidth, and some of the NR PUCCH resources sets, the total number of PRBs used for the transmission of a PUCCH may be so large that the PUCCH resource partitioning in frequency domain is no longer possible without maintaining orthogonality among the resource sets. In particular, the aforementioned issue arises for PUCCH format 1, for 480 KHz SCS. For instance, when 480 KHz SCS and system bandwidth of 400 MHz are used. In this case,

- If NRB >11, 4 of the resource sets (index 6, 10, 14, and 15) can no longer be used;

- If 1<= NRB <=11, index 15 can no longer be used.

For instance, if we assume 480 MHz SCS with system bandwidth of 400 MHz and 66 PRBs available, and a value of NRB=12, which has highlighted in [7] may be a valid configured value for fixed wireless deployments, when for instance index 6 is used, the first and second hop will be colliding with resource for index 5, while in addition the resources for index 5 will not be properly spaced between one hop and the another. It is important to highlight that this issue arises for PUCCH format 1 when the system operates with 480 KHz SCS and system bandwidth of 400 MHz, and when NRB >1. In this case only 9/13 of the total resources based on Table 9.2.1-1 from TS 38.213 could be effectively used, but the remaining 4/13 could not be used.

Observation 1: For 480 KHz SCS with system bandwidth of 400 MHz, if NRB >1, up to 4/13 of the resource sets devoted for PUCCH format 1 could not be used since the total number of PRBs would not be sufficient to support at least same partitioning in frequency domain as legacy design.

Given that the aforementioned issue limits not only the resource sets that can be used, but more importantly the multiplexing capability of the system, RAN1 should further discuss how to enhance the PUCCH common resource sets so that for each resource group at least the same number of orthogonal resources as legacy design are supported. In this matter, while we acknowledge that the following note has been agreed during a prior RAN1 meeting [5],

Note: No further enhancements on RB shortage issue and frequency hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration.

our understanding is that this note was agreed under the assumption that the maximum number of PRBs used for enhanced PUCCH would remain the same as per initial agreement (12/3/2 for 120, 480 and 960 KHz SCS), and in that scenario/assumptions we indeed thought that there was no technical motivation to enhance the resources sets before RRC configuration. However, since now based on the recent agreements made during the last meeting [6] the number of PRBs could be potentially configured up to 16 for 480 KHz SCS, and as shown in [7]12 PRBs is actually a realistic value that could be indeed used, we have concerns that if we leave the resources sets before RRC configuration as in legacy, the available bandwidth would not be enough to support frequency domain partitioning, and multiplexing capability will be highly constrained even for feasible and frequently used values. We feel that coverage for initial access signals/channels is equally important and should be enhanced appropriately. In this case, similarly as done for Rel.16 NR-U, once the specific resource sets that need to be enhanced are identified, an additional starting symbol or OCC index could be added for them so that to mitigate the aforementioned issue.

Proposal 4: RAN1 should further discuss possible enhancements to PUCCH resource sets before dedicated PUCCH resource configuration such as additional starting symbol or OCC index similarly as done in Rel.16 NR-U.

# Beamforming Indication

One important aspect to consider when properly configuring the number of PRBs over which the enhanced PUCCH format 0/1 and 4 spans, is the influence of UE’s transmit beamforming gain, TxBF. In this context, in [7] it has been shown that the maximum number of PRBs required for different TxBF values highly depends on the value of transmit beamforming gain. However, when the gNB configures the number of PRBs that a UE may use, it is currently unaware of the specific value of the UE’s TxBF.

In order to measure the impact of the gNB when this wrongly guesses the UE’s TxBF, each subfigure within Figure 4 shows two different curves: i) via transparent bars it is shown the maximum achievable MIL for different UE’s output power and EIRP when the UE has a TxBF=0 dBi, and the gNB makes the right assumption; ii) via colored bars it is shown the maximum achievable MIL for different UE’s output power and EIRP when the UE has a TxBF=0 dBi, and the gNB makes the assumption that the TxBF= 6dB. Figure 4 provides results for 120, 480 and 960 kHz SCS and shows that due to the mismatch between the assumed and actual TxBF, the MIL performance may degrade quite significantly (order of 1-5 dBs) for certain choice of number of PRBs and the UE’s output power and for different values of EIRP.

Figure 4 - Achievable MIL performance when the UE’s TxBF is the same as that assumed by the gNB (transparent bars) and achievable MIL performance when the UE’s TxBF is different than what assumed by the gNB. From left to right: SCS=120kHZ(left subfigure), SCS=480kHZ(middle subfigure), and SCS=960kHz (right subfigure).

Observation 2:  Regardless of the SCS used and the maximum UE’s output power and EIRP, there is a big loss in MIL if the gNB is not aware of the correct UE’s transmit beamforming gain.

While a gNB could potentially take always a pessimistic approach and assume the UE’s beamforming gain is 0 dBi, this will come at the cost of a very inefficient spectrum utilization with reduce multiplexing capability, especially since it is quite likely that UEs may employ directional transmissions which require/utilize much higher beamforming gains, whose effectively require much smaller number of PRBs to achieve same coverage. In this matter, Table I, II and III show for different subcarrier spacing, the number of PRBs that may need to be configured (accounting for the fact that the maximum number of PRBs that can be configured is 16) to achieve maximum MIL when the beamforming gain is 0dBi (first value in black) or 6dBi (second value in red) for different values of UEs EIRP and output power.

Table I – Number of PRB required for 12 KHz when TxBF = 0 dBi and TxBF = 6 dBi

Table II – Number of PRB required for 480 KHz when TxBF = 0 dBi and TxBF = 6 dBi

Table III – Number of PRB required for 960 KHz when TxBF = 0 dBi and TxBF = 6 dBi

This tables show that impact in terms of spectrum efficiency is quite large, since in may cases a UE may be configure to use up to more than 5 times the number of PRBs that may require.

Observation 3:  If the gNB is not aware of the correct UE’s transmit beamforming gain, by using a pessimistic approach and assuming that the UE’s transmit beamforming gain is 0 dBi, the gNB may configure up to more than 5 times the number of PRBs that would be otherwise needed.

Proposal 5: RAN1 should discuss a proper framework to implicitly or explicitly indicate the UE’s beamforming gain to the gNB.

# Conclusions

In this contribution, we discussed several aspects related to PUCCH enhancements for extending NR up to 71 GHz, and we derived the following proposals, and observations:

Proposal 1: Independently of the PUCCH resource set the number of RBs used for each PUCCH resource is the same as that configured through SIB1.

Proposal 2: The first PRB index for each PUCCH resource is function of the offset RBBWPoffset  provided in Table 9.2.1-1, but also the number of RBs, NRB, over which the PUCCH transmission spans.

Proposal 3: TP1 is supported.

Observation 1: For 480 KHz SCS with system bandwidth of 400 MHz, if NRB >1, up to 4/13 of the resource sets devoted for PUCCH format 1 could not be used since the total number of PRBs would not be sufficient to support at least same partitioning in frequency domain as legacy design.

Proposal 4: RAN1 should further discuss possible enhancements to PUCCH resource sets before dedicated PUCCH resource configuration such as additional starting symbol or OCC index similarly as done in Rel.16 NR-U.

Observation 2:  Regardless of the SCS used and the maximum UE’s output power and EIRP, there is a big loss in MIL if the gNB is not aware of the correct UE’s transmit beamforming gain.

Observation 3:  If the gNB is not aware of the correct UE’s transmit beamforming gain, by using a pessimistic approach and assuming that the UE’s transmit beamforming gain is 0 dBi, the gNB may configure up to more than 5 times the number of PRBs that would be otherwise needed.

Proposal 5: RAN1 should discuss a proper framework to implicitly or explicitly indicate the UE’s beamforming gain to the gNB.

# References

- RP-193228, “New SID Study on supporting NR above 52.6 GHz”, Intel Corporation, 3GPP TSG RAN Meeting #86, Dec. 2019.

- RP-212637, “Revised WID: Extending current NR operation to 71GHz,” Qualcomm, Intel, 3GPP TSG RAN Meeting #93-e, Dec. 2020.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #104 e-Meeting, February 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #104b e-Meeting, April 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #105 e-Meeting, May, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106 e-Meeting, August, 2021.

- R1-2107579,“ Discussion on PUCCH Format 0/1/4 Enhancements for NR up to 71 GHz”, Intel Corporation, 3GPP TSG RAN WG1 #106-e meeting, August 2021.