3GPP TSG RAN WG1 Meeting #106-bis-e	R1-2109603

e-Meeting, October 11 – 19, 2021

Source:	Intel Corporation

Title:	Discussion on Channel Access Mechanism for extending NR up to 71 GHz

Agenda item:	8.2.6

Document for:	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #86, a new SID on studying the support of NR from 52.6 GHz to 71 GHz (RP-193228) [1] was approved. Updated working item (WI) has been approved with the aim to extending NR up to 71 GHz [2] in the 3GPP TSG RAN Meeting #93-e. As part of the objectives of the WI, the following aspects were included:

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement - Radio interface protocol architecture and procedures [RAN2]: - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified. - Core specifications for UE, gNB and RRM requirements [RAN4]: - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range. - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - Specify RRM/RLM/BM core requirements. Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives. Note 2: UEs supporting a band in the range of 52.6GHz-71GHz are not required to support 480kHz SCS and 960kHz SCS. Note 3: The maximum FFT size required to operate the system in 52.6GHz-71GHz frequency is 4096, and the maximum of RBs per carrier is 275 RBs. Note 4: the system is designed to support both single-carrier and multi-carrier operation. Note 5: RAN plenary will decide whether new FR (e.g. FR3) shall be defined for the frequency range from 52.6GHz-71GHz or the existing FR2 shall be extended to cover frequency range from 52.6GHz-71GHz. Similar to regular NR and NR-U operations below 52.6GHz, NR/NR-U operation in the 52.6GHz to 71GHz can be in stand-alone or aggregated via CA or DC with an anchor carrier.

In this context, during the past RAN1 meetings [3-6] the following agreements were made:

RAN1#104-e [3]: Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP Agreement: For LBT for single carrier transmission, consider the following alternatives - Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) - Alt SC.2. gNB/UE performs LBT over the transmission bandwidth (from the lowest RB to the highest RB used for the transmission) - Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth For LBT for multi-carrier transmission in intra-band CA, consider the following alternatives - Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately - Alt CA.2. gNB/UE performs single LBT over all CCs - Alt CA.3. gNB/UE performs multiple LBT, one for each CC over the transmission bandwidth (from the lowest RB in to the highest RB used for the transmission in the CC) - Alt CA.4. gNB/UE performs LBT over the transmission bandwidth over all CCs (from the lowest RB in the lowest CC to the highest RB in the highest CC used for the transmission) - Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC Note: supporting more than one alternative for at least multi-carrier transmission in intra-band CA is not precluded. Agreement: For energy measurement in 8us deferral period, down-select from the following: - Alt 1. Two energy measurements are required - Alt 2. One measurement is required - Alt 3. Extend the 8us to 10us and perform two measurements, one in each 5us segment For energy measurement in 5us observation slot, perform single measurement - FFS minimum duration of the measurement - FFS location of the measurement Agreement: On maximum gap within a COT to allow COT sharing without LBT, down-select from - Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration - Alt 2. Define a maximum gap X, such that a later transmission can share the COT without LBT only if the later transmission starts within X from the end of the earlier transmission - FFS: Value for X - Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT - FFS: Value for Y - FFS: How to define the one-shot LBT Agreement: For Cat 2 LBT, down-select from the following alternatives - Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation - Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation Agreement: If Cat 2 LBT is introduced, the following use cases can be further studied: - Resume transmission after a gap Y: Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y) - COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT - Multi-Beam LBT: Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z) - Rx-Assistance: Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling Other use cases not precluded. FFS if Cat 2 LBT is mandated for each use case or not. Agreement: For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion - Alt 1. Legacy RSSI measurement and reporting with possible enhancements - Alt 2. AP-CSI report with possible enhancements - Alt 3. LBT at receiver - Alt 3.1 eCCA - Alt 3.2 Cat2 LBT Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of "cover" - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch Agreement: Define Type A and Type B multi-channel channel access as: - Type A: Perform independent eCCA for each channel - Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot Down-selection between - Alt1: Support Type A multi-channel channel access only - Alt2: Support both Type A and Type B multi-channel channel access. Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed Agreement: - SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signalling based SSB transmission is not met - Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams - FFS: If any difference from a multi-beam COT LBT needs to be introduced RAN1#104b-e [4]: Working assumption: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT. Agreement: - Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH. - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule - FFS: Whether this can be applied to all supported SCS or specific SCS. - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval) - FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc Working assumption: For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us. Agreement: For LBT for single carrier transmission, continue down selection between - Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) - Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth For LBT for multi-carrier transmission in intra-band CA, continue down selection between - Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately - Alt CA.2. gNB/UE performs single LBT over all CCs - Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between - Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication - Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication - FFS: Whether the indication of the decision on applying LBT mode or no-LBT mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation) - FFS: Whether a gNB and its UE(s) can have different mode - FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication Agreement: For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission. - RMSI PDCCH and RMSI PDSCH - Other broadcast PDSCH - PDSCH without user-plane data - PDCCH - CSI-RS - PRS - Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst) Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval. FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission. RAN1#105-e [5]: Agreement: For energy measurement in 8us deferral period, continue down-selection between the following alternatives - Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us - Alt 2. One measurement is required - FFS where the measurement is located Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2) Agreement: On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives - Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration - Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode - Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc RAN1 #106-e [6]: Agreement: For energy measurement in 8us deferral period, at least a single measurement within 8us is performed, and the measurement duration is selected from one of the following alternatives: - Alt 1: At least 3+X us (FFS X, such as X=1). - Alt 2: At least X us, where X is the same as the minimum measurement duration in a 5 us observation slot and is within the 5 us observation slot. - Alt 3: At least a contiguous duration of X+Y us where the Y us part of the measurement is done at the end of the first 3 us and X is the same as the minimum measurement duration in a 5 us observation slot and is at the beginning of the 5 us duration. Agreement: For energy measurement in 8us deferral period, Alt 2 is supported while Alt 1 and Alt 3 can be considered as gNB/UE implementation (Alt. 1/2/3 are defined as per previous agreement) Conclusion: There is no consensus in RAN1 to support the functionality of accessing a carrier if there is interference in part of the carrier in frequency. Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements) Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support a multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded Agreement: For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e - Scheme 1: L1-RSSI based receiver assistance - Resource used for RSSI measurement - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS - FFS: any enhancement needed for ZP-CSI-RS for this purpose (eg., ZP-CSI-RS over all REs in BWP over one or more symbols). - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval - L1-RSSI is reported in an AP-CSI report - L1-RSSI trigger in UL grant - FFS if L1-RSSI trigger can also be carried in DL grant - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed - FFS: How to indicate the measurement beam for L1-RSSI - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc - Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - FFS if the downlink data transmission can be granted with the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission, in which case, the CCA or eCCA is performed for at least the first UL PUCCH/SRS transmission - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - Scheme 3: CCA or eCCA based receiver assistance with new RTS/CTS type transmission - New RTS/CTS-like signaling introduced. - gNB sends RTS-like signaling to UE. UE performs CCA or eCCA and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed CCA or eCCA. After detecting the CTS-like signal, the data transmission happens - Scheme 4: Legacy L3-RSSI with potential enhancements - FFS potential enhancements, e.g., supporting gNB indicating the beam used for UE RSSI measurement, supporting gNB indicating new reference SCS and measurement bandwidths - Note: The schemes listed above are not mutually exclusive and should be discussed separately.

In this contribution, the following aspects related to channel access are discussed:

- Further details on general aspects related to the LBT procedure;

- Further details related to the COT sharing procedure;

- Discussion on the additional uses for CAT-2 LBT;

- Discussion on LBT procedure for multi-beam transmissions;

- Discussion on CWS and CAPC;

- Discussion on receiver-aided-LBT procedure;

- Discussion on the relationship between measurement and TX beam;

- Discussion related to short control signal exemption.

# Discussion on Channel Access Mechanism

## Further Details on the General Aspects of the LBT Procedure

While during prior meetings [3-6] the framework related to the LBT procedure has been established, some details still require some discussion. For instance, among others while it has been agreed when and how the measurements are done within a 5us and an 8us observation window, it is still left for further discussion how long the measurements should last. In this matter, while it would not be wise to define a specific value, since this may depend on implementation and hardware constrains, it may be still important to define at least a minimum value so that to make sure each device may properly implement this procedure so that to properly address co-existence and interference mitigation. With that said, to resample the IEEE 802.11ad/11ay, the minimum measurement window could be set to 2us.

Proposal 1: Within a 5us or 8us observation window, a device must perform a measurement of the medium for at least 2us.

During the RAN1 #104-e meeting [3], it has been agreed to use as a baseline the following formula for the ED threshold calculation:

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

where Pmax is the RF output power limit, and Pout is the maximum EIRP of the initiating device during a COT, where Pout≤Pmax. However, a few additional details were left for further study. In particular one aspect related to the ED threshold calculation that is important to discuss is that depending on the wideness of the beam that is used during the LBT procedure, this may change the assessment on whether a channel is idle or not. During the SI, some study was conducted [8], and it was found that in general when low ED thresholds are used, LBT performed with more directional beams may overperform LBT performed using a wider beam given that the level of protection offered by the later gets increased, which may help sufficiently mitigate the hidden node issue bringing this in par with that of LBT performed with more directional beams while still offering better spatial reuse than that. In this matter, it may be beneficial within the ED threshold calculation to also account more specifically for the measurement beam used so that to exploit the advantage described above. In this matter our view is to add an additional component to the formula provided above, where this component would depend on the relationship between a sensing and a transmission beam. In particular, this component may be equal to 0 if the sensing beam is equal to the transmit beam.

Proposal 2: When operating in unlicensed 60 GHz band, the ED threshold calculation shall account for the sensing beam used to perform the LBT procedure through an additional component which is added to the already agreed ED threshold formula.

In the context of the ED threshold calculation, another aspect that must be considered is the case when the absence of an incumbent technology can be guaranteed due to a specific use case or deployment. In this case, it may be appropriate to leave up to the gNB to decide which value of the ED threshold should be used. Note that similar design consideration was drawn in both LAA and NR-U.

Proposal 3: In case the network is able to assess the absence of any other incumbent technology, the ED threshold value that a device may use during the LBT procedure is up to the gNB and may be configured via higher layer signaling.

## Further details related to the COT Sharing Procedure

In previous meeting [6] the following agreement has been made:

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed

In this matter, a few details have been left for further study and discussion. One among them is how the gNB would indicate whether Alt-1 and Alt-3 should be used when a UE is capable of supporting both procedures. In this matter, dynamic signaling could be used, and it is preferred since generally the UE is not aware of the gaps across bursts. In particular, the fields defined during Rel.16, which are contained within the scheduling DCIs (i.e, ChannelAccess-CPext and ChannelAccess-CPext-CAPC) and that carry information related to channel access information, could be repurposed and utilized.

Proposal 4: When a UE is capable to perform Cat-2 LBT, whether to operate with or without Cat-2 LBT would be dynamically indicated by the gNB via scheduling DCIs.

Another detail that needs to be also resolved is related to how to define the maximum gap Y. In order to simplify the design and avoid appending useless transmissions that would be otherwise needed if at the end of the Cat-2 a transmission may not align with a symbol boundary, our preference is for Option 2. In particular, our preference is to define the gap Y to be the minimum number of OFDM symbols that would be at least 8us long, so that to guarantee a sufficient gap to perform the 8us observation period which composes the Cat-2 LBT. In this matter, the length of Y would depend on the subcarrier spacing and would be equal to:

- 1 OFDM symbol for 120 KHz SCS,

- 4 OFDM symbols for 480 KHz SCS,

- 8 OFDM symbols for 960 KHz SCS.

Proposal 5: Y is defined as:

- 1 OFDM symbol for 120 KHz SCS,

- 4 OFDM symbols for 480 KHz SCS,

- 8 OFDM symbols for 960 KHz SCS.

Another aspects that should be also discussed is related to the whether Alt-3 should be also applied to the initiating device when it transmits bursts which are not contiguous in time domain, and there may be pauses in between. In this matter, the regulatory requirement for Japan [11], which is mandated by ARIB, is very generic and does not define any concept of initiating or responding device, but rather distinguishes a device from a transmitter to a receiver.

- Interference mitigation function - Sending and receiving identification signals (Enforcement Article 6-2) (Facilities Article 9-4) Shall automatically transmit or receive identification codes. - Carrier Sense (Facilities Article 49-20) If the transmission power of the transmitter exceeds 10 mW, provide a carrier sense that will operate at beginning of the transmission.

Therefore, in our understanding the aforementioned agreement should be also extended in case of pauses.

Proposal 6: If an initiating device is capable to perform Cat-2 LBT, and if the initiating device performs an additional burst within the initiated COT which may be separated with any prior burst of at least a minimum gap Y, then under Alt-3 a Cat 2 LBT is needed before the initiating device transmission.

## Discussion on the Additional Uses for CAT-2 LBT

While in prior RAN#1 meeting [6] it has been agreed to support the use of CAT-2 for at least for COT sharing, RAN1 has identified a few additional possible use cases where CAT-2 LBT could be additionally used. For instance, the list includes:

- RX-Assisted LBT

- Type B channel access mode

- Beam switching

- Multi-beam TDM COT

As discussed in Sec. 2.6, the introduction of a RX-assisted LBT with the use of Cat-2 LBT is technically well motivated and has been supported by simulation campaigns provided by several companies during the SI [11]. Also as discussed later more in details within this document, among all the other proposed schemes for RX assisted LBT, the RX-assisted LBT with the use of Cat-2 LBT seems to be the most feasible considering the possible specification impact, and amount of time to discuss all the details.

Proposal 7: In addition to support CAT-2 LBT for COT-sharing procedure, the gNB may configure the UE to use CAT-2 LBT for RX-assisted LBT

During both LTE LAA and Rel.16 NR-U, following the ETSI BRAN guidelines for sub-6 GHz Band [9], the following two types of LBT procedure were adopted for multi-carrier operation:

- Type A – an initiating device performs channel access on each carrier by performing Cat-4 LBT.

- Type B - the eNB selects a single carrier, namely primary channel, which can be either uniformly picked across all carriers or selected. In either cases, the eNB cannot change the primary channel more than once every second. The initiating device can sense other carriers with Cat-2 LBT only if the initiating device completes the Cat-4 LBT for the primary channel.

However, it is important to note, that for above 60 GHz band, the regulatory requirements [10] do not support CAT-2 LBT for type B channel access mode, and applying CAT-2 LBT to initiate a COT may be against the adaptivity mechanism defined for this band.

Proposal 8: Do not support, type B channel access mode for multi-carrier operation.

As for CAT-2 applied to multi-beam switching and multi-beam TDM COT, there doesn’t seems to be a strong technical motivation to support these procedures, if the intention is to utilize a wide beam LBT to firstly acquire all the beams, and then apply a single shot LBT for each beam each time a switching occurs. If instead, the intention is to apply a CAT-4 LBT to sense the channel over a narrow beam, and then apply CAT-2 each time a beam switching occurs on a new narrow beam, similarly as type B channel access, this may not be allowed by the ETSI BRAN [10] since testing would need to be performed per transmitting beam, and the LBT procedure used may not be complaint with the adaptivity mechanism defined in this band for each beam. In both case, Cat-2 LBT for multi-beam switching and multi-beam TDM COT should not be supported.

Proposal 9: Do not support, Cat-2 LBT for multi-beam switching and multi-beam TDM COT.

## LBT Procedure for Multi-Beam Transmissions

Another important topic to discuss is related to the LBT procedure to employ in case of a MU-MIMO transmissions. In this matter, two alternatives were identified in prior meetings: i) Alt-1: single LBT sensing with wide beam cover performed at the start of the COT; ii) Alt-2: independent per-beam LBT performed at the start of the COT. Furthermore, different ways to interpret Alt-2 have been identified:

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

In our view, both Alt-1 and Alt-2 should be supported at the gNB since whether the CCA measurement may be performed on a wide-beam or a per-beam manner is capability dependent, and therefore whether to use one or the other could be left up to implementation. As for Alt-2, both Alt-A-2 and Alt-B could be considered, where Alt-A-2 is preferred to other Alt-A sub-options since this limits the time between when the measurement is performed, and when the channel is actually acquired, which inherently mitigates either co-existence and possible interferences.

Proposal 10: For a COT with MU-MIMO, both Alt-1 and Alt-2 are supported.  As for Alt-2 both Alt-A-2 and Alt-B could be considered.

Similar considerations can be also drawn for the LBT procedure of a COT within which beam switching occurs. In this matter, both a single wide-beam measurement or a multiple per-beam measurement could be performed depending on the device capability. Also for the case when multiple per beam measurements could be performed, RAN1 should consider to adopt both simultaneous as well as measurements operated in a TDM fashion where a device attempts to acquire the channel soon after assessing that the channel is idle.

Proposal 11: For a COT with beam switching, both single LBT sensing with wide beam and independent per-beam LBT sensing at the start of the COT are supported.

In the context of multi-beam operation, it is also important to highlight two aspects:

- beams with overlapping coverage could concurrently perform LBT, and therefore multiple overlapping COT acquired through different beams could be possible, and the related handling should be considered;

- given that a device could perform directional sensing over individual beam, the COT should be considered to be acquired only in the transmission beams for which the LBT is performed and the LBT measurements have indicated that the channel is idle. If the device instead performs LBT over a single beam, and assumes instead that the COT is acquired for all other transmission beams, by performing spatial multiplexing across different transmission beams, the system may induce unwanted interference in directions where transmissions may be already occurring, since the device has not performed any LBT measurements in that direction.

Proposal: 12: An initiating device is able to initiate multiple overlapping COT over different beams.

Proposal 13: When directional sensing is performed, the COT should be considered to be acquired only in the transmission beams for which the LBT is performed and the LBT measurements have indicated that the channel is idle.

If directional LBT is concurrently used to acquire the channel in multiple directions, an initiating device by potentially performing multiple CCA procedures over those directions it would be able to initiate multiple concurrent COTs. In this case multiple concurrent COTs could be active, and these should be considered separately, unless these overlap in terms of beams when performing the LBT measurements. An example of this issue if provided in Figure 7. Figure 7 illustrates the case when the gNB attempts to initiate three COTs through three different directional LBT. The COT for UE1 is acquired with LBT performed over beam#1 and beam#2, the COT for UE2 is acquired with LBT performed over beam #2 and beam #3, and the COT for UE3 is acquired with LBT performed over beam #3 and #4. In this specific case, the beam #2 and #3 may belong to two different group of beams. In this last case, it is important to define how the MCOT would be counted if multiple COTs have overlapping beams that have been used during their respective CCA procedures.

Figure 7- Illustration of directional LBT over multiple directions

Proposal 14: When directional sensing is performed, and multiple concurrent COTs are acquired, these should be independently treated unless LBT measurements have overlapping beams. In this case, RAN1 should define some rules on how to handle these cases.

As described above, if directional LBT is concurrently performed in multiple directions, an initiating device would be able to initiate multiple concurrent COTs. However, if it is equipped with a narrow-beam receiver it may be able to receive transmissions from one direction at the time. For this reason, a gNB may need to schedule time domain resource allocation (TDRA) across UEs so that these are not overlapping either within or outside its COTs, and they are allocated so that the gNB is potentially able to perform beam pair with a specific UE before this can transmit. However, given that LBT is required by an initiating device to acquire a COT and if directional LBT is used a link cannot be established in a deterministic manner since it is subject to the success of LBT from the initiating device and possibly also by the responding device on top of being subject to the successful exchange of RTS/CTS like messages among the two, then it may be possible that a transmission to or from a UE may never take place, so if some resources are exclusively allocated to a UE, those will be inevitable lost with consequent degradation of the spectral efficiency of the system.  However, this degradation may still arise even if TDRAs for the UEs overlap. In fact, while the gNB may exactly know which UEs may be transmitting, it may not know which one would be able to transmit, and by the time it may determine which UE may be transmitting some of UL transmissions may be lost. Given this problematic, RAN1 should further discuss a valid solution.

Proposal 15: RAN1 should further study how to efficiently allow beam-pairing due to LBT success.

## Discussion on CWS and CAPC

The channel access procedure mandated by ETSI EN 302 567 [10], and agreed during the SI [11] has some similarities with the CAT-4 channel access procedure defined and used in both LAA and Rel.16 NR-U. However, for CAT-4 channel access procedure four channel access priority classes (CAPCs) were available to accommodate for different types of traffic and quality of service (QoS), as also mandated by the text in ETSI BRAN 301 893 [9]. In addition, for each CAPC, the minimum Zmin and maximum Zmax values of the contention window, were changed from time to time based on the HARQ feedback information related to a reference portion of the prior transmission burst to update the back-off counter window based on the contention of the medium. Furthermore, the maximum supported COT differs for each CAPC, which was linked to specific QoSs. While these concepts are not explicitly captured in the ETSI EN 302 567 [10], they are neither precluded as well. Therefore, given that these procedures are well established in the specification, and allow to address different channel and traffic conditions that may impact the channel access procedure, RAN1 should consider to adopt them in the above 52.6 GHz band with the necessary modifications.

Proposal 16: For operation unlicensed 60 GHz band, when LBT is used within the COT, the principle of the type 1 channel access procedure defined for the sub-6 GHz band should be reused, and the channel access parameters should be modified in accordance with numerologies provided by the ETSI BRAN Harmonized Standard.

Proposal 17: The procedure specified in NR-U related to the CWS adjustment should be considered for operation in unlicensed 60 GHz band. RAN1 should further discuss and identify the values Zmin and Zmax.

## Discussion Receiver-Aided-LBT Procedure

In prior RAN#1 meeting [6], in the context of receiver-aided LBT, it was made a comprehensive agreement which listed four different schemes:

- Scheme 1 - L1-RSSI based receiver assistance

While possible gains from this scheme have not being provided by companies, we believe that it may potentially provide some gains by providing information related to the channel occupancy status at the UE. However, as indicated in one of the note of the related agreement, for this scheme to be useful the timeline for L1-RSSI reporting should be tightened, which is strictly related to the UE’s processing time. For this purpose, if scheme 1 is supported, the use of this scheme should be left up to UE’s capability.

- Scheme 2 - CCA or eCCA based receiver assistance with existing phy channel/signals:

This among the four schemes the only one that seems to be more technically motivated and for which companies have disclosed the potential advantage during the simulation campaign done during the SI [11]. In this matter, in order to minimize the LBT overhead at the receiver, which for a CAT-4 LBT may be quite substantial, and may overcome the gains in performing a receiver assisted procedure, only options which employ a CAT-2 LBT at the receiver should be supported. Furthermore, given that CAT-2 itself is supported based on UE’s capability, then this whole scheme should be left up to UE’s capability.

- Scheme 3 - CCA or eCCA based receiver assistance with new RTS/CTS type transmission:

Considering the amount of time left for discussion for this WI, agreeing on scheme 3 is very ambitious, and considering that this won’t rely on existing channels/signals it may require a lot of spec impact. Therefore, scheme 3 should not be considered further.

- Scheme 4 - Legacy L3-RSSI with potential enhancements

While this scheme is the one that would require less specification impact, its benefits are dubious, since the reporting of the channel occupancy status may be infrequent enough that the effective channel occupancy status at a given time would not be well captured. In this matter, the use of the channel occupancy status report may not be counter-productive and actually may end up degrading overall system performance.

With that said, while scheme 1 and 2 could be supported, scheme 3 and 4 should not be considered further. Furthermore, for scheme 1 and 2, it use should be left up to UE’s capability.

Proposal 18: For receiver-assisted LBT procedure both scheme 1 and 2 could be supported, where both scheme 1 and 2 could be used up to UE’s capability.

## Discussion on the Relationship Between Measurement and TX Beam

In the last RAN1 meeting [6], two alternatives to define relative relationship between all applicable sensing beam(s) and the transmission beam(s) were considered. Furthermore, it was agreed to target down-selecting between the two alternatives during this meeting. The following is copy of the agreement:

Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support a multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded

Among the two alternatives, we believe alternative 2 is the right approach for systems operating in FR2-2. As discussed in our previous contribution [12], standardizing well defined behavior for UEs is quite critical for optimizing cell deployments and stabilizing network operations. From the specification perspective, we believe behavior definition can focus on the UE and we do not need to necessarily define behavior definitions for the gNB. This is because regulatory requirements that impact the gNB do not just limit measurements and when signals can be transmitted, but impact the entire scheduling process, which include selection of RBs, power loading of the signals, how and when to schedule UEs, etc. Furthermore, gNB needs to work with single beam, and multi-beam in a complex manner. It would be impractical to consider all the complex scheduling possibilities that the gNB can perform in the behavior set for performing LBT as they are integrally tied together. On the other hand, in order for the gNB and the network as whole to optimize the network well, a well predictable UE’s behavior is required. If LBT sensing and transmission direction selections are completely left up to UEs, it would be extremely difficult for the gNB to predict and assess best scheduling strategies for the given network. As such, we suggest focusing on defining LBT measurement behaviors for the UE.

Proposal 19: Take Alt 2, “extending the beam correspondence framework and QCL, TCI, SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s)” as baseline for further discussion and standards development.

For the gNB side, there are two options for Alt 2.

- Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation. No testing or enforcement introduced in 3GPP spec for this option

- Option 2: Beam correspondence at gNB side is assumed. Support define gNB behaviors.

As discussed previously, gNB transmission beam selection can be quite complicated and impacts algorithms developed to handle scheduling decisions.  Therefore, we believe it is best to leave the beam selection aspect up to implementation (option 1). The impact of varying degree of transmission behavior and beam selection among various gNBs is expected to transmission and beam dynamic stemming from scheduling various UEs with various application requirements. Therefore, the effect of unifying gNB behaviors is not expected to be critical for deployments compared to unifying UE behaviors. There are more UEs that can be deployed in the network and in order for cells to optimize the network performance, it is critical to have predictable UE behaviors.

Proposal 20: Support option 1, “The selection of eligible sensing beam for a transmission beam is left for gNB implementation”, for gNB side of alternative 2.

For the UE side, to resolve the FFS for alternative 2, “What if beam correspondence is not supported at UE.”, we believe 3GPP should mandate support of beam correspondence for FR2-2 unlicensed operation. In order for UEs operating in Europe to comply with the ETSI BRAN requirements, they will need to go through the conformance testing procedure for checking medium access rules as met. One of the key components for ETSI BRAN testing included within EN 302 567 v2.2.0 [6] is pasted here:

“When performing testing on a UUT with a directional antenna system (such as array capable of beam-forming), the wanted communication link (between UUT and companion device) and the interference test signals shall be aligned to the direction corresponding to the UUT's maximum EIRP.”

This text implies that a device must have some beam correspondence implemented. Otherwise, if device have completely uncorrelated and decoupled Tx and Rx beams, it can potentially fail the aforementioned ETSI BRAN conformance test. Considering the nature of narrow beam operation in 60 GHz band, we think requiring beam correspondence for the UEs that are operating in the above 52.6 GHz unlicensed band is something that would be required. Given that mandatory features with capability signaling are feature that can be determined to be set to 1 (i.e. enable) by 3GPP, we suggest to enable the beam correspondence in the above 52.6 GHz unlicensed band for all UEs that support operation in this band.

Proposal 21: Beam correspondence mandatory capability signaling is set to 1 for all supported unlicensed bands in FR2-2.

For resolving the second FFS, how and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework, we think use of QCL framework doesn’t necessarily limit how the UE can form beams, as the QCL framework only provides indirect method of controlling how UE should perform Rx beamforming. Therefore, supporting wide sensing beam can be left up to UE implementation, i.e. option 1.

Proposal 22: Support option 1, up to UE implementation on how to support wide sensing beaming using QCL/TCI framework, for UE side of alternative 2.

Defining UE’s behavior to directly or indirectly control spatial filters used for transmission and reception is already supported in NR. These features are QCL framework for downlink and spatial relation information signaling framework for uplink. QCL indication is used to implicitly let the UE know what Rx beams could be used for reception of a signal/channel. The same principle can be applied to LBT measurement resources. When PDCCH schedules a PUSCH transmission, gNB could indicate the QCL type-D indication for LBT measurement resources such that source RS for QCL type-D is the same source for spatial relation information for the SRS resource indicated for the PUSCH. This will implicitly inform the UE that LBT measurement should be using the same beam direction as the transmit beam direction as the PUSCH. An illustration of the explained example is provided in Figure 8.

Figure 8 – illustration of how QCL framework can be used to guide beam directivity for LBT measurement prior to transmission.

As an alternative to using QCL framework, spatial relation information signaling framework can be leveraged for LBT measurements as well. Spatial relation information allows the UE to know which transmit beam to use for PUSCH/PUCCH/SRS transmission by indicating either DL reference signal, SSB or CSI-RS, or UL reference signal, SRS. The former is used when beam correspondence is available to the UE. Assuming beam correspondence is available, the spatial relation information can be used to directly indicate the spatial filter that should be used for operations. In the case of LBT measurements, we could simply state that the UE should use the same spatial filter as the intended transmission during the LBT measurement period. An illustration of an example using spatial relation information framework for LBT measurement is shown in Figure 9.

Figure 9 – illustration of how spatial relationship information framework can be used to guide beam directivity for LBT measurement prior to transmission.

Between using QCL and spatial relation information framework, use of QCL framework will provide a bit more flexibility in terms of how Rx beams are formulated during LBT, while governing the general directivity of the LBT measurement beams. At the same time, we expect a bit more work to be done within the specification as we would need to define how and when to use specific QCL type-D sources for LBT measurements for the different cases. The spatial relation information framework is expected to require much less specification effort, and a simple text that just states that UE should use the same spatial filter as the transmission may be able to cover the required UE’s behaviors. However, the drawback in this case is that the use of the same spatial filter may be quite strict in terms of how a UE may need to be implement the Rx beams.

Proposal 23: RAN1 should extend the QCI or Spatial Relation Info framework to define and indicate the sensing beam associated with a transmission beam.

## Discussion on Short Signal Exemption

Short signal exemption (SSE) is a mechanism to send relatively short signaling transmissions (compared to regular data transmissions) without sensing the channel for presence of other signals. For frequency ranges from 52.6 GHz up to 71 GHz, SSE is described in the most recent draft of EN 302 567 v2.2.0 [1] with application to short management and control frames. The total duration of the short control signaling transmissions is required to be constrained to less than 10 msec within 100 msec observation period per device.

During the RAN1 #104-e meeting [3], it has been agreed that SSB transmission with LBT should be supported at least when the SSE conditions are not met. Furthermore, during RAN1 #104b-e meeting [4], it has been agreed that SSE exemptions could be applied to the transmission of SS/PBCH. However, in this regards it was left for further study on what other DL signals and channels could be multiplexed with SS/PBCH transmission under the SSE exemption, and to which subcarrier spacing (SCS) this could be applied. In this matter, it is important to note that while EN 302 567 v2.2.0 [10] classifies “short control signal” as control and management frames, which are terminology used in IEEE 802.11 system, these control and management frames do not have specific definition in EN 302 567, and it is up to RAN1 to determine which signals and channel could qualify as control and management frames. While potentially the SSE could be applied to any DL signals, it is important to highlight that this exemption was introduced for control and management information, and if applied to any types of transmission even though their length would be bounded to 10ms per each 100ms observation period, this could potentially create coexistence problem and cause the gNB to block device from transmitting. With this in mind, in the context of applying the SSE to SS/PBCH multiplexed with other channels, RAN1 could either identify a set of SCSs and signal/channels for which the 10% duty cycle over a 100ms observation period is always met, or it could be left up to gNB’s implementation to enforce this exemption. The disadvantage of the first option is that in order to define a specific set of rules the worst case scenarios may need to be considered, and SSE exemption may be used in a very restrictive manner, when in reality this is not necessary.

Proposal 24: It is left up to gNB to decide and apply SSE to any signals/channels which are additionally multiplexed with SS/PBCH, as long as when it does the 10% duty cycle over a 100ms observation period is met.

As for the specific SCS over which the SSE could be applies, for 960 kHz SSB and Type0-PDCCH, if we assume the worst case transmission overhead and 20 msec SSB periodicity, the total transmission duration would be less than 7.5 msec (32 slots required for 64 SSB, 64 slots required for 64 Type0-PDCCH and PDSCH carrying RMSI results in 96 slots, which is approximately 1.5 msec in duration every 20 msec). Therefore, there may not be much difficulty in utilizing short control signal exemption for SSB and Type0-PDCCH and corresponding PDSCH.

Proposal 25: SSB transmission with no LBT is supported at least for 960 kHz and type0-PDCCH.

In previous RAN1 #104b-e meeting [4], it was also discussed whether the SSE should be also applied to the discovery burst (DB). In this matter, with similar motivations as above, this exemption could be applied by the gNB to the DB as long as the 10% duty cycle over a 100ms observation period is met.

Proposal 26: It is up to the gNB to decide and apply SSE to the discovery burst, as long as when it does the 10% duty cycle over a 100ms observation period is met.

In previous RAN1 #105-e meeting [5], it has concluded that the SSE rules can apply to the transmission of mg1 for the 4-step RACH and MsgA for the 2-step RACH for all supported SCS as long as the 10% duty cycle is met. However, it was argued how to actually interpret this constraint, and whether the 10% duty cycle is applied per cell or from the perspective of a UE. As indicated along this section, the EN 302 567 v2.2.0 [1] is written from the point of view of the device (equipment), and the limitations imposed for a SSE are meant to be applied per device. Therefore, when applying this to UL channels, the maximum 10 % transmission over an observation of 100ms should be measured from the UE’s perspective.

Observation 3: The contention exempt control signaling rules is interpreted as if the 10% over any observation period of 100ms is applicable per device.

Proposal 27: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective.

For PRACH transmission, given that PRACH is only transmitted during initial access and other rare occasions, we suggest to always consider utilizing short control signal exemption. UE is expected to retransmit PRACH only after RAR reception failure, therefore even with repetitive PRACH transmissions, random access protocol creates a natural gap between two consecutive PRACH transmissions. From our analysis, even if we utilize 120 kHz SCS for PRACH, we do not believe the UE could never exceed total transmission duration of 10 msec within 100 msec observation period. Therefore, we would like to extend the applicability of SSE also to mgs3 for the 4-step RACH for all the supported SCSs.

Observation 4: For 120 kHz, 480kHz, and 960 kHz PRACH transmission, UE does not exceed total transmission duration of 10 msec for PRACH within a 100 msec observation period.

Proposal 28: Contention Exempt Short Control Signaling rules apply to the transmission of msg3 for the 4-step RACH for all supported SCS.

During prior discussion, it was also left up to further study on whether to apply SSE to any other UL signal/channel, such as SRS, PUCCH, PUSCH without user plain data, etc. In our view, while HARQ-ACK feedbacks, PUSCH without user plain data and SRS transmissions may be more frequent than PRACH, as long as the 10% duty cycle over any 100 ms observation period is met, it should be left up to UE’s implementation to decide whether to qualify these transmissions as short control signaling. However, the gNB should be still capable to cell-specifically restrain or allow a UE to apply such a restriction by indicating which signals or channels could be qualifies as SCSs using for instance the following RRC parameter:

ShortControlSignalling-r17 ::=   SEQUENCE {

TxSRS                          ENUMERATED {true}                   OPTIONAL

TxPUCCH                        ENUMERATED {true}                   OPTIONAL

TxPUSCHwithoutUserPlane        ENUMERATED {true}                   OPTIONAL

}

Proposal 29: It is up to the UE to decide and apply SSE to SRS, PUSCH without user plain data,  and PUCCH, as long as when it does the 10% duty cycle over a 100ms observation period is met.

Proposal 30: The gNB indicates through a cell-specific RRC parameter which specific channels/signals could be qualifies as short control signaling.

# Conclusions

In this contribution, we discussed several aspects related to channel access enhancements for extending NR up to 71 GHz, and we derived the following proposals, and observations:

Proposal 1: Within a 5us or 8us observation window, a device must perform a measurement of the medium for at least 2us.

Proposal 2: When operating in unlicensed 60 GHz band, the ED threshold calculation shall account for the sensing beam used to perform the LBT procedure through an additional component which is added to the already agreed ED threshold formula.

Proposal 3: In case the network is able to assess the absence of any other incumbent technology, the ED threshold value that a device may use during the LBT procedure is up to the gNB and may be configured via higher layer signaling.

Proposal 4: When a UE is capable to perform Cat-2 LBT, whether to operate with or without Cat-2 LBT would be dynamically indicated by the gNB via scheduling DCIs.

Proposal 5: Y is defined as:

- 1 OFDM symbol for 120 KHz SCS,

- 4 OFDM symbols for 480 KHz SCS,

- 8 OFDM symbols for 960 KHz SCS.

Proposal 6: If an initiating device is capable to perform Cat-2 LBT, and if the initiating device performs an additional burst within the initiated COT which may be separated with any prior burst of at least a minimum gap Y, then under Alt-3 a Cat 2 LBT is needed before the initiating device transmission.

Proposal 7: In addition to support CAT-2 LBT for COT-sharing procedure, the gNB may configure the UE to use CAT-2 LBT for RX-assisted LBT

Proposal 8: Do not support, type B channel access mode for multi-carrier operation.

Proposal 9: Do not support, Cat-2 LBT for multi-beam switching and multi-beam TDM COT.

Proposal 10: For a COT with MU-MIMO, both Alt-1 and Alt-2 are supported.  As for Alt-2 both Alt-A-2 and Alt-B could be considered.

Proposal 11: For a COT with beam switching, both single LBT sensing with wide beam and independent per-beam LBT sensing at the start of the COT are supported.

Proposal: 12: An initiating device is able to initiate multiple overlapping COT over different beams.

Proposal 13: When directional sensing is performed, the COT should be considered to be acquired only in the transmission beams for which the LBT is performed and the LBT measurements have indicated that the channel is idle.

Proposal 14: When directional sensing is performed, and multiple concurrent COTs are acquired, these should be independently treated unless LBT measurements have overlapping beams. In this case, RAN1 should define some rules on how to handle these cases.

Proposal 15: RAN1 should further study how to efficiently allow beam-pairing due to LBT success.

Proposal 16: For operation unlicensed 60 GHz band, when LBT is used within the COT, the principle of the type 1 channel access procedure defined for the sub-6 GHz band should be reused, and the channel access parameters should be modified in accordance with numerologies provided by the ETSI BRAN Harmonized Standard.

Proposal 17: The procedure specified in NR-U related to the CWS adjustment should be considered for operation in unlicensed 60 GHz band. RAN1 should further discuss and identify the values Zmin and Zmax.

Proposal 18: For receiver-assisted LBT procedure both scheme 1 and 2 could be supported, where both scheme 1 and 2 could be used up to UE’s capability.

Proposal 19: Take Alt 2, “extending the beam correspondence framework and QCL, TCI, SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s)” as baseline for further discussion and standards development.

Proposal 20: Support option 1, “The selection of eligible sensing beam for a transmission beam is left for gNB implementation”, for gNB side of alternative 2.

Proposal 21: Beam correspondence mandatory capability signaling is set to 1 for all supported unlicensed bands in FR2-2.

Proposal 22: Support option 1, up to UE implementation on how to support wide sensing beaming using QCL/TCI framework, for UE side of alternative 2.

Proposal 23: RAN1 should extend the QCI or Spatial Relation Info framework to define and indicate the sensing beam associated with a transmission beam.

Proposal 24: It is left up to gNB to decide and apply SSE to any signals/channels which are additionally multiplexed with SS/PBCH, as long as when it does the 10% duty cycle over a 100ms observation period is met.

Proposal 25: SSB transmission with no LBT is supported at least for 960 kHz and type0-PDCCH.

Proposal 26: It is up to the gNB to decide and apply SSE to the discovery burst, as long as when it does the 10% duty cycle over a 100ms observation period is met.

Proposal 27: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective.

Observation 4: For 120 kHz, 480kHz, and 960 kHz PRACH transmission, UE does not exceed total transmission duration of 10 msec for PRACH within a 100 msec observation period.

Proposal 28: Contention Exempt Short Control Signaling rules apply to the transmission of msg3 for the 4-step RACH for all supported SCS.

Proposal 29: It is up to the UE to decide and apply SSE to SRS, PUSCH without user plain data,  and PUCCH, as long as when it does the 10% duty cycle over a 100ms observation period is met.

Proposal 30: The gNB indicates through a cell-specific RRC parameter which specific channels/signals could be qualifies as short control signaling.

# References

- RP-193228, “New SID Study on supporting NR above 52.6 GHz”, Intel Corporation, 3GPP TSG RAN Meeting #86, Dec. 2019.

- RP-212637, “Revised WID: Extending current NR operation to 71GHz,” Qualcomm, Intel, 3GPP TSG RAN Meeting #93-e, Dec. 2020.

- RAN1Chairman’s notes, 3GPP TSG RAN WG1 #104 e-Meeting, February 2021.

- RAN1Chairman’s notes, 3GPP TSG RAN WG1 #104b e-Meeting, April 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #105 e-Meeting, May, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106 e-Meeting, August, 2021.

- ARIB STD-T117, “Low Power Data Communication System / 60 GHz-band Wireless LAN for Very High Throughput Data Communications”, September 2016.

- R1-200980, “Channel Access Procedure for NR in 52.6 - 71 GHz”, Intel Corporation, 3GPP TSG RAN1 Meeting #103-e, Nov. 2020.

- ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, (2017-05).

- Draft ETSI EN 302 567 V2.2.0, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,” Dec. 2020.

- 3GPP TR 38.808, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)”, Nov. 2020.

- R1-2107582, “Discussion on Channel Access Mechanism for extending NR up to 71 GHz,” Intel Corporation, 3GPP TSG RAN Meeting #106-e, August 2021.