3GPP TSG RAN WG1 #106bis-e			 R1-2109631

e-Meeting, October 11th – 19th, 2021

Source:	Intel Corporation

Title:	Sidelink Resource Allocation Schemes for UE Power Saving

Agenda item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

The RAN WG approved work item on NR Sidelink Enhancements [1], that includes the following objective:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution, we express our view on sidelink enhancements targeting UE power saving and considering agreements made by the RAN WG1 at the previous meeting(s).

Our views on other sidelink enhancement objectives as well as LS responses are provided in companion contributions [2] and [3].

# Baseline Features for Sidelink Power Saving

According to the work item objective, the “baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation Mode 2” which are discussed in the subsections below, following the RAN WG1 agreements made at the previous meeting(s).

- Partial sensing-based RA is supported as a power saving RA scheme - FFS details - Random resource selection is supported as a power saving RA scheme - FFS any changes or enhancement - FFS on conditions to apply random resource selection - In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

## Sidelink Random Resource Selection Design

For random resource selection we follow agreements made by RAN WG1 at the previous meeting(s):

- For random resource selection, - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation. - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any) - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected. - FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions). - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

### Conditions to Perform Random Resource Selection

One of the additional open aspects for random resource selection is a condition(s) to apply this scheme. In our view, at least the following conditions should be considered:

- Sidelink resource pool is (pre)-configured to enable sidelink transmissions based on random resource selection

- UE meets either of the following criteria

  - UE does not have sidelink RX chain to perform sensing (i.e. sidelink TX only UE – Type A UE)

  - UE is configured to use random resource selection as a power saving resource allocation scheme.

  - Random resource selection can be applied if

    - UE does not have sidelink RX chain to perform sidelink sensing (i.e. Type A UE)

    - UE is configured to operate in power saving resource allocation mode

### Random Resource Selection with Low Priority

Regarding a mixture of different RA schemes in the same resource pool the follow agreements were made by RAN WG1 at the previous meeting:

Agreement - For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

In our opinion the simple approach is to define priority threshold to control priority levels for random resource selection. Transmissions below such threshold should either apply the following enhancements or not be transmitted at all:

Resource pool partitioning

  - In this case, transmissions below the threshold are only allowed to a subset of the resource configured in the resource l. The resource patterns for this would need to be defined in the resource pool configuration (e.g. subset of sub-channels)

The changes in Option 2 and Option 7 are not backwards compatible. This means transmissions with these changed 1st stage SCI fields cannot be interpreted by Rel. 16 devices. Therefore, they would not be able to respond to the additional information. In our opinion having Rel. 16 and Rel. 17 SL devices in the same resource pool should be supported, thus we cannot support these options.

  - Support Option 1 for random resource selection in a resource pool with full/partial sensing

    - For transmission with a priority value above the threshold specific resources for low priority transmissions can be configured in the resource pool

## Partial Sensing Based Resource Selection Design

In this section, we discuss design of partial sensing-based resource selection as a power saving resource allocation scheme. We follow agreements made by RAN WG1 at the previous meeting(s) on this aspect:

Agreements in RAN1#104-e - In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17. - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking Agreements in RAN1#105-e - For the set of Preserve values in periodic-based partial sensing, - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList. - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList - This is per mode 2 Tx resource pool (pre-)configuration - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx - FFS whether the monitoring can be mandatory - In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources. - The processing time restriction includes Tproc,0SL and Tproc,1SL. - Aspects relating to sensing during SL DRX are to be discussed separately - Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE - In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative - TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc.) - FFS details - FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc). Agreements in RAN1#106-e - In periodic-based partial sensing, UE monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection. - Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met: - L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool - FFS: When the trigger will be received by L1 - The resource pool is (pre-)configured to enable partial sensing - Partial sensing is configured by higher layer in the UE - For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing, - Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc - Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable. - FFS: If additional enhancements are needed for enabling/disabling - The triggering of re-evaluation and pre-emption checking is as in R16. - When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n, - The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - On the sensing window [n+TA, n+TB] for CPS, - Details of TA and TB values based on the agreements from previous RAN1 meetings - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability - FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability - Whether the set SA is confined within a set of Y candidate slots within the RSW - UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4 - Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately. - FFS: Details on T1 - When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately - When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS - UE performs contiguous partial sensing in [n+TA, n+TB] according to for resource exclusion from the initialized candidate resource set (SA) - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately - FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled

According to agreements, the RAN WG1 need to address the following FFS aspects which are discussed in the next sub-sections:

- Aspect 1: Contiguous partial sensing always required for resource (re)-selection

- Aspect 2: Monitoring/sensing window settings - TA, TB around resource (re)-selection trigger

- Aspect 3: Periodic-based partial sensing process for resource (re)-selection without periodic reservation

- Aspect 4: Range of values for (pre)-configuration of Y

- Aspect 5: Reduction of UE Active Time for TB Transmission

- Aspect 6: Implications of Multiple Sensing Occasions for Resource Reservation Period

- Aspect 7: Handling the cases with incomplete periodic-partial sensing information

- Aspect 8: Preemption handling

- Aspect 9: Conditions to perform periodic-based, contiguous partial sensing and PSSCH demodulation

### Contiguous partial sensing always required for resource (re)-selection

From RAN1 discussions, it seems there are different interpretations whether contiguous partial sensing is always assumed and is always combined with periodic partial sensing or not. In our design, contiguous partial sensing is always enabled when resource reselection is triggered. Periodic partial sensing is applied on top for semi-persistent transmissions based on applicable set of Preserve. Therefore, monitoring periodic sensing occasions between resource (re)-selection trigger in slot n and the first candidate slot is covered by the contiguous sensing considerations assuming that periodic and contiguous partial sensing overlap in time (running in parallel).

  - Contiguous partial sensing is always triggered for partial sensing-based resource (re)-selection

  - Periodic-based partial sensing is used only for semi-persistent transmissions

  - Contiguous partial sensing intervals may overlap in time with periodic partial sensing intervals and thus may be used in combination with periodic partial sensing

  - For periodic based partial sensing for each value of Preserve only transmission with PRSVP_Rx = Preserve are considered

### TA, TB Settings for Monitoring/Sensing Window

#### Resource Pools with Semi-persistent Reservations

In resource pools, where semi-persistent reservations are disabled, the maximum sensing window duration may be limited by the SCI signaling window size (32 logical slots). In this configuration scenario, UE may still have periodic and/or aperiodic traffics that need to be discussed separately.

Monitoring window settings (TA, TB) for aperiodic traffic

Aperiodic traffic has unknown packet arrival time. It is not possible to wake up and sense the radio-environment right before the packet arrival and therefore partial sensing procedure can be activated directly by resource (re)-selection trigger. In this case, two options are possible:

- Option 1: The UE performs partial sensing for M logical slots (e.g. M = 32 - SCI signaling window duration) and then triggers resource selection procedure, while continue sensing till the last retransmission of a TB.

- Option 2: The UE simultaneously triggers both partial sensing and resource selection procedure. In this case, UE can eventually transmit before it aggregates sensing information from M logical slots (e.g. M = 32 - SCI signaling window duration).

In our view, Option 2 is more general, since Option 1 can be achieved by UE implementation if the packet delay budget is sufficiently large. Note that the main purpose for sensing M logical slots before each transmission is to detect potential collisions with sidelink retransmissions.

Therefore, the following monitoring window settings can be used:

- TA settings – start of the monitoring window

  - Alt.1: TA ≤ ∆A

    - In this case, the value of ∆A depends on the maximum time required for switching from sleep state to the monitoring state (it can be different for different sleep states: micro, light, deep). It assumes that UE is in a sleep state when resource (re)-selection trigger is received. In case of aperiodic traffic, the monitoring window start time depends on time required to switch from sleep state which shortens PDB.

  - Alt.: TA = ∆A = 1 slot, if monitoring window starts from slot ‘n+1’

    - In this case UE implementation ensures that switching to monitoring state by the time the physical layer gets the resource (re)-selection trigger from higher layer. This means that the higher layer delayed the resource (re-)selection trigger till the device woke up from any possible sleep state. This has the advantage that the device specific wake-up time can be accurately accounted for.

- TB settings – end of the monitoring window

  - TB = ∆B – T3 ≤ PDB

    - In this case, the value ∆B is determined by time instance (slot index) corresponding to the last retransmission of a given TB or HARQ feedback, T3 – processing delay in slots to account the sensing and processing delays (e.g. Tproc,0 and Tproc,1).

We want to emphasize here that our selection of TB is dependent on retransmissions due to HARQ feedback. Prior to the feedback it is unknown if additional retransmissions including potential additional sensing is required, we think contiguous sensing should thus be enabled till no more retransmissions can be expected.

Monitoring window settings (TA, TB) for periodic traffic

Although semi-persistent transmissions/reservations are not supported by the resource pool, the UE by implementation can still predict the packet arrival time in case of periodic traffic and thus it can wakeup right before upcoming transmission of a TB and sense the radio-environment. In this case, UE partial sensing can be triggered M logical slots ahead of resource re-selection trigger (e.g. M = 32 - SCI signaling window duration), so that sensing data over duration of M logical slots are available before new TB/packet arrival. In this case, the following two options are possible:

- Option 1: The UE triggers partial sensing ahead of the upcoming resource re-selection trigger on N logical slots (e.g. M = 32 logical slots - SCI signaling window duration) and continues sensing till the last retransmission of a TB.

- Option 2: The UE simultaneously triggers both partial sensing and resource re-selection procedures. In this case, the UE can eventually transmit by the time it aggregated sensing information from N logical slots (e.g. M = 32 logical slots - SCI signaling window duration).

Option 1 is optimal for periodic traffic when semi-persistent reservations are disabled in a resource pool. It may be also applied for better coexistence of periodic and aperiodic traffic, for the case when semi-persistent reservations are enabled in a sidelink resource pool. Therefore Option 1 can be recommended, if similar behavior is agreed for the case of enabled semi-persistent reservations in a resource pool, as it can provide higher reliability to transmissions associated with periodic traffic. As the value of T1 is up to UE implementation and can be equal to 0 in the extreme case, sensing needs to start at least N logical slots before n.

Therefore, the following monitoring window settings TA and TB can be used:

- TA

  - – max((∆A + tn-M), resource selection window size) ≤ TA ≤ 1 slot, where tn-M is the distance in physical slots to the slot that is N logical slots before the slot with physical index n (e.g. M = 32 logical slots - SCI signaling window duration)

- TB

  - Same options as for aperiodic traffic

The value of M can be configurable as it may be dependent on resource pool configuration.

#### Resource Pools without Semi-persistent Reservations

Monitoring window settings (TA, TB) for aperiodic traffic

For dynamic sidelink transmissions, the same partial sensing and resource selection behavior as described for aperiodic traffic in case of disabled semi-persistent reservations can be used, i.e. TA and TB are the same as for aperiodic transmissions w/o semipersistent reservations.

Monitoring window settings (TA, TB) for periodic traffic

For periodic sidelink transmissions, the same partial sensing and resource selection behavior as described for periodic traffic in case of disabled semi-persistent reservations can be used, i.e. TA and TB are the same as for periodic transmissions w/o semipersistent reservations.

Combining the aspects discussed in this subsection we have the following proposal:

  - For dynamic resource selection (aperiodic traffic) or resource selection for the first TB of semi-persistent process

    - UE performs partial sensing together with resource re-selection/re-evaluation procedures within monitoring window [n+TA, n+TB], where

      - n - slot when UE gets trigger for resource selection from higher layers

      - Two alternatives for TA

        - Alt.1: TA = 1 slot,

        - Alt.2: TA ≤ ∆A, where the value of ∆A depends on the maximum time required for switching from sleep state to the monitoring/sensing state

      - TB = ∆B – T3 ≤ PDB, where the value ∆B is determined by slot corresponding to the last retransmission of a given TB or HARQ feedback, T3 – processing delay in slots

  - For semi-persistent resource (re)-selection (periodic traffic) or dynamic resource selection for periodic traffic

    - UE performs partial sensing together with resource re-selection/re-evaluation procedures within monitoring window [n+TA, n+TB], where

      - n – slot when UE gets trigger for resource (re)-selection from higher layers

      - Two alternatives for TA

        - TA - is within a range: –max(tn-M, resource selection window size) ≤ TA ≤ 1 slot, where tn-N is the distance in physical slots to the slot that is M logical slots before the slot with physical index n

        - TA - is within a range: –max((∆A + tn-M), resource selection window size) ≤ TA ≤ 1 slot, where tn-N is the distance in physical slots to the slot that is M logical slots before the slot with physical index n, the value of ∆A depends on the maximum time required for switching from sleep state to the monitoring/sensing state

      - TB - is determined in the same way as for dynamic resource reservation

### Periodic-based Partial Sensing for Resource (Re)-selection without Periodic Reservation

- The main problem of periodic-based partial sensing in application to aperiodic traffic is unknown packet arrival time. Therefore, its applicability should be limited to periodic traffic only (traffic with predictable arrival time). If semi-persistent reservations are enabled in a resource pool but a UE does not utilize the semi-persistent reservation processes for periodic traffic, it may still be valuable for such UE to perform periodic based partial sensing, so that it can take into account periodic transmissions from other UEs. For handling aperiodic traffic, the periodic-based partial sensing will increase power consumption without guaranteeing that periodic sensing data will be used for resource selection. Therefore, we do not see benefits to define periodic-based partial sensing for transmissions without semi-persistent reservations. At the same time, we assume that periodic-based partial sensing should not be precluded by specification and thus can be left up to UE implementation.

  - Specification does not support periodic-based partial sensing for transmissions without semi-persistent reservation

  - In resource pool with enabled semi-persistent reservations, the use of periodic-based partial sensing for transmissions without semi-persistent reservation is left up to UE implementation

### Range of values for (pre)-configuration of Y

In our understanding, the value range for the minimum number of candidate slots Y adjusts the number of slots that a UE using partial sensing needs to monitor. For partial sensing in LTE V2X, a value in the range of 1 to 13 was defined. In general, we do not see much motivation to define the minimum number of candidate slots Y for NR-V2X. However, if it is introduced then it should be defined per priority level as the minimum resource selection window determined by T2,min is also a function of priority.

For defining the range of Y, the following should be considered. As the minimum resource selection window for the case of 15 kHz can be configured to be 1 slot, the minimum for Y should also be one slot. For LTE the maximum resource selection window are 100 slots. In contrast for NR this is dependent on the implementation and the remaining PDB. However, the largest minimum resource selection window would be 160 slots for the case of 120 kHz SCS. If we apply the same ratio of 13/100 for LTE, we arrive at 21 slots. To allow for implementation specific larger values with the same ratio the maximum configurable value of Y should be set to 32.

  - The minimum number of candidate slots Y for partial sensing is (pre)-configured per priority level

  - The minimum number of candidate slots Y is configurable in the range of 1 to 32 slots

### Reduction of UE Active Time for TB Transmission

When UE operates in partial sensing mode, the amount of power saving gain depends on the amount of time necessary to complete transmission of a TB. Therefore, the faster UE completes transaction the more power saving gain can be achieved. To reduce amount of overall time for TB transmission, the UE can optimize resource selection procedure and among identified candidate resources for transmission, pick the one out of the N first in time candidate resources.

Our system level evaluation analysis provided in Section 3 shows that additional power saving gains ~12% can be achieved without noticeable loss in PRR performance if early in time resource selection is applied on top of partial sensing.

  - To reduce time for transmission of a TB and improve power saving, randomly pick one out of N first in time candidate resources, where the value of N is pre-configured

### Implications of Multiple Sensing Occasions for Resource Reservation Period

The following agreement was made by RAN WG1 for sensing occasions for a given resource reservation periodicity:

- For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE

In our view, the agreement made during RAN1#105-e on monitoring of multiple periodic sensing occasions for a given reservation period instead of using only the most recent one has impact on mode 2 resource set determination procedure described in clausa 8.1.4 of TS 38.214 [7]. The agreed functionality has impact on resource exclusion in step 5) and 6) of clausa 8.1.4 of TS 38.214 [7]. To structure the discussion, we split it into the following sub-topics:

- Resource exclusion due to sidelink transmissions by UE re-selecting resources

- Aggregation of SL-RSRP measurements and priority information from multiple sensing occasions

Resource exclusion due to sidelink transmissions by UE re-selecting resources

Figure 1 illustrates an example for one reservation period for the case of two sensing occasions (k = 2) configured. It describes scenario when UE transmits and thus not monitoring one or multiple slots due to the half duplex constraint in one of the periodic sensing occasions (the latest one). The R16 behavior requires UE monitoring only the latest occasion preceding resource selection and the related slot(s) in the resource selection window are excluded. If multiple occasions are configured, it is not clear if these slots should be excluded or not. In general, assuming k occasions are configured there are 2k different combinations of monitored and not monitored slots.

Figure 1: Illustration for one reservation period and two configured sensing occasions

This general problem can be solved in different ways. A slot should not be a candidate slot if all the k sensing occasions for each Preserve were not monitored for this slot.

SL-RSRP measurement and priority considerations for multiple sensing occasions

For the resource exclusion in step 6) of clause 8.1.4 of 38.214 [7] the SL-RSRP value and priority level from transmission in the latest occasion before resource (re)-selection are used. For the case of monitoring multiple periodic occasions, it needs to be discussed whether SL-RSRP measurements and priority information from multiple sensing occasions can be combined. Given that preemption and sidelink collisions may happen for each transmission the combining of measurements or priority information across multiple occasions can only be possible if it can be verified that sidelink transmissions belong to the same semi-persistent process and belong to the same originating source. Therefore, the decoding of the stage-2 SCI may be needed to verify originating source during sensing operation.

To properly handle multiple sensing occasions for the resource exclusion, the following options can be analyzed:

Option 1: Independent processing of semi-persistent transmissions from multiple sensing occasions

  - Semi-persistent transmissions across multiple sensing occasions are treated independently. Thus, even for multiple sensing occasions with the same periodicity these transmissions are treated as independent procedures for the resource exclusion in step 6) of clausa 8.1.4 in 38.214 [7].

Option 2: Aggregation/combining of semi-persistent transmissions from multiple sensing occasions

  - Semi-persistent transmissions originating from the same source and reserving the same periodic resources can be combined. This step requires the decoding of the 2nd stage SCI to detect the SRC ID. The R16 design does not imply decoding of the 2nd stage SCI for sensing operation. For aggregation the following parameters should match across aggregated occasions: periodicity/priority/number of subchannels and its allocation /SRC ID. If any of these parameters do not match the system automatically falls back to the case of treating each of these transmissions independently.

  - For periodic based partial sensing, with semi-persistent transmissions enabled per resource pool and multiple sensing occasions configured for Preserve

    - A slot is excluded if all of the k sensing occasions for each Preserve were not monitored for this slot

    - Each sensing occasions is treated independently for the SL-RSRP based candidate resource exclusion

Maximum value of k

Considering the aspects discussed in the preceding subsections we think that value range of k should be quite small e.g k is configurable up to 4. Thus, k should be configurable in the range of 1 to 4.

  - For periodic based partial sensing, with semi-persistent transmissions enabled per resource pool the number of sensing occasions k is configurable in the range from 1 to 4

### Handling Cases with Incomplete Partial Sensing Information

- There are at least two scenarios where resource (re)-selection with incomplete periodic-based partial sensing results could arise:

Scenario 1: Periodic reservation for transmission of the first (initial) TB of the semi-persistent process or TBs for which periodic-based partial sensing data is not sufficient

- To handle this scenario, at least the following two alternatives can be considered:

  - Alt.1: - Immediate semi-persistent reservation of resources

    - In this case, UE indicates semi-persistent reservation starting from the first TB of the given semi-persistent process. Partial sensing procedure designed for aperiodic transmission can be applied for transmission of the 1st TB. If it happens that periodic based partial sensing results are available (applicable for given resource selection window) the UE can use this information to improve resource selection for transmission of the first TB and trigger periodic-based partial sensing for transmission to be used in the next resource reselection period which can be determined based on the following set of sub-alternatives.

      - Alt. 1A: The next resource reselection period can be determined based on expiration of TB counter initialized with random number of TBs for transmission w/o reselection

      - Alt. 1B: The next resource reselection period can be determined by the event when enough of periodic-partial sensing data is available at UE side

      - Alt. 1C: The next resource reselection is determined based on the earliest time instance resulting from Alt. 1A and Alt. 1B

  - Alt.2: - Postponed semi-persistent reservation of resources

    - In this case, UE can immediately trigger periodic based partial sensing and transmit the first N TBs of semi-persistent process without resource reservation for the following TBs. For the transmission of the first N TBs, the UE can apply resource selection procedure defined for aperiodic transmission while at the same time aggregate periodic-based and contiguous partial sensing data for transmission of the N first TBs. Once a sufficient amount of periodic-based partial sensing information is available, the UE can perform resource (re)-selection and indicate semi-persistent reservation for the transmission of the future TBs of a given semi-persistent process. The amount of sufficient partial sensing information can be dependent on UE transmission period and can be controlled by pre-configuration of N per transmission period.

- For both described above alternatives, in case of upcoming (predictable) semi-persistent resource reselection of a given semi-persistent process, the UE is expected to apply periodic-based partial sensing for resource reselection process.

Scenario 2: Periodic reservation for transmission of the pre-empted TBs of the semi-persistent process

- Preemption of semi-persistent transmission may cause resource re-selection for transmission of each TB. It still needs to be discussed whether periodic based partial sensing need to be applied for transmission of each TB if preemption and semi-persistent reservations are enabled per resource pool. Whether to use periodic based partial sensing for preemption check for the transmission of each TB needs to be concluded first. Depending on this conclusion the following scenarios are possible:

  - Scenario 2A: Periodic based partial sensing is triggered only for the case of the next resource reselection

  - Scenario 2B: Periodic based partial sensing is triggered for transmission of each TB

- Irrespective of the Scenario 2A or 2B the design alternatives applicable for Scenario 1 can be reused.

If periodic-based partial sensing information is not sufficient, UE selects resources based on partial sensing procedures designed for dynamic transmissions considering incomplete periodic based partial sensing information

If UE does not have sufficient periodic based partial sensing information, UE is not allowed to use semi-persistent reservation

Sufficient amount of periodic-based partial sensing information is determined by the following condition:

  - N⸱PTX > Preserve_threshold where

    - N – number of TBs transmitted without semi-persistent reservation configured for given PTX

    - PTX – reservation period for transmission

    - Preserve_threshold – pre-configured threshold which is one of the Preserve values

### Preemption Handling

If preemption is enabled, UE performing semi-persistent transmissions should perform preemption check before each transmission of a TB. The partial sensing window of size max {resource selection window size, SCI signaling window size (N = 32 logical slots)} can be used for preemption check. It needs to be discussed further whether to support periodic-based partial sensing for preemption check for every TB or before the resource reselection only. In our view, periodic-based partial sensing for every TB with preemption check may result in significant power consumption. The power consumption for periodic traffic/semi-persistent transmissions may become much higher than for aperiodic traffic with the same intensity. Considering this aspect, it makes sense to have configurability among the following two options for preemption check:

- Option 1: Periodic-based partial sensing is enabled for every TB transmission

- Option 2: Periodic-based partial sensing is enabled for resource reselection events

  - UE preemption behavior in terms of resource yielding and re-selection is reused from NR-V2X R16 for each partial sensing time interval, determined from partial sensing windows

  - For preemption check in case of periodic-based partial sensing, support configurability among the following two options

    - Option 1: Preemption check and periodic-based partial sensing are enabled for every TB transmission

    - Option 2: Preemption check and periodic-based partial sensing are enabled for resource reselection events

### Conditions to Perform Periodic-based, Contiguous Partial Sensing and PSSCH Demodulation

In our opinion the possibility to perform periodic-based partial sensing and/or contiguous partial sensing should be dependent on the following parameters:

Priority:

  - Transmissions with a high priority should have the possibility to perform contiguous partial sensing.

QoS constraints of the packet:

  - It is important to consider QoS criteria of the packet for the selection where partial sensing can be used.

  - Partial sensing can be enabled / disabled per transmission priority level and the quality-of-service requirements

  - For power saving, UE can skip PSSCH demodulation depending on sidelink transmission priority level

## Adaptation of Power Saving Resource Allocation Schemes

At the previous meeting, different types of UE behaviors beneficial for power consumption were discussed

- Type A: No SL reception

- Type B: only PSFCH/PSBCH reception

- Type-D: reception of all sidelink physical channels

The Type-A and Type B behaviors are associated with random resource selection, while for Type-D both partial sensing and full sensing operation modes are possible. The UE adaptation between random, partial, full sensing resource selection schemes can benefit UE power saving. One of the possible solutions here is to apply filtered CBR criteria for adaptation. The L3 filtered CBR measurements can be compared with the pre-configured thresholds to switch b/w power saving resource allocation schemes. The intention beyond this approach is to apply random resource selection in low loading, partial sensing in medium, and full sensing in high loading scenarios, for the case when UE needs to save power. The different power saving resource allocation schemes can be also activated depending on the service provided by UE. The mechanism for association/mapping of services to power saving resource allocation schemes can be defined.

  - For UE sidelink power saving, NR supports adaptation of sidelink power saving resource allocation schemes (i.e. b/w random, partial, or full sensing-based resource selection)

## Additional Aspects of Sidelink Power Saving Resource Allocation Schemes

### Sidelink HARQ Feedback

UE should be able to request HARQ feedback in all resource allocation schemes (including random resource selection). The mandatory condition for requesting feedback is monitoring of the related resources where feedback is expected. If UE does not have capability of sidelink reception, it shall not request for sidelink feedback. A UE deciding to transmit with HARQ feedback request shall monitor PSFCH resources for all potential re-transmissions of a TB unless re-transmissions are dropped.

  - UEs using partial sensing or random resource selection for transmissions with enabled HARQ feedback are required to monitor the associated PSFCH resources

### Congestion Control

The following agreements were made by RAN WG1 regarding sidelink congestion control:

- Further study congestion control based on CBR and CR for power saving RA schemes - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

In R16, NR congestion control is based on Channel Busy Ratio (CBR) measurements defined in a 100ms window preceding each sidelink transmission. In case of partial sensing, it is possible to perform CBR measurements across multiple partial sensing windows, right before the actual sidelink transmission. This approach may be feasible for semi-persistent allocation when partial sensing is implemented across multiple windows corresponding to different configured periods. For dynamic resource reservations, multiple partial sensing window(s) may not be available right before sidelink transmission of a TB and thus previous CBR measurements and measurements performed within the ongoing partial sensing time interval can potentially be used.

As the current definition of the CBR measurement interval sl-TimeWindowSizeCBR can only take one of the two values 100 ms or 100 slots, it might be necessary to introduce additional values for CBR time windows applicable for UEs operating in partial sensing. Depending on the values of the partial sensing window, CBR measurements can be done across multiple partial sensing intervals.

In our view, more design details on partial sensing operation as well as more studies for candidate congestion control options are needed before further discussion on potential congestion control schemes for UEs operating in partial sensing mode. The major potential problem here is that CBR measurements may be quite rough and vary significantly even among re-transmissions of a TB. Various solutions can be considered if this problem is confirmed, e.g. definition of separate CBR table, CBR averaging/filtering, etc.

  - Decisions related to congestion control for partial sensing UEs are discussed when design of partial sensing mechanism is completed (at least enough details are defined)

If SL reception is not supported by a UE, there may be no CBR measurement available at the UE. In this case, CBR may be provided by network using the Uu air-interface. However, this option does not work for out of coverage and PC5 only UEs. Another potential approach is to simply put constraints on maximum amount of resources or CR value in a predefined time interval.

  - FFS how to support congestion control in case of random resource selection if no SL reception is supported by UE

### Open Loop Power Control

In our opinion the only change necessary for SL Open Loop Power Control (OLPC) is related to the transmit power constraints due to congestion control. This means if the interaction of partial sensing and random resource selection and congestion control is clarified this will also solve all required changes of the OLPC. This even hold true for devices that cannot receive the SL as they would fall back to the case that the PSSCH and PSCCH power is only dependent on the CBR measurement and the maximum power.

### Sidelink DRX Behaviour

A LS from RAN2 0 related to working assumption made on SL DRX was received at the previous meeting. In our opinion, the introduction of SL DRX has limited impact on the physical layer procedures. However, it needs to be ensured that there is enough time to perform all required measurements as well as perform time and frequency synchronization. The required measurement time is dependent on which resource allocation scheme is used. To enable discovery based on broadcast communication, alignment of the SL DRX for different devices is necessary. During the unicast or groupcast communication setup a minimal set of time slots during which participating devices are active needs to be agreed.

In our opinion the setup of the SL DRX should consider the physical layer communication requirements. For example, to achieve reliable, periodic communication the SL DRX periodicity needs to have at a minimum the same periodicity. In the same fashion if the SL DRX configuration of the recipient of the communication is known it is beneficial to consider its activity pattern during the resource selection. Otherwise, the transmitter might select resource that cannot be received as the receiving UE is inactive.

These considerations are dependent on the details of the SL DRX implementation. In our understanding at this stage the design of the SL DRX in RAN2 is not sufficiently complete to discuss all details in RAN1.

  - The design of SL DRX cycle needs to ensure that UE partial sensing behavior is respected (i.e. UE wake up time intervals for the purpose of partial sensing need to be aligned with On duration intervals, as well as traffic characteristics)

In our view, design of SL DRX and partial sensing can be decoupled. Periodic-based partial sensing should be possible in the SL DRX inactive time. However, special cases can be handled according to the discussion about incomplete information in the Section 2.2.7. As this leads to some cases where the sensing needs to be performed during the SL DRX inactive time, we think this should be left to UE implementation.

  - Defined partial sensing procedure (periodic and contiguous) need to be fulfilled irrespective of the SL DRX status

  - Sensing in the SL DRX inactive time is up to UE implementation

# Partial Sensing Evaluation Results

In this section, we provide sidelink power saving system level evaluation results. The SL power model follows the power model described in [4] with adaptation for the SL being agreed in chair notes for RAN1#102-e [5] and RAN1#103-e [6] meetings.

The evaluation assumptions are summarized in section 6.1. We compared the performance of pedestrian (P-UEs) in terms of power consumption and PRR in a mixed scenario with P-UEs and V-UEs present. The analysis was done for P2V and V2V communication assuming that 1) all UEs (P-UEs and V-UEs) perform full sensing and 2) P-UEs perform partial sensing and V-UEs use full sensing mode.

From the power consumption analysis in Figure 2, there are three cases evaluated:

For the case of full sensing operation, it is seen that UEs consume most of their power for processing sidelink control and shared channels.

For the partial sensing case, we see that the power consumption is dominated by the GNSS synchronization and the power consumption in the sleep states (transition b/w sleep and wake up states).

Finally, the combination of partial sensing with early in time resource selection provides additional power saving gain w/o loss in PRR.

Figure 3 provides PRR performance for the P2V or V2V traffic. There is no loss in performance of the transmissions originating from P-UEs in case of aperiodic traffic with large PDB. It can be explained by the fact that once resource reselection is triggered, P-UEs can wait for a certain amount of time and acquire necessary amount of sensing data (i.e. 32 slots) to perform resource selection. Obviously, if the P-UE selects another strategy and selects resources right after resource selection trigger some loss of V2V and P2V can be expected.

  - Partial sensing-based resource selection can substantially reduce UE power consumption (i.e. increase UE power saving)

  - In considered urban scenario, no PRR degradation for P2V and V2V traffic was observed for considered partial sensing scheme

  - Power saving gain is increased if early in time resource selection is applied on top of partial sensing

    - Up to ~12% gain is achieved with respect to partial sensing without noticeable PRR degradation

| - Full sensing (98.8) | (b) Partial sensing (25.6) |
| --- | --- |
| (c) Partial sensing w/ early in time resource selection (22.5) |  |
| Figure 2: P-UE mean power consumption per slot. |  |

Figure 3: Average PRR over the distance 
P2V (left) and V2V (right) traffic for full sensing / partial sensing and partial sensing with early in time resource selection P-UEs.

Conclusions

In this contribution, we have provided our views on potential power saving features and concepts for NR sidelink communication. In summary, we have following list of proposals for NR sidelink power saving:

  - Random resource selection can be applied if

    - UE does not have sidelink RX chain to perform sidelink sensing (i.e., Type A UE)

    - UE is configured to operate in power saving resource allocation mode

  - Support Option 1 for random resource selection in a resource pool with full/partial sensing

    - For transmission with a priority value above the threshold specific resources for low priority transmissions can be configured in the resource pool

  - For dynamic resource selection (aperiodic traffic) or resource selection for the first TB of semi-persistent process

    - UE performs partial sensing together with resource re-selection/re-evaluation procedures within monitoring window [n+TA, n+TB], where

      - n - slot when UE gets trigger for resource selection from higher layers

      - Two alternatives for TA

        - Alt.1: TA = 1 slot,

        - Alt.2: TA ≤ ∆A, where the value of ∆A depends on the maximum time required for switching from sleep state to the monitoring/sensing state

      - TB = ∆B – T3 ≤ PDB, where the value ∆B is determined by slot corresponding to the last retransmission of a given TB or HARQ feedback, T3 – processing delay in slots

  - For semi-persistent resource (re)-selection (periodic traffic) or dynamic resource selection for periodic traffic

    - UE performs partial sensing together with resource re-selection/re-evaluation procedures within monitoring window [n+TA, n+TB], where

      - n – slot when UE gets trigger for resource (re)-selection from higher layers

      - Two alternatives for TA

        - TA - is within a range: –max(tn-M, resource selection window size) ≤ TA ≤ 1 slot, where tn-N is the distance in physical slots to the slot that is M logical slots before the slot with physical index n

        - TA - is within a range: –max((∆A + tn-M), resource selection window size) ≤ TA ≤ 1 slot, where tn-N is the distance in physical slots to the slot that is M logical slots before the slot with physical index n, the value of ∆A depends on the maximum time required for switching from sleep state to the monitoring/sensing state

      - TB - is determined in the same way as for dynamic resource reservation

  - Specification does not support periodic-based partial sensing for transmissions without semi-persistent reservation

  - In resource pool with enabled semi-persistent reservations, the use of periodic-based partial sensing for transmissions without semi-persistent reservation is left up to UE implementation

  - The minimum number of candidate slots Y for partial sensing is (pre)-configured per priority level

  - The minimum number of candidate slots Y is configurable in the range of 1 to 32 slots

  - To reduce time for transmission of a TB and improve power saving, randomly pick one out of N first in time candidate resources, where the value of N is pre-configured

  - For periodic based partial sensing, with semi-persistent transmissions enabled per resource pool and multiple sensing occasions configured for Preserve

    - A slot is excluded if all of the k sensing occasions for each Preserve were not monitored for this slot

    - Each sensing occasions is treated independently for the SL-RSRP based candidate resource exclusion

  - For periodic based partial sensing, with semi-persistent transmissions enabled per resource pool the number of sensing occasions k is configurable in the range from 1 to 4

If periodic-based partial sensing information is not sufficient, UE selects resources based on partial sensing procedures designed for dynamic transmissions considering incomplete periodic based partial sensing information

If UE does not have sufficient periodic based partial sensing information, UE is not allowed to use semi-persistent reservation

Sufficient amount of periodic-based partial sensing information is determined by the following condition:

  - N⸱PTX > Preserve_threshold where

    - N – number of TBs transmitted without semi-persistent reservation configured for given PTX

    - PTX – reservation period for transmission

    - Preserve_threshold – pre-configured threshold which is one of the Preserve values

  - UE preemption behavior in terms of resource yielding and re-selection is reused from NR-V2X R16 for each partial sensing time interval, determined from partial sensing windows

  - For preemption check in case of periodic-based partial sensing, support configurability among the following two options

    - Option 1: Preemption check and periodic-based partial sensing are enabled for every TB transmission

    - Option 2: Preemption check and periodic-based partial sensing are enabled for resource reselection events

  - Partial sensing can be enabled / disabled per transmission priority level and the quality-of-service requirements

  - For power saving, UE can skip PSSCH demodulation depending on sidelink transmission priority level

  - For UE sidelink power saving, NR supports adaptation of sidelink power saving resource allocation schemes (i.e., b/w random, partial, or full sensing-based resource selection)

  - Decisions related to congestion control for partial sensing UEs are discussed when design of partial sensing mechanism is completed (at least enough details are defined)

  - FFS how to support congestion control in case of random resource selection if no SL reception is supported by UE

  - The design of SL DRX cycle needs to ensure that UE partial sensing behavior is respected (i.e. UE wake up time intervals for the purpose of partial sensing need to be aligned with On duration intervals, as well as traffic characteristics)

  - Defined partial sensing procedure (periodic and contiguous) need to be fulfilled irrespective of the SL DRX status

  - Sensing in the SL DRX inactive time is up to UE implementation

References

- RP-202846, “WID revision: NR sidelink enhancement”, LGE, December 2020.

- R1-2109632, “Design of Inter-UE Coordination Solutions for Sidelink Communication”, Intel Corporation, October 2021.

- R1-2109589, “Response to RAN2 LS Related to Resource Selection”, Intel Corporation, October 2021.

- 3GPP TR 38.840, “NR; Study on User Equipment (UE) power saving in NR (Release 16)”, v16.0.0, June 2019.

- R1-2007501, “Final Report of 3GPP TSG RAN WG1#102-e”, v1.0.0, October 2020.

- R1-210xxxx, “Draft Report of 3GPP TSG RAN WG1#103-e”, v0.2.0, Available at: https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Report/Draft_Minutes_report_RAN1%23103-e_v020.zip, November 2020.

- 3GPP TS 38.214, “NR; Physical layer procedures for data (Release 16)”, v16.7.0, September 2021.

Appendix – A: Evaluation Assumptions

## Assumptions for Evaluations with partial sensing

Table 1: System level evaluation assumptions power saving evaluations

| Parameter | Value |
| --- | --- |
| Deployment scenario | - NR V2X Urban scenario - Vehicle speed: 60 km/h - Number of pedestrian UEs: 500 |
| Channel model | TR 37.885, NR V2X Channel Model |
| Spectrum allocation | Carrier frequency: 6GHz Simulated bandwidth: 40 MHz |
| Subcarrier spacing | 30 kHz |
| Communication Mode(s) | Groupcast, connection-less, 150 m range |
| UE-to-UE Association | - UE is considered as member of the group based on location principle - Each UE may be a member of a multiple groups |
| TTI structure | NR Slot TTI: - Slot w/o PSFCH zone: 1 AGC symbol, 1 GAP symbol, 2 PSSCH symbols w/ pilots - Slot w/ PSFCH zone: 1 AGC symbol, 2 GAP symbol, 2 PSSCH symbols w/ pilots, 2 symbols for PSFCH |
| PSFCH Zone Allocation | Enabled, in every 2nd slot |
| Sidelink control TX parameters | 64 Bits QPSK Modulation |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB - PSSCH: 35 PRB |
| SCI/Data time resource allocation | - PSCCH: 3 Symbols - PSFCH: 2 Symbols - PSSCH: Remaining symbols in slot |
| Data Packet Tx parameters | Aperiodic variable packet size evaluations: - 200 B packet: QPSK, 2 TTI - 400 B packet: 16-QAM, 2 TTI - 600 B packet: 16-QAM, 2 TTI - 800 B packet: 16-QAM, 2 TTI - 1000 B packet: 16-QAM, 3 TTI - 1200 B packet: 16-QAM, 3 TTI - 1400 B packet: 64-QAM, 3 TTI - 1600 B packet: 64-QAM, 3 TTI - 1800 B packet: 64-QAM, 4 TTI - 2000 B packet: 64-QAM, 4 TTI |
| HARQ Feedback | Disabled |
| Vehicle UE |  |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| V2V Traffic model | Aperiodic variable packet size traffic (TR 37.885 Aperiodic Model 1 traffic): - Packet size: uniform in the range [200..2000] B with quantization step of 200 B - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Latency requirement: 50 ms |
| Resource selection | - Resource selection with full sensing - Resource selection window size: 32 slots |
| Pedestrian UE |  |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| P2V Traffic model | Aperiodic variable packet size traffic: - Packet size: uniform in the range [200..800] B with quantization step of 200 B - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms - Latency requirement: 100 ms |
| Resource selection | - Full sensing - Resource selection window size: 32 slots - Partial sensing - Sensing window is allocated right after resource selection trigger event - Sensing is performed until the last TTI transmission - Resource selection window starts with 16 ms offset after resource selection trigger event - Resource selection window size: 32 slots - Partial sensing with early in time resource selection - Sensing window is allocated right after resource selection trigger event - Sensing is performed until the last TTI transmission - Resource selection window starts with 8 ms offset after resource selection trigger event - Resource selection window size: 32 slots |