3GPP TSG RAN WG1 #106bis-e                                             R1-2109646

e-Meeting, October11th – 19th, 2021

Source: 	Intel Corporation

Title: 	UE Features for NR Positioning Enhancement

Agenda item:	8.17.5

Document for: 	Discussion and Decision

# Introduction

In this contribution, we provide our initial views on UE features for NR positioning based on preliminary input provided in [R1-2108679].

# Views on UE Features for NR Positioning Enhancement

## NR Positioning in RRC_INACTIVE State

In Rel.17, the support for NR positioning for RRC_INACTIVE UEs has been added. The corresponding FGs need to be added to UE feature list.

### Consideration on FGs for DL PRS and SRS for positioning resources

In Rel.16, the FGs for DL PRS resources, SRS for positioning resources for various NR positioning techniques were defined. We assume that the same FGs are applicable for R17 UEs supporting NR positioning enhancements including RRC-INACTIVE UEs.

### Measurement reports by RRC_INACTIVE UEs

In Rel.16 the UE feature groups have been defined for measurement reports for each positioning technique. The same principle can be applied for RRC_INACTIVE UEs.

  - Add the following FGs for NR positioning measurement reports by UEs in RRC_INACTIVE state:

    - Support of DL PRS Measurement Report for DL-AoD in RRC_INACTIVE state

    - Support of DL PRS Measurement Report for DL-TDoA in RRC_INACTIVE state

    - Support of UE Rx-Tx Measurement Report for Multi-RTT in RRC_INACTIVE state

    - Support of SS-RSRP RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of SS-RSRQ RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of CSI-RSRP RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of CSI-RSRQ RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of simultaneous DL-AoD and DL-TDoA processing in RRC_INACTIVE state

    - Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state

The potential alternative way for Rel.17 UEs could be to extend relevant Rel.16 FGs for support by RRC_INACTIVE UEs.

## SRS for positioning transmission by RRC_INACTIVE UEs

RAN1 has agreed on SRS for positioning transmission by RRC_INACTIVE UEs. The discussion on type of SRS for positioning to be supported (except periodic SRS for positioning) is still open. The support of OLPC and spatial relationship was agreed to be reused from Rel.16. The support of SRS for positioning by RRC_INACTIVE UEs should be reflected in UE features.

  - Add the following FGs for SRS for positioning transmission by UEs in RRC_INACTIVE state, i.e.:

    - Support of periodic SRS for positioning resources by UE in RRC_INACTIVE UEs

    - [Support of semi-persistent SRS for positioning resources by UE in RRC_INACTIVE UEs]

    - [Support of aperiodic SRS for positioning resources by UE in RRC_INACTIVE UEs]

    - OLPC for SRS for positioning based on DL PRS from serving cell

    - OLPC for SRS for positioning based on DL PRS from neighboring cells

    - OLPC for SRS for positioning based on SSB from serving cells

    - Pathloss maintenance

    - Spatial relation for SRS for positioning based on SSB from serving cell

    - Spatial relation for SRS for positioning based on CSI-RS from serving cell

    - Spatial relation for SRS for positioning based on DL PRS from serving cell

    - Spatial relation for SRS for positioning based on SSB from non-serving cells

    - Spatial relation for SRS for positioning based on DL PRS from non-serving cells

    - Spatial relation maintenance

The potential alternative way for Rel.17 UEs that could be discussed is to extend relevant Rel.16 FGs for support by RRC_INACTIVE UEs.

## Proposed NR Positioning in RRC_INACTIVE State

In addition, we have the following proposals and corrections to the table with a preliminary input:

  - Add FG for support of DL PRS measurement with low-latency MG mechanism

    - 27-y [Support of DL PRS measurements with low-latency measurement gap allocation mechanism]

  - Update the table based on corrections provided in Table 1

  - Remove details for UE DL PRS processing outside measurement gaps as those were not agreed

Table 1: Proposed corrections to preliminary input on UE features for NR positioning enhancements

| 7-x1 | Mitigation of UE Rx timing errors | The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA and Multi-RTT positioning Values: up to [8] |
| --- | --- | --- |
| 27-x2 | Mitigation of UE Tx timing errors | The maximum number of UE-TxTEG per UE, which is supported and reported by UE for UL TDOA and Multi-RTT positioning Values: up to [8] |
| 27-x3 | Mitigation of UE RxTx timing errors | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning Values: up to [8] |
| 27-x4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. Values: up to [8] |
| 27-z1 | Support of DL PRS RSRP of the first path | UE’s capability to support providing the PRS RSRP of the first path Note: Applicable for DL-TDOA and Multi-RTT |
| 27-z2 | Support of DL PRS RSRP reporting for more than 8 measurements. | UE capability to support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Limit the maximum number of DL PRS RSRP associated with the same Rx beam index |
| 27-y | Support of DL PRS measurements with low-latency measurement gap allocation mechanism |  |
| 27-u1 | Support of M-sample measurements. | The capability to support providing a measurement based on measuring M samples (instances) of a DL PRS resource set M = [1, 4]. FFS: other values. If the UE does not provide the capability, the UE supports M = 4 only. |
| 27-u5 | DL PRS measurement outside MG | [ ] |
| 27-u6 | DL PRS Processing Capability outside MG |  |
| 27-v1 | LOS/NLOS Indicator | UE’s capability to support reporting LoS/NLoS indicator to LMF for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning. Values: [0:0.1:1] |
| 27-w1 | Support of on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS request signalling |

# Conclusions

In this contribution, we have provided our initial views on UE features for the Rel.17 WI on NR positioning enhancements. In summary, we have following proposals:

  - Add the following FGs for NR positioning measurement reports by UEs in RRC_INACTIVE state:

    - Support of DL PRS Measurement Report for DL-AoD in RRC_INACTIVE state

    - Support of DL PRS Measurement Report for DL-TDoA in RRC_INACTIVE state

    - Support of UE Rx-Tx Measurement Report for Multi-RTT in RRC_INACTIVE state

    - Support of SS-RSRP RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of SS-RSRQ RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of CSI-RSRP RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of CSI-RSRQ RRM measurements for NR E-CID Positioning in RRC_INACTIVE state

    - Support of simultaneous DL-AoD and DL-TDoA processing in RRC_INACTIVE state

    - Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state

  - Add the following FGs for SRS for positioning transmission by UEs in RRC_INACTIVE state, i.e.:

    - Support of periodic SRS for positioning resources by UE in RRC_INACTIVE UEs

    - [Support of semi-persistent SRS for positioning resources by UE in RRC_INACTIVE UEs]

    - [Support of aperiodic SRS for positioning resources by UE in RRC_INACTIVE UEs]

    - OLPC for SRS for positioning based on DL PRS from serving cell

    - OLPC for SRS for positioning based on DL PRS from neighboring cells

    - OLPC for SRS for positioning based on SSB from serving cells

    - Pathloss maintenance

    - Spatial relation for SRS for positioning based on SSB from serving cell

    - Spatial relation for SRS for positioning based on CSI-RS from serving cell

    - Spatial relation for SRS for positioning based on DL PRS from serving cell

    - Spatial relation for SRS for positioning based on SSB from non-serving cells

    - Spatial relation for SRS for positioning based on DL PRS from non-serving cells

    - Spatial relation maintenance

  - Add FG for support of DL PRS measurement with low-latency MG mechanism

    - 27-y [Support of DL PRS measurements with low-latency measurement gap allocation mechanism]

  - Update the table based on corrections provided in Table 1

  - Remove details for UE DL PRS processing outside measurement gaps as those were not agreed

# References

- RP-210903	, Revised WID on NR Positioning Enhancements, Intel Corporation, CATT

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), August 2021

# 27. Preliminary input on UE features for NR positioning enhancements from [R1-2108679]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-x1 | Mitigation of UE Rx timing delays | The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFF: whether to have separate values for DL TDOA and/or Multi-RTT positioning |  | No |  | Mitigation of UE Rx timing delays is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x2 | Mitigation of UE Tx timing delays | The maximum number of UE-TxTEG per UE, which is supported and reported by UE for UL TDOA and/or Multi-RTT positioning FFS; the values (>1). FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FF: whether a UE supports different values for UL TDOA and/or Multi-RTT positioning |  | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. FFS: whether gNB needs to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x3 | Mitigation of UE RxTx timing delays | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning FFS; the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated |  | No |  | Mitigation of UE RxTx timing delays is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS; The values (>1). | 27-x1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. |  |
| 27. NR_pos_enh | 27-z1 | Support of PRS RSRP of the first path | UE’s capability to support providing the PRS RSRP of the first path |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-z2 | Support of DL PRS RSRP reporting for more than 8 measurements. | UE capability to support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Limit the maximum number of DL PRS RSRP associated with the same Rx beam index |  | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u1 | Support of M-sample measurements. | The capability to support providing a measurement based on measuring M samples (instances) of a DL PRS resurce set M=[1, 4]. FFS: other values. If the UE does not provide the capability, the UE is assume to support M=4 only. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u5 | PRS measurement outside MG | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG. Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 2B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2C refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u6 | PRS Processing Capability outside MG | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-u5 | No |  |  | Per band | n/a |  |  | FFS |  |
| 27. NR_pos_enh | 27-v1 | LOS/NLOS Indicator | UE’s capability to support reporting LoS/NLoS indicator to LMF for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning. FFS: whether to have separate capability component for RSTD and UE Rx-Tx time difference measurements. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-w1 | Support of on-demand PRS | UE’s capability to support UE-initiated on-demand PRS. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |