3GPP TSG RAN WG1 #106bis-e			R1-2109650

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.10

Source: 	Intel Corporation

Title: 	UE features for IAB

Document for:	Discussion and decision

# 1   Introduction

It has been summarized in [1] with preliminary inputs on RAN1 UE features for Rel-17 NR. Regarding Rel-17 NR IAB enhancements, the following feature groups have been listed (full version is attached in the end of this document).

- [31-1]: Guard symbols

- [31-2]: Restricted DU beam indication

- [31-3]: Recommended/[non-preferred] MT beam indication

- [31-4]: Timing case indication

- [31-5]: Case 7 timing offset

- [31-6]: DL TX power adjustment reporting

- [31-7]: DL TX power adjustment reception

- [31-8]: Desired UL TX power adjustment reporting

In this contribution, based on those preliminary inputs, we further discuss on UE features for IAB with current agreements and conclusions.

# 2   Support DCI format 2_5 enhancements for frequency-domain soft availability indication

Semi-static frequency-domain H/S/NA configuration has been agreed in RAN1#105e and per-resource type frequency-domain H/S/NA configuration has been further agreed in RAN1-106e.

- The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

- The semi-static configuration of H/S/NA resource type in frequency domain is provided per RB set, per D/U/F resource type within a slot.

For frequency-domain soft availability indication, RAN1#105e-RAN1#106e has agreed to reuse DCI format 2_5 to support soft resource availability indication for frequency-domain resources, with three alternatives for down-selection.

- DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources

  - FFS: If additional enhancements are necessary

- To support soft resource availability in the frequency domain, the existing DCI 2_5 format is reused according to one of the following alternatives:

- Alt. 1: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 2: Multiple DCI format 2_5 can be received indicating availability with the granularity of one or more RB set(s) for different RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 3: A single DCI format 2_5 can be received indicating availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell.

Comparing with Rel-16 per-slot DCI format 2_5 time-domain soft availability indication, all three alternatives are per-resource type frequency-domain soft availability indication. Therefore, all three alternatives need DCI format 2_5 enhancements for frequency-domain soft availability indication. Hence, new feature group is needed to support DCI format 2_5 enhancements for frequency-domain soft availability indication.

Proposal 1: New feature group is needed to support DCI format 2_5 enhancements for frequency-domain soft availability indication.

# 3   Support Multiplexing Mode Adaptation Reporting

Regarding multiplexing mode adaptation, RAN1#104bis-e and RAN1#105-e have the following agreements.

- Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following):

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (0including whether the adaptation is dynamic or semi-static)

FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

- The parent IAB-node is dynamically provided with conditions/parameters to facilitate adaptation between multiplexing operation modes:

  - FFS: Required number of guard symbols for switching of multiplexing mode (FFS: per timing mode or per multiplexing mode) for IAB-DU

  - FFS: Signalling procedure

  - FFS: Required guard band for FDM

  - FFS: other conditions, e.g. required timing mode, required power control parameters, and preferred TCI.

Therefore, an IAB-MT can dynamically provide parent IAB-node with conditions/parameters to facilitate multiplexing modes adaptation. Accordingly, new feature group is needed to support multiplexing mode adaptation reporting.

Proposal 2: New feature group is needed to support multiplexing mode adaptation reporting.

# 3   Conclusion

In this contribution, we discussed feature groups regarding Rel-17 NR UE features for IAB.  It is summarized by the following proposals.

Proposal 1: New feature group is needed to support DCI format 2_5 enhancements for frequency-domain soft availability indication.

Proposal 2: New feature group is needed to support multiplexing mode adaptation reporting.

# 4   References

[1] R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, RAN1#106e, August 2021.

# NR_IAB_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception |  | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | Restricted DU beam indication | Support [Simultaneous Operation DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | Recommended/[Non-preferred] MT beam indication | Support [Simultaneous Operation MT Beam Indication] transmission |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | Timing case indication | Support [Timing Case Indication] reception |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing offset | Support [Case7 Timing Offset] reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | support [Desired DL TX Power Adjustment] reporting |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting, is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | DL TX power adjustment | support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-8 | Desired UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |