3GPP TSG RAN WG1 #106bis-e                                             R1-2109651

e-Meeting, October11th – 19th, 2021

Source: 	Intel Corporation

Title: 	UE features for NR SL enhancement

Agenda item:	8.17.11

Document for: 	Discussion and Decision

# Introduction

In this contribution, we provide our initial views on UE features for NR sidelink enhancements based on preliminary input provided in [1].

# Views on UE Features for NR Sidelink Enhancement

## Power Saving Resource Allocation Schemes

So far RAN1 has not had much discussion regarding support of dedicated FG 32-2. In our view FG 32-2 can be merged as a component of FG 32-1. The functionality to receive PSFCH and S-SSB can be beneficial for UE power saving especially for the case of random resource selection with sidelink HARQ enabled but it does not need to be a separate FG. Therefore, we have following proposal:

  - Delete FG 32-2 and define FG 32-1 with following component

    - UE can be configured to receive NR PSCCH / PSSCH / PSFCH / S-SSB or receive PSFCH / S-SSB only

## Inter-UE coordination schemes

In Rel.17, two inter-UE coordination schemes were agreed for sidelink communication (scheme 1 and scheme 2). These schemes have different mechanisms and can operate independently. Therefore FG 32-5 can be divided at least into two FGs:

32-x5 Inter-UE coordination scheme 1 in NR sidelink mode 2

32-y5 Inter-UE coordination scheme 2 in NR sidelink mode 2

Further progress on design needs to be made to decide whether further split is needed. For instance, the FG for inter-UE coordination scheme 1 can be further divided into the following FGs:

Request-based inter-UE coordination with the feedback of preferred resource set

Request-based inter-UE coordination with the feedback of non-preferred resource set

Condition-based inter-UE coordination with the feedback of preferred resource set

Condition-based inter-UE coordination with the feedback of non-preferred resource set

These aspects can be further discussed once more design details are available.

At this stage, we have following proposal for inter-UE coordination FG:

  - Split FG 32-5 into two FGs:

    - 32-x5: Inter-UE coordination scheme 1 in NR sidelink mode 2

    - 32-y5: Inter-UE coordination scheme 2 in NR sidelink mode 2

  - Further discuss if additional split is needed based on progress of inter-UE coordination framework

## Proposed Corrections to Preliminary Table

In addition, we propose the following corrections to the table with a preliminary input for UE sidelink features:

  - Update preliminary input on UE features for sidelink enhancements based on corrections provided in Table 1

Table 1: Proposed corrections to preliminary input on UE features for NR sidelink enhancements

| 32-1 | [Receiving NR sidelink of PSCCH/PSSCH/PSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None |
| --- | --- | --- | --- |
| 32-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by NR Uu or pre-configuration. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [32-1] |
| 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or pre-configuration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. 4) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or pre-configuration. | [32-1], [32-3] |
| 32-x5 | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of preferred resource set only and/or non-preferred resource set. | [32-1] |
| 32-y5 | Inter-UE coordination scheme 2 in NR sidelink mode 2 | - UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [32-1] |

# Conclusions

In this contribution, we have provided our initial views on UE features for R17 WI on NR sidelink enhancements. In summary, we have following proposals:

  - Delete FG 32-2 and define FG 32-1 with following component

    - UE can be configured to receive NR PSCCH / PSSCH / PSFCH / S-SSB or receive PSFCH / S-SSB only

  - Split FG 32-5 into two FGs:

    - 32-x5: Inter-UE coordination scheme 1 in NR sidelink mode 2

    - 32-y5: Inter-UE coordination scheme 2 in NR sidelink mode 2

  - Further discuss if additional split is needed based on progress of inter-UE coordination framework

  - Update preliminary input on UE features for sidelink enhancements based on corrections provided in Table 1

# References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), August 2021

- Preliminary input on UE features for NR sidelink enhancements from [R1-2108679]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by NR Uu or preconfiguration. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [32-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [32-1], [32-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [32-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |