3GPP TSG RAN WG1 Meeting #106bis-e 	     R1-2109712

E-meeting, October 11 - October 19, 2021

Agenda Item:	8.17.7

Source:	NTT DOCOMO

Title:	Rel-17 UE features for UE power saving

Document for:	Discussion and Decision

Introduction

In this paper, we share our views on Rel-17 UE features for UE power saving. The discussions here are in reference to the R1-2108679 [1].

Rel-17 UE features for UE power saving

We itemized our suggested corrections to the existing UE features as follows.

- FG 29-1:

- For the Components: In the RAN plenary#93, support of PDCCH based PEI is agreed and the following should be added.

  - New DCI format

In addition to the above, TRS availability indication via PEI is working assumption. Thus, this should be added with bracket.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication 3. Support New DCI Format 4. [Support TRS availability] |

- FG 29-3:

- For the Components: In the RAN1 #106-e, there is Working Assumption at most 3 SSSGs is supported to be configured. As the behaviour [2B] is necessary only for configuring 3 SSSGs,  behavior 2/2A, i.e., 2 SSSG, and behavior [2B], i.e., 3 SSSG, should be separately indicated as capability indication.

- For the Components: Following note should be added: ‘Support of PDCCH monitoring adaptation behaviour 1/1A’, ‘Support of PDCCH monitoring adaptation behaviour 2/2A’ , ‘Support of PDCCH monitoring adaptation behaviour [2B]’ and combination of behaviors may be separately indicated based on further RAN1 discussion/decision.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | (1). Support of PDCCH monitoring adaptation behaviour 1/1A (2). Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] (3). Support of PDCCH monitoring adaptation behaviour [2B] Note: (1), (2), (3) and combinations of behaviors may be separately indicated based on further RAN1 discussion/decision. |

According to the above comments, the following revision is proposed for UE power saving.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-1:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication 3. Support New DCI Format 4. [Support TRS availability] |

- FG 29-3:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | (1). Support of PDCCH monitoring adaptation behaviour 1/1A (2). Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] (3). Support of PDCCH monitoring adaptation behaviour [2B] Note: (1), (2), (3) and combinations of behaviors may be separately indicated based on further RAN1 discussion/decision. |

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-1:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication 3. Support New DCI Format 4. [Support TRS availability] |

- FG 29-3:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | (1). Support of PDCCH monitoring adaptation behaviour 1/1A (2). Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] (3). Support of PDCCH monitoring adaptation behaviour [2B] Note: (1), (2), (3) and combinations of behaviors may be separately indicated based on further RAN1 discussion/decision. |

Reference

[1].	R1-2108679, 	Preliminary RAN1 UE features list for Rel-17 NR, 	AT&T, NTT DOCOMO, INC.