3GPP TSG RAN WG1 #106bis-e			R1-2109714

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.8

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NR coverage enhancement

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.17.8 regarding UE features for NR coverage enhancement and captures the following email discussion.

[106bis-e-R17-UE-features-CovEnh-01] Email discussion UE features for NR coverage enhancement – Shinya (DOCOMO) - 1st check point: October 14 - Final check point: October 19

In the preliminary RAN1 UE features list for Rel-17 NR [1], there are following feature groups for NR coverage enhancement.

- 30-1	Increased maximum number of PUSCH Type A repetitions

- 30-1a	Increased maximum number of Type 2 configured grant PUSCH Type A repetitions

- 30-2	PUSCH Type A repetitions based on available slots

- 30-2a	Configured grant PUSCH Type A repetitions based on available slots

- 30-3	TB processing over multi-slot PUSCH

- 30-4	[The maximum duration for DM-RS bundling]

- 30-4a	[DM-RS bundling for PUSCH repetition type A]

- 30-4b	[DM-RS bundling for PUSCH repetition type B]

- 30-4c	[DM-RS bundling for TB processing over multi-slot PUSCH]

- 30-4d	[DMRS bunding for PUCCH repetitions]

- 30-4e	[Inter-slot frequency hopping with inter-slot bundling for PUSCH]

- 30-4f	[Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling]

- 30-4g	[Restart DM-RS bundling after the events that violate power consistency and phase continuity]

- 30-5	Dynamic PUCCH repetition indication

- 30-6	Msg3 repetition

Based on the discussions summarized in Sections 2-6, following is the suggested list of issues to be discussed and priority order considering RAN2 impact especially for capability signaling design, which are tagged and colour coded with High priority, Medium priority, or Low priority.

FL proposal of list of issues/proposals and priority:

- High priority issues (such as a certain FG is necessary or not):

  - Discuss whether/how to separate/merge FGs 30-1 to 30-2a

  - Discuss whether/how to separate FG 30-3

  - Discuss whether/how to include the capability for the repetition of TB processing over multi-slot PUSCH

  - Discuss whether/how to include the capability for the maximum concurrent TBOMS transmissions supported by a UE across all carriers when operating in UL-CA

  - Discuss whether UE can report different values of maximum duration for DMRS bundling for (a) different modulation orders, (b) back-to-back and non-back-to-back transmissions

  - Discuss whether/how to revise the structure for FGs 30-4 and 30-4x

  - Discuss whether FG 30-5 can be kept as “Slot based Dynamic PUCCH repetition indication”

  - Discuss whether FG 30-6 is necessary or not

- Medium priority issues (such as components and type that have capability signaling impacts):

  - Discuss whether the column for “Mandatory/Optional” in FGs 30-1 to 30-2a can be confirmed as “Optional with capability signaling”

  - Discuss whether the type of FGs 30-1 to 30-2a should be per UE or per band

  - Discuss whether the column for “Mandatory/Optional” in FG 30-3 can be confirmed as “Optional with capability signaling”

  - Discuss whether the type of FG 30-3 should be per UE or per band

  - Discuss xDD/FRx differentiation for FG 30-3

  - Discuss whether the column for “Mandatory/Optional” in FGs 30-4 and 30-4x can be confirmed as “Optional with capability signaling”

  - Discuss whether the type of FGs 30-4 and 30-4x should be per UE, per band, or per FS

  - Discuss xDD/FRx differentiation for FGs 30-4 and 30-4x

  - Discuss whether the column for “Mandatory/Optional” in FG 30-5 can be confirmed as “Optional with capability signaling”

  - Discuss whether the type of FG 30-5 should be per UE or per band

  - Discuss FDD/TDD differentiation for FG30-5

  - Discuss whether capability signaling is necessary for FG 30-6, i.e., whether to support as optional with capability signaling or optional without capability signaling

  - Discuss whether the type of FG 30-6 should be per UE or per band

  - Discuss FDD/TDD differentiation for FG 30-6

- Low priority issues (such as components that do not have capability signaling impacts)

  - Discuss whether/how to revise the sentence in “Consequence if the feature is not supported by the UE” in FGs 30-1 to 30-2a

  - Discuss whether/how to revise the prerequisite feature groups for FGs 30-1 to 30-2a

  - Discuss whether/how to revise any other contents in FGs 30-1 to 30-2a which do not have capability signaling impacts

  - Discuss whether/how to revise the prerequisite feature groups for FG 30-3

  - Discuss whether/how to revise any other contents in FG 30-3 which do not have capability signaling impacts

  - Discuss whether/how to revise the prerequisite feature groups for FGs 30-4 and 30-4x

  - Discuss whether/how to revise any other contents in FGs 30-4 and 30-4x which do not have capability signaling impacts

  - Discuss whether/how to revise the prerequisite feature groups for FG 30-5

  - Discuss whether/how to revise any other contents in FG 30-5 which do not have capability signaling impacts

  - Discuss whether/how to revise any other contents in FG 30-6 which do not have capability signaling impacts

- 30-1 to 30-2a: Enhancements for PUSCH Type A repetitions

In [1], FGs 30-1 to 30-2a are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configured grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | ZTE | In [1], two separate UE feature groups (FG 30-1 and 30-1a) are introduced for dynamic grant (DG) and configured grant (CG) PUSCH respectively. In our view, there is no need two separate UE FGs as there is no complexity difference to support increased maximum number of repetitions between DG and CG PUSCH. This is similar as Rel-16 URLLC FG 11-5, where only one FG is introduced for dynamic PUSCH repetition indication for both DG and CG PUSCH. Based on above, we propose to merge FG 30-1 and FG 30-1a. In such case, the prerequisite FG should be updated correspondingly. Regarding the reporting type, we think per UE reporting is sufficient. Proposal 1: Merge FG 30-1 and FG 30-1a with the following revisions. Features; Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 30. NR_cov_enh; 30-1; Increased maximum number of PUSCH Type A repetitions; Support the number of repetitions K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 for PUSCH Type A repetitions times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list, by adding an additional column for the number of repetitions in the TDRA table. A row index of the TDRA list is indicated by a DCI or a Type 1 configured grant configuration.; [5-17] One of {5-14, 5-16, 5-17}; [Per UE]. 30. NR_cov_enh; 30-1a; Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. - The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration.; [5-16], [30-1]; [Per UE]. Similar as Proposal 1, we propose to merge FG 30-2 and FG 30-2a. The prerequisite FG is not needed as legacy FG is based on counting by physical slots. Regarding the reporting type, we think per UE reporting is sufficient. As for the ‘Need of FDD/TDD differentiation’, it depends on further discussion in AI 8.8.1.1. Proposal 2: Merge FG 30-2 and FG 30-2a with the following revisions. Features; Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation. 30. NR_cov_enh; 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions are determined on the basis of available slots including both dynamic grant and configured grant PUSCH repetition type A. RV is cycled across transmission occasions.; [5-17]; [Per UE]; FFS. 30. NR_cov_enh; 30-2a; Configurecd grant PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions.; [5-14 or 5-16], [30-2]; [Per UE]; FFS. |
| --- | --- | --- |
| [4] | Huawei, HiSilicon | As enhanced features in Rel-17, considering UE capabilities and flexibility, all the UE features for coverage enhancement listed in [1] should be optional with capability signaling. Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling. |
| [5] | Samsung | Current FGs for enhanced PUSCH repetition Type A are separated for dynamic grant (DG, FG 30-1/30-2) and for configured grant (CG, FG 30-1a/30-2a). It is the approach taken in Rel-15 for PUSCH repetition, e.g., FG 5-16/5-17. On the other hand, in Rel-16, a single capability (FG 11-5) consists of DG and CG. In our view, the Rel-16 approach simplifies the capability signalling. Even for UE implementation perspective, we do not see the value of differentiating DG and CG from UE capability. In case of FG 30-2/30-2a, it is our understanding that as long as a UE supports enhanced PUSCH repetition Type A, the UE is also capable of counting available slots. Hence, we do not see the need for a separate UE capability on counting method. In these regards, FG 30-1/30-1a/30-2/30-2a can be combined into a single feature group. Proposal 1: Index; Feature group; Components. 30-1; Increased maximum number of Enhanced PUSCH Type A repetitions; 1) K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. 2) For dynamic grant, a row index of the TDRA list is indicated by a DCI. 3) For Type 2 configured grant, a row index of the TDRA list is indicated by a Type 1 configured grant configuration. 30-1a; Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration.. 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions.. 30-2a; Configurecd grant PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions.. |
| [6] | Intel | As listed in the Appendix, UE feature groups 30-1 and 30-1a are included for increased maximum number of repetitions for PUSCH repetition type A enhancement. Note that as agreed in RAN1#104-e meeting, the maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH [2]. Following this agreement, it may be more appropriate to further divide the UE feature group for type 1 and type 2 based CG-PUSCH for the increased maximum number of repetitions. The same principle can also apply for the PUSCH repetition type A based on available slots. Based on the discussions above, Table 1 illustrates suggested update for UE feature groups for PUSCH repetition type A enhancement. Table 1. UE feature groups for PUSCH repetition type A enhancement Index; Feature group; Components. 30-1; Increased maximum number of PUSCH Type A repetitions for DG-PUSCH; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI.. 30-1a; Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions for Type 1 CG-PUSCH; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration.. 30-1b; Increased maximum number of PUSCH type A repetitions for Type 2 CG-PUSCH; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 repetitions. 30-2; PUSCH Type A repetitions based on available slots for DG-PUSCH; Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions.. 30-2a; Configurecd grant PUSCH Type A repetitions based on available slots for Type 1 CG-PUSCH; Transmission occasions for K repetitions for Type 1 configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions.. 30-2b; PUSCH Type A repetitions based on available slots for Type 2 CG-PUSCH; Transmission occasions for K repetitions for Type 2 configured grant PUSCH are determined on the basis of available slots.. Proposal 1 - For PUSCH repetition type A enhancement, divide UE feature groups into DG-PUSCH, Type 1 and Type 2 CG-PUSCH. - Consider Table 1 as a starting point for discussion of feature groups for PUSCH repetition type A enhancement. |
| [7] | Sharp | One discussion point is what are the prerequisite features for the increased maximum number of PUSCH Type A repetitions. More specifically, whether the Rel-17 CovEnh feature supporting up to 32 repetitions requires Rel-16 URLLC feature supporting up to 16 repetitions as a prerequisite feature. In our view, coverage enhancement should be more fundamental feature and to be implemented wider range of UE types, compared to ultra-reliability. Therefore, FG 30-1 should not require feature groups other than FG 5-17, i.e., Rel-15 PUSCH repetition. Proposal 1: Prerequisite feature groups for 30-1 should be FG 5-17 only. Nothing else is needed. |
| [8] | Ericsson | For Type A PUSCH repetition, a set of UE features discussed so far are summarized and updated in Table 1. Main updates proposed are: 30-1: The Rel-15 repetition factor of PUSCH repetition is configured in a semi-static way. While in Rel-16, it can be dynamically indicated by DCI 0_1 or DCI 0_2. Based on the agreement in RAN1#106e that DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists, we think Rel-17 PUSCH repetition Type A with increased maximum repetition numbers is based on Rel-16 PUSCH repetition Type A, which is [11-6]. 30-1a: 2 Typos are corrected. And given [30-1] already includes [11-6] which is for both CG Type 2 PUSCH and DG PUSCH, we should remove [5-16]. 30-2: Time domain resource allocation of Rel-17 PUSCH repetition based on available slots is configured in the same way as PUSCH repetition with increased number of repetitions. Therefore, its prerequisite feature group is Rel-16 PUSCH repetition with dynamic indication of number of repetitions [11-6]. 30-2a: Since whether Type 1 PUSCH will be enhanced is still open, no agreement has been made yet. So we should remove [5-14] (Type 1 configured PUSCH repetitions over multiple slots) at this stage unless RAN1 agrees on enhancing CG Type 1 PUSCH repetition to support repetition factors counted based on available slot, and clarify that Type 2 is supported in the feature group name. And given [30-2] already includes [11-6] which is for both CG Type 2 PUSCH and DG PUSCH, we should also remove [5-16]. Table 1: Capabilities for PUSCH Repetition Type A Enhancement Index; Feature group; Components; Prerequisite feature groups; Comments. 30-1; Increased maximum number of PUSCH Type A repetitions; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI.; [5-17] [11-6]. 30-1a; Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 12 configured grant configuration.; [5-16], [30-1]. 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions.; [5-17] [11-6]. 30-2a; Type 2 configured grant PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions for configured grant Type 2 PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions.; [5-14 or 5-16], [30-2]. UE features for PUSCH Repetition Type A Enhancement are defined according to Table 1 |
| [9] | Qualcomm | General remark applicable to all features for NR coverage enhancement Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. 30. NR_cov_enh; 30-1; Increased maximum number of PUSCH Type A repetitions; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI.; [5-17]; Yes; N/A; UE does not support more than 16 repetitions. In some cases repetitions are limited to 8 (legacy type 1 CG PUSCH); [Per UE] Per Band; No N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-1a; Increased maximum number of Type 2 configured grant PUSCH Type A repetitions Is this for Type 1 or Type 2? Please add 30-2b for the other type.; K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. The above text mentions Type 1, but feature group description points to Type 2. Needs clarification.; [5-16], [30-1] Dependence on [30-1] is not necessary.; Yes; N/A; UE does not support more than 16 repetitions for Type 2 configurecd grant PUSCH. In some cases repetitions are limited to 8 (legacy type 1 CG PUSCH); [Per UE] Per Band; No N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions for PUSCH scheduled by DCI are determined on the basis of available slots. RV is cycled across transmission occasions.; [5-17]; Yes; N/A; UE does not support PUSCH repetitions counted on the basis of available slots.; [Per UE] Per Band; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-2a; Configured grant PUSCH Type A repetitions based on available slots; Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions.; [5-14 or 5-16], [30-2] Dependence on [30-2] is not necessary.; Yes; N/A; UE does not support configured grant PUSCH repetitions counted on the basis of available slots.; [Per UE] Per Band; FFS N/A; No N/A; N/A; [Optional with capability signalling]. |
| [10] | Nokia, Nokia Shanghai Bell | Detailed comments on UE features are listed below. For reference, the feature list is available in the Annex. - 30-1: - Move candidate values to notes column. No need to list the legacy values, hence only list values for K>16. - Add FG11-6 (PUSCH repetition Type A) as pre-requisite - 30-1a: - Similarly to FG30-1, move values to notes column and restrict range to K>16 - 30-2: - Add 30-1 as pre-requisite - 30-2a: - Replace pre-requisite FG5-16 with FG30-1a Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list. |

Discussion

[FL1] High priority question 2-1:

- Companies are encouraged to provide views on whether/how to separate/merge FGs 30-1 to 30-2a, e.g.,

  - Option 1:

    - Merge FGs 30-1 and 30-1a into an FG

    - Merge FGs 30-2 and 30-2a into an FG

  - Option 2:

    - Merge FGs 30-1, 30-1a, 30-2, and 30-2a into an FG

  - Option 3:

    - Split 30-1 and 30-1a into 3 separate FGs: 1st one for DG, 2nd one for type 1 CG, 3rd one for type 2 CG

    - Split 30-2 and 30-2a into 3 separate FGs: 1st one for DG, 2nd one for type 1 CG, 3rd one for type 2 CG

  - Option 4:

    - Any other FG structures

| Company | Comment |
| --- | --- |
| QC | Option 3 would be preferred at this stage since how we support the increased number of repetitions for CG-PUSCH is still being debated. |
| NTT DOCOMO | We prefer Option 1. It’s up to RRC discussion, on the other hand we expect single RRC configuration (e.g. numberOfRepetitions-r17 or AvailableSlotCounting.) can be used for both DG-PUSCH and CG-PUSCH, so that FGs can be merged. |
| Intel | We prefer Option 3. It would be more reasonable to separate the support of maximum number of repetitions and counting based on available slots for DG-PUSCH, Type 1 and Type 2 CG-PUSCH. |
| Nokia, NSB | We do not support splitting the FGs. No strong option on merging though, as long as individual components cannot be disabled by the corresponding signaling. |
| ZTE | We prefer Option 1 or Option 2. This is similar as Rel-16 URLLC FG 11-5, where only one FG is introduced for dynamic PUSCH repetition indication for both DG and CG PUSCH. |
| vivo | No need to change current FG structure on 30-1,30-1a,30-2,30-2a. |
| Sharp | Our 1st preference is to keep the current FG structure, but we can live with the other options as well. One comment on type 1 CG from our side is that whether type 1 CG supports up to 32 repetitions or not is still under discussion in AI 8.8.1.1. Therefore, we suggest adding “if supported” for type 1 CG in Option 3. |
| Samsung | We support Option 2. - As also pointed out in our contribution, there is a trade-off among the options: UE implementation/testing efforts vs. capability signaling overhead, UE implementation fragmentation. Having the precedent in Rel-16 for combining DG and CG into a single FG (i.e., FG 11-5), we don’t see the need for separate FG for DG and CG. - It is our understanding that as long as a UE supports Rel-17 enhanced PUSCH repetition Type A, the UE is also capable of counting available slots. Hence, we do not see the need for a separate UE capability on counting method. |
| CMCC | Option 2 (1st preference) and options 1(2nd ) are supported. Both the enhancement of maximum repetition number and available slot should be supported by UE for Rel-17 CE. Which type of PUSCH or which feature are activated should be based on RRC configurations. Whether type 1 CG-PUSCH is supported are still under discussion. Then option 3 is not supported. |
| Lenovo, Motorola Mobility | Our preference is Option 1 to keep increased number of repetitions and available slot counting as two separate features. |
| Huawei, Hisilicon | We prefer option 1, where increasing the maximum repetition number and the counting based on available slot could be two independent features. |
| Apple | Our first preference is option 3, the second choice is option 2. The reason is that supporting 32 repetitions for type 1 and type 2 CG PUSCH is still open, different solution could be adopted for both CG type. |
| FL2 | According to the comments provided so far, companies have different views: - Option 1: DOCOMO, Nokia, NSB, ZTE, CMCC Lenovo, Motorola Mobility, Huawei, Hisilicon - Option 2: Nokia, NSB, ZTE, Samsung, CMCC, Apple - Option 3: Qualcomm, Intel, Apple, MediaTek - Option 4 (Current structure is kept): vivo, Sharp Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| Ericsson | Prefer Option 1. Whether type 1 CG-PUSCH is supported are still under discussion as CMCC points out. |
| MediaTek | Prefer Option 3. It may not be necessary or required for UE to enhance all types of repetitions. |
| FL4 | According to the comments provided so far, companies have different views: - Option 1: DOCOMO, Nokia, NSB, ZTE, CMCC Lenovo, Motorola Mobility, Huawei, Hisilicon, Ericsson - Option 2: Nokia, NSB, ZTE, Samsung, CMCC, Apple - Option 3: Qualcomm, Intel, Apple, MediaTek - Option 4 (Current structure is kept): vivo, Sharp Given that Option 1 has the most support, following proposals is made 1) to merge FGs 30-1a into 30-1a and to merge FGs 30-2a into 30-2 and 2) FFS whether/how to separate FGs 30-1/30-2 into multiple FGs. Please note that here the proposal is discussing whether FGs 30-1/30-2 are necessary or not. any contents highlighted in yellow mean FFS and to be discussed further. [FL4] High priority proposal 2-1: - FG 30-1a is merged into FG 30-1 as follows - FG 30-2a is merged into FG 30-2 as follows 30. NR_cov_enh; 30-1; Increased maximum number of PUSCH Type A repetitions; - K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions for DG-PUSCH. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. - K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions for Type 2 CG PUSCH. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether/how to separate FG 30-1 into multiple FGs; [5-16], [5-17]; Yes; N/A; UE does not support more than 16 repetitions.; [Per UE]; No; No; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-2; PUSCH Type A repetitions based on available slots; - Transmission occasions for K repetitions for DG-PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. - Transmission occasions for K repetitions for CG-PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. FFS whether/how to separate FG 30-2 into multiple FGs; [5-14 or 5-16], [5-17]; Yes; N/A; UE does not support PUSCH repetitions counted on the basis of available slots.; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| NTT DOCOMO | We support the FL4 proposal. |
| QC | The FG structures need to mirror UE feature groups. Given the precedence set in R15 to split DG and CG, we prefer to keep them separated. We therefore do not agree with the FL proposal. |
| Samsung | We are fine with the [FL4] proposal 2-1 above. We understand the same principle (i.e., with “FFS ….”) is applied between [FL4] proposal 2-1 and [FL4] proposal 3-1. Whether or not to split DG and CG: There is another example in Rel-16 FGs (e.g., FG 11-5, 15-2, 15-25) where DG and CG are merged into a single FG. It is observed that the companies who have the preference on either option 1 or option 2 are along with merging DG and CG. |
| Nokia, NSB | We are fine with FL4. |
| Sharp | Fine with the FL4 proposal. |
| Ericsson | Support |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study the FG structure toward the next RAN1 meeting considering the comments provided so far. |

Medium priority question 2-2:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FGs 30-1 to 30-2a can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| Nokia, NSB | Optional with capability signaling |
| Sharp | Fine with “Optional with capability signaling” for all of them. |
| Samsung | All Rel-17 FGs should be “Optional with capability signaling”. We may need to decide which FG is the basic FG for Rel-17 Coverage Enhancement. |
| CMCC | FG 30-1 to 30-2a could be mandatory for Rel-17 CE. |
| Huawei, Hisilicon | Optional with capability signalling. |
| Ericsson | Fine |
| Lenovo, Motorola Mobility | Yes, fine to confirm “Optional with capability signalling” |

Medium priority question 2-3:

- Companies are encouraged to provide views on whether the type of FGs 30-1 to 30-2a should be per UE or per band

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE should be sufficient |
| Sharp | Either is fine. |
| CMCC | Per UE should be supported. |
| Huawei, Hisilicon | Per UE is sufficient |
| Qualcomm | Please see our first proposal in our tdoc. With UE testing and certification in mind, we would like this feature type to be per band. |
| Ericsson | Per UE is fine |
| Lenovo, Motorola Mobility | Per UE |

Low priority question 2-4:

- Companies are encouraged to provide views on whether/how to revise the sentence in “Consequence if the feature is not supported by the UE”, e.g.,

  - Add “In some cases repetitions are limited to 8 (legacy type 1 CG PUSCH)” in FGs 30-1 and/or 30-1a

| Company | Comment |
| --- | --- |

Low priority question 2-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 30-1 to 30-2a

| Company | Comment |
| --- | --- |
| vivo | If increased number of repetitions can only be triggered by numberOfRepetitions in TDRA table, FG 11-6 should be perquisite for FG 30-1 and FG 30-1a. |
| Sharp | Prefer to keep the current prerequisite FG for 30-1 |
| Ericsson | As mentioned in [8] and quoted above, we provide our views briefly. For 30-1, as the column components says, The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI, this is Rel-16 enhanced PUSCH repetition Type A, the prerequisite feature for 30-1 is [11-6] Rel-16 PUSCH repetition Type A, rather than [5-17] Rel-15 slot aggregation which is configured in a semi-static way. Similarly, 30-1a also actually depends on 11-6, rather than 5-16. However, given [30-1] already includes [11-6] which is for both CG Type 2 PUSCH and DG PUSCH, we should remove [5-16]. 30-2: Time domain resource allocation of Rel-17 PUSCH repetition based on available slots is configured in the same way as [30-1] PUSCH repetition with increased number of repetitions. Therefore, its prerequisite feature group is Rel-16 PUSCH repetition with dynamic indication of number of repetitions [11-6]. 30-2a: Since whether Type 1 PUSCH will be enhanced is still open, no agreement has been made yet. So we should remove [5-14] (Type 1 configured PUSCH repetitions over multiple slots) at this stage unless RAN1 agrees on enhancing CG Type 1 PUSCH repetition to support repetition factors counted based on available slot, and clarify that Type 2 is supported in the feature group name. And given [30-2] already includes [11-6] which is for both CG Type 2 PUSCH and DG PUSCH, we should also remove [5-16]. |

Low priority question 2-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 30-1 to 30-2a which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Ericsson | Since whether Type 1 PUSCH will be enhanced for PUSCH repetition based on available slots is still open, no agreement has been made yet. So at this stage we advise to clarify that Type 2 is supported in the feature group name unless RAN1 agrees on enhancing CG Type 1 PUSCH repetition to support repetition factors counted based on available slot. Namely, 30-2a Type 2 configured grant PUSCH Type A repetitions based on available slots |

- 30-3: TB processing over multi-slot PUSCH

In [1], FG 30-3 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | ZTE | For UE FG 30-3 for TBoMS, we basically agree with that one FG is sufficient based on the agreements so far. More components could be added once all related functionalities are clear. However, we don’t think the prerequisite FG is needed as FG 11-6 is for single TB operation and TBoMS only borrows part of the features from FG 11-6, i.e., the time domain location, while not others e.g., RV design. Regarding the reporting type, we think per UE reporting is sufficient. As for the ‘Need of FDD/TDD differentiation’, it depends on further discussion in AI 8.8.1.2. As a result, we have the following proposal. Proposal 3: For FG 30-3 for TBoMS, no prerequisite FG is needed and per UE reporting is sufficient. Features; Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation. 30. NR_cov_enh; 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode.; [11-6]; [Per UE]; FFS. |
| --- | --- | --- |
| [3] | vivo | In RAN1#106e, it has been agreed that the number of slots for TBoMS is counted on available slots for UL transmission, and the available slots is determined based on the mechanisms determined for type-A PUSCH repetition counted on available slots. Hence, feature 30-2, i.e., type-A PUSCH repetitions counted on available slots, should be considered as perquisite feature for feature 30-3. Besides, the value range of number of slots for a single TBoMS is discussing in AI 8.8.1.2, and it should be captured in the component for feature 30-3. Proposal 1: Feature 30-2 should be considered as perquisite feature for feature 30-3. And value range of number of slots for a single TBoMS should be captured in the component for Feature 30-3. The revised UE Feature 30-3 is provided in following table. 30. NR_cov_enh; 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode. Supported value range for a single TBoMS including { FFS }; [11-6] 30-2; Yes; N/A; UE does not support TB processing over multi-slot PUSCH.; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. |
| [4] | Huawei, HiSilicon | As enhanced features in Rel-17, considering UE capabilities and flexibility, all the UE features for coverage enhancement listed in [1] should be optional with capability signaling. Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling. For FG 30-3 to FG 30-5, there is no justification or evidence to support the need for FDD/TDD differentiation. The “FFS” should be “No”. Proposal 2: For FG 30-3, FG 30-4, FG 30-4x and FG 30-5, there is no need of FDD/TDD differentiation for all the UE features. |
| [6] | Intel | As agreed in the RAN1#106-e meeting, TB processing over multiple slots (TBoMS) is supported for both configured grant and dynamic grant [3]. In this case, it is more appropriate to differentiate DG-PUSCH and Type 1/2 CG-PUSCH for the UE feature group. Table 2 illustrates suggested update for UE feature groups for TBoMS. Table 2. UE feature groups for TBoMS Index; Feature group; Components. 30-3; TB processing over multi-slot PUSCH for DG-PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode for DG-PUSCH.. 30-3a; TB processing over multi-slot PUSCH for Type-1 CG-PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode for Type 1 CG-PUSCH.. 30-3b; TB processing over multi-slot PUSCH for Type-2 CG-PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode for Type 2 CG-PUSCH.. Proposal 2 - For TBoMS, divide UE feature groups into DG-PUSCH, Type 1 and Type 2 CG-PUSCH. - Consider Table 2 as a starting point for discussion of feature groups for TBoMS. |
| [7] | Sharp | In RAN1#106-e, it was agreed that the counting based on available slots is used to derive the number of slots for TBoMS. Agreement The number of slots allocated for TBoMS is counted based on the available slots for UL transmission. - The determination of available slots for PUSCH repetition type A, as defined in AI 8.8.1.1, is reused. - Note: Available slots for FDD or SUL could be revisited according to discussion in AI 8.8.1.1. So far, there is no other agreement on the derivation of the number of the slots for TBoMS. Therefore, FG 30-2 “PUSCH Type A repetitions based on available slots” should be a prerequisite feature for FG 30-3 “TB processing over multi-slot PUSCH”. Proposal 2: 30-2 “PUSCH Type A repetitions based on available slots” should be a prerequisite feature for 30-3 “TB processing over multi-slot PUSCH”. In addition, if we have two different feature groups, FG 30-2, i.e., DG-PUSCH with the counting based on available slots, and FG 30-2a, i.e., CG-PUSCH with the counting based on available slots. It is more natural to introduce two different TBoMS feature groups, one is for DG-PUSCH and the other is for CG-PUSCH. Proposal 3: Introduce two different TBoMS feature groups, one is for DG-PUSCH and the other is for CG-PUSCH. |
| [8] | Ericsson | 30-3: The transmission of a single TBoMS is over at least two slots. This needs indication of UE’s capability. [11-6] is the enhanced PUSCH repetition Type A in Rel-16 with the number of repetitions jointly coded with SLIV in TDRA table. Although TBoMS uses Type A-like resource allocation, it is a different type of UL transmission. We don’t see the need of a prerequisite feature beyond basic PUSCH transmission. 30-3a: As agreed in RAN1#106e, TBoMS with repetition is supported, which can be considered as sub-feature of TBoMS. Table 2: Capabilities for Transport Block over Multi-slot PUSCH Index; Feature group; Components; Prerequisite feature groups; Comments. 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode.; [11-6]. 30-3a; Repetition of TB processing over multi-slot PUSCH; Support of repetition of TB processing over multi-slot PUSCH in RRC connected mode.; [30-3]. UE features for transport block over multi-slot PUSCH are defined according to Table 2. |
| [9] | Qualcomm | General remark applicable to all features for NR coverage enhancement Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. On TBOMS Proposal 5: Consider the following additional feature for TBOMS: - Maximum concurrent TBOMS transmissions supported by a UE across all carriers when operating in UL-CA. Additional comments and suggested changes to the UE features table: Proposal 6: Additional comments and suggested changes to the UE features table for coverage enhancement are included in the following table, with changes marked in red. 30. NR_cov_enh; 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode.; [11-6]; Yes; N/A; UE does not support TB processing over multi-slot PUSCH.; [Per UE] Per Band; FFS N/A; No N/A; N/A; [Optional with capability signalling]. |

Discussion

[FL1] High priority question 3-1:

- Companies are encouraged to provide views on whether/how to separate FG 30-3, e.g.,

  - Option 1:

    - Split 30-3 into 3 separate FGs: 1st one for DG, 2nd one for type 1 CG, 3rd one for type 2 CG

  - Option 2:

    - Split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG

  - Option 3:

    - Any other FG structures

| Company | Comment |
| --- | --- |
| QC | Option 1 or 2 would be preferred. We think separate FGs for DG and CG could be useful. PUSCH repetition counting could either be based on physical slots or available slots, and irrespective of the counting method, we think TBOMS is equally applicable. |
| NTT DOCOMO | Our first preference is Option 2. As DG and type 2 CG are expected to have the same allocated slot indication mechanism in common, they can be merged into one FG. This follows the same structure of FG 11-6, where TDRA-based repetition factor indication FG covers both DG and type 2 CG. |
| Intel | We prefer Option 1. Similar comments as for enhancement on repetition type A. |
| Nokia, NSB | We do not see a need to split the FGs |
| ZTE | Option 3. We don’t see the need for split. |
| vivo | Both opt-1 and opt-2 are fine to us. |
| Sharp | This should follow whether FG30-2/2a/2b are separate or merged. |
| Samsung | No need to split FG 30-3, i.e., current FG is ok. When we look at the TBoMS feature, it matters whether UE supports to determine the TBS for multiple slots, while DG or CG will matter to how UE determine the slots. So with UE already signalled that whether it supports Type1-CG or Type2-CG, UE doesnot need to further separate it for TBoMS feature. |
| CMCC | Option 3. No need to split the FG |
| Lenovo, Motorola Mobility | We don’t see the need to split the feature group individually for DG, type 1 CG and type 2 CG |
| Huawei, Hisilicon | No need to split the features. Current FG is fine. |
| Apple | Either option 1 or option 2 is fine for us. |
| FL2 | According to the comments provided so far, companies have different views: - Option 1: Qualcomm, Intel, vivo, Apple, MediaTek - Option 2: Qualcomm, DOCOMO, vivo, Apple, MediaTek - Option 3: - Current structure: Nokia, NSB, ZTE, Samsung, CMCC, Lenovo, Motorola Mobility, Huawei, Hisilicon - Follow the structure for FGs 30-2/2a/2b: Sharp Given that majority companies prefer to keep current structure, following proposal is made to confirm FG 30-3 is kept as “TB processing over multi-slot PUSCH” [FL2] High priority proposal 3-1: - FG 30-3 is kept as “TB processing over multi-slot PUSCH” as follows 30. NR_cov_enh; 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode.; [11-6]; Yes; N/A; UE does not support TB processing over multi-slot PUSCH.; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Nokia, NSB | We support the FL proposal. |
| Qualcomm | We do not agree. The prerequisites go back to R15 features 5-16 and 5-17. We prefer to preserve separate lineage of prerequisites and not merge them into one. |
| NTT DOCOMO | Fine with the proposal. |
| Ericsson | Support the FL proposal |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| MediaTek | We prefer Option 1 or Option 2. Separated FGs are preferred. |
| FL4 | Given that companies still have different view, FFS is added in the column of components to further discuss whether/how to separate FG 30-3 into multiple FGs [FL4] High priority proposal 3-1: - FG 30-3 is kept as “TB processing over multi-slot PUSCH” as follows 30. NR_cov_enh; 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether/how to separate FG 30-3 into multiple FGs; [11-6]; Yes; N/A; UE does not support TB processing over multi-slot PUSCH.; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| NTT DOCOMO | Support the proposal. |
| QC | Its easier to merge than to split at a later stage. Given the precedence, we don’t understand why we wouldn’t split them up. Separation between DG and CG is helpful to have. |
| Samsung | We are fine with the [FL4] proposal 3-1 above. We understand the same principle (i.e., with “FFS ….”) is applied between [FL4] proposal 2-1 and [FL4] proposal 3-1. |
| Nokia, NSB | We support the FL proposal. |
| Sharp | Fine with the proposal. |
| Ericsson | Support the proposal. We prefer Option 2. |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study the FG structure toward the next RAN1 meeting considering the comments provided so far. |

[FL1] High priority question 3-2:

- Companies are encouraged to provide views on whether/how to include the capability for the repetition of TB processing over multi-slot PUSCH

| Company | Comment |
| --- | --- |
| QC | Don’t see a strong need. |
| NTT DOCOMO | We support introducing the capability for the repetition of TB processing over multi-slot PUSCH. |
| Intel | We slightly prefer to introduce capability for repetition of TBoMS. |
| ZTE | Ok to introduce one FG for repetition of TBoMS, with prerequisite on support of FG 30-3. |
| vivo | No need. |
| Sharp | OK to include it. |
| Samsung | No need to introduce a separate capability for the repetition of TBoMS. If a UE supports TBoMS and PUSCH repetition independently, it means the UE also supports the repetition of TBoMS. We believe this simplifies the capability signalling. |
| CMCC | Currently no need |
| Lenovo, Motorola Mobility | We are fine to include a separate capability for repetition of TB over multi-slot PUSCH |
| Huawei, Hisilicon | No need to introduce a separate feature. |
| Apple | We are ok to introduce the capability for repetition of TBoMS. |
| FL2 | According to the comments provided so far, companies have different views: - Need to introduce an FG: DOCOMO, ZTE, Sharp, Lenovo, Motorola Mobility, Apple - No need to introduce an FG: Qualcomm, Intel, vivo, Samsung, CMCC, Huawei, Hisilicon Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| Ericsson | We are OK to introduce a separate capability for TBoMS repetition. This is similar to Rel-15 where UL slot aggregation is an independent feature from single-slot TB transmission without repetition. Here TBoMS is analogous to single-slot TB, and its repetition can be a separate capability. |
| FL4 | According to the comments provided so far, companies still have different views: - Need to introduce an FG: DOCOMO, ZTE, Sharp, Lenovo, Motorola Mobility, Apple, Ericsson - No need to introduce an FG: Qualcomm, Intel, vivo, Samsung, CMCC, Huawei, Hisilicon Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| QC | We revise our position. We prefer to introduce a separate capability to better align with R15 structure. We would like to have R15 slot aggregation feature as a prerequisite. |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study whether/how to include the capability for the repetition of TB processing over multi-slot PUSCH toward the next RAN1 meeting considering the comments provided so far. |

[FL1] High priority question 3-3:

- Companies are encouraged to provide views on whether/how to include the capability for the maximum concurrent TBoMS transmissions supported by a UE across all carriers when operating in UL-CA

| Company | Comment |
| --- | --- |
| QC | This is quite important as TBOMS transmissions are likely to lock up certain UE resources over long periods of time. Some additional memory and state mananagement across slots may be required. |
| NTT DOCOMO | Since it has not been agreed yet to support concurrent TBoMS transmission in UL-CA, this capability is not necessary at this point. |
| Intel | Suggest to defer the discussion at later stage. |
| ZTE | Support TBoMS for CA with a separate UE capability |
| Samsung | UL CA (and UL MIMO) is not the target scenario for coverage enhancement. No need to discuss this aspects in UE feature session. |
| CMCC | Since TBOMS in UL CA are not supported, no need to further discuss this. |
| Lenovo, Motorola Mobility | Currently nothing has been agreed on this feature, so don’t see the point to discuss this now |
| Huawei, Hisilicon | No need to discuss this currently. |
| Apple | This is depending on the progress on AI 8.12.2 whether UL CA is supported. |
| FL2 | According to the comments provided so far, companies have different views: - Need to introduce an FG: Qualcomm, ZTE - Wait for some progress in AI 8.8.1.2: DOCOMO, Intel, Samsung, CMCC, Lenovo, Motorola Mobility, Huawei, Hisilicon, MediaTek Therefore, no additional proposal is made for now, but any company can propose to add the FG when some progress is made AI 8.8.1.2. |
| Ericsson | Agree with other companies and with FL that this does not need to be discussed now. |

Medium priority question 3-4:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FG 30-3 can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| Nokia, NSB | OK to confirm |
| Sharp | Fine with “Optional with capability signaling” |
| Samsung | All Rel-17 FGs should be “Optional with capability signaling”. We may need to decide which FG is the basic FG for Rel-17 Coverage Enhancement. (same comment above) |
| CMCC | Fine with “Optional with capability signaling” |
| Huawei, Hisilicon | OK to confirm. |
| Ericsson | Fine |

Medium priority question 3-5:

- Companies are encouraged to provide views on whether the type of FG 30-3 should be per UE or per band

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE |
| Sharp | Should follow the type of FG 30-2 |
| Samsung | We are open to discuss it. It is our understanding that other types are not precluded, e.g., per BC, per FS, and per FSPC. |
| CMCC | Per UE |
| Huawei, Hisilicon | Per UE |
| Qualcomm | Per band. Please see our general comment on UE feature type for all R17 features. |
| Ericsson | Per UE is our first preference |

Medium priority question 3-6:

- Companies are encouraged to provide views on xDD/FRx differentiation for FG 30-3

| Company | Comment |
| --- | --- |
| Ericsson | No differentiation between TDD and FDD is needed, as it was agreed that a single TBoMS can cross non-consecutive slots. We don’t see a need of FR differentiation, but can discuss. |

Low priority question 3-7:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 30-3

| Company | Comment |
| --- | --- |

Low priority question 3-8:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 30-3 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 30-4 to 30-4g: [DM-RS bundling]

In [1], FGs 30-4 to 30-4g are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | ZTE | Regarding DMRS bundling among PUSCH/PUCCH transmissions, the current FG list is a good starting point. However, we may not be able to further refine these FGs before more input from RAN4, or we can also decide some of the UE FGs could be left for RAN4, i.e., regarding as RAN4 FGs. For instance, RAN4 is still discussing whether the UE should report a maximum duration and the potential factors may have impact on the duration. Proposal 4: RAN1 further revisits the FGs related to DM-RS bundling among PUSCH/PUCCH transmissions based on further input from RAN4 or leave some of UE FGs as RAN4 FGs. If a maximum duration can be reported by UE, it would imply the UE supports DM-RS bundling at least for one of PUSCH repetition type A, repetition type B and TBoMS once the UE reports a value for maximum duration. Therefore, at least one of FG 30-4a, 30-4b and 30-4c should be merged into FG 30-4 if it is agreed by RAN4. In addition, RAN4 has agreed the maximum duration should be the same for different cases for both PUSCH and PUCCH. Therefore, we don’t think a separate FG 30-4d for DMRS bundling for PUCCH is needed. Proposal 5: If RAN 4 would agree a maximum duration can be reported by UE (i.e. confirming FG 30-4), at least one of FG 30-4a, 30-4b and 30-4c should be merged into FG 30-4, and a separate FG 30-4d for DMRS bundling for PUCCH is not needed. |
| --- | --- | --- |
| [3] | vivo | For DMRS bundling related features, it depends on detailed RAN4 FR requirements on power consistency, phase continuity, and it may be FR specific or band specific, may be not per UE feature. In [R4-2114991], RAN4 reply that the maximum duration (feature 30-4) maybe per band or per FR dependent, and Feature 30-4 is considered as perquisite for other DMRS bundling features. Hence, whether feature 30-4 and 30-4(a)-(g) is per band feature, or need FR1/FR2 differentiation, need to be confirmed by RAN4. Proposal 2: Whether feature 30-4 and 30-4(a-g) is per ‘band/UE’, or ‘need of FR1/FR2 differentiation’ should be confirmed by RAN4. The suggested revision DMRS bundling related UE features are provided in following table. 30. NR_cov_enh; 30-4a; [DM-RS bundling for PUSCH repetition type A]; Support DM-RS bundling for PUSCH repetition type A; [30-4], [30-1] or [30-2]; Yes; N/A; UE does not Support DM-RS bundling for PUSCH repetition type A; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4b; [DM-RS bundling for PUSCH repetition type B]; Support DM-RS bundling for PUSCH repetition type B; [30-4], [11-5] [30-1]; Yes; N/A; UE does not Support DM-RS bundling for PUSCH repetition type B; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4c; [DM-RS bundling for TB processing over multi-slot PUSCH]; Support DM-RS bundling for TB processing over multi-slot PUSCH; [30-4], [30-3]; Yes; N/A; UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4d; [DMRS bunding for PUCCH repetitions]; Support DM-RS bundling for PUCCH repetitions; [30-4], [4-23]; Yes; N/A; UE does not support DMRS bunding for PUCCH repetitions; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4e; [Inter-slot frequency hopping with inter-slot bundling for PUSCH]; Support inter-slot frequency hopping with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c]; Yes; N/A; UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4f; [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling]; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d]; Yes; N/A; UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4g; [Restart DM-RS bundling after the events that violate power consistency and phase continuity]; Support restarting DM-RS bundling after the events that violate power consistency and phase continuity; [30-4]; Yes; N/A; UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity; [Per UE] [Per band]; FFS; No [YES]; N/A; [Optional with capability signalling]. |
| [4] | Huawei, HiSilicon | As enhanced features in Rel-17, considering UE capabilities and flexibility, all the UE features for coverage enhancement listed in [1] should be optional with capability signaling. Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling. For FG 30-3 to FG 30-5, there is no justification or evidence to support the need for FDD/TDD differentiation. The “FFS” should be “No”. Proposal 2: For FG 30-3, FG 30-4, FG 30-4x and FG 30-5, there is no need of FDD/TDD differentiation for all the UE features. The values of the maximum duration are related to the phase continuity and power consistency tolerance, which is under discussion in RAN4 now. The tolerance should be the same for different bands. However, for the same tolerance, the CFOs of different operating frequency are different, which may cause the “compensation leftover for frequency error” be different. And UE’s implementation on different FR range to maintain the phase continuity could be different. Thus, it is a good choice to define the UE feature based on the granularity of Per UE and with FR1/FR2 differentiation. Proposal 3: For FG 30-4, - The UE feature of the maximum duration for DM-RS bundling should be based on the granularity of Per UE. - There is need of FR1/FR2 differentiation. |
| [5] | Samsung | It is understood that FGs 30-4/30-4a/30-4b/30-4c/30-4d/30-4e/30-4f/30-4g are within square bracket because the corresponding agreement is still working assumption or FFS. It can be further addressed based on RAN1 progress. FG 30-4a/30-4b/30-4c/30-4d define specific DM-RS bundling capability combined with PUSCH Type A repetition, PUSCH Type B repetition, PUSCH TBoMS, and PUCCH, respectively. Similar with above discussion on FG 30-1/30-1a, a single capability for DM-RS bundling would be enough. For example, if a UE supports DM-RS bundling and PUSCH TBoMS independently, it means the UE also supports DM-RS bundling for TBoMS (if agreed). In case of frequency hopping with DM-RS bundling (FG 30-4e/30-4f), there would be no functional difference between PUSCH and PUCCH. Note that RAN1 agreement states “to strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible”. Therefore, FG 30-4e and 30-4f can be consolidated. As for FG 30-4g, we do not agree with introducing this capability (even with square bracket). If a UE is capable of DM-RS bundling, it would be capable to do DM-RS bundling before/after an event. No need for a separate capability for after the event. Proposal 2: 30-4; [The maximum duration for DM-RS bundling]; The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH. 30-4a; [DM-RS bundling for PUSCH repetition type A]; Support DM-RS bundling for PUSCH repetition type A and PUCCH. 30-4b; [DM-RS bundling for PUSCH repetition type B]; Support DM-RS bundling for PUSCH repetition type B. 30-4c; [DM-RS bundling for TB processing over multi-slot PUSCH]; Support DM-RS bundling for TB processing over multi-slot PUSCH. 30-4d; [DMRS bunding for PUCCH repetitions]; Support DM-RS bundling for PUCCH repetitions. 30-4e; [Inter-slot frequency hopping with inter-slot bundling for PUSCH/PUCCH]; Support inter-slot frequency hopping with inter-slot bundling for PUSCH/PUCCH. 30-4f; [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling]; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling. 30-4g; [Restart DM-RS bundling after the events that violate power consistency and phase continuity]; Support restarting DM-RS bundling after the events that violate power consistency and phase continuity. |
| [6] | Intel | In the previous RAN1 meetings [2][3][4], the following use cases were agreed to be supported for joint channel estimation of PUSCH: - back-to-back PUSCH transmissions (of the same TB) for repetition type A and B scheduled by dynamic grant or configured grant - back-to-back PUSCH transmissions for TB processing over multiple slots - non-back-to-back PUSCH transmissions across consecutive slots (of the same TB) for repetition type A and type B scheduled by dynamic grant or configured grant. Considering different use cases as mentioned above, it is more appropriate to divide the UE feature groups for back to back PUSCH transmission and non-back to back PUSCH transmission. At the RAN1#106-e meeting, it was agreed that for DMRS bundling for PUCCH repetitions, RAN1 at least prioritize use cases 3 and 4a in R1-2104119 [3]. Hence, the same principle can also apply for DMRS bundling for PUCCH repetitions, i.e., UE feature groups can be divided into back to back PUCCH transmission and non-back to back PUCCH transmission. Note that in RAN4 reply LS [5], certain conditions need to be met in order to maintain phase continuity in case of other UL signals/channels in the gap between repetitions as follows: RAN4 has agreed for the case of other signals/channels in the gap between repetitions, it is not considered for UE to transmit other channels in the gap with different settings. For the case of other UL signals/channels in the gap between repetitions with same settings, as communicated in R4-2105417, RAN4 has further refined the conditions when phase continuity can be met as follows: - Signals/channels with repetitions and other UL signals/channels in the gap have the same: - PAPR and average power, e.g., PUSCH/PUCCH part of repetitions and SRS has same PAPR and average power. - Allocated number and locations of PRBs transmitted - Antenna port settings. Hence, addition UE feature group may need to be added for the case when other UL signals/channels are inserted in the gap between repetitions with same setting. Proposal 3 - For UE features for DMRS bundling, consider back to back PUSCH/PUCCH transmission and non-back to back PUSCH/PUCCH transmission. - For non-back to back PUSCH/PUCCH transmissions, add UE feature group for the case when other UL signals/channels are inserted in the gap between repetitions with same setting. |
| [7] | Sharp | For the support of DM-RS bundling for PUSCH repetition type A, we do not see the need to limit its use to the Rel-17 PUSCH repetition type A. It should also be applicable to Rel-15/16 PUSCH repetition Type A. Therefore, in our view the prerequisite feature groups for FG 30-4a are not only FG 30-1 and FG 30-2 but also FG 5-14, FG 5-16 and FG 5-17. Proposal 4: The Prerequisite feature groups for FG 30-4a should be FG 30-4 and at least one of FG 5-14, FG 5-16, FG 5-17, FG 30-1 and FG 30-2. |
| [8] | Ericsson | Features for joint channel estimation are described in the Table 3 below. Our rationale for each of the features is as follows: 30-4, 30-4a, & 30-4d: A basic UE feature supporting joint channel estimation should be defined jointly for PUCCH and PUSCH, given the RAN1#104 agreement to strive for a common design for PUSCH/PUCCH with DMRS bundling as much as possible. So we think 30-4, 30-4a and 30-4d can be merged into 30-4. DMRS bundling for PUSCH will be supported for Rel-15, -16, and -17 PUSCH repetition Type A and B as well as TBoMS. We note that 11-6, PUSCH Repetition Type A, depends on one of 5-16 and 5-17. Since prerequisites are what is required to configure a feature, and not what can be configured with a feature, this dependency on 5-16 and 5-17 means that 11-6 is not technically a prerequisite. Therefore, the prerequisites are 5-14, 5-16, 5-17, 11-5, or 30-3. As suggested (tentatively) by the moderator, the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), seems needed for PUCCH bundling for slots format 1/3/4. Similarly, the new Rel-17 feature 25-2, “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8” is needed for formats 0/2. The values for the time domain window size are still being discussed in RAN4, and so the candidate values are t.b.d. 30-4b: The working assumption confirmed in RAN1#106 identifies support for PUSCH repetition type B in a UE configured joint channel estimation as an additional UE capability. Therefore, in addition to the basic support of DMRS bundling, the UE will also need to support PUSCH repetition Type B, i.e. Rel-16 feature 11-5. We suggest clarifying the wording of 30-4b to be “Support DM-RS bundling when configured for PUSCH repetition type B” because DM-RS bundling does not change when configured for PUSCH repetition Type B, since the agreement is that it reuses only those joint channel estimation specification enhancements defined to support repetition Type A. 30-4c: Similar to 30-4b, our understanding is that DM-RS bundling for TBoMS is not any different than for Type A repetition, so it is not specifically for TBoMS, but the support is for DM-RS bundling when TBoMS is configured. Suggest: “Support DM-RS bundling when configured for TB processing over multi-slot PUSCH”. 30-4e: As discussed in [2] and [3], new inter-slot frequency hopping patterns provide notable gains for both PUSCH joint channel estimation and for TBoMS. PUSCH Type A scheduling (2-16) is assumed as a baseline for both of these features and so should be a prerequisite. Lastly, especially since UEs in a cell not supporting JCE or TBoMS should be able to hop with UEs configured for JCE and/or TBoMS, and since enhanced frequency hopping patterns can have gains for UEs not configured for JCE and/or TBoMS, 2-16 should be a sufficient prerequisite, and e.g. 30-4 and 30-3 should not be prerequisites. 30-4f: While we think enhanced frequency hopping for PUCCH is interesting, it is not included in the NR coverage enhancement WID and has not been developed during the work item in our understanding. So we suggest removing this for now. However, if similar patterns to those we agree for PUSCH can be used we are open to investigating this further. 30-4g: Agree with moderator’s approach, but suggest to rephrase somewhat. As agreed in RAN1#106, there is a UE capability for when a configured time domain window is split into multiple actual windows and DMRS bundling is restarted in the new window. Since whether the terminology ‘actual window’ and ‘configured window’ is used in specifications is FFS, describing the capability more directly as is done in 30-4g seems appropriate. The phrase ‘restarting DM-RS bundling’ seems a bit imprecise, and the event timing could be more clear. We suggest clarifying with the component description with “Support restarting PUSCH and PUCCH DM-RS bundling of slots remaining in a bundling window after a slot for which the an events that violates power consistency and phase continuity requirements” 30-4h: The working assumption confirmed in RAN1#106 requires that non-back-to-back PUSCH transmissions be a UE capability. RAN4 has indicated that it is possible to support phase continuity and power consistency for non-back-to-back transmission only where the gap is 13 symbols or less (in R4-2114991), but is still discussing whether continuity/consistency with DL slots between the bundled PUSCH repetitions is feasible. Furthermore, RAN4 has indicated that continuity/consistency can be maintained for back-to-back slots if the UE transmits other channels or signals between bundled PUSCHs, however the conditions are quite restrictive, and RAN4 is asking RAN1 about the consequences if bundling cannot be maintained in these cases. Furthermore, for PUCCH, RAN1 has agreed in RAN1#106 to prioritize the use case where UE does not transmit between the bundled PUCCHs. Therefore, the capability for non-back-to-back operation here only includes the case where UE does not transmit PUSCH or PUCCH in adjoining slots. The intention is that if support for additional non-back-to-back operation is agreed, e.g. for bundling across DL slots or where UE transmits other channels/signals between PUSCH/PUCCH repetitions, then additional capabilities can be defined. Table 3: Capabilities for PUSCH and PUCCH Joint Channel Estimation Index; Feature group; Components; Prerequisite feature groups; Comments. 30-4; [The maximum duration for DM-RS bundling] DMRS bundling; 1) DMRS bundling for PUSCH 2) DMRS bundling for PUCCH 3) Maximum window duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH; 1: [5-14, 5-16, 5-17, 11-5, or 30-3] 2: 4-23 or 25-2; Component-3: Candidate values T.B.D.. 30-4a; [DM-RS bundling for PUSCH repetition type A]; Support DM-RS bundling for PUSCH repetition type A; [30-4], [30-1] or [30-2]. 30-4b; [DM-RS bundling for PUSCH repetition type B]; Support DM-RS bundling when configured for PUSCH repetition type B; [30-4], [11-5] [30-1]. 30-4c; [DM-RS bundling for TB processing over multi-slot PUSCH]; Support DM-RS bundling when configured for TB processing over multi-slot PUSCH; [30-4], [30-3]. 30-4d; [DMRS bunding for PUCCH repetitions]; Support DM-RS bundling for PUCCH repetitions; [30-4], [4-23]. 30-4e; [Enhanced Inter-slot frequency hopping with inter-slot bundling for PUSCH]; Support enhanced inter-slot frequency hopping pattern with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c] 2-16. 30-4f; [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling]; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d]. 30-4g; [Restart DM-RS bundling after the events slots that violate power consistency and phase continuity]; Support restarting PUCCH and PUSCH DM-RS bundling of slots remaining in a bundling window after a slot for which the an events that violates power consistency and phase continuity requirements; [30-4]. 30-4h; DMRS bundling for discontinuous transmission between adjoining slots; Support for DMRS bundling when UE does not transmit between bundled PUSCH/PUCCH repetitions in adjoining slots; 30-4. UE features for PUSCH and PUCCH joint channel estimation are defined according to Table 3 |
| [9] | Qualcomm | General remark applicable to all features for NR coverage enhancement Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. On DMRS bundling for PUSCH/PUCCH repetitions Proposal 2: On UE features 30-4a to 30-4d: split these features into two components, one for back-to-back transmissions and another for non-back-to-back transmissions. Proposal 3: On UE features 30-4 to 30-4g: all features on DMRS Bundling (PUSCH and PUCCH) to be indicated at the per FS level. This is required due potential impact of operations such as uplink tx switching, uplink full power transmission, and interactions with other features such as UE’s MIMO coherence capability. Proposal 4: On UE feature 30-4 for maximum duration for DMRS bundling, UE may report different values for (a) different modulation orders, (b) back-to-back and non-back-to-back transmissions. Consider splitting this into multiple components. 30. NR_cov_enh; 30-4; [The maximum duration for DM-RS bundling]; The maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH. Max duration is only applicable to modulation order not higher than QPSK. For other modulation orders, DMRS bundling is not supported. TBD: (a) whether another row for pi/2 BPSK is required. (b) Candidate values (c) whether different values for back to back and non back to back transmissions are reported.; Yes; N/A; UE does not support DM-RS bundling for PUSCH/PUCCH; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4a; [DM-RS bundling for PUSCH repetition type A with back-to-back transmission]; Support DM-RS bundling for PUSCH repetition type A. This is applicable for back-to-back transmissions.; [30-4], [30-1] or [30-2]; Yes; N/A; UE does not Support DM-RS bundling for PUSCH repetition type A with back-to-back transmission; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4h; DMRS-bundling for PUSCH Repetition Type A with non-back-to-back transmission; Support DM-RS bundling for PUSCH repetition type A. This is applicable for non-back-to-back transmissions.; [30-4]; Yes; N/A; UE does not Support DM-RS bundling for PUSCH repetition type A with non-back-to-back transmission; Per FS; N/A; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4b; [DM-RS bundling for PUSCH repetition type B with back-to-back transmission]; Support DM-RS bundling for PUSCH repetition type B. This is applicable for back-to-back transmissions.; [30-4], [11-5] [30-1]; Yes; N/A; UE does not Support DM-RS bundling for PUSCH repetition type B with back-to-back transmission; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4i; DMRS-bundling for PUSCH Repetition Type B with non-back-to-back transmission; Support DM-RS bundling for PUSCH repetition type B. This is applicable for non-back-to-back transmissions.; Yes; N/A; UE does not Support DM-RS bundling for PUSCH repetition type B with non-back-to-back transmission; Per FS; N/A; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4c; [DM-RS bundling for TB processing over multi-slot PUSCH with back-to-back transmissions]; Support DM-RS bundling for TB processing over multi-slot PUSCH. This is applicable only for back-to-back transmissions.; [30-4], [30-3], [30-4a]; Yes; N/A; UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH with back-to-back transmissions; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4j; DM-RS bundling for TB processing over multi-slot PUSCH with non-back-to-back transmission; Support DM-RS bundling for TB processing over multi-slot PUSCH. This is applicable only for non-back-to-back transmissions.; [30-4], [30-3],[30-4h]; Yes; N/A; UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH with non-back-to-back transmissions; Per FS; N/A; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4d; [DMRS bunding for PUCCH repetitions with back-to-back transmissions]; Support DM-RS bundling for PUCCH repetitions with back-to-back transmissions; [30-4], [4-23]; Yes; N/A; UE does not support DMRS bunding for PUCCH repetitions with back-to-back transmissions; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4k; DMRS bunding for PUCCH repetitions with non-back-to-back transmissions; Support DM-RS bundling for PUCCH repetitions with non-back-to-back transmissions; [30-4], [4-23]; Yes; N/A; UE does not support DMRS bunding for PUCCH repetitions with non-back-to-back transmissions; Per FS; N/A; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4e; [Inter-slot frequency hopping with inter-slot DMRS bundling for PUSCH]; Support inter-slot frequency hopping with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c]; Yes; N/A; UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4f; [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] Unclear why this says “enhanced” but not 30-4e.; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d]; Yes; N/A; UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. 30. NR_cov_enh; 30-4g; [Restart DM-RS bundling after the events that violate power consistency and phase continuity within a configured TDW]; Support restarting DM-RS bundling after the events that violate power consistency and phase continuity; [30-4]; Yes; N/A; UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity within a configured TDW; [Per UE] Per FS; FFS N/A; No N/A; N/A; [Optional with capability signalling]. |

Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether UE can report different values of maximum duration for DMRS bundling for (a) different modulation orders, (b) back-to-back and non-back-to-back transmissions

| Company | Comment |
| --- | --- |
| QC | Yes to both (a) and (b). Else, UE will be forced to under-report based on worst case scenario and this is not ideal. |
| NTT DOCOMO | We are not sure why different maximum duration is preferred according to the modulation order. Since RAN4 is still in the discussion on maximum duration, we should wait for the reply from RAN4. |
| Intel | For (a), it may be better to wait RAN4 response first. For (b), it seems not necessary to report different values as this is still based on consecutive slots. |
| ZTE | We have similar view as Intel. |
| vivo | We think it may be beneficial to separately report the max duration for (a) and (b). it can be up to RAN4 to confirm the necessity. |
| Sharp | OK to keep them separate for now, as their necessities depend on RAN4 discussions. |
| Samsung | For (a), it should be further discussed based on RAN4 feedback. If the maximum duration depends on the modulation order, then UE can report different values for maximum duration. For (b) back-to-back and non-back-to-back transmissions, we don’t think the different values of maximum duration are needed. |
| Lenovo, Motorola Mobility | Similar views as Intel |
| Huawei, HiSilicon | We should postpone this discussion until the RAN4 discussion on the maximum duration progresses. |
| Apple | Agee to wait for RAN4’s reply. |
| FL2 | According to the comments provided so far, companies have different views: (a) different modulation orders - UE can report different values: Qualcomm - UE reports a single value: - Wait for RAN4 reply: DOCOMO, Intel, ZTE, vivo, Sharp, Samsung, Lenovo, Motorola Mobility, Apple, MediaTek (b) back-to-back and non-back-to-back transmissions - UE can report different values: Qualcomm - UE reports a single value: Intel, ZTE, Samsung, Lenovo, Motorola Mobility - Wait for RAN4 reply: vivo, Sharp, Apple, MediaTek Therefore, no additional proposal is made for now, but any company can propose them when RAN1 gets RAN4 reply. |
| Ericsson | Agree that we can wait for RAN inputs for both a) and b). |
| MediaTek | Agree to wait for RAN4 reply. |

[FL1] High priority question 4-2:

- Companies are encouraged to provide views on whether/how to revise the structure for FGs 30-4 and 30-4x, e.g.,

  - Q1: Whether to wait for RAN4 input before discussing the structure

  - Q2: Whether to split to back-to-back transmission and non-back-to-back transmission

    - Q2a: For non-back-to-back transmission, whether to add an FG for the case when other UL signals/channels are inserted in the gap between repetitions with same setting

  - Q3: Whether to merge FGs 30-4a, 30-4b, 30-4c and 30-4d into FG 30-4

  - Q4: Whether to merge FGs 30-4b, 30-4c, 30-4d, and 30-4g into FG 30-4a

  - Q5: Whether to merge FG 30-4f into FG 30-4e or remove FG 30-4f from the UE feature list

| Company | Comment |
| --- | --- |
| Qualcomm | Q1: We seem to have most of the necessary information to make top-level decisions. Okay to proceed in this meeting without additional RAN4 input. Open to asking for RAN4 input if some critical items are identified in this meeting. Q2: Splitting based on back-to-back and non-back-to-back is necessary. The underlying complexities are quite different and its conceivable that there can be scenarios where only one of the two are supported. We do not anticipate RAN1 supporting cases where there are other uplink channels in the middle --- the RAN4 requirements for phase continuity make this scenario practically irrelevant. Q3-5: We suggest not deleting any of the FGs at this moment. Open to considering 30-4a to 30-4d as sub-features of 30-4. |
| NTT DOCOMO | Since whether or not the maximum duration is further dependent on UE capabilities is still under discussion based on RAN4 LS (R4-2114991), we should wait for RAN4 inputs before discussing the structure. |
| Intel | Q2: We are fine to 1) split back-to-back transmission and non-back-to-back transmission and add 2) add an FG for the case when other UL signals/channels are inserted in the gap between repetitions with same setting for non-back-to-back transmission Q3/Q4: no need to merge as based on agreement/working assumption, it is subject to UE capability to support joint channel estimation for repetition type B, TBoMS, etc. Q5: no need to remove FG 30-4f. it should be a separate UE FG |
| Nokia, NSB | Same as above. We do not see the need for splitting back-to-back and non-back-to-back transmission. Similarly, we think 30-4e and 30-4f should be merged. |
| ZTE | In general, we think we need to wait for RAN4 as it is still discussing whether the UE should report a maximum duration and the potential factors may have impact on the duration. We don’t see need to split the FG according to whether it is back-to-back transmission or not. |
| vivo | Q1: in our understanding RAN1 can discuss the structure for FG30-4 and 30-4x, RAN4 information may have impacts on candidate value of the max duration, the FG is per UE/band , whether it is FR1/FR2 differentiated. And additional components and sub-features can be further included if necessary based on RAN4 information. Q3-Q5: merge on the sub-features under FG30-4 is not necessary. |
| Sharp | OK to keep them separate for now |
| Samsung | Q2: No need to split to back-to-back and non-back-to-back transmission. The sub-bullet would be discussed in [106bis-e-NR-R17-CovEnh-03]. Q3/Q4: We support to merge FGs 30-4b, 30-4c and 30-4d into FG 30-4a. A single capability would be sufficient Q5: Merger FG 30-4f into FG 30-4e. A single capability would be enough for inter-slot frequency hopping with DMRS bundling for PUSCH/PUCCH. Other: For the FG 30-4g, our view is that if a UE is capable of DMRS bundling by FG 30-4, it would be capable to do DMRS bundling before/after an event. Therefore, there is no need to introduce the FG 30-4g. |
| Lenovo, Motorola Mobility | Q1: We prefer to wait for RAN4 input Q2: We don’t see the need to split the FG between back-to-back and non-back-to-back transmission Q3: Yes, we prefer to merge Q4: Yes, we prefer to merge Q5:FG 30-4f should be included and merger with FG 30-4e |
| Huawei, Hisilicon | Some of the details may have to wait for input from RAN4. For Q2, it is necessary to split non-back-to-back transmission with other UL signals/channels in the gap from other cases. Open to Q3~Q5. |
| Apple | Q2: RAN4 is still investigating UL transmission in the middle of PUSCH transmission for non back to back case. Q3/Q4: prefer to keep the FG as it is. The precondition for each FGs are quite different. If therse are merged together, it would require the UE supporting many features at the same time, e.g., repetition type A/B, TBoMS, etc. |
| FL2 | According to the comments provided so far, companies have different views: Q1: Whether to wait for RAN4 input before discussing the structure - Yes: DOCOMO, ZTE, Lenovo, Motorola Mobility, Huawei, Hisilicon, Apple - No: Qualcomm, vivo Q2: Whether to split to back-to-back transmission and non-back-to-back transmission - Yes: Qualcomm, Intel, Sharp, MediaTek - Q2a: For non-back-to-back transmission, whether to add an FG for the case when other UL signals/channels are inserted in the gap between repetitions with same setting - Yes: Qualcomm, Intel, Huawei, Hisilicon, MediaTek - No: - No: Nokia, NSB, ZTE, Samsung, Lenovo, Motorola Mobility Q3: Whether to merge FGs 30-4a, 30-4b, 30-4c and 30-4d into FG 30-4 - Yes: Lenovo, Motorola Mobility - No: Intel, Sharp, Apple, MediaTek Q4: Whether to merge FGs 30-4b, 30-4c, 30-4d, and 30-4g into FG 30-4a - Yes: Lenovo, Motorola Mobility - No: Intel, Sharp, Samsung (remove FG 30-4g), Apple, MediaTek Q5: Whether to merge FG 30-4f into FG 30-4e or remove FG 30-4f from the UE feature list - Yes: Nokia, NSB, Lenovo, Motorola Mobility - No: Intel, Sharp, MediaTek Therefore, no additional proposal is made for now, but any company can propose them when RAN1 gets RAN4 reply or some progress is made in AI 8.8.1.3. |
| Ericsson | Q1: We can discuss some topics now. Q2/Q2a: Can’t answer at this time; wait for more discussions in RAN4. Q3: No. PUCCH should be different, and JCE for TBoMS was agreed to be a capability. 4, 4a, and 4b might be merged though. Q4: No. DMRS restart was agreed as a capability. Q5: Not for now, but can be further discussed according in 8.8.1.3 and 8.8.2. |
| MediaTek | Adding our view in FL summary table. In general, the features should be set separately for different channels (PUCCH/PUSCH) and the different use case (back-to-back，non-back-to-back) and slot level (for Type B PUSCH repetition, there is within-slot repetition and cross-slot B2B repetitions respectively). Moreover, PUCCH may not be the bottleneck for the most cases/bands, it is reasonable to separate it with PUSCH. |

Medium priority question 4-3:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FGs 30-4 and 30-4x can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| Nokia, NSB | OK to confirm |
| Samsung | All Rel-17 FGs should be “Optional with capability signaling”. We may need to decide which FG is the basic FG for Rel-17 Coverage Enhancement. (same comment above) |
| Huawei, Hisilicon | OK to confirm |
| QC | Yes. |
| Ericsson | OK to confirm |
| Lenovo, Motorola Mobility | Fine to confirm |

Medium priority question 4-4:

- Companies are encouraged to provide views on whether the type of FGs 30-4 and 30-4x should be per UE, per band, or per FS

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE |
| vivo | It depends on RAN4 further inputs, it could be per band based on previous RAN4 LS in R4-2114991. |
| Huawei, Hisilicon | Per UE |
| QC | Due to interactions with other operations such as uplink tx switching, we prefer all DMRS bundling related features to be indicated per FS. DMRS bundling is RF implementation dependent, so per FS would be the most natural choice. |
| Ericsson | Agree with vivo |
| Lenovo, Motorola Mobility | Per UE |

Medium priority question 4-5:

- Companies are encouraged to provide views on xDD/FRx differentiation for FGs 30-4 and 30-4x

| Company | Comment |
| --- | --- |
| vivo | It depends on RAN4 further inputs, it could be FR dependent based on previous RAN4 LS in R4-2114991. |
| Huawei, HiSilicon | Considering UE’s implementation on FR1 and FR2 to maintain the phase continuity could be different, FRx differentiation may be necessary. No need to differentiate xDD |
| QC | Since we suggested feature type be indicated as per FS, this question is N/A. |
| Ericsson | Agree with vivo |

Low priority question 4-6:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 30-4 and 30-4x

| Company | Comment |
| --- | --- |
| Ericsson | 30-4a: We note that 11-6, PUSCH Repetition Type A, depends on one of 5-16 and 5-17. Since prerequisites are what is required to configure a feature, and not what can be configured with a feature, this dependency on 5-16 and 5-17 means that 11-6 is not technically a prerequisite. Therefore, the prerequisites are 5-14, 5-16, 5-17, or 11-5 (if merged with 4b). 30-4d: As suggested (tentatively) by the moderator, the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), seems needed for PUCCH bundling for slots format 1/3/4. Similarly, the new Rel-17 feature 25-2, “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8” is needed for formats 0/2. The values for the time domain window size are still being discussed in RAN4, and so the candidate values are t.b.d. 30-4e: As discussed in [2] and [3], new inter-slot frequency hopping patterns provide notable gains for both PUSCH joint channel estimation and for TBoMS. PUSCH Type A scheduling (2-16) is assumed as a baseline for both of these features and so should be a prerequisite. Lastly, especially since UEs in a cell not supporting JCE or TBoMS should be able to hop with UEs configured for JCE and/or TBoMS, and since enhanced frequency hopping patterns can have gains for UEs not configured for JCE and/or TBoMS, 2-16 should be a sufficient prerequisite, and e.g. 30-4 and 30-3 should not be prerequisites. |

Low priority question 4-7:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 30-4 and 30-4x which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Ericsson | 30-4b: If not merged with 4a, we suggest clarifying the wording of 30-4b to be “Support DM-RS bundling when configured for PUSCH repetition type B” because DM-RS bundling does not change when configured for PUSCH repetition Type B, since the agreement is that it reuses only those joint channel estimation specification enhancements defined to support repetition Type A. 30-4c: Similar to 30-4b, our understanding is that DM-RS bundling for TBoMS is not any different than for Type A repetition, so it is not specifically for TBoMS, but the support is for DM-RS bundling when TBoMS is configured. Suggest: “Support DM-RS bundling when configured for TB processing over multi-slot PUSCH”. 30-4g: Agree with moderator’s approach, but suggest to rephrase somewhat. As agreed in RAN1#106, there is a UE capability for when a configured time domain window is split into multiple actual windows and DMRS bundling is restarted in the new window. Since whether the terminology ‘actual window’ and ‘configured window’ is used in specifications is FFS, describing the capability more directly as is done in 30-4g seems appropriate. The phrase ‘restarting DM-RS bundling’ seems a bit imprecise, and the event timing could be more clear. We suggest clarifying with the component description with “Support restarting PUSCH and PUCCH DM-RS bundling of slots remaining in a bundling window after a slot for which the an events that violates power consistency and phase continuity requirements” |

- 30-5: Dynamic PUCCH repetition indication

In [1], FG 30-5 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-5 | Dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | ZTE | For dynamic PUCCH repetition indication, we agree that one single FG is sufficient. We also confirm that the prerequisite FG 4-23 is needed and per UE reporting is sufficient. As for the ‘Need of FDD/TDD differentiation’, we don’t identify any differentiation is needed. Proposal 6: Adopt the following revisions for FG 30-5 for dynamic PUCCH repetition indication. Features; Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation. 30. NR_cov_enh; 30-5; Dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication; [4-23]; [Per UE]; FFS No. |
| --- | --- | --- |
| [4] | Huawei, HiSilicon | As enhanced features in Rel-17, considering UE capabilities and flexibility, all the UE features for coverage enhancement listed in [1] should be optional with capability signaling. Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling. For FG 30-3 to FG 30-5, there is no justification or evidence to support the need for FDD/TDD differentiation. The “FFS” should be “No”. Proposal 2: For FG 30-3, FG 30-4, FG 30-4x and FG 30-5, there is no need of FDD/TDD differentiation for all the UE features. |
| [8] | Ericsson | Features for PUCCH repetition enhancement are described in the Table 4. Our rationale is as follows: The URLLC work item in agenda item 8.3.1.1 also developed PUCCH repetition enhancements, but focused on PUCCH formats 0 and 2 and sub-slot based dynamic and semi-static PUCCH repetition. Based on the agreements from agenda item 8.3.1.1 below, we understand that slot based PUCCH repetition for formats 0 and 2 is to be supported for slot based (as well as sub-slot based) operation as a UE capability. This is captured in the proposed 25-2, “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8”, in [1]. Features 25-3, “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH sub-slots with configured K = 2, 4, 8”, and 25-3a “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication” address semi-static and dynamic sub-slot PUCCH repetition, respectively. Therefore, only dynamic slot based PUCCH repetition needs to be specified in the context of the coverage enhancement work item. While the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), we are not sure whether it is needed for dynamic PUCCH repetition factor indication, as these could be duplicate functionality. Dynamic PUCCH repetition in our understanding is implemented as configuring a repetition factor for a PUCCH resource, while 4-23 is configured per PUCCH format. Configuring per PUCCH resource can have the same behavior without additional DCI overhead, as configuring per PUCCH format. Therefore, we would like to discuss further whether 4-23 is really needed as a prerequisite, and suggest the square brackets around 4-23 be kept for now. Regarding the 30-5 feature description for dynamic PUCCH repetition indication, we suggest it be labeled as slot based to differentiate from the sub-slot based repetition specified for URLLC. Also, since dynamic PUCCH repetition is implemented by configuring a repetition factor per PUCCH resource, it can be supported together e.g. with repetition of PUCCH formats 0 and 2, so 30-5 is a general mechanism that supports PUCCH formats 0, 1, 2, 3, and 4. Agreement: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH - Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary - FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK - Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17 - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed Agreement: Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. - FFS: Support for slot-based PUCCH repetition Agreement Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation. - The support is subject to independent UE capability indication Conclusion The dynamic repetition indication solution for slot-based PUCCH repetition from the RAN1#105-e working assumption from Cov. Enh. WI can be directly applied for dynamic repetition indication for sub-slot based PUCCH repetition. Agreement For sub-slot based PUCCH repetition for HARQ-ACK, semi-static configured PUCCH repetition (i.e. using nrofSlots) and dynamic repetition factor based operation is supported. - Sub-slot based PUCCH repetition based on semi-static configuration (i.e. using nrofSlots) and based on dynamic indication is subject to separate UE capabilities Table 4: Capabilities for PUCCH Repetition Enhancement Index; Feature group; Components; Prerequisite feature groups; Comments. 30-5; Slot based dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication Support for configuring a repetition factor per PUCCH resource for slot based PUCCH formats 0, 1, 2, 3, and 4; [4-23]. UE features for PUCCH repetition enhancement are defined according to Table 4 |
| [9] | Qualcomm | General remark applicable to all features for NR coverage enhancement Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. 30. NR_cov_enh; 30-5; Dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication; [4-23]; Yes; N/A; UE does not support Dynamic PUCCH repetition indication; [Per UE] Per band; FFS N/A; No N/A; N/A; [Optional with capability signalling]. |

Discussion

[FL1] High priority question 5-1:

- Companies are encouraged to provide views on whether FG 30-5 can be kept as “Slot based Dynamic PUCCH repetition indication”

  - Note: Subslot based dynamic PUCCH repetition indication is currently captured as FG 25-3a “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication” in [1]

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We support to keep “Slot based Dynamic PUCCH repetition indication”, since we focused on slot based repetition in the CovEnh WI. |
| Intel | For CovEnh, support the current version to keep “Slot based Dynamic PUCCH repetition indication” |
| Nokia, NSB | OK to keep it. |
| ZTE | Fine to keep it. |
| vivo | Support to have ‘slot based’ in FG30-5. |
| Sharp | Agree to keep it. |
| Samsung | Fine with the proposed clarification. |
| Lenovo, Motorola Mobility | Fine to keep it |
| Huawei, Hisilicon | OK to keep it. |
| FL2 | According to the comments provided so far, all companies are fine to keep FG 30-5. Therefore, following proposal is made to confirm FG 30-5 is kept as “Dynamic PUCCH repetition indication” [FL2] High priority proposal 5-1: - FG 30-5 is kept as “Dynamic PUCCH repetition indication” as follows 30. NR_cov_enh; 30-5; Dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication; [4-23]; Yes; N/A; UE does not support Dynamic PUCCH repetition indication; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Nokia, NSB | We support the FL proposal. |
| Apple | Although marked as yellow/FFS, in our view the feature should be “per-band” (if a finer granularity is not needed), not Per-UE |
| NTT DOCOMO | Based on the companies view, we assume that proposal 5-1 might be as follows. •FG 30-5 is kept as “Slot based Dynamic PUCCH repetition indication” as follows |
| Ericsson | Support the FL proposal |
| FL3 | Please note that any contents highlighted in yellow mean FFS and to be discussed further. Here the proposal is discussing whether FG 30-5 is necessary or not. Since no concerns/objections have been received so far (more than 24 hours from FL2) regarding keeping FG 30-5, the same proposal (with editorial revision) is set for email endorsement at 1st check point (Oct 14th). @DOCOMO: Thank you for pointing out my copy & paste error. High priority proposal 5-1: - FG 30-5 is kept as “Slot based dynamic PUCCH repetition indication” as follows 30. NR_cov_enh; 30-5; Slot based dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication; [4-23]; Yes; N/A; UE does not support Dynamic PUCCH repetition indication; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. |
| MediaTek | We should clarify the format for PUCCH repetition to be supported in this FG, e.g., PUCCH format 3. Because there is no intention in CovEnh to support PUCCH format 0 repetitions which is the separated UE feature under discussion in RedCap. |
| FL4 | Following was agreed at the 1st check point (October 14) Agreement - FG 30-5 is kept as “Slot based dynamic PUCCH repetition indication” as follows 30. NR_cov_enh; 30-5; Slot based dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication; [4-23]; Yes; N/A; UE does not support Dynamic PUCCH repetition indication; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. Let’s further discuss the contents highlighted in yellow in the next step. |

[FL4] Medium priority question 5-2:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FG 30-5 can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| Nokia, NSB | OK to confirm |
| Sharp | Fine |
| Samsung | All Rel-17 FGs should be “Optional with capability signaling”. We may need to decide which FG is the basic FG for Rel-17 Coverage Enhancement. (same comment above) |
| Huawei, Hisilicon | Fine. |
| Ericsson | Fine. |
| Lenovo, Motorola Mobility | Fine to confirm |
| NTT DOCOMO | We are fine to confirm. |
| FL5 | All companies are fine to support FG 30-5 as optional with capability signaling. Therefore, following proposal is made Medium priority proposal 5-2: - FG 30-5 is supported as optional with capability signaling |
| FL | Following was agreed at the final check point (October 19) Agreement FG 30-5 is supported as optional with capability signaling |

[FL4] Medium priority question 5-3:

- Companies are encouraged to provide views on whether the type of FG 30-5 should be per UE or per band

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE |
| Huawei, Hisilicon | Per UE. |
| Ericsson | Per UE |
| Lenovo, Motorola Mobility | Per UE |
| NTT DOCOMO | Per UE |
| QC | Per band. |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study the appropriated granularity toward the next RAN1 meeting considering the comments provided so far. |

[FL4] Medium priority question 5-4:

- Companies are encouraged to provide views on FDD/TDD differentiation for FG30-5

| Company | Comment |
| --- | --- |
| NTT DOCOMO | TDD/FDD differentiation is not necessary. |
| Nokia, NSB | In principle we agree with DOCOMO here that TDD/FDD differentiation is not necessary. However, one could leave this issue FFS until agreement on DM-RS bundling activation granularity is achieved in AI8.8.2. |
| Ericsson | No differentiation is needed |
| FL5 | It seems more input is necessary for this issue. No additional proposal is made so far. Companies are encouraged to study whether FDD/TDD differentiation is necessary or not toward the next RAN1 meeting considering the comment provided so far. |

[FL4] Low priority question 5-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 30-5

| Company | Comment |
| --- | --- |
| Ericsson | While the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), we are not sure whether it is needed for dynamic PUCCH repetition factor indication, as these could be duplicate functionality. Dynamic PUCCH repetition in our understanding is implemented as configuring a repetition factor for a PUCCH resource, while 4-23 is configured per PUCCH format. Configuring per PUCCH resource can have the same behavior without additional DCI overhead, as configuring per PUCCH format. Therefore, we would like to discuss further whether 4-23 is really needed as a prerequisite, and suggest the square brackets around 4-23 be kept for now. |
| FL5 | It seems more input is necessary for this issue. No additional proposal is made so far. Companies are encouraged to study appropriate prerequisite feature groups toward the next RAN1 meeting considering the comment provided so far. |

[FL4] Low priority question 5-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 30-5 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| FL5 | Closed |

- 30-6: Msg3 repetition

In [1], FG 30-6 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | ZTE | As whether to introduce FG 30-6, it has been discussed once in AI 8.8.3. Below our view on the necessity of this FG is provided. - If a UE requests Msg3 repetition, it implicitly means the UE reports its capability. However, gNB would not know how many of UEs in the cell is capable of Msg3 repetition. Because, only those Msg3 capable UEs in poor coverage will make a request. Allowing UE to report its capability of Msg3 repetition after initial access could let gNB know this information (i.e., how many of UEs in the cell is capable of Msg3 repetition), then gNB can know the maximum number of separate PRACH resources needed. In addition, NW generally has the knowledge of channel variation of a UE (e.g., moving) by various means (e.g., mobility management). Therefore, knowing the number of UEs needed Msg3 repetition (by request) and the total number of UEs supporting Msg3 repetition (by capability reporting after initial access) would certainly help NW find a more appropriate PRACH configuration based on it’s monitoring. In this sense, reporting the capability after initial access is beneficial in general including CBRA case. - For CFRA case, it allows gNB can configure less separate PRACH resources for CE UEs. Because, in CFRA case, even if UE uses legacy PRACH resource for transmission, gNB can still schedule Msg3 with or without repetition based on gNB's measurement, since gNB would know whether the UE has the capability or not, thanks to the capability reporting after initial access. - Theoretically, if a UE requests Msg3 repetition and the RACH procedure is successfully completed, NW can know UE’s capability of supporting Msg3 repetition. While, the RACH procedure with Msg3 repetition may also fail, and if UE accesses to the NW without requesting Msg3 repetition later, capability reporting after initial access is also needed in such case. Regarding the reporting type, we think per UE reporting is sufficient. In addition, it may or may not need to differentiate FDD or TDD based on further discussion of HD FDD UEs regarding how to handle SSB and Msg3 PUSCH transmission in RedCap WI. At this moment, we prefer to leave this open. Proposal 7: Adopt the following revisions for FG 30-6 for Msg3 PUSCH repetition. Features; Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation. 30. NR_cov_enh; 30-6; Msg3 repetition; Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode.; [Per UE]; No FFS. |
| --- | --- | --- |
| [3] | vivo | For msg3 repetition related features 30-6, whether the capability reporting is needed depends on RAN2 further discussion, according to RAN2 agreements as follows. Agreements - Msg3 repetition is applicable to all cases that trigger 4-step CBRA procedure (can come back if we identify that some specific case should not be covered) - A separate RSRP threshold is introduced for requesting Msg3 repetition - Extension of ra-ResponseWindow and ra-ContentionResolutionTimer are not needed for Msg3 repetition. - RAN2 confirms enhancing MAC RAR for indicating MSG3 repetition is not supported. - Postpone the discussion on UE capability (i.e. whether explicit UE capability is needed for indicating the support of Msg3 repetition).. Hence, whether Features 30-6 is ‘Optional with capability signaling’ or ‘Optional without capability signaling’ can be up to RAN2 discussion. Proposal 3: Whether Features 30-6 is ‘Optional with capability signaling’ or ‘Optional without capability signaling’ can be up to RAN2 discussion. |
| [4] | Huawei, HiSilicon | As enhanced features in Rel-17, considering UE capabilities and flexibility, all the UE features for coverage enhancement listed in [1] should be optional with capability signaling. Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling. |
| [6] | Intel | For Msg3 PUSCH repetition, it is supported for RRC IDLE/INACTIVE mode UEs. For RRC CONNECTED mode, existing PUSCH with repetitions as defined in Rel-15/16 can be reused. Table 3 illustrates suggested update for UE feature groups for Msg3 PUSCH repetition. Table 3. UE feature group for Msg3 PUSCH repetition Index; Feature group; Components. 30-6; Msg3 repetition; Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC idle/inactive connected mode.. Proposal 4 - For UE features for Msg3 PUSCH repetition, support Msg3 repetition for initial transmission and re-transmission in RRC idle/inactive mode. |
| [7] | Sharp | Msg3 repetitions can be applicable to all cases that trigger 4-step CBRA procedure. If the capability signaling is not supported for msg3 repetition, network will confuse when providing CBRA resource for a connected mode UE. For example, when the network triggers the handover or PSCell addition, the network needs to decide what resource (CBRA with/without msg3 repetition request resource or CFRA) should be provided to the UE. If the capability signaling is not supported, the network decision will be complicated, or the network needs to allocate CBRA with msg3 repetition request resource all the time. The same thing happens for scheduling BFR resource. In other situation, the network may configure two BWPs (e.g., one with CBRA resource without msg3 repetition request resource, and another with CBRA with/without msg3 repetition request resource). If the network wants to distribute UEs in the two BWPs, the capability signaling is necessary. Therefore, we should confirm that the capability signaling for msg3 repetition is supported. Proposal 5: Confirm that a capability signaling for msg3 repetition (FG 30-6) is supported. |
| [8] | Ericsson | It was agreed in RAN1#105e that a UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. So only if UE is capable of Msg3 repetition and its RSRP is below the threshold, will it request Msg3 repetition with specific preamble. The gNB can’t distinguish between a UE that doesn’t request Msg3 repetition because it is incapable of Msg3 repetition and a UE that has higher RSRP. Therefore, UEs should report their capability for Msg3 repetition after random access. Msg3 repetition can be used in RRC Idle or RRC connected mode, so we suggest to remove the words “in RRC connected mode”. Furthermore, since in RAN1 specification, PUSCH scheduled by RAR is used for Msg3 in CBRA and a PUSCH scheduled by RAR in CFRA. Thus, we propose to use term “PUSCH scheduled by RAR” instead for Msg3 initial transmission and the PUSCH transmission scheduled by RAR in CFRA. Table 5: Capabilities for Type A PUSCH repetition for Msg3 Index; Feature group; Components; Prerequisite feature groups; Comments. 30-6; Msg3 repetition of PUSCH scheduled by RAR or Msg3 retransmission scheduled by DCI.; Support of Msg3 repetition of PUSCH scheduled by RAR for Msg3 initial transmission and Msg3 re-transmission scheduled by DCI in RRC connected mode.. UE features for Type A PUSCH repetition for Msg3 are defined according to Table 5 |
| [9] | Qualcomm | General remark applicable to all features for NR coverage enhancement Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. 30. NR_cov_enh; 30-6; Msg3 repetition [This feature group seems unnecessary since UE capability of msg3 repetition is implicitly indicated by UE RACH transmission. Indication of msg3 repetition is not necessary after initial access.]; Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode.; Yes No, gNB doesn’t not need to know this via UE capability reporting.; N/A; UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode.; [Per UE] Per band; No N/A; No N/A; N/A; [Optional with capability signalling] Optional without capability signalling. |

Discussion

[FL1] High priority question 6-1:

- Companies are encouraged to provide views on whether FG 30-6 is necessary or not

| Company | Comment |
| --- | --- |
| NTT DOCOMO | The related discussion is pending and will be treated in RAN2. We should wait for the conclusions by RAN2. |
| Intel | It seems not necessary for this. If Msg3 repetition is supported for initial access, it should be supported for RRC connected mode. |
| Nokia, NSB | FG 30-6 is necessary for the reasons described by ZTE and Ericsson. |
| ZTE | Support FG 30-6 as a UE supports Msg3 repetition may not request Msg3 repetition. |
| vivo | FG 30-6 is necessary; the only thing need to be discussed is whether it should be optional w/ or w/o capability signaling. |
| Sharp | Yes. This is necessary. |
| Samsung | It is not necessary to have FG 30-6. The msg3 repetition, in addition to a UE capability nature, it is more represented as a status nature, which means it matters when UE needs to do msg3 repetition on top of UE already supports msg3 repetition. Since we already define the RACH resource partition for msg3 repetition. This separate UE feature signaling is not needed. |
| CMCC | Support FG 30-6, as it was concluded that whether UE supported Msg 3 repetition should be also reported. |
| Huawei, Hisilicon | FG 30-6 is necessary |
| FL2 | According to the comments provided so far, companies have different views: - Wait for RAN2 progress: DOCOMO - FG 30-6 is necessary: Nokia, NSB, ZTE, vivo, Sharp, CMCC, Huawei, Hisilicon - FG 30-6 is not necessary: Intel, Samsung Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| Apple | We also think FG 30-6 is not necessary, as UE implicitly indicates the capability to support Msg3 repetition by PRACH |
| Ericsson | Necessary as we commented earlier. |
| FL4 | According to the comments provided so far, companies have different views: - Wait for RAN2 progress: DOCOMO - FG 30-6 is necessary: Nokia, NSB, ZTE, vivo, Sharp, CMCC, Huawei, Hisilicon, Ericsson - FG 30-6 is not necessary: Intel, Samsung, Apple Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| Ericsson | While RAN2 may be discussing this, I think we can discuss from a RAN1 perspective. Please note also that RAN2 has also made agreements that msg3 repetitions may be configured in connected mode, and find our earlier comments on the FG definition below in question 6-5. |
| FL5 | Given that companies still have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study whether FG 30-6 is necessary or not toward the next RAN1 meeting considering the comments provided so far. |

Medium priority question 6-2:

- Companies are encouraged to provide views on whether capability signaling is necessary for FG 30-6, i.e., whether to support as optional with capability signaling or optional without capability signaling

| Company | Comment |
| --- | --- |
| Nokia, NSB | Optional with capability signaling. |
| vivo | Whether FG 30-6 should be optional w/ or w/o capability signaling can be up to RAN2. |
| Sharp | For allocating random access resource dedicated for RRC_CONNECTED UE with CBRA (e.g., for BFR, handover), capability signaling is required. |
| Huawei, Hisilicon | Optional with capability signaling. |
| Ericsson | Optional with capability signaling. |

Medium priority question 6-3:

- Companies are encouraged to provide views on whether the type of FG 30-6 should be per UE or per band

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE |
| Huawei, Hisilicon | Per UE. |
| Ericsson | Per UE. |

Medium priority question 6-4:

- Companies are encouraged to provide views on FDD/TDD differentiation for FG 30-6

| Company | Comment |
| --- | --- |

Low priority question 6-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 30-6 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Ericsson | Regarding the column of component, as discussed in question 6-1, UEs should report their capability for Msg3 repetition after random access. Msg3 repetition can be used in RRC Idle or RRC connected mode, so we suggest to remove the words “in RRC connected mode”. As to the name of the feature group, since in RAN1 specification, PUSCH scheduled by RAR is used for Msg3 in CBRA and a PUSCH scheduled by RAR in CFRA. Thus, we propose to use term “PUSCH scheduled by RAR” instead for Msg3 initial transmission and the PUSCH transmission scheduled by RAR in CFRA. |

- Conclusions

Following agreements were made in this RAN1 meeting:

Agreement

FG 30-5 is kept as “Slot based dynamic PUCCH repetition indication” as follows.

| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 30-5 is supported as optional with capability signaling

References

[1]	R1-2108679	Preliminary RAN1 UE features list for Rel-17 NR	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2108859	Discussion on UE features for NR coverage enhancement	ZTE

[3]	R1-2109019	Discussion on UE features for NR coverage enhancement	vivo

[4]	R1-2109150	Rel-17 UE features for NR coverage enhancement	Huawei, HiSilicon

[5]	R1-2109531	UE features for NR coverage enhancement	Samsung

[6]	R1-2109649	Discussion on UE features for NR coverage enhancement	Intel Corporation

[7]	R1-2110007	UE feature for Rel-17 coverage enhancement	Sharp

[8]	R1-2110128	UE Features for NR Coverage Enhancement	Ericsson

[9]	R1-2110226	UE Features for Coverage Enhancements	Qualcomm Incorporated

[10]	R1-2110271	On UE features for NR coverage enhancement	Nokia, Nokia Shanghai Bell