3GPP TSG RAN WG1 #106bis-e	R1-2109733

e-Meeting, October 11th – 19th, 2021

Title:	Discussion on UE features for NR sidelink Enhancement

Source: 	ZTE, Sanechips

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

This paper discusses UE features for NR sidelink enhancement taking into account the input from [1].

Discussion

Based on the following agreement from RAN1#103-e and 104-e,

Conclusion - SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17. - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e) - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is. Conclusion: - PSFCH reception is not included for Type A UE - S-SSB reception is not included for Type A UE - SL reception Type B is additionally added - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception - Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Three types of UE shall have corresponding UE capability indication:

Type A: UE can not receive PSSCH/PSCCH/PSFCH/S-SSB. Thus UE can only perform random selection only due to lack of sensing results.

Type B: UE can receive PSFCH/SSB only. Thus UE can only perform random selection only due to lack of sensing results.

Type D: This UE capability can be indicated using Rel-16 legacy signalling.

To reflect the above capability,  the original 4-1(32-1) needs to be removed and be replaced by the signalling for Type A UE as shown in the following modification

| Not receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB | 1) UE can not receive NR PSCCH/PSSCH/PSFCH/S-SSB. 2)UE can perfom random resource selection only |
| --- | --- |
| [Receiving NR sidelink of PSFCH/S-SSB only] | - UE can receive NR PSFCH/S-SSB only. - UE can perfom random resource selection only |

Moreover, it's suggested all the FGs listed shall be optional, thus the note for each FG "the FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported." can be removed.

For 4-3(32-3) Transmitting NR sidelink mode 2 with full sensing, it can be indicated already by Rel-16 signalling 15-3, it can be removed.

- In summary, the following proposal is made

Adopt the following modification to the relevant FGs

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. (32)[NR_SL_enh] | 4(32)-1 | Not receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB | 1) UE can not receive NR PSCCH/PSSCH/PSFCH/S-SSB. 2)UE can perform random resource selection only | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1)UE can receive NR PSFCH/S-SSB only. 2)UE can perform random resource selection only | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by LTE Uu. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [4-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by LTE Uu. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [4-1], [4-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)--5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [4-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |

Conclusion

- Adopt the following modification to the relevant FGs

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. (32)[NR_SL_enh] | 4(32)-1 | Not receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB | 1) UE can not receive NR PSCCH/PSSCH/PSFCH/S-SSB. 2)UE can perform random resource selection only | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1)UE can receive NR PSFCH/S-SSB only. 2)UE can perform random resource selection only | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by LTE Uu. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [4-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by LTE Uu. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [4-1], [4-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4(32)--5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [4-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |

Reference

- R1-2108678, Preliminary RAN1 UE features list for Rel-17 LTE, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106-e

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106-e