3GPP TSG RAN WG1 #106-bis-e	R1-2109737

e-Meeting, October 11th – October 19th, 2021

Agenda item:      8.14.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications

Document for:     Discussion

# Introduction

RAN#91 approved a revised SID on XR Evaluations for NR [1]. The XR SI is now ongoing with focus on related system-level performance evaluation to assess how well the currently standardized NR system can support such services. In this document, we present our preliminary results of XR and CG services in 5G NR. Two deployment scenarios are considered for evaluating the performance of XR and CG services, as agreed in [2]: (1) Indoor Hotspot (InH) and (2) Dense Urban (UMa). In these two deployments, we evaluate DL and UL system capacity of Cloud Gaming (CG) and Virtual Reality/Augmented Reality (VR/AR) applications according to the DL traffic models discussed in [3].

# Statistical Models and KPI for XR Traffic

In this section, we discuss the parameters we used in our performance evaluation for the statistical model for XR traffic. The parameters have been set according to the discussion in [2,4,5]. Table 1 illustrates the parameters of the traffic models used for generating DL and UL traffic. Table 2 and Table 3 illustrate the traffic models of XR applications as well as their QoS constraints used to evaluate user satisfaction and system capacity for DL and UL, respectively. Note that the maximum packet size of the transport layer is fixed to the maximum frame size to avoid segmentation at transport layer.

Table 1 – Parameters of DL and UL traffic models for XR applications.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Type of content | Video (DL) | Video (DL) | Video (UL) | Pose (UL) |
| Traffic model | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Single stream |
| Number of streams | 1 | 1 | 1 | 1 |
| Packet size distribution | Truncated Gaussian |  |  | Constant |
| Bitrate | 30 Mbps | 45 Mbps | 10 Mbps | 200 kbps |
| Packet size (avg) | 62500 Bytes | 93750 Bytes | 20833 Bytes | 100 Bytes |
| Packet size (std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) | 2188 Bytes (10.5% of mean value) | 0 |
| Packet size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) | 31250 Bytes (150% of mean value) | 100 Bytes |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) | 10417 Bytes (50% of mean value) | 100 Bytes |
| Transport protocol | UDP |  |  |  |
| Maximum transport packet size | 93750 Bytes 1 packet = 1 PDU | 140625Bytes 1 packet = 1 PDU | 31250 Bytes 1 packet = 1 PDU | 100 Bytes 1 packet = 1 PDU |
| Packet Rate | 60 fps | 60 fps | 60 fps | 250 pps |
| Inter-arrival time distribution | Truncated Gaussian |  |  | Periodic |
| Interarrival time (avg) | 16.67 ms |  |  | 4 ms |
| Interarrival time (std) | 2 ms |  |  | 0 |
| Interarrival time (max) | 20.67 ms |  |  | 4 ms |
| Interarrival time (min) | 12.67 ms |  |  | 4 ms |

Table 2 – Traffic models and QoS constraints used to evaluate XR applications in DL direction.

| Application | CG | VR | AR |
| --- | --- | --- | --- |
| Traffic model | Video single-stream | Video single-stream | Video single-stream |
| Bitrate | 30 Mbps | {30, 45} Mbps | {30, 45} Mbps |
| Packet rate | 60 fps | 60 fps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 10 ms |
| Number of streams | 1 | 1 | 1 |

Table 3 – Traffic models and QoS constraints used to evaluate XR applications in UL direction.

| Application | CG | VR | AR | AR |
| --- | --- | --- | --- | --- |
| Traffic model | Pose single-stream | Pose single-stream | Video single-stream | Video+Pose dual-stream |
| Bitrate | 200 kbps | 200 kbps | 10 Mbps | 10Mbps + 200kbps |
| Packet rate | 250 pps | 250 pps | 60 fps | 60 fps + 250 pps |
| Packet Delay Budget (PDB) | 10 ms | 10 ms | 30 ms | 30 ms + 10ms |
| Number of streams | 1 | 1 | 1 | 1 |

As detailed in [2,4,5], the system capacity is measured in terms on “number of satisfied UEs”. More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs. In our evaluation, we adopt X=99% as the percentage of frames to be successfully received within the PDB to identify satisfied UEs, while Y=90% as the percentage of satisfied UEs to measure the system capacity.

# Capacity Evaluation

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in Indoor Hotspot and Dense Urban deployments. Results are illustrated in Section 3.1 and 3.2, respectively. System configurations used for the two deployment scenario are described in Appendix. In our evaluation, we independently evaluate the DL and UL capacity.  For both deployment scenarios, we first illustrate the DL system capacity for “Video Single-Stream” traffic model with parameters for CG and VR/AR applications. Then, we illustrate the UL system capacity for “Pose Single Stream” traffic model for CG/VR and AR “Video Single-Stream”. We selected the even load random placement of UEs in the simulation area to be sure that all cells get the same number of connected UEs. Specifically, UEs are randomly dropped in the simulated area one by one according to a spatial uniform distribution. This process is repeated for all UEs until each cell served exactly the same number of UEs. Note that cell selection for a UE is based on RSRP.

## Indoor Hotspot (InH) scenario

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in InH scenario.

### DL System Capacity: Cloud Gaming (Video Single-Stream)

Figure 1 shows the ratio of satisfied UEs for CG computed with 99% of frames received within the PDB of 15ms as a function of the average cell load, which is measured in number of UEs per cell. Results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and a for data rate of 30Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we conclude that in the Indoor Hotspot deployment we can support up to 5 UEs per cell (i.e., 60 UEs in the network) in FR1, with the capacity showing a significant increase in FR2 with up to 11 satisfied UEs per cell (i.e., 132 UEs in the network). The higher capacity for FR2 is mainly due to the use of the gNB GoB and the multi panel UE.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |

Figure 1 – DL capacity evaluation of CG traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 4.

Table 4 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | 30Mbps |
| --- | --- |
| FR1 | 5 |
| FR2 | 11 |

### DL System Capacity: Virtual/Augmented Reality (Video Single-Stream)

In this section, we illustrate the results of the DL system capacity obtained for VR/AR applications with PDB=10ms. Figure 2 shows the ratio of satisfied UEs computed with 99% of frames received within the PDB as a function of the average cell load, which is measured in number of UEs; results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and data rates of 30Mbps and 45Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Indoor Hotspot deployment in FR1 we can support 5 UEs per cell running VR/AR applications at 30Mbps (i.e., 60 UEs in the network), with such capacity increasing to 10 UEs per cell (i.e., 120 UEs in the network) for the same data rate when using FR2. For a 45Mbps data rate, in FR1 we can support 3 UEs per cell (i.e., 36 UEs in the network) whereas in FR2 up to 5 UEs per cell (i.e., 60 UEs in the network) can be supported.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) VR/AR in FR1 at 45Mbps with X=99%. | (d) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 2 –  DL capacity evaluation of VR/AR traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 2: When we consider VR/AR traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Table 5 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 5 | 3 |
| FR2 | 10 | 6 |

### UL System Capacity: Cloud Gaming and Virtual Reality (Pose Single-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Pose Single-Stream” traffic model for CG and VR applications at 200kbps, with PDB=10ms. Figure 3 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB for an FR1 configurations as a function of the average cell load, which is measured in number of satisfied UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 54 UEs per cell running XR services and generating pose information can be supported in UL in FR1.

Figure 3 – UL capacity evaluation of CG and VR traffic (Pose Single-Stream) in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 3: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 6.

Table 6 – System capacity in UL for CG and VR (Pose Single-Stream) at 200kbps [#UEs/cell]

|  | Rate: 200 Kbps |
| --- | --- |
| FR1 | 54 |

### UL System Capacity: Augmented Reality (Video Single-Stream and Video+Pose Dual-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Video Single-Stream” and “Video+Pose Dual-Stream” traffic models for AR applications. Video stream generates 10Mbps and its packets have a PDB=30ms. Pose information generates a bitrate of at 200kbps and its packets have PDB=10ms. Figure 3 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB=10ms and 99% of video frame received withing PDB=30ms as a function of the average cell load, which is measured in number of satisfied UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 4 UEs per cell running AR services can be supported in UL in FR1 for both traffic models.

| (a) AR Single-Stream in FR1 at 10Mbps with X=99%. | (b) AR Dual-Stream in FR1 at 10Mbps and 200kbps with X=99%. |
| --- | --- |

Figure 4 – UL capacity evaluation of AR traffic models (Video Single-Stream and Video+Pose Dual-Stream) in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 4: When we consider baseline AR traffic models for UL (Pose Single-Stream and Video+Pose Dual-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 7.

Table 7 – System capacity in UL for AR traffic models (PDB=30ms for video and PDB=10ms for pose) at 10Mbps (Video stream) and 200kbps (Pose stream) [#UEs/cell] in an InH deployment

|  | Single-Stream at 10Mbps | Dual-Stream at 10Mbps and 200kbps |
| --- | --- | --- |
| FR1 | 4 | 4 |

## Dense Urban (DU) scenario

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in DU scenario.

### DL System Capacity: Cloud Gaming (Video Single-Stream)

In this section, we illustrate the results on the DL system capacity obtained considering the “Video Single-Stream” traffic model for CG application at 30Mbps, with PDB=15ms. Figure 5 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 8 UEs per cell running CG services can be supported in DL in both FR1 and FR2.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |

Figure 5 – DL Capacity evaluation of CG traffic (Video Single-Stream) for Dense Urban deployment in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 5: When we consider baseline CG traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 8.

Table 8 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps |
| --- | --- |
| FR1 | 8 |
| FR2 | 8 |

### DL System Capacity: Virtual/Augmented Reality (Video Single-Stream)

In this section, we illustrate the results on the DL system capacity obtained considering the “Video Single-Stream” traffic model for VR/AR application, namely with PDB=10ms. As in the previous analysis, UE are placed in the simulation area to obtain evenly loaded cells in terms of connected UEs. Figure 6 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. Given the smaller PDB, we can observe that up to 6 UEs per cell running VR/AR services at 30Mbps can be supported in DL in FR1 and FR2. Furthermore, we can observe that up to 4 and 3 UEs per cell running VR/AR services at 45Mbps can be supported in DL in FR1 and FR2, respectively.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) VR/AR in FR1 at 45Mbps with X=99%. | (d) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 6 – DL capacity evaluation of VR/AR traffic (Video Single-Stream) for Dense Urban deployment in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 6: When we consider baseline VR/AR traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Table 9 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 6 | 4 |
| FR2 | 6 | 3 |

### UL System Capacity: Cloud Gaming and Virtual Reality (Pose Single-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Pose Single-Stream” traffic model for CG and VR applications in FR1 at 200kbps, with PDB=10ms. Figure 7 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe between 45 and 50 UEs per cell running XR services generating Pose information can be supported in UL in FR1.

Figure 7 – UL capacity evaluation of CG and VR traffic (Pose Single-Stream) for Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 7: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 10.

Table 10 – System capacity in UL for CG and VR (Pose Single-Stream) at 200kbps [#UEs/cell]

|  | Rate: 200Kbps |
| --- | --- |
| FR1 | 45-50 |

### UL System Capacity: Augmented Reality (Video Single-Stream)

In this section, we illustrate the results of the UL system capacity obtained considering the “Video Single-Stream” traffic model for AR applications in FR1 at 10Mbps, with PDB=30ms. Figure 8 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe between  4 UEs per cell running XR services generating Pose information can be supported in UL in FR1.

Figure 8 – UL capacity evaluation of AR traffic (Video Single-Stream) for Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 8: When we consider baseline AR traffic model for UL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 11.

Table 11 – System capacity in UL for AR (Video Single-Stream) at 10Mbps [#UEs/cell]

|  | Rate: 10Mbps |
| --- | --- |
| FR1 | 4 |

# Power Saving Evaluation

In this section, we illustrate the results for power saving evaluation obtained with our system level simulator in Indoor Hotspot and Dense Urban deployments. Results are illustrated in Section 4.1 and 4.2, respectively. System configurations used for the two deployment scenarios are described in Appendix. In our evaluation, we independently evaluate the DL and UL capacity. In our evaluation we consider CDRX as power saving scheme and analyzed its performance in term of power saving gain and capacity loss as agreed in [2]. We consider the following five CDRX configurations to evaluate the trade-off between power saving and achievable capacity with respect to the baseline scheme (UE “Always ON”):

- CDRX 1: (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2)

- CDRX 2: (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4)

- CDRX 3: (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8)

- CDRX 4: (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2)

- CDRX 5: (DRX long cycle, inactivity timer value, On duration timer value)=(10, 5, 5)

We observe that the first three CDRX configurations are not fully Rel-15/16 compliant, since 4ms, 8ms, and 16ms are not supported as value for long cycle. However, these configurations can be obtained by configuring the short and long cycle parameters as follows:

- CDRX 1:  (long DRX cycle, inactivity timer value, On duration timer value, short DRX cycle, short DRX cycle timer)= (32,2,2,4,16)

- CDRX 2:  (long DRX cycle, inactivity timer value, On duration timer value, short DRX cycle, short DRX cycle timer)= (32,4,4,8,16)

- CDRX 3:  (long DRX cycle, inactivity timer value, On duration timer value, short DRX cycle, short DRX cycle timer)= (32,8,8,16,16)

The DRX Start Offset is selected randomly across all UEs and across all simulation drops in order to evaluate time misalignment/desynchronization between XR traffic and long DRX cycle.

We compare tested PS schemes using two lexicographic ordering schemes:

- Best system-level PS scheme: the PS scheme with the highest number of satisfied UEs per cell with PS scheme enabled. If two or more PS schemes have the same number of satisfied UEs per cell, then order PS schemes in descending order of mean PS gain and pick the first scheme of the ordered set. Mean PS gain is computed considering all UEs (option 1).

- Best UE-level PS scheme: the PS scheme with the highest mean PS gain provided that at least 1 UE per cell is satisfied. If two or more PS schemes have the same mean PS gain, then order PS schemes in descending order of number of satisfied UEs per cell and pick the first scheme of the ordered set. Mean PS gain is computed considering only satisfied UEs (option 2).

## Indoor Hotspot (InH) scenario

In the following, we illustrate the measured power saving and capacity in Indoor Hotspot deployment when CDRX is enabled. The proposed configurations are compared to the results with DRX disabled shown in Section 3 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### DL Power Evaluation in FR1

#### Cloud Gaming

In this section, we present the power consumption evaluation obtained considering the CG application at 30Mbps in FR1, with PDB=15ms. Table 12 and Table 13 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 12  shows the PS gain considering all UEs (option 1), whereas Table 13 shows the PS gain considering only satisfied UEs (option 2).

Table 12 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 20.9 | 22 | 21.7 | 16.6 | 4.84 / 5 | 5 | 96.8 |
| (8, 4, 4) | 18.2 | 20 | 18.9 | 13.9 | 4.84 / 5 | 5 | 96.7 |
| (16, 8, 8) | 16.2 | 18.3 | 16.7 | 11.6 | 4.75 / 5 | 5 | 95 |
| (10, 8, 2) | 9.3 | 10.4 | 9.3 | 7.5 | 4.93 / 5 | 5 | 98.5 |
| (10, 5, 5) | 17.3 | 20.3 | 17.5 | 13 | 4.82 / 5 | 5 | 96.3 |

Table 13 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 21.5 | 22 | 21.7 | 21 | 4.84 / 5 | 5 | 96.8 |
| (8, 4, 4) | 18.9 | 20 | 18.9 | 18.7 | 4.84 / 5 | 5 | 96.7 |
| (16, 8, 8) | 17.1 | 18.4 | 16.8 | 18.4 | 4.75 / 5 | 5 | 95 |
| (10, 8, 2) | 9.5 | 10.4 | 9.3 | 11 | 4.93 / 5 | 5 | 98.5 |
| (10, 5, 5) | 17.9 | 20.3 | 17.6 | 16.5 | 4.82 / 5 | 5 | 96.3 |

Observation 9: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 12 and Table 13 considering all UEs and satisfied UEs, respectively.

Observation 10: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 11: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for CG applications in DL in Indoor Hotspot deployment in FR1.

#### Virtual/Augmented Reality

##### Virtual/Augmented Reality at 30Mbps

In this section, we illustrate the results for the power consumption evaluation obtained considering AR/VR applications at 30Mbps and 45Mbps in FR1, with PDB=10ms. Table 14 and Table 15 report the PS gain and system capacity loss of the five CDRX configurations for the case of AR/VR application at 30Mbps with respect to Always ON policy. PS gain has been evaluated considering both options. Table 14 shows the PS gain considering only satisfied UEs (option 1), whereas Table 15  shows the PS gain considering all UEs (option 2).

Table 14 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 20.9 | 22 | 21.7 | 16.6 | 4.5 / 5 | 5 | 90 |
| (8, 4, 4) | 18.2 | 20 | 18.9 | 13.9 | 4.15 / 5 | 4 | 83 |
| (16, 8, 8) | 16.2 | 18.3 | 16.7 | 11.6 | 0 / 5 | 0 | 0 |
| (10, 8, 2) | 9.3 | 10.4 | 9.3 | 7.5 | 4.65 / 5 | 5 | 93 |
| (10, 5, 5) | 17.3 | 20.3 | 17.5 | 13 | 3.7 / 5 | 4 | 74 |

Table 15 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 5 | - | - |
| (4, 2, 2) | 22.2 | 22 | 21.7 | 25.6 | 4.5 / 5 | 5 | 90 |
| (8, 4, 4) | 19.8 | 20 | 19 | 24 | 4.15 / 5 | 4 | 83 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 5 | 0 | 0 |
| (10, 8, 2) | 10 | 10 | 9.4 | 13.5 | 4.65 / 5 | 5 | 93 |
| (10, 5, 5) | 19.2 | 20.6 | 18.4 | 22.5 | 3.7 / 5 | 4 | 74 |

##### Virtual/Augmented Reality at 45Mbps

Table 16 and Table 17 report the PS gain and system capacity loss of the five CDRX configurations for the case of AR/VR application at 45Mbps with respect to Always ON policy. PS gain has been evaluated considering both options. Table 16 shows the PS gain considering only satisfied UEs (option 1), whereas Table 17 shows the PS gain considering all UEs (option 2).

Table 16 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 3 | - | - |
| (4, 2, 2) | 15.7 | 16.8 | 16.4 | 11.5 | 2.85 / 3 | 3 | 95 |
| (8, 4, 4) | 12.1 | 14 | 12.8 | 6.9 | 2.54 / 3 | 2 | 84.7 |
| (16, 8, 8) | 9.4 | 12 | 10.2 | 3.3 | 0 / 3 | 0 | 0 |
| (10, 8, 2) | 4 | 5.3 | 4.5 | 0.4 | 2.91 / 3 | 3 | 97 |
| (10, 5, 5) | 10.8 | 14.2 | 11.5 | 4.8 | 1.89 / 3 | 2 | 63 |

Table 17 - Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 3 | - | - |
| (4, 2, 2) | 16.2 | 16.8 | 16.4 | 14.6 | 2.85 / 3 | 3 | 95 |
| (8, 4, 4) | 13 | 14 | 13 | 11.6 | 2.54 / 3 | 2 | 84.7 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 3 | 0 | 0 |
| (10, 8, 2) | 4.3 | 5.3 | 4.5 | 2.7 | 2.91 / 3 | 3 | 97 |
| (10, 5, 5) | 12.4 | 14.5 | 12.3 | 10.7 | 1.89 / 3 | 2 | 63 |

Observation 12: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 14 and Table 15 considering all UEs and satisfied UEs, respectively.

Observation 13: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 16 and Table 17 considering all UEs and satisfied UEs, respectively.

Observation 14: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.4% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 15: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, in DL in Indoor Hotspot deployment in FR1 for AR/VR applications both at 30Mbps and 45Mbps.

Observation 16: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

### UL Power Evaluation in FR1

#### Virtual/Augmented Reality

In this section, we present the power consumption evaluation for UL considering the “Video Single Stream” traffic model for AR applications at 10Mbps in FR1, with PDB=30ms. Table 18 reports the PS gain and system capacity loss considering all UEs (option 1) for five CDRX configurations with respect to Always ON policy.

Table 18 – Evaluation of UE power saving schemes for {Indoor Hotspot, AR “Video Single Stream”, UL Only, 10Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 21.64 | 22.06 | 21.85 | 21.38 | 3.9 / 4 | 4 | 99% |
| (8, 4, 4) | 18.27 | 18.66 | 18.44 | 17.8 | 3.9 / 4 | 4 | 99% |
| (16, 8, 8) | 13.5 | 14.42 | 13.59 | 13 | 3.9 / 4 | 4 | 99% |
| (10, 8, 2) | 8.67 | 8.83 | 8.77 | 8.31 | 3.9 / 4 | 4 | 99% |
| (10, 5, 5) | 16.67 | 17.08 | 16.82 | 16.08 | 3.9 / 4 | 4 | 99% |

Observation 17: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Indoor Hotspot deployment in FR1 are indicated in Table 18  considering all UEs.

Observation 18: CDRX reduces UE mean power consumption between 8.67% and 21.64% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 19: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level  PS scheme, among the modelled ones,  for AR “Video Single-Stream” applications in UL in Indoor Hotspot deployment in FR1, since it achieves the same capacity as Always ON and the highest PS gain.

### DL Power Evaluation in FR2

#### Cloud Gaming

In this section, we present the power consumption evaluation obtained considering the CG application at 30Mbps in FR2, with PDB=15ms. Table 19 and Table 20 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 19 shows the PS gain considering all UEs (option 1), whereas Table 20 shows the PS gain considering only satisfied UEs (option 2).

Table 19 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR2}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 11 | - | - |
| (4, 2, 2) | 18.5 | 19.2 | 18.8 | 16.9 | 10.81 / 11 | 11 | 98.33 |
| (8, 4, 4) | 15.4 | 16.3 | 15.6 | 14 | 10.77 / 11 | 11 | 98 |
| (16, 8, 8) | 11.6 | 13.7 | 11.8 | 9.5 | 8.59 / 11 | 10 | 78.1 |
| (10, 8, 2) | 7.6 | 8.1 | 7.9 | 7.2 | 10.8 / 11 | 11 | 98.2 |
| (10, 5, 5) | 13.7 | 15.1 | 13.9 | 12.2 | 10.66 / 11 | 11 | 96 |

Table 20 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR2}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 11 | - | - |
| (4, 2, 2) | 18.8 | 19.2 | 18.8 | 17.5 | 10.81 / 11 | 11 | 98.33 |
| (8, 4, 4) | 15.6 | 16.3 | 15.6 | 14.6 | 10.77 / 11 | 11 | 98 |
| (16, 8, 8) | 12.2 | 13.8 | 12 | 10.9 | 8.59 / 11 | 10 | 78.1 |
| (10, 8, 2) | 7.8 | 8 | 7.9 | 7.2 | 10.8 / 11 | 11 | 98.2 |
| (10, 5, 5) | 14 | 15.1 | 13.9 | 12.7 | 10.66 / 11 | 11 | 96 |

Observation 20: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR2 are indicated in Table 19 and Table 20 considering all UEs and satisfied UEs, respectively.

Observation 21: CDRX reduces UE mean power consumption between 7.6% and 18.8% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 22: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS, among the modelled ones, scheme for CG applications in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON and the highest PS gain.

#### Virtual/Augmented Reality

##### Virtual/Augmented Reality at 30Mbps

In this section, we present the power consumption evaluation obtained considering the VR/AR application at 30Mbps in FR2, with PDB=10ms.  Table 21and Table 22 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for VR/AR application. PS gain has been evaluated considering both options. Table 21 shows the PS gain considering all UEs (option 1), whereas Table 22 shows the PS gain considering only satisfied UEs (option 2).

Table 21 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR2}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 10 | - | - |
| (4, 2, 2) | 19.58 | 20 | 19.66 | 19.12 | 8.5 / 10 | 9 | 85.58 |
| (8, 4, 4) | 16.41 | 17.27 | 16.45 | 15.78 | 2 / 10 | 8 | 20.66 |
| (16, 8, 8) | 13.16 | 15 | 13.22 | 11.58 | 0 | 0 | 0 |
| (10, 8, 2) | 8.21 | 8.43 | 8.26 | 8.03 | 9.24 / 10 | 10 | 92.41 |
| (10, 5, 5) | 14.92 | 16.29 | 14.96 | 13.76 | 0.7 / 10 | 7 | 7.16 |

Table 22 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR2}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 10 | - | - |
| (4, 2, 2) | 19.68 | 20 | 19.68 | 19.25 | 8.5 / 10 | 9 | 85.58 |
| (8, 4, 4) | 16.92 | 17.42 | 16.9 | 16.41 | 2 / 10 | 8 | 20.66 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| (10, 8, 2) | 8.26 | 8.43 | 8.27 | 7.98 | 9.24 / 10 | 10 | 92.41 |
| (10, 5, 5) | 16.22 | 16.66 | 16.23 | 15.79 | 0.7 / 10 | 7 | 7.16 |

##### Virtual/Augmented Reality at 45Mbps

In this section, we present the power consumption evaluation obtained considering the VR/AR application at 45Mbps in FR2, with PDB=10ms. Table 23 and Table 24 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for VR/AR application. PS gain has been evaluated considering both options. Table 23 shows the PS gain considering all UEs (option 1), whereas Table 24 shows the PS gain considering only satisfied UEs (option 2).

Table 23 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR2}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 18 | 18.3 | 18.1 | 17.6 | 4.5 / 6 | 5 | 75.56 |
| (8, 4, 4) | 15 | 15.7 | 15 | 14.5 | 0.56 / 6 | 4 | 9.4 |
| (16, 8, 8) | 11.6 | 13.1 | 11.7 | 10.1 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 7.5 | 7.7 | 7.6 | 7.3 | 5.4 / 6 | 6 | 90 |
| (10, 5, 5) | 13.5 | 14.6 | 13.5 | 12.6 | 0.19 / 6 | 1 | 3.33 |

Table 24 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR2}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 18.1 | 18.3 | 18.1 | 17.9 | 4.5 / 6 | 5 | 75.56 |
| (8, 4, 4) | 15.5 | 15.8 | 15.3 | 15.3 | 0.56 / 6 | 4 | 94.44 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 7.6 | 7.7 | 7.6 | 7.4 | 5.4 / 6 | 6 | 90 |
| (10, 5, 5) | 13.4 | 14.6 | 13.5 | 12.6 | 0.19 / 6 | 1 | 3.33 |

Observation 23: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 21 and Table 22 considering all UEs and satisfied UEs, respectively.

Observation 24: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 23 and Table 24 considering all UEs and satisfied UEs, respectively.

Observation 25: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 19.58% for 30Mbps and between 0% and 18.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 26: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS, among the modelled ones, scheme for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON.

Observation 27: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the highest mean PS gain with respect to Always ON.

## Dense Urban (DU) scenario

In the following, we illustrate the measured power saving and capacity for DL in Dense Urban deployment when CDRX is enabled. The proposed configurations are compared to the results with DRX disabled shown in Section 3 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### DL Power Evaluation in FR1

#### Cloud Gaming

In this section, we present the power consumption evaluation obtained considering the CG application at 30Mbps in FR1, with PDB=15ms.  Table 25 and Table 26 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 25 shows the PS gain considering all UEs (option 1), whereas Table 26 shows the PS gain considering only satisfied UEs (option 2).

Table 25 – Evaluation of UE power saving schemes for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 8 | - | - |
| (4, 2, 2) | 20 | 21.6 | 21 | 15.5 | 7.04 / 8 | 7 | 88 |
| (8, 4, 4) | 16.7 | 18.9 | 17.6 | 11.9 | 6.72 / 8 | 7 | 84 |
| (16, 8, 8) | 13.6 | 16.8 | 14.4 | 8.3 | 5.6 / 8 | 6 | 70 |
| (10, 8, 2) | 8.8 | 9.9 | 9 | 7 | 7.44 / 8 | 8 | 93 |
| (10, 5, 5) | 15.4 | 19.4 | 15.7 | 10 | 6.08 / 8 | 7 | 76 |

Table 26 – Evaluation of UE power saving schemes for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 8 | - | - |
| (4, 2, 2) | 21.6 | 21.6 | 21.2 | 22.4 | 7.04 / 8 | 7 | 88 |
| (8, 4, 4) | 18.6 | 19 | 17.9 | 19.5 | 6.72 / 8 | 7 | 84 |
| (16, 8, 8) | 16.6 | 17 | 15.3 | 20 | 5.6 / 8 | 6 | 67 |
| (10, 8, 2) | 9.7 | 9.9 | 9.1 | 10.5 | 7.44 / 8 | 8 | 93 |
| (10, 5, 5) | 17.8 | 19.6 | 16.8 | 19.5 | 6.08 / 8 | 7 | 76 |

Observation 28: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 25 and Table 26 considering all UEs and satisfied UEs, respectively.

Observation 29: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 30: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 31: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

#### Virtual/Augmented Reality

##### Virtual/Augmented Reality at 30Mbps

In this section, we illustrate the results for the power consumption evaluation obtained considering VR/AR applications at 30Mbps in FR1, with PDB=10ms. Table 27 and Table 28 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 27 shows the PS gain considering all UEs (option 1), whereas Table 28 shows the PS gain considering only satisfied UEs (option 2).

Table 27 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 21 | 21.9 | 21.5 | 17.8 | 4.98 / 6 | 5 | 83 |
| (8, 4, 4) | 18 | 19.7 | 18.5 | 14.3 | 3.66 / 6 | 5 | 61 |
| (16, 8, 8) | 15.8 | 18 | 16.2 | 11.4 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 9.2 | 10 | 9.3 | 8 | 5.58 / 6 | 6 | 93 |
| (10, 5, 5) | 17 | 20.4 | 17.1 | 12.8 | 3.12 / 6 | 4 | 52 |

Table 28 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 2: PS gain computed with Satisfied UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | - | - |
| (4, 2, 2) | 22.1 | 21.9 | 21.7 | 24.5 | 4.98 / 6 | 5 | 83 |
| (8, 4, 4) | 19.6 | 19.9 | 18.9 | 22.8 | 3.66 / 6 | 5 | 61 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 6 | 0 | 0 |
| (10, 8, 2) | 9.9 | 9.9 | 9.4 | 11.8 | 5.58 / 6 | 6 | 93 |
| (10, 5, 5) | 19.2 | 20.7 | 18.5 | 21.2 | 3.12 / 6 | 4 | 52 |

##### Virtual/Augmented Reality at 45Mbps

In this section, we illustrate the results for the power consumption evaluation obtained considering VR/AR applications at 45Mbps in FR1, with PDB=10ms. Table 29 and Table 30 report the PS gain and system capacity loss of the five CDRX configurations with respect to Always ON policy measured for CG application. PS gain has been evaluated considering both options. Table 29 shows the PS gain considering all UEs (option 1), whereas Table 30 shows the PS gain considering only satisfied UEs (option 2).

Table 29 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 45Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 14.5 | 16.4 | 15.9 | 5.8 | 2.76 / 4 | 3 | 69 |
| (8, 4, 4) | 10.8 | 13.3 | 12 | 1.3 | 1.6 / 4 | 3 | 40 |
| (16, 8, 8) | 7.9 | 11.1 | 9.2 | 0 | 0 / 4 | 0 | 0 |
| (10, 8, 2) | 3.2 | 5 | 4.2 | 0 | 3.52 / 4 | 3 | 88 |
| (10, 5, 5) | 9.5 | 13.2 | 10.5 | 0.3 | 0.96 / 4 | 2 | 24 |

Table 30 – Evaluation of UE power saving schemes for {Dense Urban, AR/VR, DL Only, 45Mbps, FR1}. Option 1: PS gain computed with Satisfied UEs

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 16.1 | 16.4 | 16 | 16.1 | 2.76 / 4 | 3 | 69 |
| (8, 4, 4) | 12.8 | 13.6 | 12.6 | 12.8 | 1.6 / 4 | 3 | 40 |
| (16, 8, 8) | 0 | 0 | 0 | 0 | 0 / 4 | 0 | 0 |
| (10, 8, 2) | 4.3 | 5 | 4.3 | 3.4 | 3.52 / 4 | 3 | 88 |
| (10, 5, 5) | 12.3 | 13.6 | 12.1 | 11.9 | 0.96 / 4 | 2 | 24 |

Observation 32: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Dense Urban deployment in FR1 are indicated in Table 27 and Table 28 considering all UEs and satisfied UEs, respectively.

Observation 33: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Dense Urban deployment in FR1 are indicated in Table 29 and Table 30 considering all UEs and satisfied UEs, respectively.

Observation 34: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 35: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 36: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

Observation 37: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for AR/VR applications at 45Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity and the highest mean PS gain with respect to Always ON.

Observation 38: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

### UL Power Evaluation in FR1

#### Augmented Reality (Video Single-Stream)

In this section, we present the power consumption evaluation for UL considering the “Video Single Stream” traffic model for AR applications at 10Mbps in FR1, with PDB=30ms. Table 31 reports the PS gain and system capacity loss considering all UEs (option 1) for five CDRX configurations with respect to Always ON policy.

Table 31 – Evaluation of UE power saving schemes for {Dense Urban, AR “Video Single Stream”, UL Only, 10Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 4 | - | - |
| (4, 2, 2) | 14.6 | 22 | 18.7 | 3.7 | 3.65 / 4 | 4 | 91.2 |
| (8, 4, 4) | 10.8 | 18.4 | 14.7 | 0.1 | 3.66 / 4 | 4 | 91.4 |
| (16, 8, 8) | 7.5 | 13.8 | 10.7 | 0 | 3.66 / 4 | 4 | 91.4 |
| (10, 8, 2) | 5.8 | 8.6 | 7.5 | 1.5 | 3.65 / 4 | 4 | 91.3 |
| (10, 5, 5) | 9.7 | 16.8 | 13.5 | 0 | 3.66 / 4 | 4 | 91.4 |

Observation 39: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Dense Urban deployment in FR1 are indicated in Table 31 considering all UEs.

Observation 40: CDRX reduces UE mean power consumption between 5.8% and 14.6% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 41: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4) is the best system-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 42: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

# Coverage Evaluation

In this section, we illustrate the results for coverage evaluation obtained with our system level simulator in Indoor Hotspot and Dense Urban deployments. Results are illustrated in Section 5.1 and 5.2, respectively. System configurations used for the two deployment scenarios are described in Appendix. In our evaluation, we evaluate coverage measuring the coupling gain with B UEs/cell (B=1 and B=”system capacity”).

## Indoor Hotspot (InH) scenario

Figure 9 shows the empirical CDF of the coupling gain measured in the InH deployment scenario in FR1, while Table 32 shows the 5%-tile of the distribution.

Figure 9 – Empirical CDF of Coupling Gain [dB] in InH deployment in FR1.

Table 32 – 5%-tile of coupling gain [dB] in InH deployment measured with DL traffic model.

|  | 5%-tile of coupling gain |  |  |  |
| --- | --- | --- | --- | --- |
|  | B=1 CG/AR/VG | B=5 CG 30Mbps | B=5 AR/VR 30Mbps | B=3 AR/VR 45Mbps |
| FR1 | -72 | -71 | -71 | -70 |

Observation 43: The 5%-tile of coupling gain in InH deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 9.

## Dense Urban (DU) scenario

Figure 10 shows the empirical CDF of the coupling gain measured in the InH deployment scenario in FR1, while Table 33 shows the 5%-tile of the distribution.

Figure 10  – Empirical CDF of Coupling Gain [dB] in DU deployment in FR1.

Table 33 – 5%-tile of coupling gain [dB] in DU deployment.

|  | 5%-tile of coupling gain |  |  |  |
| --- | --- | --- | --- | --- |
|  | B=1 CG/AR/VR | B=8 CG 30Mbps | B=8 AR/VR 30Mbps | B=6 AR/VR 45Mbps |
| FR1 | -123 | -126 | -126 | -126 |

Observation 44: The 5%-tile of DL coupling gain in DU deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 10.

# Mobility Evaluation

Mobility and, especially, handover events may crucially affect the performance and reliability of the XR services in certain setups. To estimate the potential impact of handover events on the characteristics of XR services, a simple still credible analytical study is presented in this section. First, the realistic handover durations are identified for existing handover techniques by directly following the current specifications. Then, the average number of consecutive XR frames to be lost during the handover are calculated from the obtained handover time durations. Finally, the lower bound is obtained of the handovers impact on the agreed XR UE satisfactory metrics, when considering a possible (optimistic) UE trajectory through the cells.

## Handover interruption time

This subsection illustrates Step 1 of the mobility evaluation presented in [6].

At every successful handover there is a temporary interruption of the data connectivity, called the interruption time. The requirements for HO interruption times captured in 3GPP TS 38.133 [7], Clause 6.1.1.2.2 (as per earlier agreements in RAN4). Based on [7], we have summarized the HO interruption times in Table 34 below for the different HO methods. The interruption time roughly varies from 40-80 ms depending on the conditions and we expect a reduction of approximately 10 ms compared to baseline handover of Rel. 15 if Conditional Handover (CHO) is used along with early data forwarding, as the time for RRC procedure delay can be saved since the UE can still receive data from the source cell while decoding conditional reconfiguration to be executed. The text in [7] also defines the interruption time for DAPS, which can be down to 2 ms for intra-frequency synchronous case in FR1. But, although the radio interruption on the Uu interface is down to 2 ms in some DAPS scenarios, there is an additional end-to-end delay for UL only since the target cell cannot forward the buffered UL packets to UPF/Serving Gateway before it receives the final SN Status Transfer message. For XR cases, the HO interruption can be a real problem if it causes violation of the PDB for multiple XR frames.

Table 34 – Summary of HO interruption time for different HO methods as per the requirements in 3GPP TS 38.133 [7].

| Component | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS (FR1-FR1) |
| --- | --- | --- | --- | --- | --- |
| 1 | RRC Reconfiguration procedure delay | 10 ms | 0 (UE can still receive from source cell while decoding) | 0 (UE can still receive from source cell while decoding) | Downlink: Up to Tinterrupt1 on source cell and Tinterrupt2 on target cell [3] For intra-frequency DAPS HO, Tinterrrupt1= 1 ms and Tinterrupt2 = 1ms (assuming same BWP) Uplink: CBRA: UL switch (sending new UL PDCP SDU and the non-acknowledged PDCP PDUs to target cell) occurs after MAC CE contention resolution is received. Target will forward the UL packets to UPF after ~ 10 ms (2 Xn messages for Handover Success+ SN Status Transfer). Total interruption can be up to 10 ms + Tinterrupt2 = 11 ms CFRA = UL switch is performed when RAR is received. Additional delay for sending RRC Reconfiguration Complete which is 8 ms in FR1. Total interruption can be up to ~ 19 ms |
| 2 | Target cell search Tsearch | 0 (if target cell is known) | 0 (if target cell is known) | 0 (if target cell is known) |  |
| 3 | UE processing time Tprocessing | 20 ms (upper limit for FR2) | 20 ms (upper limit for FR2) | 20 ms (upper limit for FR2) |  |
| 4 | Fine time tracking and acquiring full timing information of the target cell TΔ | 20, 10 ms on average (default value for SMTC period) | 20, 10 ms on average (default value for SMTC period) | 20, 10 ms on average (default value for SMTC period) |  |
| 5 | Tmargin (time for SSB post-processing) | 2 ms | 2 ms | 2 ms |  |
| 6 | Delay to acquire the first available PRACH in target gNB TIU | up to 20, 10 on average (for smallest value of x =1 defined in tables 6.3.3.2-2 and 6.3.3.2-3 of [4] for FR1 and table 6.3.3.2-4 for FR2) | up to 20, 10 on average (for smallest value of x =1 defined in tables 6.3.3.2-2 and 6.3.3.2-3 of [4] for FR1 and table 6.3.3.2-4 for FR2) | up to 20, 10 on average (for smallest value of x =1 defined in tables 6.3.3.2-2 and 6.3.3.2-3 of [4] for FR1 and table 6.3.3.2-4 for FR2) |  |
| 7 | PRACH preamble transmission | 1 slot (FR1/FR2: 1/0.125 ms) | 1 slot (FR1/FR2: 1/0.125 ms) | 1 slot (FR1/FR2: 1/0.125 ms) |  |
| 8 | UL Allocation + TA for UE | 10 slots (FR1/FR2: 10/1.25 ms) | 10 slots (FR1/FR2: 10/1.25 ms) | 10 slots (FR1/FR2: 10/1.25 ms) |  |
| 9 | UE sends RRC Reconfiguration Complete | 8 slots (FR1/FR2: 8/1 ms) | 8 slots (FR1/FR2: 8/1 ms) | 8 slots (FR1/FR2: 8/1 ms) |  |
| 10 | Additional Interruption time from data forwarding | 0 ms | 10 ms (2 Xn messages for Handover Success+ SN Status Transfer) | 0 ms |  |
|  | Ball-park total delay [ms] (FR1/FR2) | 71/54.4 ms | 71/54.4 ms | 61/44.5 ms |  |

Observation 45: Appropriate durations and processing times incorporated in the HO, CHO, and DAPS, as detailed in TS 38.133, are computed in Table 34. The observed ball-park total delay values are considerably high and may lead to a loss of several consecutive XR frames, depending on the traffic characteristics and PDB.

## Number of lost XR frames due to HO events

This subsection illustrates Step 2a of the mobility evaluation presented in [6].

Once the HO interruption time values, Y, are revealed, it becomes possible to estimate the number of consecutive frames that would be lost, N, due to a single HO. Particularly, we present an estimation of N in Table 35 and Table 36 below, assuming that all XR frames within “OFF time = HO time - PDB” (i.e., 71ms – 10ms = 61ms) are lost while other frames (i.e., arriving close to the end of the HO event) are not lost.

Table 35 – Number of lost XR frames due to HO events for FR1.

|  | Application | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS (FR1-FR1) |
| --- | --- | --- | --- | --- | --- | --- |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 3.37 (OFF time = 56ms) | 3.37 (OFF time = 56ms) | 3.07 (OFF time = 46ms) | 0 (OFF time = 0ms) |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 3.67 (OFF time = 61ms) | 3.67 (OFF time = 61ms) | 2.77 (OFF time = 51ms) | 0 (OFF time = 0ms) |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 3.67 (OFF time = 61ms) | 3.67 (OFF time = 61ms) | 2.77 (OFF time = 51ms) | 0 (OFF time = 0ms) |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 15.25 (OFF time = 61ms) | 15.25 (OFF time = 61ms) | 12.75 (OFF time = 51ms) | 2.25 (OFF time = 9ms) |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 15.25 (OFF time = 61ms) | 15.25 (OFF time = 61ms) | 12.75 (OFF time = 51ms) | 2.25 (OFF time = 9ms) |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 2.47 (OFF time = 41ms) | 2.47 (OFF time = 41ms) | 1.87 (OFF time = 31ms) | 0 (OFF time = 0ms) |

Table 36 – Number of lost XR frames due to HO events for FR2.

|  | Application | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS |
| --- | --- | --- | --- | --- | --- | --- |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 2.37 (OFF time = 39.4ms) | 2.37 (OFF time = 39.4ms) | 1.78 (OFF time = 29.5ms) | Currently not specified for FR2 |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 2.67 (OFF time = 44.4ms) | 2.67 (OFF time = 44.4ms) | 2.08 (OFF time = 34.5ms) |  |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 2.67 (OFF time = 44.4ms) | 2.67 (OFF time = 44.4ms) | 2.08 (OFF time = 34.5ms) |  |
|  | ===== | ====== | ====== | ====== | ====== |  |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 11.1 (OFF time = 44.4ms) | 11.1 (OFF time = 44.4ms) | 8.63 (OFF time = 34.5ms) |  |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 11.1 (OFF time = 44.4ms) | 11.1 (OFF time = 44.4ms) | 8.63 (OFF time = 34.5ms) |  |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 1.47 (OFF time = 24.4ms) | 1.47 (OFF time = 24.4ms) | 0.87 (OFF time = 14.5ms) |  |

Observation 46: The number of consecutive XR frames that will be lost during a single HO event can be estimated from the HO interruption time and PDB, as given in Table 35 for FR1 and Table 36 for FR2.

The values reported in Table 35 and Table 36 are of a particular interest, recalling that they present the number of consecutive frames to be lost. Loosing several frames in a row is more crucial than losing the same number of frames over a long period of time, as the former may cause severe drops in QoS and user experience, including video “freezing”, video “blinking”, among other negative effects.

## Performance of XR services in the presence of mobility

In this section, we proceed further with mobility analysis connecting the obtained number of lost frames and OFF duration with the agreed definition for an XR UE satisfactory criteria.

This subsection illustrates Step 2b of the mobility evaluation presented in [6].

At RAN1#104-bis-e, the following was further agreed for DL and UL:

Agreement: In case of single stream per UE in DL, a UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within a given air interface PDB. - The baseline X value is 99.

Agreement: On UL Traffic model and QoS parameters - … - A UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within the given air interface PDB. - The baseline X value is 99.

Hence, even losing several XR consecutive XR frames due to a handover does not immediately lead to the UE becoming “non satisfied”. Instead, this conclusion will depend on the frequency of the HO events.

To estimate a lower bound on the HO impact on the UE satisfaction, an analysis is presented below. It is important to note that this is an optimistic lower bound, as the following assumption is made:

- Assumption: It is assumed that HO events are the only source of XR frames losses or delays in the system. In other words, UE is declared non satisfied, if more than 1% of frames are lost due to HO interruptions. In reality, as there would be multiple sources of losses, the impact of HO interruption would be higher (loss budget for HO events would be lower than 1%).

Hence, the maximum allowed periodicity of HO events for XR UEs can be computed from the values reported in Table 35 and Table 36, as “min HO period = OFF time/(100% - 99%)” given below in Table 37 and Table 38 for FR1 and FR2, respectively.

Table 37  – Minimum allowed time interval (max allowed periodicity) between HO events for FR1.

|  | Application | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS (FR1-FR1) |
| --- | --- | --- | --- | --- | --- | --- |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 5.6s | 5.6s | 4.6s | 0s |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 6.1s | 6.1s | 5.1s | 0s |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 6.1s | 6.1s | 5.1s | 0s |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 6.1s | 6.1s | 5.1s | 0.9s |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 6.1s | 6.1s | 5.1s | 0.9s |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 4.1s | 4.1s | 3.1s | 0s |

Table 38 – Minimum allowed time interval (max allowed periodicity) between HO events for FR2.

|  | Application | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS |
| --- | --- | --- | --- | --- | --- | --- |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 3.94s | 3.94s | 2.95s | Currently not specified for FR2 |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 4.44s | 4.44s | 3.45s |  |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 4.44s | 4.44s | 3.45s |  |
|  | ===== | ====== | ====== | ====== | ====== |  |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 4.44s | 4.44s | 3.45s |  |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 4.44s | 4.44s | 3.45s |  |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 2.44s | 2.44s | 1.45s |  |

Observation 47: The periodicity (inter-arrival time) of the HO events that leads to the XR UE becoming unsatisfied is given in Table 37 for FR1 and in Table 38 for FR2. If the HO events happen more frequently than the corresponding values given in these tables, an XR UE cannot be satisfied.

Observation 48: The values presented in Table 37 and in Table 38 present important details, as they limit the maximum allowed frequency of the HO events for mobile XR UEs.

## Impact of UE speed on UE satisfactory when running XR services

This subsection illustrates Step 3 of the mobility evaluation presented in [6].

If the speed and trajectory of UE mobility through the cells is known or can be estimated, it becomes possible to estimate the number of HO events as a function of the UE speed. This estimate and values presented in Table 37 and Table 38 enable to calculate the critical speed for XR UEs: if a UE moves at a speed higher than a reported value, XR UE becomes unsatisfied, since XR UE will experience a higher number of HO events than the maximum value.

To be consistent with other RAN1 tracks for Release 17, the following UE trajectory is incorporated as an example, as has been already agreed in 8.1 item on feMIMO [8] [9] and correctly pointed out in [10] for RAN1#106-e. We particularly focus on DU, as FR1 mobility will be more crucial in outdoor scenarios. The same analysis can be repeated for InH, if needed. It is assumed that the HO events appear only at the borders of the cells/sectors. In real deployments there will be more HO events happening due to the presence of fading and other reasons.

At RAN1#102-e, the following was agreed [9]:

Agreement The three proposals on R1-2007151 on the evaluation methodology for multi-beam enhancement are agreed. … Dense Urban: Here X (in meter) is a uniformly distributed random variable U[26,34]. One UE is dropped and starts at P and moves along the 120-deg line downward to Q.

Hence, a moving UE will experience 7 HO events on a path of a length varying between 492m and 501m [11]. So, there will be at least one HO event every Z = 501/7 = 71.5m. Other approaches to estimate Z can be optionally considered (i.e., making it the minimal distance between the HO events instead of the average one).

Observation 49: Following the defined trajectory, in DU a moving UE will experience one HO event at least every 71.5m.

Proceeding further with the mobility impact on UE satisfactory, we obtain the values of “critical UE speed”, V, (if UE moves faster, then the XR UE becomes non satisfied for different XR services) as V = 71.5m / T, where T is the minimum allowed time interval between HO events, calculated in Section 6.3 (Table 37 and Table 38). The approach is illustrated in Figure 11 below.

Figure 11 – Deriving critical speed of an XR UE leading to the UE becoming unsatisfied.

The following tables summarize critical UE speed values for FR1 and FR2, consequently.

Table 39 – Critical UE speed, V, leading to the XR UE becoming non satisfied for FR1.

|  | Application | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS (FR1-FR1) |
| --- | --- | --- | --- | --- | --- | --- |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 45.96 km/h | 45.96 km/h | 55.96 km/h | + Inf |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 42.20 km/h | 42.20 km/h | 50.47 km/h | + Inf |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 42.20 km/h | 42.20 km/h | 50.47 km/h | + Inf |
|  | ===== | ====== | ====== | ====== | ====== | ====== |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 42.20 km/h | 42.20 km/h | 50.47 km/h | 285.55 km/h |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 42.20 km/h | 42.20 km/h | 50.47 km/h | 285.55 km/h |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 62.78 km/h | 62.78 km/h | 83.03 km/h | + Inf |

Table 40 – Critical UE speed, V, leading to the XR UE becoming non satisfied for FR2.

|  | Application | Description | Baseline HO | Conditional Handover (Late data forwarding) | Conditional Handover (Early data forwarding) | DAPS |
| --- | --- | --- | --- | --- | --- | --- |
|  | ===== | ====== | ====== | ====== | ====== | Currently not specified for FR2 |
| DL | CG | Singe-stream video (60fps), 16.6ms periodicity, PDB=15ms | 65.33 km/h | 65.33 km/h | 87.25 km/h |  |
|  | VR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 57.97 km/h | 57.97 km/h | 74.61 km/h |  |
|  | AR | Singe-stream video (60fps), 16.6ms periodicity, PDB=10ms | 57.97 km/h | 57.97 km/h | 74.61 km/h |  |
|  | ===== | ====== | ====== | ====== | ====== |  |
| UL | CG | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 57.97 km/h | 57.97 km/h | 74.61 km/h |  |
|  | VR | Single-stream pose/control information, 4ms periodicity, PDB = 10ms | 57.97 km/h | 57.97 km/h | 74.61 km/h |  |
|  | AR | Single-stream video (60 fps), 16.6ms periodicity, PDB = 30ms | 105.49 km/h | 105.49 km/h | 177.52 km/h |  |

Observation 50: Critical UE speed leading to the XR UEs becoming non satisfied is given in Table 39 for FR1 and in Table 40 for FR2.

The following main observations can be made from the two tables above:

Observation 51: Recalling that the XR traffic is bi-directional (both UL and DL), all FR1 XR UEs (CG, VR, and AR) become unsatisfied at speeds higher than 42.2 km/h for “Baseline HO” and “Conditional HO with late data forwarding”, as well as at speeds higher than 50.47 km/h for “Conditional HO with early data forwarding”.

Observation 52: Recalling that the XR traffic is bi-directional (both UL and DL), all FR2 XR UEs (CG, VR, and AR) become unsatisfied at speeds higher than ~58 km/h for “Baseline HO” and “Conditional HO with late data forwarding”, as well as at speeds higher than 74.61 km/h for “Conditional HO with early data forwarding”.

Observation 53: The DAPS handover method offers the lowest handover interruption times where the number of impacted XR frames from handovers is minimized. However, DAPS is currently not specified for intra-frequency FR2-FR2 cases. Specifying DAPS for intra-frequency FR2-FR2 cases is desirable to minimize violation of XR frames PDB for FR2 deployments due to handovers.

It is also worth to note that the obtained speed values present an optimistic picture following the assumptions made in Section 6.3. In real deployments, where HO events are not the only source of XR frames losses/delays, the critical speed could be even lower. Therefore, mobility is an important KPI to be considered for further design and evaluation of support for XR services in NR.

# Summary

In this contribution we have discussed performance results in indoor and outdoor deployments and bottlenecks that could be improved to increase 5G NR system capacity and UE power saving. Those naturally require further studies and discussions. In summary, following observations have been made for system capacity evaluation:

The following observations have been made for system capacity evaluation in Indoor Hotspot:

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 4.

Observation 2: When we consider VR/AR traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Observation 3: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 6.

Observation 4: When we consider baseline AR traffic models for UL (Pose Single-Stream and Video+Pose Dual-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 7.

The following observations have been made for system capacity evaluation in Dense Urban:

Observation 5: When we consider baseline CG traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 8.

Observation 6: When we consider baseline VR/AR traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Observation 7: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 10.

Observation 8: When we consider baseline AR traffic model for UL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 11.

The following observations have been made for UE power saving evaluation in Indoor Hotspot:

Observation 9: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 12 and Table 13 considering all UEs and satisfied UEs, respectively.

Observation 10: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 11: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for CG applications in DL in Indoor Hotspot deployment in FR1.

Observation 12: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 14 and Table 15 considering all UEs and satisfied UEs, respectively.

Observation 13: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR1 are indicated in Table 16 and Table 17 considering all UEs and satisfied UEs, respectively.

Observation 14: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.4% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 15: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, in DL in Indoor Hotspot deployment in FR1 for AR/VR applications both at 30Mbps and 45Mbps.

Observation 16: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

Observation 17: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Indoor Hotspot deployment in FR1 are indicated in Table 18  considering all UEs.

Observation 18: CDRX reduces UE mean power consumption between 8.67% and 21.64% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 19: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Indoor Hotspot deployment in FR1, since it achieves the same capacity as Always ON and the highest PS gain.

Observation 20: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR2 are indicated in Table 19  and Table 20  considering all UEs and satisfied UEs, respectively.

Observation 21: CDRX reduces UE mean power consumption between 7.6% and 18.8% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 22: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for CG applications in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON and the highest PS gain.

Observation 23: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 21 and Table 22 considering all UEs and satisfied UEs, respectively.

Observation 24: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Indoor Hotspot deployment in FR2 are indicated in Table 23 and Table 24 considering all UEs and satisfied UEs, respectively.

Observation 25: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 19.58% for 30Mbps and between 0% and 18.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 26: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the same capacity as Always ON.

Observation 27: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps and 45Mbps in DL in Indoor Hotspot deployment in FR2, since it achieves the highest mean PS gain with respect to Always ON.

The following observations have been made for UE power saving evaluation in Dense Urban:

Observation 28: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 25 and Table 26 considering all UEs and satisfied UEs, respectively.

Observation 29: CDRX reduces UE mean power consumption between 9% and 22% across all UEs, but, at the same time, it also affects the system capacity. A non-optimal configuration of CDX parameters can cause a significant drop in the number of satisfied users per cell.

Observation 30: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 31: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for CG applications in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

Observation 32: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 30Mbps in Dense Urban deployment in FR1 are indicated in Table 27 and Table 28 considering all UEs and satisfied UEs, respectively.

Observation 33: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications at 45Mbps in Dense Urban deployment in FR1 are indicated in Table 29 and Table 30 considering all UEs and satisfied UEs, respectively.

Observation 34: CDRX reduces UE mean power consumption  for VR/AR applications between 0% and 22% for 30Mbps and between 0% and 16.1% for 45Mbps, but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 35: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(10, 8, 2) is the best system-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 36: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR/VR applications at 30Mbps in DL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

Observation 37: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best system-level and UE-level PS scheme, among the modelled ones, for AR/VR applications at 45Mbps in DL in Dense Urban deployment in FR1, since it achieves the same capacity and the highest mean PS gain with respect to Always ON.

Observation 38: CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) archives zero capacity due to the misalignment between the start of the DRX cycle and the AR/VR traffic for both 30Mbps and 45Mbps. Therefore, an improper configuration of the DRX start offset of the CDRX configuration negatively affects system capacity for AR/VR services. A shorter duration of the long DRX cycle helps to reduce the effect of the improper configuration of the DRX start offset.

Observation 39: Power saving gain and UL capacity loss with different CDRX configurations for AR “Video Single-Stream” applications in Dense Urban deployment in FR1 are indicated in Table 31 considering all UEs.

Observation 40: CDRX reduces UE mean power consumption between 5.8% and 14.6% across all UEs, and although it seems not to affect the UL system capacity for AR “Video Single-Stream” applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 41: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4) is the best system-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the same capacity as Always ON.

Observation 42: The CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2) is the best UE-level PS scheme, among the modelled ones, for AR “Video Single-Stream” applications in UL in Dense Urban deployment in FR1, since it achieves the highest mean PS gain with respect to Always ON.

The following observations have been made for the coverage evaluation:

Observation 43: The 5%-tile of coupling gain in InH deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 9.

Observation 44: The 5%-tile of DL coupling gain in DU deployment scenario computed at B UEs/cell (B=1 and B=“system capacity”) is indicated in Figure 10.

The following observations have been made for the mobility evaluation:

Observation 45: Appropriate durations and processing times incorporated in the HO, CHO, and DAPS, as detailed in TS 38.133, are computed in Table 34. The observed ball-park total delay values are considerably high and may lead to a loss of several consecutive XR frames, depending on the traffic characteristics and PDB.

Observation 46: The number of consecutive XR frames that will be lost during a single HO event can be estimated from the HO interruption time and PDB, as given in Table 35 for FR1 and Table 36 for FR2.

Observation 47: The periodicity (inter-arrival time) of the HO events that leads to the XR UE becoming unsatisfied is given in Table 37 for FR1 and in Table 38 for FR2. If the HO events happen more frequently than the corresponding values given in these tables, an XR UE cannot be satisfied.

Observation 48: The values presented in Table 37 and in Table 38 present important details, as they limit the maximum allowed frequency of the HO events for mobile XR UEs.

Observation 49: Following the defined trajectory, in DU a moving UE will experience one HO event at least every 71.5m.

Observation 50: Critical UE speed leading to the XR UEs becoming non satisfied is given in Table 39 for FR1 and in Table 40 for FR2.

Observation 51: Recalling that the XR traffic is bi-directional (both UL and DL), all FR1 XR UEs (CG, VR, and AR) become unsatisfied at speeds higher than 42.2 km/h for “Baseline HO” and “Conditional HO with late data forwarding”, as well as at speeds higher than 50.47 km/h for “Conditional HO with early data forwarding”.

Observation 52: Recalling that the XR traffic is bi-directional (both UL and DL), all FR2 XR UEs (CG, VR, and AR) become unsatisfied at speeds higher than ~58 km/h for “Baseline HO” and “Conditional HO with late data forwarding”, as well as at speeds higher than 74.61 km/h for “Conditional HO with early data forwarding”.

Observation 53: The DAPS handover method offers the lowest handover interruption times where the number of impacted XR frames from handovers is minimized. However, DAPS is currently not specified for intra-frequency FR2-FR2 cases. Specifying DAPS for intra-frequency FR2-FR2 cases is desirable to minimize violation of XR frames PDB for FR2 deployments due to handovers.

All previous observations on system capacity and DL UE power saving gains (option 1) are captured and summarized by the following tables:

|  |  | CG (DL) | VR/AR (DL) |  | CG / VR (UL – Pose) | AR (UL – Video) | AR (UL – Video+Pose) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Deployment | Frequency Range | Rate: 30Mbps | Rate: 30Mbps | Rate: 45Mbps | Rate: 200Kbps | Rate: 10Mbps | Rate: 10Mbps+200Kbps |
| Indoor Hotspot (DL) | FR1 | 5 | 5 | 3 | 54 | 4 | 4 |
|  | FR2 | 11 | 10 | 6 | - | - | - |
| Dense Urban (DL) | FR1 | 8 | 6 | 4 | 45-50 | 4 | - |
|  | FR2 | 8 | 6 | 3 | - | - | - |

|  |  | Cloud Gaming (30Mbps) |  | Virtual/Augmented Reality (30Mbps) |  | Virtual/Augmented Reality (45Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Deployment | PS Scheme | Mean PS gain | Capacity | Mean PS gain | Capacity | Mean PS gain | Capacity |
| Indoor Hotspot (FR1) | Always ON | - | - / 5 | - | - / 5 | - | - / 3 |
|  | CDRX (4, 2, 2) | 20.9 | 4.84 / 5 | 20.9 | 4.5 / 5 | 15.7 | 2.85 / 3 |
|  | CDRX (8, 4, 4) | 18.2 | 4.84 / 5 | 18.2 | 4.15 / 5 | 12.1 | 2.54 / 3 |
|  | CDRX (16, 8, 8) | 16.2 | 4.75 / 5 | 16.2 | 0 / 5 | 9.4 | 0 / 3 |
|  | CDRX(10,8,2) | 9.3 | 4.93 / 5 | 9.3 | 4.65 / 5 | 4 | 2.91 / 3 |
|  | CDRX(10,5,5) | 17.3 | 4.82 / 5 | 17.3 | 3.7 / 5 | 10.8 | 1.89 / 3 |
| Indoor Hotspot (FR2) | Always ON | - | - / 11 | - | - / 10 | - | - / 6 |
|  | CDRX (4, 2, 2) | 18.5 | 10.81 / 11 | 18.5 | 8.5 / 10 | 18 | 4.5 / 6 |
|  | CDRX (8, 4, 4) | 15.4 | 10.77 / 11 | 15.4 | 2 / 10 | 15 | 0.56 / 6 |
|  | CDRX (16, 8, 8) | 11.6 | 8.59 / 11 | 11.6 | 0 | 11.6 | 0 / 6 |
|  | CDRX(10,8,2) | 7.6 | 10.8 / 11 | 7.6 | 9.24 / 10 | 7.5 | 5.4 / 6 |
|  | CDRX(10,5,5) | 13.7 | 10.66 / 11 | 13.7 | 0.7 / 10 | 13.5 | 0.19 / 6 |
| Dense Urban (FR1) | Always ON | - | - / 8 | - | - / 6 | - | - / 4 |
|  | CDRX (4, 2, 2) | 20 | 7.04 / 8 | 21 | 4.98 / 6 | 14.5 | 2.76 / 4 |
|  | CDRX (8, 4, 4) | 16.7 | 6.72 / 8 | 18 | 3.66 / 6 | 10.8 | 1.6 / 4 |
|  | CDRX (16, 8, 8) | 13.6 | 5.6 / 8 | 15.8 | 0 / 6 | 7.9 | 0 / 4 |
|  | CDRX(10,8,2) | 8.8 | 7.44 / 8 | 9.2 | 5.58 / 6 | 3.2 | 3.52 / 4 |
|  | CDRX(10,5,5) | 15.4 | 6.08 / 8 | 17 | 3.12 / 6 | 9.5 | 0.96 / 4 |

# References

- RP-210460 Revised SID on XR Evaluations for NR, RAN#91

- R1-2009812 FL Summary of RAN1 103-e agreements and email discussions on Rel-17 SI on XR evaluations for NR

- RAN1 Chairman’s Notes (v018) 3GPP TSG RAN WG1 Meeting #104-e e-Meeting, January 25th – February 5th, 2021

- R1-2101866 Summary of [104-e-NR-XR-02]_Other Eval Methodology.

- Chair notes RAN1, 104-bis-e Chair_notes_104b-e

- R1-2109738 Development of the Evaluation Methodology for XR Study, Nokia Tdoc for RAN1#106-bis-e

- 3GPP, TS 38.133 "NR; Requirements for support of radio resource management”

- R1-2007151 Moderator summary for multi-beam enhancement: EVM, RAN1#102-e, 8.1.1

- 3GPP, RAN1 Chairman’s Notes, RAN1#102-e, v22, https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_102-e/Inbox/Chairman_notes/

- R1-2106630 Remaining issues on evaluation methodologies for XR, VIVO Tdoc for RAN1#106-e

- 3GPP, RAN1#106-e, 8.14.2, Summary on evaluation methodology for XR, v15, https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_106-e/Inbox/drafts/8.14.2/

# Appendix A – Simulation settings

## Indoor Hotspot (InH) scenario

The carrier frequency is set to 4 GHz and 30 GHz for FR1 and FR2, respectively. System bandwidth is assumed to be 100 MHz for FR1 and for FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz and 120 kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1, and 24dBm with (23dBm per 80MHz) in FR2. Table 37 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 43 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |  |
| Channel model | InH |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |
| System bandwidth | FR1: 100 MHz | FR2: 100 MHz |
| BS height | 3 m |  |
| UE height | 1.5 m |  |
| BS noise figure | FR1: 5 dB | FR2: 7 dB |
| UE noise figure | FR1: 9 dB | FR2: 13 dB |
| BS receiver | LMMSE-IRC |  |
| UE receiver | LMMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) | FR2: 24 dBm (23dBm per 80MHz) |
| UE Tx max power | 23 dBm |  |
| TDD Frame structure | DDDSU |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |  |
|  | Configuration in FR1: - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) | Configuration in FR2: - 2 TxRU - (M, N, P, Mg, Ng) = (16, 8, 2,1,1) - (dH, dV) = (0.5, 0.5) λ - (Mp, Np) = (1,1) Grid of Beams: - Azimuth angles (degrees): {90, 90, 90, 112.5, 112.5, 112.5, 67.5, 67.5, 67.5, 140, 140, 140, 40, 40, 40} - Elevation angles (degrees): {-30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30} |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) | - Pattern: UE radiation pattern model 1 (TR 38.901) - Gain: 5 dBi - Configuration (Option 1): (M, N, P) = (1, 4, 2), (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,1) 3 panels (left, right, top) |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

## Dense Urban (DU) Scenario

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth has been fixed to 100MHz for both FR1 and FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 38 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 44– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |  |
| Channel model | Uma |  |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |  |
| System bandwidth | FR1: Option 1: 100 MHz | FR2: Option 1: 100 MHz |  |
| BS height | 25m |  |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |  |
| BS noise figure | FR1: 5 dB |  | FR2: 7 dB |
| UE noise figure | FR1: 9 dB |  | FR2: 13 dB |
| BS receiver | MMSE-IRC |  |  |
| UE receiver | MMSE-IRC |  |  |
| Channel estimation | Realistic (with ideal CSI) |  |  |
| UE speed | 3 km/h |  |  |
| MCS | Up to 256QAM |  |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |  |
| UE Tx max power | FR1: 23 dBm |  | FR2: 23 dBm, maximum EIRP 43 dBm |
| TDD Frame structure | Option 1: DDDSU |  |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |  |
| Cell Selection | RSRP Slow Fading |  |  |
| BS antenna configuration | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: 2TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH, dV) = (0.5λ, 0.5λ) Grid of Beams - Azimuth angles: {33.75, 56.25, 78.75, 101.25, 123.75, 146.25, 33.75, 56.25, 78.75, 101.25, 123.75, 146.25} degrees - Elevation angles: {-12.5, -12.5, -12.5, -12.5, -12.5, -12.5, -57.5, -57.5, -57.5, -57.5, -57.5, -57.5} degrees |  |
| UE antenna configuration | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) | FR2: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) (Mp, Np)=(1, 1) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |  |
| PDCCH overhead | Modelled |  |  |
| Target BLER | 10% for first transmission |  |  |
| Max HARQ transmission | 3 |  |  |
| HARQ scheme | Chase combining |  |  |