3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109753

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon

Title:	Further considerations and simulations of CSI enhancement for Rel-17

Document for:	Discussion and Decision

Introduction

In this contribution, more simulation results and details of CSI enhancement are discussed for Rel-17 to provide supplementary information for companion paper [1].

- Rel-17 PS CB enhancement

- Simulation results for parameter combinations

- More considerations on UCI design

- CSI enhancement for MTRP

- Views on CBSR

Additional simulation results for parameter combinations

According to RAN1#106-e agreements, parameter combinations represented by (alpha, Mv, beta) with K1 = alpha*P for Rel-17 PS codebook. Moreover, candidate values of alpha are {1/2, 3/4, 1}, Mv = {1, 2}, and values of beta = {[1/4], 1/2, 3/4, 1}. In order to decide exact parameter combinations, the performance and overhead of all supported parameter combinations for 8 ports and 32 ports are given in [1]. In this part, more simulation results will be given for the parameter combinations.

In the simulation, the performance and overhead of all supported parameter combinations for 16 ports are shown in Figure 1. As shown in the Figure 1, the performance and overhead of all the supported parameter combinations can be described by the following six curves:

  - Curve 1：Mv=1,α=1, β={0.25,0.5,0.75,1}

  - Curve 2：Mv=1,α=0.75, β={0.25,0.5,0.75,1}

  - Curve 3：Mv=1,α=0.5, β={0.25,0.5,0.75,1}

  - Curve 4：Mv=2,α=1, β={0.25,0.5,0.75,1}

  - Curve 5：Mv=2,α=0.75, β={0.25,0.5,0.75,1}

  - Curve 6：Mv=2,α=0.5, β={0.25,0.5,0.75,1}

Similar as the simulation results of 8 ports and 32 ports in [1], the cures with {alpha = 0.5, Mv=1}, {alpha = 1, Mv=1} and {alpha = 1, Mv=2} can constantly provide better performance compared with other parameter combinations of (alpha, Mv) for a ranges of # of CSI-RS ports.

(a-1) Fixed Rank 1: Curve 1 ~ 6                               (a-2) Fixed Rank 1: Curve 1, 3, 4

(b-1) Up to Rank 2: Curve 1 ~ 6                                (b-2) Up to Rank 2: Curve 1, 3, 4

Figure 1. The performance and overhead of all the supported parameter combinations for 16 ports

Furthermore, for detailed parameter combinations to distribute mean UPT gain equally as much as possible and also support a wide range of payload sizes, following 8 parameter combinations, which is also same as [1], are shown in Figure 2 for 16 ports,

Table 1 The selected parameters combinations for 16 ports

| paramCombination-r17 | α | Mv | β For Rank 1~2 | β For Rank 3~4 | Payload of Rank 2 (16 ports) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.5 | 1 | 0.25 | 0.125 | ~50 bits |
| 2 | 0.5 | 1 | 0.5 | 0.375 | ~80 bits |
| 3 | 0.5 | 1 | 0.75 | 0.625 | ~110 bits |
| 4 | 1 | 1 | 0.5 | 0.375 | ~150 bits |
| 5 | 1 | 1 | 0.75 | 0.625 | ~200bits |
| 6 | 1 | 1 | 1 | 0.875 | ~260bits |
| 7 | 1 | 2 | 0.5 | 0.375 | ~300bits |
| 8 | 1 | 2 | 0.75 | 0.625 | ~400 bits |

Figure 2. The performance and overhead of all the supported parameter combinations for 16 ports

Observation 1: Considering additional simulation cases, e.g.PCSI-RS=16, similar observations can be found that {alpha = 0.5, Mv=1}, {alpha = 1, Mv=1} and {alpha = 1, Mv=2} and 8 parameter combinations on Table 1 can provide better performance and CSI feedback overhead tradeoff.

UCI design

Using ceil(log2(K1*MV)) bits to report the strongest coefficient indication (SCI) per layer has been agreed last meeting. Different from the SCI design as Rel 16, the strongest coefficient indication (SCI) per layer included the position of selected port and FD base for the strongest coefficient. Therefore, as proposed in [1], The FD indicator should be assigned to G0 for Rel-17 PS codebook enhancement, at the same level of port indicator.

In addition to above modification, considering that the UL and DL have reciprocity of angles and delays, if the ports are precoded in ascending order of the coefficients for the angle-delay bases, the coefficients also basically satisfy that a smaller-index port corresponds to a larger coefficient, and a larger-index port corresponds to a smaller coefficient. The value of MV is also quite small (MV=1 or 2) in Rel-17. Therefore the priority order of coefficients should be 1st layer index, 2nd port index, and then 3rd FD bases index.

Based on above analysis, the following proposal can be considered for the priority order of coefficients for Rel-17 Type II PS CB.

Proposal 1: The priority order of Rel-17 Type II PS coefficients should be 1st layer index, 2nd FD bases index, and 3rd port index.

CBSR for CSI enhancement for Multi-TRP

To avoid reporting the precoders which may induce the strong interference, the UE can be configured with CBSR to be informed that the reported PMI is not allowed to correspond to the associated precoders. In NCJT, the precoders causing the strong interference on the cooperative TRPs can be very different. It is reasonable to configure CBSR per TRP. Specifically, for CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE is configured with two CBSRs. When the UE derives the PMI over a CMR associated with the first CMR group, the reported PMI is restricted by the first CBSR associated to that CMR group, and similarly for the second CMR group in the same CMR set. Note that the payload of PMI is independent to the configuration of CBSR. For example, for a given NZP CSI-RS resource set for channel measurement with 4 NZP CSI-RS resource, i.e., {{#0 #1}, {#2, #3}} with N=1 so that {#0 #2} is for NCJT measurement hypothesis. {#0 #1} is the first CMR group associated with the first CBSR #1 and {#2, #3} is the second CMR group associated with the second CBSR2. Therefore, for NCJT CSI calculation, both CBSR #1 and #2 configured for corresponding CSI-RS resources are applied jointly when determining NCJT PMI. On the other hand, if single-TRP CSI calculation is considered, CBSR #1 and #2 configured for corresponding CSI-RS resources are applied independently for single-TRP measurement hypotheses.

Proposal 2: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, two CBSRs can be configured for a given reporting setting whereas each of them corresponds to one CMR group in a CMR set, i.e. per TRP.

Conclusions

This contribution provides our views on further considerations and simulations CSI enhancement for Rel-17. In summary, the following proposals and observations are made.

Observation 1: Considering additional simulation cases, e.g.PCSI-RS=16, similar observations can be found that {alpha = 0.5, Mv=1}, {alpha = 1, Mv=1} and {alpha = 1, Mv=2} and 8 parameter combinations on Table 1 can provide better performance and CSI feedback overhead tradeoff.

Proposal 1: The priority order of Rel-17 Type II PS coefficients should be 1st layer index, 2nd FD bases index, and 3rd port index.

Proposal 2: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, two CBSRs can be configured for a given reporting setting whereas each of them corresponds to one CMR group in a CMR set, i.e. per TRP.

- References

R1- 2108762, Discussion on CSI enhancements for Rel-17, Huawei, HiSilicon.

- Appendix A.

Table A-1 SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro |
| Frequency Range |  | FR1 only, 2.1GHz, with duplexing gap of 200MHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ Other configuration is not precluded. |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 [21] |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with fixed Rank 2 SU-MIMO with fixed Rank 4 |
| CSI feedback |  | Feedback assumption: - CSI feedback periodicity (full CSI feedback) : 5ms - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| SRS Configuration |  | - SRS periodicity with 10ms - Comb: 2 - Number of OFDM symbols: 2 |
| SRS Error Model |  | SRS error Modelling in Table A.1-2 in 36.897. Δ=9dB and detailed derivation of Δ can be found in R1-144943[20]. |
| Calibration error model at gNB |  | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN amplitude error (expressed in decibels) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation for simulation bandwidth , respectively |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI reporting overhead |