3GPP TSG RAN WG1 #106bis-e			R1-2109763

e-Meeting, October 11th – 19th, 2021

Agenda Item: 	8.4.1

Source: 	Baicells

Title:	Discussion on timing relationship enhancement for NTN

Document for:	Discussion and Decision

Introduction

In RAN1#106-e, some agreements have been achieved to enhance the timing relationship for NTN

Agreement:

- The UE-specific K_offset can be provided and updated by network with MAC CE.

- FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration

  - FFS: Details on whether and how the two solutions work together

Agreement:

For random access procedure initiated by a PDCCH order received in downlink slot , UE determines the next available PRACH occasion after uplink slot  to transmit the ordered PRACH.

- Note: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  , where  is assumed for PDCCH ordered PRACH.

- FFS: Which value of  should be applied

- FFS: Whether the  timing relationship is impacted by UE behavior within or after the validity duration.

Agreement:

The unit of K_offset is number of slots for a given subcarrier spacing.

- FFS: one subcarrier spacing value or different subcarrier spacing values for different scenarios.

Agreement:

The information of K_mac is carried in system information.

Agreement:

The unit of K_mac is number of slots for a given subcarrier spacing.

- FFS: one subcarrier spacing value or different subcarrier spacing values for different scenarios.

Agreement:

In the estimate of UE-gNB RTT, which is equal to the sum of UE’s TA and K_mac, for delaying the starts of ra-ResponseWindow and msgB-ResponseWindow, the UE’s TA is equal to  with .

Agreement:

For defining value range(s) of K_offset, down-select one option from below:

- Option 1: One value range of K_offset covering all scenarios.

- Option 2: Different value ranges of K_offset for different scenarios.

In this contribution, we provide some further considerations on timing relationship enhancement for NTN.

Discussion on timing relationship enhancement for NTN

K_offset update

At the GTW session on August 17, 2021, the following agreement was made:

Agreement:

- The UE-specific K_offset can be provided and updated by network with MAC CE.

- FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration

  - FFS: Details on whether and how the two solutions work together

Based on the views expressed, it can be seen that there are a number of companies that would like to support the RRC reconfiguration method, but there are also several companies that question the necessity and raise some issues.

Moderator recommendation on Issue #1:

Proponents are encouraged to further justify the necessity of the RRC reconfiguration method for updating UE-specific K_offset at the next RAN1 meeting.

Our opinion on this issue is:

Based on the agreement that UE-specific K_offset can be provided and updated by network with MAC CE, UE-specific K_offset update by RRC reconfiguration may introduce complexity and is not necessary.

Proposal 1: The UE-specific K_offset can be provided and updated by network with MAC CE, and not by RRC reconfiguration.

Cell-specific K_offset value determination

In RAN1#106-e, one issue has been mentioned that cell-specific K_offset may have misalignment between gNB and UE.

Our opinion is: Yes, misalignment of cell-specific K_offset between gNB and UE does exist due to large RTT between gNB and UE. The issue may cause errors in scheduling in a time window right after the network update cell-specific K_offset. The more often cell-specific K_offset is updated, more likely errors will be caused. To solve this issue we propose to configure a time window after cell-specific K_offset update and before it become effective at the gNB side.

Proposal 2: For cell-specific K_offset update, a time window shall be configured by the network to delay it taking effective.

Beam-specific K_offset

Regarding whether or not beam-specific K_offset is supported in initial access, it has been discussed in several meetings from 103-e to 106-e meeting.

Our opinion is to support beam-specific K_offset because it is a good compromising between cell-specific and UE-specific K_offset. Beam-specific K_offset can take the rolls of cell-specific K_offset and even UE-specific K_offset, greatly decreasing the complexity and signaling overhead introduced by UE-specific K_offset. Beam-specific K_offset can be indicated by system information. We would also like to propose to enhance the system information to be beam-specific also, so that the signaling overhead to broadcast beam-specifc K_offset can be significantly reduced.

Proposal 3: Support beam-specific K_offset.

Proposal 4: Support beam-specific system information, which can carry beam-specific K_offset and is dedicated for a particular beam.

Conclusions

In this contribution, we discussed enhancements for timing relationship in NTN. Then we get the following observations and proposals:

Proposal 1: The UE-specific K_offset can be provided and updated by network with MAC CE, and not by RRC reconfiguration.

Proposal 2: For cell-specific K_offset update, a time window shall be configured by the network to delay it taking effective.

Proposal 3: Support beam-specific K_offset.

Proposal 4: Support beam-specific system information, which can carry beam-specific K_offset and is dedicated for a particular beam.

- References

[1] RAN1#106-e Chairman's Notes