3GPP TSG RAN WG1 #106b-e	                           R1-2109791

e-Meeting, October 11th –19th, 2021

Agenda Item: 	8.5.2

Source:	Sony

Title:	Considerations on enhancements for UL-AoA

Document for:	Discussion & Decision

# Introduction

In RANP#91e, a work item on NR positioning enhancements was updated [1]. The objective of the work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. One of the objectives of this work item is:

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

In RAN1#106e [2], the following agreements on the accuracy enhancements for UL-AoA positioning were made:

Agreement: The maximum number of UL-AOAs values (pair of AOA & ZOA values) to be reported per SRS resource for the first arrival path corresponding to the same timestamp is 8. Conclusion: It is up to RAN3 to decide how to support indication of UL AoA/ZoA assistance information in LCS for LCS to GCS translation Agreement: Further study and conclude whether association of UL-AOA, UL-TDOA, Multi-RTT measurements with ARP (Antenna Reference Point) information is supported at RAN1#106bis-e. Agreement: Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF - FFS: Reporting of RSRP for the first arrival path - Note: The use of SRS for MIMO resource is transparent to the UE - FFS: Reporting of gNB Rx-Tx Agreement: Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF FFS: Reporting of RSRP for the first arrival path

In this contribution, we discuss and provide our view on the remaining issues of the potential solution to enhance the UL-AoA positioning accuracy.

# Potential enhancements

## Assistance Information for Expected AoA/ZoA

In our view, gNB may use the expected AoA/ZoA information as a filter. This filter will exclude all the paths that are outside of range. Therefore, it is important that gNB can receive an accurate range from LMF. And at least, the expected AoA/ZoA range should cover the whole UE activity range that a UE may exist.

In some cases, the expected AoA/ZoA may not be valid due to UE’s movement. This also means the expected AoA/ZoA can only be used during a certain period. The usage of the expected AoA/ZoA while the UE has moved to the area outside the expected AoA/ZoA range can result in positioning accuracy degradation. For example, if a gNB is informed by LMF of an expected AoA range but this UE happens to move outside of the range before the measurement, gNB may miss the signal from the direction to the UE and lead to positioning estimation inaccuracy.

To address this issue and to ensure the assistance information on AoA range is used correctly, we consider LMF to provide a time validity of an expected AoA/ZoA so that the gNB can identify how long the related assistance information  can be used. For example, LMF can inform the gNB that the assistance information is valid within certain time. Hence, the SRS reception should be obtained within a certain time range as informed by LMF.

Proposal 1: Support LMF to provide a time validity information associated with the expected AoA/ZoA range.

In addition, we also consider the case where LMF is not able to estimate a certain validity period. In such case, LMF can report a one-time expected AoA/ZoA range that can only be used in one positioning measurement. To prevent a long gap between expected range report and a positioning measurement, LMF can reports the one-time expected range in a positioning request, which is correct before the positioning measurement is performed.

Proposal 2: Expected AoA/ZoA range assistance information can be explicitly indicated to be used for one positioning measurement.

## Quality matrix enhancement

Most of the TRPs are equipped MIMO antenna with multiple element rows and columns. A typical antenna sizes are 4 by 4 MIMO in FR1 and 4 by 8 in FR2. In UL-AoA positioning, each element row can be regarded as a linear array. The AoA can be measured at each element row and the measurements are independent. Likewise, the ZoA can be independently measured by each  column of elements. As a result of it, there could be multiple independent AoA and ZoA measurements at different element rows or columns. To illustrate, we provide the following figure about multiple AoA/ZoA measurements by FR1 MIMO antenna model. In FR1 MIMO antenna model as shown in Figure 1, there could be 4 independent AoA measurements and 4 ZoA measurements in total.

AoA measurement 1

AoA measurement 2

AoA measurement 3

AoA measurement 4

ZoA1

ZoA2

ZoA3

ZoA4

Figure 1: Illustration of multiple AoA/ZoD measurements given by different rows/columns.

Our study shows that the standard deviation of multiple AoA/ZoA measurements at different rows/columns can also be utilized to indicate the channel quality and the reliability of the AoA/ZoA measurement. This parameter reflects the statistical property of the measured AoA results. For simplicity, we name this new parameter as “uAoA”.

Similar to the usage of SRS-RSRP, the LMF can use uAoA to filter out some unreliable AoA measurements. The qualified AoA measurement would be used in the calculation of the UE position estimation. In order to verify the proposed method, a simulation on UE positioning estimation in indoor factory (InF) scenario was carried out.

We have performed simulations on InF-sparse high (SH) scenario in FR2 frequency range. The simulation results are shown in Figure 2. The CDF of the legacy UL-AoA positioning accuracy (with RSRP reporting) is also given in the simulation as the reference. It is shown that reporting uAoA as the AoA measurement indicator improve the UE positioning estimation accuracy. Particularly on the horizontal accuracy for 90% CDF of the UE, one can observe 68% gain in positioning accuracy over the legacy UL-AoA in InF-SH FR2 scenario.

Figure 2: Comparison between UL-AoA positioning with AoA uncertainty reporting and the legacy UL-AoA positioning in terms of positioning accuracy in meter.

Observation 1: The statistic property of the AoAs measurement at different MIMO elements can be used as a channel quality indicator. The usage of this new parameter can improve UL-AoA positioning accuracy. For InF-SH FR2, this parameter can obtain 68% gain over the legacy AoA .

We propose to enhance the reporting from gNBs to the LMF by including the statistical property of the measured AoA in which it can reflect the uncertainty of the AoA (uAoA). This is to be reported as part of the measurement report (ie. New parameter in the quality matrix) which enable the LS to process (filtering) the AoA measurement effectively. The statistical property is basically the measurement quality indication.

In RAN1#105, it was agreed upon that multiple AoA reports based on the first path can be provided by gNB. We consider this agreement is also aligned with our proposal on providing the statistical property of the measured AoA. In practice, gNB may have obtained multiple AoA results. The number of multiple AoA that can be reported may be limited. Hence, the statistical property of the measured AoA is a measure of all AoA results that the gNB has obtained. The LMF can use this information for the UE positioning estimation.

Proposal 3: Support measurement of a AoA statistical property (e.g., statistical property / standard deviation of multiple AoA/ZoA measurements in UL-AoA measurement) as a new quality indicator in the quality matrixes, reported from gNB to LMF.

## UE SRS resource ID reporting

Another approach to enhance AoA positioning method is by providing UE TX beam related information to the LS. UE transmits UL SRS for positioning with certain TX beam direction / properties and it also transmit in SRS resource that has been configured by the gNB. The detected received SRS (e.g. SRS that has high RSRP) in a specific resource can represent UE TX beam. For example, if there are 3 gNBs located in various directions then each gNB may detect different SRS resource that produce high RSRP. We propose gNB to report the SRS resource ID that has been used for the AoA measurement. Having this SRS resource ID information enable the LS to filter out undesired report from certain gNB or basically to filter out any anomaly results.

Furthermore, the LMF can estimate the AoD in the UE coordinate because each SRS resource ID is associated with different UE Tx beams and different AoDs. Although the orientation of the UE is unknown and the AoD can’t be transferred from LCS to GCS, LMF can still calculate the AoD differences between reference TRP and other TRP as in the figure shown below:

TRP1

Reference TRP

TRP2

UE with unknown orientation

θ1

θ2

SRS 1

SRS 2

SRS 3

SRS 4

SRS 5

Figure 3: The UE AoD differences can be estimated if the SRS resource IDs at different TRPs are known in LMF.

If more than 3 TRPs are used, LMF can use the AoD differences to estimate the UE location without knowing the UE orientation. Moreover, this information can assist the UE location estimation by the AoA and improve the positioning accuracy.

Proposal 4: Support SRS resource ID information of the associated reported AoA/SRS-RSRP measurement in UL-AoA measurement report from gNB to LMF.

# Conclusion

In this contribution, we discuss some aspects of positioning accuracy improvements using UL-AoA method.  We provide the following observations:

Observation 1: The statistic property of the AoAs measurement at different MIMO elements can be used as a channel quality indicator. The usage of this new parameter can improve UL-AoA positioning accuracy. For InF-SH FR2, this parameter can obtain 68% gain over the legacy AoA .

We also provide the following proposals:

Proposal 1: Support LMF to provide a time validity information associated with the expected AoA/ZoA range.

Proposal 2: Expected AoA/ZoA range assistance information can be explicitly indicated to be used for one positioning measurement.

Proposal 3: Support measurement of a AoA statistical property (e.g., statistical property / standard deviation of multiple AoA/ZoA measurements in UL-AoA measurement) as a new quality indicator in the quality matrixes, reported from gNB to LMF.

Proposal 4: Support SRS resource ID information of the associated reported AoA/SRS-RSRP measurement in UL-AoA measurement report from gNB to LMF.

# Reference

RP-210903, Revised WID on NR Positioning Enhancements

RAN1 Chairman’s Notes, RAN1#106e, e-Meeting, August 16th – 27th, 2021