3GPP TSG RAN WG1 #106bis-e			R1-2109833

e-Meeting, October 11th – 19th, 2021

Agenda Item:	   8.1.2.3

Source:			   FGI, Asia Pacific Telecom

Title:	Discussion of enhancements on beam management for multi-TRP

Document for:	   Discussion

# Introduction

In previous meetings, agreements related to Mul-TRP beam managements have been made and part of them are quoted below.

Beam failure recovery Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC. Agreement A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group - NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group Agreement For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR - FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1) Agreement Select one of the following alternatives with possible modification in RAN1#106-e - Alt 2.5.2 A: - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 B: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 C: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 D: - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

Based on the above quotations and FeMIMO WID [1], we provide our views on some related issues, especially TRP-specific BFR, in the following paragraphs.

# Discussion

## TRP-specific BFR procedure

As shown above, in RAN1#104e, RAN1 has agreed that at most two PUCCH-SR resources for TRP-BFR can be configured in a cell group. However, there were some issues staying unresolved after previous two meetings. One discussion focuses on how to select the PUCCH-SR resource in case of that there are two PUCCH-SR resources configured. RAN1 has listed four alternatives and respective brief introductions are shown as below:

- Alt 2.5.2 A: UE implementation

- Alt 2.5.2 B: upon particular case, select PUCCH-SR associated with failed TRP

- Alt 2.5.2 C: upon particular case, select PUCCH-SR associated with non-failed TRP

- Alt 2.5.2 D: revert agreement that two PUCCH-SR can be configured

In our views, the selection could be totally left to UE implementation, i.e. Alt 2.5.2 A. The whole per-TRP BFR procedure can work well since network is supposed to monitor both PUCCH-SR resources anyway, regardless of whether there is a selection rule. Some companies have argued that two PUCCH-SR resources can be associated with two TRPs respectively. Through pre-determined selection rule, network can know which TRP is failed in advance (e.g., via choosing a PUCCH-SR associated with a failed or non-failed TRP). In addition, network may be able to decide which TRP to allocate the UL grant for per-TRP BFRQ transmission. However, it provides marginal benefit. One reason is that UE does not always need a PUCCH-SR to request UL resource for transmitting TRP-BFR MAC-CE to convey information of failed TRP(s). UE can transmit a TRP-BFR MAC-CE whenever there is available PUSCH resource. Another one is UE behaviour would be unclear when TRP failure status is different across cells. Under such case, we may need other rules or a default PUCCH-SR for acquiring UL grant. In details, the claimed benefit exists when beam failure happens for only one TRP in only one serving cell. Hence, we suggest leaving selection of PUCCH-SR to UE implementation. In our opinion, one possible UE implementation includes transmission of two PUCCH-SR resources. Meanwhile, we can also accept Alt 2.5.2 D since configuration of two PUCCH-SR does not really provide much benefit given the reasons shown above.

Proposal 1: Support Alt 2.5.2 A: selection of PUCCH-SR resource is up to UE implementation.

- Note: this includes transmission of two PUCCH-SR resources.

During the offline discussion after RAN1#105(e) and following online/offline discussions in RAN1#106(e), companies had some discussions on whether to configure association of PUCCH-SR and TRP. After the fruitful discussion, the group came out a proposal related to the association of PUCCH-SR and TRP. We see the benefit and support it in general. However, we would like to modify it a bit since the association does not only benefit the PUCCH-SR selection (if agreed to have a rule). Therefore, we have the proposal below.

Proposal 2: Support configuring an association between a TRP (e.g., BFD-RS set) on SpCell and PUCCH resource (e.g., PUCCH-SR resource) on SpCell.

In addition, the group also had another issue related to PUCCH-SR resource transmission. Given that it has been agreed in M-TRP PUCCH enhancement that a PUCCH resource can be activated two UL beams for repetition, one issue is that whether the configured PUCCH-SR can be activated with two UL beams as well. In our opinion, since this feature has been introduced, there is no prohibition to network from configuring PUCCH-SR with such feature. However, if PUCCH-SR is activated with two UL beams, both beams should be used to transmit for repetition purpose. We do not agree to add additional selection rule on these two activated beams, in which we see no benefit and increase SPEC impact instead.

Proposal 3: PUCCH-SR resource is allowed to be activated two spatial relations for repetition.

- Note: no further selection is imposed on activated spatial relations of PUCCH-SR resource, if two spatial relations are activated.

In RAN1#104b(e), we left a FFS issue regarding whether cell-specific and TRP-specific BFR can be configured in the same serving cell. We propose to discuss this issue more specifically since one general BFR procedure may involve BFD-RS configuration, NBI-RS configuration, BFRQ transmission and BFR response. We do not see it is sensible to configure a number of BFD-RS sets and NBI-RS sets from cell-specific and TRP-specific BFR in one serving cell. However, it is possible to configure transmission of cell-specific BFRQ and TRP-specific BFRQ in one serving cell. In fact, we believe that cell-specific BFRQ may be triggered when two TRPs are both considered as failed TRPs. In conclusion, we propose to support configuring transmission of cell-specific BFRQ and TRP-specific BFRQ in one serving cell.

Proposal 4: Support configuring transmission of cell-specific BFRQ and TRP-specific BFRQ in one serving cell.

From discussion above, we see there is no need to configure another BFD-RS set for detecting cell-specific beam failure, which correspondingly triggers a cell-specific BFRQ. In such way, RS measuring complexity can be released in UE side. Hence, we have the following proposal.

Proposal 5: At most 2 BFD-RS sets are allowed to be configured in a BWP of a serving cell configuring BFR.

In previous meetings, RAN1 group has discussed that whether to introduce a fallback scheme for BFRQ in TRP-specific BFR procedure (like RACH-based mechanism). In addition, an involved issue is regarding the co-existence/interaction between cell-specific BFR procedure and TRP-specific procedure. In our opinion, we suggest dealing with this issue from perspective of SpCell and SCell respectively.

- From perspective of SpCell:

  - We suggest not supporting RACH-based BFRQ transmission for TRP-specific BFR when there is a PUCCH-SR configured. However, when there is no PUCCH-SR configured in the SpCell and there is no available UL grant assigned to the UE (DG or CG), it is natural to trigger a RACH based BFRQ transmission (e.g., CBRA) for TRP-BFR, even that only one TRP is failed.

  - For co-existence/interaction between cell-specific BFR and TRP-specific procedure, we should prohibit that UE monitors two TRP-specific BFD-RS sets and one cell-specific BFD-RS set at the same time. As mentioned in previous paragraphs, it may cause too much UE power consumption and resources waste. Nonetheless, when two TRP-specific BFD-RS sets are detected below certain quality, we may treat it as beam failure event of the whole serving cell. Correspondingly, we may send cell-specific (Rel-15/16) BFRQ transmission in this case. When such cell-specific BFRQ transmission is received, gNB learns that both TRP links are failed and the UE requests to recover one of TRP links. To some extent, this coexistence can be considered as fallback mechanism, but without much more additional PRACH resources demand.

- From perspective of SCell:

  - We do not see the need to introduce RACH-based BFRQ for TRP-specific BFR in SCell when there is still available UL grant or PUCCH-SR configured. We believe that only when there is no PUCCH-SR configured in belonging cell group and there is no available UL grant assigned to the UE (DG or CG), it is allowable to trigger a RACH based BFRQ transmission (e.g., CBRA) for TRP-BFR, even that only one TRP is failed.

  - For co-existence/interaction between cell-specific BFR and TRP-specific BFR, we share similar view as that of SpCell. There is no need to monitor TRP-specific BFD-RS and cell-specific BFD-RS at the same time. However, when all TRP-specific beam links are measured below quality, it is straightforward for UE to transmit a Rel-16 MAC-based BFRQ as we promoted in SpCell case. In this sense, gNB not only receives one new beam for recovery, but also learns that both TRP links of the serving cell have failed.

To conclude the above discussion, we have the following proposals.

Proposal 6: If one of the following conditions is achieved, CBRA can be triggered as TRP-specific BFRQ:

- For both SpCell and SCell, when there is no PUCCH-SR configured and there is no available UL grant;

- For SpCell, when two TRP-specific BFD-RS sets are measured below quality.

# Conclusion

According to the discussion mentioned above, we have the following proposals.

Proposal 1: Support Alt 2.5.2 A: selection of PUCCH-SR resource is up to UE implementation.

- Note: this includes transmission of two PUCCH-SR resources.

Proposal 2: Support configuring an association between a TRP (e.g., BFD-RS set) on SpCell and PUCCH resource (e.g., PUCCH-SR resource) on SpCell.

Proposal 3: PUCCH-SR resource is allowed to be activated two spatial relations for repetition.

- Note: no further selection is imposed on activated spatial relations of PUCCH-SR resource, if two spatial relations are activated.

Proposal 4: Support configuring transmission of cell-specific BFRQ and TRP-specific BFRQ in one serving cell.

Proposal 5: At most 2 BFD-RS sets are allowed to be configured in a BWP of a serving cell configuring BFR.

Proposal 6: If one of the following conditions is achieved, CBRA can be triggered as TRP-specific BFRQ:

- For both SpCell and SCell, when there is no PUCCH-SR configured and there is no available UL grant;

- For SpCell, when two TRP-specific BFD-RS sets are measured below quality.

# Reference

- RP-202024, Revised WID: Further enhancements on MIMO for NR, Samsung