3GPP TSG RAN WG1 #106b-e                                    R1-2109850

e-Meeting, October 11th - 19th, 2021

Source:	ZTE

Title:	Discussion on UE features for NR-NTN

Agenda Item:	8.17.4

Document for:   Discussion

- Introduction

The initial set of UE feature for NR-NTN is provided in [1]. In this contribution, our views on some features are elaborated with corresponding updates.

- Discussion on the UE feature for NR-NTN

As listed in the Table 1, the updates on the FG 26-3/26-4/26-7 and 26-8 have been made. More specifically,

1. Regarding the FG 26-3/26-4, these two features are related to the UE’s behavior on TA pre-compensation, and the prerequisite feature groups should be FG 26-1 instead of FG 26-3.

2. Regarding the FG 26-7, according to the current discussion, there is no intention to explicit couple the supports on the larger aggregation factor for the case that the HARQ feedback is disabled. Then, the existing description in the column for consequence should be removed. Moreover, it should be noticed that if no new value is introduced, this feature group should be removed.

3. Regarding the FG 26-8, according to the current discussion, the indication of polarization information of DL or UL from gNB side is only for information instead of configuration to UE. In this case, there is no mandate behaviour or requirement to force the UE to support any polarization. Then, the whole FG-26-8 should be removed. If the further agreement on the polarization information reporting is achieved, new feature for this purpose can be defined.

In addition, in our view, the UE’s behaviour on frequency offset pre-compensation is mainly the UE’s implementation. It may not be needed to define the dedicated UE feature group for it. Then, FG-26-2 can be removed.

In conclusion, the updates on UE features for NR-NTN can be found in Table 1.

Proposal-1: The updates on the UE features for NR-NTN listed in Table-1 should be supported.

Table 1 UE features for NR-NTN

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. | Mandatory |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | Mandatory |
| 26. NR_NTN_solutions | 26-3 | Start of RAR window | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow - delaying the start of msgB-ResponseWindow | 26-2 26-1 | No | No | Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 26-2 26-1 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | [Optional] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - Support on the maximal HARQ process number is up to UE capability |  | Yes | No | Increasing the number of HARQ processes avoids HARQ stalling, |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |
| 26. NR_NTN_solutions | 26-6 | HARQ CODEBOOK Enhancement | - Enhancement on Type-2 HARQ codebook in NTN - Enhancement on Type-1 HARQ codebook in NTN |  | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory |
| 26. NR_NTN_solutions | 26-7 | NTN Performance enhancement | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 8, 16 or 32 |  | Yes | No | Particularly beneficial in case HARQ feedback is disabled |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - UE polarization capability (RHCP, LHCP, Linear) - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. |  | No | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |

# References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, AT&T, NTT DOCOMO.