3GPP TSG RAN WG1 #106bis-e	R1-2109854
e-Meeting, October 11th – 19th, 2021

Source:				Panasonic

Title:	RAN1 aspects for RAN2-led features for RedCap

Agenda item:	8.6.2

Document for:			Discussion and Decision

Introduction

This document discusses UE type description for Reduced capability NR devices (RedCap UE).

Discussion

RedCap UE type description

In RAN1 #106, the following agreement was made:

Agreement - A RedCap UE type from RAN1 point of view supports a maximum bandwidth of 20MHz for FR1 and 100MHz for FR2 - Further discuss whether to capture also one or more of the following reduced capabilities to RedCap UE type description - Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers - Supports either FD-FDD or Type A HD-FDD operation for FR1 FDD bands - Supports either DL up to 64 QAM or up to 256 QAM for FR1 - Does not support CA/DC

According to the Rel-17 RedCap WID, the purpose of RedCap UE description is: for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs. For this purpose, we don’t see need to include the capability description on duplex type as either FD or HD-FDD can be supported by a Rel-15/16 UE as well. The remaining capabilities can be useful for this purpose and should be captured.

Proposal 1:	In addition to maximum bandwidth, the following reduced capabilities can be captured to RedCap UE type description:

- Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers

- Supports either DL up to 64 QAM or up to 256 QAM for FR1

- Does not support CA/DC

Proposal 2:	No need to capture duplex capability to RedCap UE type description.

Conclusion

Proposal 1:	In addition to maximum bandwidth, the following reduced capabilities can be captured to RedCap UE type description:

- Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers

- Supports either DL up to 64 QAM or up to 256 QAM for FR1

- Does not support CA/DC

Proposal 2:	No need to capture duplex capability to RedCap UE type description.