3GPP TSG RAN WG1 #106bis-e		                                    		R1-2109924

e-Meeting, Oct 11 – Oct 19, 2021

Agenda Item:	8.14.1

Source:	InterDigital Inc.

Title:	Performance Evaluation Results for XR

Document for:	Discussion and Decision

# Introduction

Previous RAN1 meetings have agreed on several parameters for the traffic model in terms of data rates and PDB values per application, flagging some parameters as baseline and others as optional [1-5]. In this contribution, system-level evaluations for capacity are presented for all baseline scenarios. Additionally, capacity performance for a multiple-stream model is evaluated in the UL.

# UL/DL Performance Evaluation Results for XR

System level simulations are done to evaluate capacity performance for a single DL data stream model and single and multiple UL data stream models. For the UL case, the underlying assumption is that the UEs are always ON (no power saving scheme is applied for either model). For the DL case, capacity is evaluated assuming the baseline scenario without power savings.

2.1 Traffic Model for RAN evaluation

Parameters used in the performance evaluation for the statistical model for XR traffic are listed in Table 1 for UL and Table 2 for DL. In the given set of evaluations, a UE is declared as satisfied only when each stream meets the requirement of 99% successful packet delivery within a given air interface PDB.

| UL |  | CG/VR (single stream: pose/control) | AR (single stream: Aggregated video) |
| --- | --- | --- | --- |
|  | Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
|  | Data rate | 0.2 Mbps | 10 Mbps |
|  | Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
|  | Air Interface PDB | 10ms | 30ms |
|  | Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,30ms] (baseline) |
|  | Jitter | - Periodic: 4ms (no jitter) | - Jitter: same model as for DL |

Table 1: UL Traffic models for CG/VR/AR evaluations

|  |  | CG | VR | AR |
| --- | --- | --- | --- | --- |
| DL | Data Rate | 30Mbps @60fps | 30Mbps, 45Mbps @60fps | 30Mbps, 45Mbps @60fps |
|  | FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
|  | Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
|  | Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
|  | Air Interface PDB | 15ms | 10ms | 10ms |
|  | Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [[-4, 4]ms] |  |  |

Table 2: DL Traffic models for CG/AR/VR evaluations

2.2 Simulation assumptions

The simulation assumptions employed to evaluate system level performance for FR1 are in Table 3.

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,2,2,1,1;8,2), (dH, dV) = (0.5λ, 0.8λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | Reciprocity-based precoding |  |  |
| Scheduling Algorithm | DL: MU-MIMO with PF scheduling UL: MU-MIMO with PF scheduling |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 3: Assumptions for System-level simulation

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Table 4: Baseline scenarios from RAN1 #105-e meeting notes [2]:

# Simulation results

  - Uplink results

Uplink system capacity evaluations for the single stream and multiple-stream models are presented in this section.

UL Capacity results for single stream transmission

System capacity provides the maximum number of supported UEs out of which at least Y% of UEs are satisfied where Y=90 (baseline) is used in the simulation.

#### Indoor Hotspot

UL system capacity for Indoor hotspot (parameters defined in Table 3) is evaluated for FR1 (4 GHz) using MU-MIMO with PF scheduling. A UE is declared satisfied if more than 99% of packets are successfully transmitted within PDB values of 30ms and 10ms for AR and CG/VR applications, respectively.

Figure 1: FR1 UL AR results for Indoor Hotspot scenario

Figure 2: FR1 UL CG/VR results for Indoor Hotspot scenario

Figures 1 and 2 show UL results for system capacity for AR and CG/VR respectively in an InH deployment where a single stream is simulated. It is observed that a large number of UEs/cell (>20) can be supported when only pose/control information is sent in the UL for the CG/VR case. In the AR case, where aggregated video is sent in the UL, the capacity experiences a sharp drop as the number of UEs exceed ~10 UEs/cell. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to < 12 UEs/cell. Resource utilization (%) also follows a steeper uptake as the number of UEs/cell increases when sending aggregated video in the UL vs only pose/control information.

#### Dense Urban

Figures 3 and 4 show the deployment in a dense urban scenario (with parameters as defined in Table 3) and all other evaluation assumptions kept the same (i.e., PDB values of 30ms and 10ms for CG/VR and AR applications respectively).

Figure 3: FR1 UL AR results for Dense Urban scenario

Figure 4: FR1 UL CG/VR results for Dense Urban scenario

Observation 1: 	The uplink traffic of AR (aggregated video) with 10Mbps data rates is the bottleneck of the system capacity in FR1, compared to the uplink traffic of CG/VR (pose and control information).

Observation 2: 	Transitioning from InH to DU deployment scenario has a significant impact on the capacity as shown by a large drop in #UEs/cell that can be supported for all XR applications

UL Capacity results for multiple-stream transmission

As discussed by some companies in previous meetings, several approaches can be used to model multiple streams in the uplink. One approach involves duplicating a physical UE into multiple virtual UEs (twin 1, twin 2, etc.) whereby each virtual UE is associated with a single flow. Another evaluation approach, that we adopted in our simulations, is based on multiplexing whereby multiple data flows are be mapped onto the same PUSCH. In this approach, some form of book-keeping is required to track the transmission and delay budget for each packet in a PUSCH.

#### Indoor Hotspot

UL system capacity for Indoor hotspot is evaluated for two streams in FR1 (4 GHz) using MU-MIMO with PF scheduling, for a pose/control stream and an aggregated video stream at data rates of 10 Mbps. A UE is declared satisfied if more than 99% of packets are successfully transmitted for each stream with their respective PDB values, i.e., stream 1: [99%, 10ms] AND stream 2: [99%, 30ms].

Figure 5: FR1 UL AR results for 2 streams in Indoor hotspot scenario

Figure 5 shows the UL results for system capacity for multiple-stream AR in an InH deployment. It is observed that a smaller number of UEs/cell can be supported compared to single-stream scenarios in UL. The trend mirrors the aggregated video performance displaying a drastic fall-off as the number of UEs/cell increases. In this case, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to < 8 UEs/cell (compared to < 12 UEs/cell for single stream aggregated video transmission).

#### Dense Urban

Figure 6: FR1 UL AR results for 2 streams in Dense Urban scenario

Figure 6 shows UL results for system capacity for multiple-stream AR in a Dense Urban deployment. It is observed that a smaller number of UEs/cell can be supported compared to single-stream scenarios (e.g. in pose/control only or aggregated video only scenarios) in UL. The trend mirrors the aggregated video performance displaying a drastic fall-off as the number of UEs/cell increases. In this case, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to 0 UE/cell (compared to ~ 2 UEs/cell for single stream aggregated video transmission).

Observation 3: 	UL capacity of multi-stream model is less than that of single-stream model.

Observation 4: 	Capacity trends in multi-stream model mirrors that of single-stream aggregated video in the UL (with a lower number of supported UEs/cell), leading to the following conclusions:

- Capacity bottleneck in multi-stream model is largely due to transmission of aggregated video in the UL.

- The impact of sending pose information in multi-stream model is not entirely negligible.

  - Downlink results

Downlink system capacity evaluations for the single stream model are presented in this section. For DL capacity, the traffic model parameters indicated in Table 2 are used for both Indoor hotspot and Dense Urban deployments. The simulation results for AR/VR and CG are given below:

#### Indoor Hotspot

System capacity for indoor hotspot is evaluated for FR1 (4 GHz) based on the parameter settings given in Table-3.

Figure 7: FR1 DL AR @ 45 Mbps and 30 Mbps results for Indoor Hotspot scenario

Figure 7 shows the results for DL system capacity for AR/VR in InH with MU-MIMO where 99% of packets are assumed to have arrived within 10ms PDB. For 30Mbps, up to 5 UEs per cell are able to satisfy the per-UE KPI (at 90% crossing), whereas at 45Mbps, the number satisfied UEs drops to 3 UEs per cell.

Figure 8. FR1 DL CG @ 30 Mbps results for Indoor Hotspot scenario

Figure 8 shows results for DL system capacity for CG in InH at 30Mbps with MU-MIMO where 99% of packets are assumed to have arrived within 15ms PDB. For CG, up to 7 UEs per cell are able to satisfy the per-UE KPI (at 90% crossing).

Observation 5: 	System Capacity in DL (% of satisfied UEs) for indoor hotspot scenarios is reasonable for the given data rate and PDB requirements (up to 7 UEs @30Mbps at 15ms for CG, 5 UEs @ 30Mbps at 10ms and 3 UEs @45Mbps at 10ms)

#### Dense Urban

Similar evaluation results are provided in the following for Dense Urban scenario for AR/VR (at 30Mbps and 45Mbps) and CG (at 30Mbps).

Figure 9. FR1 DL AR @ 30 Mbps and 45 Mbps results for Dense Urban scenario

Figure 9 shows results for DL system capacity for AR/VR in DU with MU-MIMO where 99% of packets are assumed to have arrived within 10ms PDB. For 30Mbps, 3 UEs per cell are able to satisfy the per-UE KPI (at 90% crossing), whereas at 45Mbps, only 2 UEs per cell satisfy the per-UE KPI.

Figure 10. FR1 DL CG @ 30 Mbps and 8 Mbps results for Dense Urban scenario

Figure 10 shows results for DL system capacity for CG in DU at 30Mbps with MU-MIMO where 99% of packets are assumed to have arrived within 15ms PDB. For CG, only 5 UEs per cell are able to satisfy the per-UE KPI (at 90% crossing).

Observation 6: 	System Capacity in DL (% of satisfied UEs) for dense urban scenarios is significantly limited (up to 5 UEs @30Mbps at 15ms for CG, 3 UEs @ 30Mbps at 10ms and 2 UEs @45Mbps at 10ms)

Observation 7: 	For DL in both InH and DU scenarios, relaxing the PDB requirement can result in significant improvement in the capacity performance

# Conclusion

In this contribution, system level simulation results are provided for baseline scenarios for single stream UL and DL, as well as capacity results for a multiple-stream model in the UL. The following observations are made:

Observation 1: 	The uplink traffic of AR (aggregated video) with 10Mbps data rates is the bottleneck of the system capacity in FR1, compared to the uplink traffic of CG/VR (pose and control information).

Observation 2: 	Transitioning from InH to DU deployment scenario has a significant impact on the capacity as shown by a large drop in #UEs/cell that can be supported for all XR applications

Observation 3: 	UL capacity of multi-stream model is less than that of single-stream model.

Observation 4: 	Capacity trends in multi-stream model mirrors that of single-stream aggregated video in the UL (with a lower number of supported UEs/cell), leading to the following conclusions:

- Capacity bottleneck in multi-stream model is largely due to transmission of aggregated video in the UL.

- The impact of sending pose information in multi-stream model is not entirely negligible.

Observation 5: 	System Capacity in DL (% of satisfied UEs) for indoor hotspot scenarios is reasonable for the given data rate and PDB requirements (up to 7 UEs @30Mbps at 15ms for CG, 5 UEs @ 30Mbps at 10ms and 3 UEs @45Mbps at 10ms)

Observation 6: 	System Capacity in DL (% of satisfied UEs) for dense urban scenarios is significantly limited (up to 5 UEs @30Mbps at 15ms for CG, 3 UEs @ 30Mbps at 10ms and 2 UEs @45Mbps at 10ms)

Observation 7: 	For DL in both InH and DU scenarios, relaxing the PDB requirement can result in significant improvement in the capacity performance

# References

RAN1 Chairman notes, RAN1#106-e, Aug 2021

RAN1 Chairman notes, RAN1#105-e, May 2021

RAN1 Chairman notes, RAN1#104-bis-e, Apr 2021

RAN1 Chairman notes, RAN1#104-e, Jan 2021

RAN1 Chairman notes, RAN1#103-e, Nov 2020