3GPP TSG RAN WG1 Meeting #106-bis-e	R1-2109936

e-Meeting, October 11th – 19th 2021

Agenda item:	8.10.1

Source: 	Lenovo, Motorola Mobility

Title: 	Resource multiplexing in enhanced IAB systems

Document for:	Discussion and Decision

# Introduction

In this paper, we provide our views on specification of resource multiplexing enhancements for simultaneous IAB-MT/IAB-DU operations in eIAB systems. Then, in a companion paper ‎[1], we provide our views on specification of timing modes, power control, and interference management related to the enhancements.

# Discussion

## Frequency domain

Discussion on resource multiplexing enhancements have led to the following agreements on FDM ‎[2]‎[3]:

Agreement

For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA

Agreement

If an IAB node is configured with a frequency-domain H/S/NA configuration down select between the following options:

- Alt. 1 Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource

  - FFS: Whether configurations are switched with per-slot, per-resource type within a slot, or per-symbol granularity

- Alt. 2 The Rel-16 H/S/NA configuration and frequency domain configuration are jointly applied

Agreement

The minimum resource size for configuring the frequency domain granularity is a set of N RBs:

- Candidate values for N: {4, 8, 16, other values TBD}

- N is at least the # PRBs that are corresponding to the MT’s # PRBs of an RBG).

- FFS: Scaling or configuration of N based on system BW or size of IAB-MT BWP

Agreement

The semi-static configuration of H/S/NA resource type in frequency domain is provided per RB set, per D/U/F resource type within a slot.

Agreement

N is a configured number of PRBs, where the CU configures N

- N = {2, 4, 8, 16, 32, 64}

- FFS: Value(s) of N in case of multiple configured BWPs at the IAB-MT

- This agreement does not revert any existing RAN1 agreement

Agreement

For a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied

It was agreed in the previous meetings to provide separate H/S/NA configurations for the frequency domain. This choice improves backward compatibility. The two approaches of joint and separate configuration are illustrated in the schematic below.

. Joint vs. separate configuration and signalling

The separate-versus-joint matter is also applicable to the IAB node’s method for applying H/S/NA attributes of resources in the time-frequency grid, which is not necessarily tied to the choice for signalling format. Indeed, while the IAB node is configured with H/S/NA attributes in the time and frequency domains separately, it is still beneficial to apply the attributes jointly.

One main benefit for joint application of time- and frequency-domain H/S/NA attributes is flexibility for a TDM fallback mode. If the H/S/NA configurations are applied separately, that will require the IAB-CU to dedicate non-overlapping resources, such as slots, to either TDM or FDM. Then, each slot may be used for only one of the multiplexing modes. That limits resource management significantly: if the IAB node is not capable of performing FDM at a moment, it does not have the option of using FDM slots as TDM, which may result in wasting all or a part of the bandwidth on those slots. This issue is illustrated in ‎Figure 2 (a).

Separate application of time- and frequency-domain H/S/NA may result in resources wasted when FDM cannot be realized at a moment.

Furthermore, separate application is a special case of joint application, e.g., IAB-CU may choose to configure IAB nodes with time- and frequency-domain H/S/NA on same slots for joint application, or alternatively, on separate slots for separate application.

Separate application of time- and frequency-domain H/S/NA is a special case of joint application.

Joint application of time- and frequency-domain attributes will allow the IAB-CU to send configurations for overlapping resources, hence allowing a higher flexibility and improving resource efficiency in the presence of intermittent limitations for performing non-TDM multiplexing. This significant benefit comes at the small cost of defining rules to obtain T-F attributes jointly based on separate configurations.

Support joint application of time- and frequency-domain H/S/NA configurations.

Specify rules for determining H/S/NA resource types for time-frequency resources based on separate configurations in time and frequency domains.

The following are examples for rules to determine an attribute for a resource jointly based on separate T-F configurations.

Alt-1: If the resource is indicated NA in at least one of time and frequency domains, the resource is considered NA. Otherwise, if a resource is indicated soft in at least one domain, the resource is considered soft. Finally, the resource is considered hard if indicated hard in both domains.

|  | Time-domain Hard | Time-domain Soft | Time-domain NA |
| --- | --- | --- | --- |
| Freq-domain Hard | Hard | Soft | NA |
| Freq-domain Soft | Soft | Soft | NA |
| Freq-domain NA | NA | NA | NA |

Alt-2: The resource preserves its hard/NA attributes in the time domain on all PRBs/RBGs. However, if the resource is indicated soft in the time domain, it is subject to availability indication in the frequency domain.

|  | Time-domain Hard | Time-domain Soft | Time-domain NA |
| --- | --- | --- | --- |
| Freq-domain Hard | Hard | Soft or Hard | NA |
| Freq-domain Soft | Hard | Soft | NA |
| Freq-domain NA | Hard | NA | NA |

Separate and joint application of H/S/NA configurations are illustrated below. In ‎Figure 2 (a), separate configurations are applied in non-overlapping resources, e.g., slot by slot. The result is reduced flexibility in utilizing resources for scheduling. In ‎Figure 2 (b), examples of rules to apply H/S/NA configurations jointly are illustrated, which result in higher flexibility when FDM is applicable.

- . (a) Separate application of H/S/NA configurations (b) Joint application

In IAB Rel-16, it was specified to allow the IAB-DU to use a soft symbol only if

- the IAB-MT does not transmit or receive in the symbol, or

- the IAB-MT would transmit or receive in the symbol, and the transmission or reception in the symbol is not changed due to a use of the symbol by the IAB-DU, or

- the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available.

The first condition is for IAB nodes capable of performing TDM only. The second condition is applicable to enhanced IAB nodes capable of FDM and/or SDM to perform non-TDM multiplexing on a ‘best-effort’ basis. The third condition allows the IAB node to use resources for communication with a child node without a concern for multiplexing constraints.

The above specification works well for TDM between upstream and downstream communications – the IAB node receives explicit indication of availability of soft resources in a guaranteed manner, while it can still perform non-TDM multiplexing based on a best-effort approach.

That, however, is not sufficient for a guaranteed performance with enhanced duplexing. On the one hand, the best-effort approach for non-TDM mode of operation does not guarantee a performance improvement, and on the other hand, relying on the IAB node’s capability to perform non-TDM operations may change frequently and unpredictably in the presence of varying beam, power, and interference and in the presence of timing alignment constraints.

The best-effort approach is insufficient for non-TDM operation in the presence of varying beam, power, interference, and timing alignment constraints.

When non-TDM multiplexing between upstream and downstream links are intended, sending availability indication for IAB-DU resources solely based on IAB node’s non-TDM capability may result in shortage of resources for IAB-MT due to beamforming, power, interference, or timing alignment constraints at a moment.

Therefore, we propose to support availability indication of soft resources whereby the IAB node can use the soft resources only when it is able to perform non-TDM with an IAB-MT operation.

Support conditional availability indication as a balance between existing best-effort non-TDM and TDM-only availability indication.

With respect to modifications to use DCI 2-5 for the frequency-domain, the following was agreed in the previous meetings ‎[2]‎[3]:

Agreement

DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources

- FFS: If additional enhancements are necessary

Agreement

To support soft resource availability in the frequency domain, the existing DCI 2_5 format is reused according to one of the following alternatives:

- Alt. 1: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 2: Multiple DCI format 2_5 can be received indicating availability with the granularity of one or more RB set(s) for different RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 3: A single DCI format 2_5 can be received indicating availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell.

The existing format for the time domain may be extended to the frequency domain, wherein the unit of one slot is replaced by a unit of an RB set (N PRBs). Higher layer configurations may follow a similar extension (AI combinations, etc.) to reduce standardization efforts.

Furthermore, DCI 2-5 for time and frequency domains may be transmitted separately, but applied jointly, similar to the principle explained for configurations. That simplifies the process and increases flexibility, again at a small cost of applying a simple rule, e.g., a resource that is configured soft in both domains is considered ‘available’ only if indicated ‘available’ in either or both domains.

With respect to the 3 alternatives, Alt. 3 seems unnecessarily inflexible when the parent IAB node may have only partial information on what frequency resources to indicate available at an early stage, hence forcing the parent IAB node to choose between indicating a smaller number of RB sets available or delaying the DCI 2-5 transmission. Alt. 1 and Alt. 2 provide higher flexibility, and particularly Alt. 2 is sufficiently flexible without adding much complexity.

Extend DCI 2-5 for frequency-domain availability indication by allowing a unit of N PRBs in addition to the unit of 1 slot. Alt. 2 is preferable for the sake of flexibility.

Regarding a scaling rule for the value of N based on system BW or IAB-MT BWP, we consider that an unnecessary complexity that would demand standardization efforts in the limited remaining time.

No need to introduce scaling of N based on system BW or IAB-MT BWP.

## Spatial domain

Discussion on spatial-domain enhancements for simultaneous operations have led to the following agreement ‎‎[2]‎[3]:

Agreement

In case of simultaneous MT/DU operation,

- the parent node can dynamically indicate to the child node at least a set of restricted beams at the IAB-DU of the child node

- the child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node

  - FFS: Whether the specification supports all reporting combinations.

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Additional semi-static signaling

- FFS: Per-panel granularity in addition to per-beam granularity

- FFS: Relationship between child IAB-MT beam indication and parent IAB-DU beam indication

- Note: This does not preclude any enhancements for either DU or MT-based CLI measurement and reports

Agreement

Spatial domain restrictions from a parent node or recommendations from a child node is limited to a subset of time resources in which simultaneous operation is applied.

- FFS: Handling of frequency resources in case of FDM operation

- FFS: Support for implicit/explicit indication of the simultaneous operation mode

Agreement

The child node indication of recommended beams to the parent node can include both IAB-MT DL beams and/or IAB-MT UL beams.

- FFS: Indication via MAC-CE or UCI transmission

- FFS: Definition of IAB-MT DL beams and/or IAB-MT UL beams (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Whether indication of “not preferred” beams is supported

Agreement

MAC-CE signaling from a parent node is supported for indication of beams of an IAB-DU in the direction of which simultaneous operation is restricted

- FFS: Details of beam indication (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Applicability to other beams

When operating at FR2, communications of an IAB node with a single antenna panel (connected to a single RF chain) are spatially limited to one beam at a time. That means if two operations are scheduled simultaneously, the signals are constrained to be transmitted or received through the same beam.

Communications of an IAB node with multiple antenna panels (each connected to a separate RF chain) are not limited to one beam, but an enhanced multiplexing case may still put spatial constraints, such as the example shown in ‎Figure 3.

. Beam/panel indication for simultaneous MT-DU operations

In this example, the IAB node has two antenna panels that are shared for IAB-MT and IAB-DU functionalities. The IAB node is connected to a parent node and a UE that are spatially separated. As a result, an antenna panel performing a TX/RX operation with the parent node cannot be used for a simultaneous operation with the UE, and vice versa.

Suppose that the parent node schedules a PDSCH to the IAB node. A TCI state indicates which of the two panels of the IAB node are selected by the parent node. If the PDSCH takes only one of the two antenna panels, the IAB node can schedule a simultaneous PUSCH for the UE, and select one of the antenna panels for the PUSCH that is not indicated used or restricted by the TCI state indication for the PDSCH.

It can be seen that if the IAB node receives the TCI state indication in advance, it can schedule the PUSCH for the UE. However, if the TCI state indication is received in the same time slot (for example), the IAB node will not have sufficient time to decode the message in a timely manner and transmit a scheduling DCI of its own to the child node.

Therefore, proper configuration of scheduling parameters such as k0 and k2 can be key to enabling simultaneous operations.

Support per-panel granularity by allowing dynamic indication of restricting/using beams and their associated antenna panels for specific time-frequency resources.

Moreover:

- The above proposed mechanism is not only applicable to multi-panel scenarios. A single-panel IAB node may also benefit from early dynamic signalling if it can accommodate simultaneous operations by one antenna panel.

- The proposed dynamic signalling allows the parent node to indicate a beam for the upstream link, from which the IAB node can determine spatial constraints for a downstream link. This approach complements the downstream indication agreed in the past meetings. The two approaches are compared in ‎Figure 4.

. Dynamic signalling for upstream indication vs. downstream indication

Each approach has its pros and cons. Advantages of ‘upstream indications’ include simplicity and less effort required for specification, configuration, and capability signalling. Advantages of ‘downstream indications,’ on the other hand, include higher flexibility for spatial resource (panel and beam) management, interference management, and so on.

Since these approaches leads to provisions that cannot be provided by one approach alone, we find it beneficial to discuss early upstream indications in addition to the agreed indication approach.

Support early beam/panel indications for the upstream link in addition to downstream beam indications.

Finally, we consider the four multiplexing cases A, B, C, D as nomenclature for the standardization process. In practice, one or multiple cases of simultaneous operations may occur depending on the number of antenna panels at an IAB node, resource configurations, hardware and operation constraints, and so on. Therefore, we propose to support beam and panel indications for specific time-frequency resources and not specific multiplexing cases. Whether and how to apply a beam/panel indication for a specific multiplexing case will then be a matter of implementation.

Support applicability of beam/panel indications to specific time-frequency resources. No need to specify indication of simultaneous operation mode in the standard.

With respect to indicating recommended beams to the parent node, we discuss and propose in the next section to address the matter under the topic of dynamic signaling of “conditions/parameter” agreed in RAN#105-e.

## Applicability of multiplexing cases

The following was agreed in the previous meetings ‎[2]:

Agreement

The parent IAB-node is dynamically provided with conditions/parameters to facilitate adaptation between multiplexing operation modes:

- FFS: Required number of guard symbols for switching of multiplexing mode (FFS: per timing mode or per multiplexing mode) for IAB-DU

- FFS: Signalling procedure

- FFS: Required guard band for FDM

- FFS: other conditions, e.g. required timing mode, required power control parameters, and preferred TCI.

The main point of this discussion is that an IAB node’s capability to perform simultaneous operations depend on two main type of constraints:

- hardware constraints, e.g., the number of antenna panels,

- operation constraints, e.g., beam, power, interference, and timing alignment constraints.

With regards to hardware constraints, it was agreed in the previous meetings that Case A (simultaneous transmission) and Case B (simultaneous reception) are considered for both single-panel and multi-panel IAB nodes. For these two cases, multi-panel IAB nodes are expected to impose less stringent requirements on baseband timing alignment. If the goal is to avoid any overlap between a transmission on one panel and a reception on another panel, even at symbol boundaries, the matter can be handled by allocating guard symbols between DL and UL resources.

However, an IAB node with a single panel/RF frontend, shared between IAB-MT and IAB-DU, may require a perfect alignment between FDM’ed operations. Enhanced timing alignment cases being discussed under agenda item 8.10.2 can be employed to satisfy the timing alignment constraint for simultaneous transmissions (Case 6) and simultaneous receptions (Case 7). Nevertheless, such a requirement can be relaxed if multiple baseband units are available, or the baseband unit is equipped with multiple IFFT/FFT windows. Whether the IAB node supports multiple baseband processing can be specified as an IAB node capability.

Support signaling IAB node capabilities for timing alignment and multi-panel and baseband characteristics.

Next, regarding operation constraints, dynamic signalling can be introduced whereby an IAB node indicates to its parent node whether a multiplexing (simultaneous operation) case is currently possible with the current operation (beam, power, interference, etc.) constraints. The indication message may be specified as an L1/L2 control message that includes the following additional information:

- time-frequency domain resources, including a temporal parameter such as a number of slots or subframes,

- spatial constraints, e.g., beams that cause excessive interference, hence not allowing a certain multiplexing case,

- TX power range or maximum TX power offset based on the TX/RX power for a downstream operation,

- guard times, guard bands, etc.

Given hardware constraints and operation constraints at the IAB node, the parent node can then respond by applying the information for signalling of scheduling, beam indication, link adaptation, power control, timing alignment control, availability indication in time/frequency domains, and so on.

Support dynamic signalling for communicating operation constraints such as beam, power, interference, and timing alignment constraints along with information of time, frequency, and spatial resources.

Furthermore, as mentioned earlier, regarding the following agreement in RAN1#106-e ‎[3]:

Agreement

The child node indication of recommended beams to the parent node can include both IAB-MT DL beams and/or IAB-MT UL beams.

- FFS: Indication via MAC-CE or UCI transmission

- FFS: Definition of IAB-MT DL beams and/or IAB-MT UL beams (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Whether indication of “not preferred” beams is supported

This signaling may be specified as part of the multiplexing applicability signaling, i.e., when a child node signals to its parent node whether it is capable of simultaneous operation, the signaling may include recommended beams.

Support indication of recommended beams to the parent node as part of dynamic signaling of “conditions/parameter” agreed in RAN#105-e.

## Signalling details for timing alignment indication

The following conclusion was made in ‎[3]:

Conclusion

Details on the design of the indication of when Case 6 timing (and Case 7 timing, if agreed) is performed at the IAB-node are to be discussed under 8.10.1.

Hence, in this paper, a part of the discussion in the companion paper ‎[1] is replicated.

The following agreements on timing alignment from the previous meetings under 8.10.2 are related to this discussion ‎[2]‎[3]:

Agreement

For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

- FFS range, granularity, and signaling details of the offset.

Agreement

An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources.

- FFS: whether the indication should be associated with another dimensions, e.g. multiplexing cases

- FFS whether an IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the IAB-node.

Agreement

An IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the parent node.

- FFS for signalling details

Case 6 timing alignment:

With respect to indication signaling, dynamic indication to activate/deactivate Case 6 timing is needed in order to allow IAB nodes to adapt to varying conditions for realizing multiplexing cases. We propose MAC CE signaling for reliable activation/deactivation of Case 6 timing.

Support MAC CE signaling for activation/deactivation of Case 6 timing.

Case 7 timing alignment:

Similarly, the parent IAB node can use MAC CE signaling for activating/deactivating Case 7 timing. The message activating Case 7 timing can additionally carry an additional TA value as offset to the legacy TA value.

Support MAC CE signaling for activation/deactivation of Case 7 timing. The activation MAC CE message can carry an additional TA value as offset to the legacy TA value.

RRC configuration:

In addition to dynamic signalling, indicating timing alignment mode by RRC can be beneficial in some scenarios.

For example, Case 6 timing alignment should be applied when UL resources are configured for IAB-MT and DL resources are configured for IAB-DU; and conversely, Case 7 timing alignment should be applied when DL resources are configured for IAB-MT and UL resources are configured for IAB-DU.

However, in a multi-hop scenario, since timing alignment modes affect child nodes’ timing, an indication of whether Case 6 or Case 7 timing can be applied to certain time-frequency resources can be determined by RRC in order to resolve timing conflicts in consecutive hops. For example, RRC can indicate to the parent node and its child node on which resources each can activate Case 6 or Case 7 timing modes.

Support RRC configuration for Case-6 and Case-7 timing alignment in addition to dynamic signaling.

The RRC indication and dynamic signalling are, of course, linked to multiplexing capability of the IAB node as well as whether the IAB node requires enhanced timing alignment as explained later in this section.

Signalling for timing alignment indication is linked to multiplexing capability of the IAB node as well as whether the IAB node requires enhanced timing alignment.

Capability signaling:

An important outstanding issue that RAN1 is yet to discuss is whether enhanced timing alignment is needed in the first place. Particularly if the IAB node has multiple IFFT/FFT windows for OFDM processing, it does not need to perform timing alignment further than Case 1, which can improve the applicability of simultaneous operations significantly. Therefore, we propose that the IAB node’s capability for timing alignment be reported to the IAB donor and/or the parent node.

Support a capability signalling, e.g., number of IFFT/FFT windows, to indicate whether the IAB node requires timing alignment between IAB-MT and IAB-DU operations. If negative, the IAB node can transmit unaligned OFDM symbols (Case A) and receive/process unaligned OFDM symbols (Case B), which simplifies signalling and configuration significantly.

## Dual connectivity

The following was agreed in the previous meetings on DC enhancements ‎‎[2]:

Agreement

In case of intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios the following are supported:

- Reusing the Rel-16 CA TDD prioritization rules in case of UL/DL conflict when applicable

  - FFS: Whether all prioritization rules apply in case of NR-DC

  - FFS: Need of new prioritization rules in case of NR-DC

- Coordinating the IAB-MT’s TDD configurations to avoid conflicts from different parent nodes in case the child IAB-MT does not support simultaneous TX and RX on different carries

  - FFS: Coordination for scheduling conflicts involving at least DCI Format 2_0 usage (e.g. usage of flexible symbols)

- Exchanging H/S/NA configurations between parent nodes/donors

Consider the following DC scenario.

. IAB DC

In this figure, both parent IAB node 1 (PN1) and parent node 2 (PN2) can be informed of H/S/NA resource configurations for the IAB node (N) according to IAB Rel-16. However, for example, if PN1 indicates soft resources available for N through DCI 2_5 signalling, PN2 does not have access to this information.

That raises issues. One issue is that if PN2 intends to avoid interference with transmissions scheduled by N on soft resources, it does not have a means to realize whether the soft resources are already available to N.

Another issue that may be raised in intra-band DC is that during transition times, when the link between N and PN1 is not available, PN2 should take control of resource availability for N and other child nodes without knowing which soft resources are already indicated available to N.

In order to address these issues, signalling can be introduced between parent nodes, or from an IAB node to its parent node, that informs the parent node of the current status of availability indication for soft resources of a dually connected IAB node.

Support signaling from an IAB node in the DC mode to its parent node for informing the parent node of the status of availability of soft resources. Further discuss specification of mechanisms to handle availability indication collisions by two parent nodes in the DC mode.

With respect to the following agreement ‎[3]:

Agreement

Select from the following alternatives to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 6. The IAB-MT is expected to operate according to the non-flexible configuration.

Alt. 6 raises issues as the IAB-MT is instructed to neglect the communication from the parent node with flexible configuration. Alt. 1 is the preferred option here and the matter can be left to implementation.

To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios, support Alt. 1. The matter can be left to implementation.

Next, with respect to the following agreement ‎[3]:

Agreement

Select from the following alternatives to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1: The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

- FFS: Consideration of the impact of parent node’s H/S/NA when specifying the prioritization rules in case of UL/DL conflict

Similar to the previous argument, Alt. 5 raises issues with respect to communication with the SCG without notice. Alt. 1 is preferred, and the matter can be left to implementation.

To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios, support Alt. 1. The matter can be left to implementation.

Next, there have been discussions on supporting per-backhaul-link resource configurations, which led to the following conclusion in the previous meeting ‎[3]:

Conclusion

Decide in RAN1#106bis-e whether the parent IAB DUs of a DC connected IAB node can have per-backhaul-link resource configurations.

- FFS: whether the per-backhaul-link configuration is provided to the dual-connected IAB node.

One main issue with this discussion is that the proposal has not been based on technical ground at a level that justifies the significant specification impact such an approach will introduce to signaling and IAB node behavior across RAN1, RAN2, and RAN3. Furthermore, this matter is a carry-over from Rel-16 and not essential to any of the objectives in the current IAB WI.

Do not support per-backhaul-link resource configuration in Rel-17.

Finally, Case-7 timing alignment can result in a timing conflict between two uplink transmissions in the intra-band DC scenario, as timing of each uplink transmission follows TA commands from the corresponding parent node. RAN1 should investigate this matter.

RAN1 to discuss resource conflicts in the DC mode due to conflicting Case-7 timing.

# Conclusions

In this contribution, we discussed resource multiplexing enhancements for simultaneous operations on child and parent links of an IAB node and made the following observations and proposals:

Observation 1:	Separate application of time- and frequency-domain H/S/NA may result in resources wasted when FDM cannot be realized at a moment.

Observation 2:	Separate application of time- and frequency-domain H/S/NA is a special case of joint application.

Observation 3:	The best-effort approach is insufficient for non-TDM operation in the presence of varying beam, power, interference, and timing alignment constraints.

Observation 4:	When non-TDM multiplexing between upstream and downstream links are intended, sending availability indication for IAB-DU resources solely based on IAB node’s non-TDM capability may result in shortage of resources for IAB-MT due to beamforming, power, interference, or timing alignment constraints at a moment.

Proposal 1:	Support joint application of time- and frequency-domain H/S/NA configurations.

Proposal 2:	Specify rules for determining H/S/NA resource types for time-frequency resources based on separate configurations in time and frequency domains.

Proposal 3:	Support conditional availability indication as a balance between existing best-effort non-TDM and TDM-only availability indication.

Proposal 4:	Extend DCI 2-5 for frequency-domain availability indication by allowing a unit of N PRBs in addition to the unit of 1 slot. Alt. 2 is preferable for the sake of flexibility.

Proposal 5:	No need to introduce scaling of N based on system BW or IAB-MT BWP.

Proposal 6:	Support per-panel granularity by allowing dynamic indication of restricting/using beams and their associated antenna panels for specific time-frequency resources.

Proposal 7:	Support early beam/panel indications for the upstream link in addition to downstream beam indications.

Proposal 8:	Support applicability of beam/panel indications to specific time-frequency resources. No need to specify indication of simultaneous operation mode in the standard.

Proposal 9:	Support signaling IAB node capabilities for timing alignment and multi-panel and baseband characteristics.

Proposal 10:	Support dynamic signalling for communicating operation constraints such as beam, power, interference, and timing alignment constraints along with information of time, frequency, and spatial resources.

Proposal 11:	Support indication of recommended beams to the parent node as part of dynamic signaling of “conditions/parameter” agreed in RAN#105-e.

Proposal 12:	Support MAC CE signaling for activation/deactivation of Case 6 timing.

Proposal 13:	Support MAC CE signaling for activation/deactivation of Case 7 timing. The activation MAC CE message can carry an additional TA value as offset to the legacy TA value.

Proposal 14:	Support RRC configuration for Case-6 and Case-7 timing alignment in addition to dynamic signaling.

Proposal 15:	Signalling for timing alignment indication is linked to multiplexing capability of the IAB node as well as whether the IAB node requires enhanced timing alignment.

Proposal 16:	Support a capability signalling, e.g., number of IFFT/FFT windows, to indicate whether the IAB node requires timing alignment between IAB-MT and IAB-DU operations. If negative, the IAB node can transmit unaligned OFDM symbols (Case A) and receive/process unaligned OFDM symbols (Case B), which simplifies signalling and configuration significantly.

Proposal 17:	Support signaling from an IAB node in the DC mode to its parent node for informing the parent node of the status of availability of soft resources. Further discuss specification of mechanisms to handle availability indication collisions by two parent nodes in the DC mode.

Proposal 18:	To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios, support Alt. 1. The matter can be left to implementation.

Proposal 19:	To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios, support Alt. 1. The matter can be left to implementation.

Proposal 20:	Do not support per-backhaul-link resource configuration in Rel-17.

Proposal 21:	RAN1 to discuss resource conflicts in the DC mode due to conflicting Case-7 timing.

# References

- R1-2109937, “Timing, interference, and power control in enhanced IAB systems,” Lenovo, Motorola Mobility, RAN1#106-bis-e, Electronic Meeting, October 2021.

- RAN1#105-e Chairman’s Notes.

- RAN1#106-e Chairman’s Notes.

- RAN1#95, RAN1#100 Chairman’s Notes.