3GPP TSG RAN WG1 #106bis-e	R1-2109940

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.3.1.1

Source: 	Lenovo, Motorola Mobility

Title: 	HARQ-ACK feedback enhancement for IIoT/URLLC

Document for:	Discussion and Decision

# Introduction

During RAN1 #106-e meeting, RAN1 made following agreements regarding Rel-17 one-shot triggering of HARQ-ACK retransmission via a DL assignment for UE HARQ-ACK feedback enhancement for Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC):

Agreements:

Confirm the following RAN1#105-e working assumption:

For at least HARQ-ACK re-transmission:

- Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17

  - Definition of enhanced Type 3 CB:

    - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration

    - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells)

- Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17

  - Details are FFS

- Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

The DCI triggering (by a DL assignment) the one-shot HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB dynamically indicates the HARQ-ACK codebook(s) / PUCCH occasions to be re-transmitted.

- FFS details

A single DCI triggering the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB can trigger the re-transmission of HARQ-ACK information of only a single HARQ-ACK CB.

The Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB is done through an explicit triggering indication in the DCI through a DCI field.

Support PHY priority handling for the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB.

- The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the re-transmitted HARQ-ACK information.

- The indicated PHY priority in the triggering DCI is used to determine the HARQ-ACK information to be re-transmitted corresponding to the indicated PHY priority.

For Rel-17 one-shot triggering for HARQ-ACK re-transmission, the UE does not expect more than one triggering DCI for Rel-17 one-shot feedback indicating the same PUCCH slot for the re-transmission of HARQ-ACK CBs of different PUCCH slots to be re-transmitted

- Note: i.e. only a single HARQ-ACK codebook / PUCCH occasion can be re-transmitted in a PUCCH slot

In this document, we discuss further details on re-transmission mechanisms of a cancelled HARQ-ACK transmission for HARQ-ACK feedback enhancement in IIoT/URLLC.

# Re-transmission of cancelled HARQ-ACK transmission

UE may have to cancel HARQ-ACK transmission due to collision between PUCCH/PUSCH resources of a high priority index and PUCCH/PUSCH resources of a low priority index, urgent DL transmissions from gNB, and/or reception of an uplink cancellation indication. Instead of incurring the system performance loss due to dropping of cancelled HARQ-ACK transmission, it is beneficial to support mechanisms to re-transmit the cancelled HARQ-ACK transmission. In this section, three different mechanisms to enable retransmission of cancelled HARQ-ACK transmission are discussed.

## DCI based one-shot triggering of HARQ-ACK re-transmission

As for DL DCI based one-shot triggering of HARQ-ACK re-transmission, a re-transmission triggering indication field in DCI can dynamically indicate a HARQ-ACK codebook/PUCCH occasion to be re-transmitted. Timing information corresponding to the indicated HARQ-ACK codebook/PUCCH occasion can be defined in terms of a slot/sub-slot offset with respect to a DL slot where a DCI format triggering the retransmission is detected.

For example, a bit field, e.g. ‘HARQ-ACK retransmission indicator’, indicates a HARQ-ACK codebook/PUCCH occasion triggered for retransmission as follows:

-	HARQ-ACK retransmission indicator – The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer harq-ackRetransmissionList if the higher layer parameter is configured; otherwise I is the number of entries in the default table shown in Table 1.

Table 1 Default table for the HARQ-ACK retransmission indicator field

| Row index | Slot/sub-slot offset of a HARQ-ACK codebook/PUCCH occasion with respect to a DL slot where a DCI format triggering HARQ-ACK retransmission is detected (i.e. a reference DL slot) |
| --- | --- |
| 1 | Re-transmission is not triggered. |
| 2 | 1 slot earlier than the reference DL slot |
| 3 | 2 slots earlier than the reference DL slot |
| 4 | 4 slots earlier than the reference DL slot |

Proposal 1: Timing information of a HARQ-ACK codebook/PUCCH occasion triggered for retransmission is defined in terms of a slot/sub-slot offset with respect to a DL slot where a DCI format triggering the retransmission is detected.

## Implicit triggering of HARQ-ACK re-transmission

If a UE receives an implicit or explicit indication of two PUCCH resources of different slots/sub-slots (i.e. different DL-to-HARQ feedback delay values) in DCI and if the UE cannot perform HARQ-ACK transmission on an earlier HARQ-ACK transmission occasion, the UE can perform retransmission of cancelled HARQ-ACK transmission on a later HARQ-ACK transmission occasion, without receiving a separate triggering DCI format. gNB may indicate implicit triggering of HARQ-ACK retransmission to UE, when an UL transmission by the UE at the earlier HARQ-ACK transmission occasion may not be guaranteed for various reasons (e.g. dynamic TDD, UL cancellation indication, LBT failure in unlicensed spectrum).

For example, a codepoint of a PUCCH resource indicator (PRI) field in DCI can indicate one or multiple PUCCH resources. Furthermore, a codepoint of a PDSCH-to-HARQ-timing-indicator field in DCI can indicate multiple values for the number of slots (or sub-slots).

In another example, an offset value of a second PDSCH-to-HARQ-timing indication value with respect to a first PDSCH-to-HARQ-timing indication value can be dynamically indicated in DCI. The UE may assume a PUCCH resource index of a second PUCCH resource is same as a PUCCH resource index of a first PUCCH resource.

Proposal 2: Support implicit triggering of HARQ-ACK retransmission based on indication of two HARQ-ACK transmission occasions in DCI.

## Autonomous one-shot HARQ-ACK re-transmission

With dynamic TDD operations in non-paired spectrum and/or operation in unlicensed spectrum, it may be difficult to guarantee uplink regions or channel occupancy for configured SPS PDSCH HARQ-ACK transmission occasions, especially when SPS PDSCH HARQ-ACK feedback occurs very frequently. As another solution approach to SPS HARQ-ACK dropping in TDD or in unlicensed spectrum, potential enhancement of Rel-16 Type-3 HARQ-ACK codebook has been discussed.

Since Rel-17 URLLC/IIoT will support UE-initiated COT in a semi-static channel access mode, if a UE is configured with configured grant (CG) PUSCH, the UE can autonomously use a CG-PUSCH resource for HARQ-ACK retransmission with Type-3 HARQ-ACK codebook. That is, when a configured PUCCH resource for HARQ-ACK transmission is not available or when a configured PUCCH transmission for HARQ-ACK feedback fails due to listen-before-talk (LBT) operation, the UE may multiplex the corresponding HARQ-ACK information in the next earliest CG-PUSCH (i.e. where the CG-PUSCH starts earlier than the earliest PUCCH resource for the HARQ-ACK feedback, and the UE performs multiplexing even when the CG-PUSCH does not overlap with a PUCCH for HARQ-ACK). The multiplexed HARQ-ACK information is determined based on the maximum allowed time duration of delaying HARQ-ACK feedbacks. The max allowed duration for delaying may be determined based on a latency requirement of a traffic (e.g. specific to an IIoT/URLLC application). A HARQ-ACK report for all HARQ processes may be transmitted in one shot in a CG-PUSCH resource so that there is no ambiguity in terms of HARQ-ACK codebook. For HARQ-ACK codebook size reduction, the UE may further be configured via RRC to include HARQ-ACK information for a subset of HARQ processes and/or a subset of component carriers of a PUCCH group.

Proposal 3:  Support autonomous one-shot HARQ-ACK re-transmission for all or a subset of HARQ processes in a CG-PUSCH resource, where the CG-PUSCH is available in an earlier slot/sub-slot than a slot/sub-slot where the earliest available PUCCH resource for HARQ-ACK is.

# Conclusion

In summary, we propose the followings for enhancements to HARQ-ACK feedback in Rel-17 URLLC/IIoT:

Proposal 1: Timing information of a HARQ-ACK codebook/PUCCH occasion triggered for retransmission is defined in terms of a slot/sub-slot offset with respect to a DL slot where a DCI format triggering the retransmission is detected.

Proposal 2: Support implicit triggering of HARQ-ACK retransmission based on indication of two HARQ-ACK transmission occasions in DCI.

Proposal 3:  Support autonomous one-shot HARQ-ACK re-transmission for all or a subset of HARQ processes in a CG-PUSCH resource, where the CG-PUSCH is available in an earlier slot/sub-slot than a slot/sub-slot where the earliest available PUCCH resource for HARQ-ACK is.

# References

- “RAN1 Chairman’s Notes”, RAN1 106-e meeting, August 16th – 27th, 2021.