3GPP TSG RAN WG1 #106bis-e	                                       	  R1-2109941

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.1.2

Source:	Lenovo, Motorola Mobility

Title:	CSI feedback enhancements for URLLC/IIoT

Document for:	Discussion

# Introduction

This contribution expresses our views on the remaining aspects of 4-bits CQI.

The following agreement made in the last meeting:

Agreement

For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration.

- This feature is subject to UE capability

- FFS: Whether wideband CQI report can be omitted

In this contribution, we address the FFS.

# Wideband CQI report

## Whether wideband CQI report can be omitted

In our view, wideband CQI (WB-CQI) should still be included as subband CQI may not be reported for all subbands, and gNB could use WB-CQI to schedule additional subbands.

## wideband CQI definition

In the last meeting, it was pointed out that wideband CQI definition can be updated to extend the range of CQI reporting (e.g., R1-2107584). As mentioned above, the existing WB-CQI is still useful when not all subband CQIs be reported leading to less motivation for updating wideband CQI definition.

Proposal:   Existing wideband CQI report cannot be omitted if 4-bits subband CQI reported for each CSI report configuration.

# Conclusion

On the FFS of omission for wideband CQI report, we propose:

Proposal:   Existing wideband CQI report cannot be omitted if 4-bits subband CQI reported for each CSI report configuration.