3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109945

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.7.1.2

Source: 	Lenovo, Motorola Mobility

Title: 	Provision of TRS/CSI-RS for idle/inactive UEs

Document for:	Discussion and Decision

# Introduction

During RAN1 #106-e meeting, RAN1 made following agreements regarding TRS/CSI-RS occasions for idle/inactive-mode UE power saving [1]:

Agreement

Support at least one of the following alternatives:

- Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion.

- Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

Note:  The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication.

Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only.

Agreement

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where

- the time duration can be determined based on at least one from the following (to be down-selected):

  - Alt-1: configured by higher layer

  - Alt-2: a predefined/configured window

  - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s)

  - Alt-4: until when the UE receives another availability indication

  - A combination of alternatives or other alternatives is not precluded.

- the reference point can be determined as at least one from the following (to be down-selected):

  - Alt-1: start of next PO or DRX cycle

  - Alt-2: time location where UE receives the indication

    - Note: the time location is subject to application delay if agreed

  - Alt-3: start of current PO or DRX cycle where UE receive the indication

  - Alt-4: a time location which is configured by higher layer

  - A combination of alternatives or other alternatives is not precluded.

Agreement

For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as

  - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

In this document, we discuss further details (e.g. configuration signaling including QCL information, availability indication) on provision of TRS for idle/inactive UEs. This document is updated from R1-2107183.

# TRS Configuration Details

## Configuration signalling structure

In this section, we provide considerations on how to reduce a signalling overhead for configuration of TRS.

Examples A-C below show possible configuration of TRS with reduced signalling overhead. A TRS resource configuration includes one or multiple TRS resource sets. Each TRS resource set consists of 2 or 4 TRS resources, where a pair of TRS resources is in a slot. All the TRS resources in a TRS resource set have the same powerControlOffsetSS, scramblingID, and periodicity. Slot offsets for all the TRS resources are determined based on the parameter periodicityAndOffset (i.e. The first and second TRS resources occur in an indicated slot, and the third and fourth TRS resources, if configured, have a slot offset same as the indicated slot offset plus 1). Note that in Rel-15/16, TRS is configured such that all non-zero power (NZP)-CSI-RS resources have the same bandwidth and subcarrier location.

#### Example A:	TRS-ResourceCommon

The IE TRS-ResourceCommon is used to cell-specifically configure one or more TRS resource sets configured with TRS in a cell where the IE is included, which an idle or inactive UE may be configured to use for time and frequency tracking.

TRS-ResourceCommon information element

-- ASN1START

-- TAG-TRS-RESOURCESETCOMMON-START

TRS-ResourceCommon ::=             SEQUENCE {

TRS-ResourceSetList      SEQUENCE (SIZE (1..maxNrofTRS-ResourceSetsPerCell)) OF TRS-ResourceSetId

}

-- TAG-TRS-RESOURCESETCommon-STOP

-- ASN1STOP

#### Example B: TRS-ResourceSet

The IE TRS-ResourceSet includes a set of TRS resources and set-specific parameters.

TRS-ResourceSet information element

-- ASN1START

-- TAG-TRS-RESOURCESET-START

TRS-ResourceSet ::=          SEQUENCE {

TRS-ResourceSetId               	TRS-ResourceSetId,

n-NZP-CSI-RS						INTEGER {2, 4}

resourceMapping                     TRS-ResourceMapping,

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset

}

-- TAG-TRS-RESOURCESET-STOP

-- ASN1STOP

TRS-ResourceSet field descriptions

periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots (see TS 38.214, clause 5.2.2.3.1) for the first TRS resource. The second TRS resource has the same slot offset as the first TRS resource. The third and fourth TRS resources, if configured, have the slot offset same as the indicated slot offset plus 1.

powerControlOffsetSS Power offset of non-zero power TRS RE to SSS RE. Value in dB (see TS 38.214, clause 5.2.2.3.1).

resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the TRS resource.

scramblingID Scrambling ID (see TS 38.214, clause 5.2.2.3.1).

#### Example C:	TRS-ResourceMapping

The IE TRS-ResourceMapping is used to configure the resource element mapping for at least one TRS resource in time- and frequency domain.

TRS-ResourceMapping information element

-- ASN1START

-- TAG-TRS-RESOURCEMAPPING-START

TRS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           BIT STRING (SIZE (4)),

firstOFDMSymbolInTimeDomain         INTEGER (0..9),

freqBand                            CSI-FrequencyOccupation,

...

}

-- TAG-TRS-RESOURCEMAPPING-STOP

-- ASN1STOP

TRS-ResourceMapping field descriptions

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for TRS. See TS 38.211, clause 7.4.1.5.3. Value 2 is supported only when dmrs-TypeA-Position equals pos3.

freqBand Wideband or partial band TRS, (see TS 38.214, clause 5.2.2.3.1).

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211, clause 7.4.1.5.3. Bitmap for row 1 of table 7.4.1.5.3-1, i.e. the value of of row 1 in Table 7.4.1.5.3-1 is given by the bitmap , , where is the bit number of the bit in the bitmap set to one.

Additionally, gNB can configure a TRS resource by updating a subset of parameters of a reference TRS resource configuration, where the reference TRS resource configuration consists of full configuration parameters, as shown in Figure 1. For example, a TRS resource 1 is a reference TRS resource with full configuration information, and a TRS resource 3 is configured with a subset of parameters (e.g. scrambling ID). The remaining parameters that are not explicitly configured for the TRS resource 3 are determined based on the reference TRS resource configuration.

Figure 1 TRS resource configuration based on a reference TRS resource configuration

Proposal 1: Support following methods to reduce the TRS configuration signalling overhead:

- Configure the parameters powerControlOffsetSS, scramblingID, and periodicityAndOffset, respectively, with same values for all TRS resources in a TRS resource set.

- A reference TRS resource has full configuration information. For other TRS resources, a subset of parameters is updated from the full configuration information of the reference TRS resource.

## Quasi-co-location (QCL) indication for TRS occasion

Considering that all 2 or 4 TRS resources in a TRS resource set have the same single antenna port, QCL information can be indicated per TRS resource set. As shown in Example B in Subsection 2.1, each TRS resource set includes a TRS resource set identity (ID), where the TRS resource set ID can be used as QCL indication. That is, a TRS resource set ID explicitly indicates an SSB index as a QCL source of the TRS resource set.

Proposal 2: QCL information for TRS configured for idle/inactive UEs is indicated per TRS resource set.

Proposal 3: A TRS resource set ID explicitly indicates an SSB index as a QCL source of the TRS resource set.

# Availability indication of TRS

A DCI format carrying paging early indication (PEI) can also include TRS availability information. UE can determine whether to and when to wake-up to monitor paging DCI based on PEI and the TRS availability information. For example, a bitfield for indicating availability of TRS on configured TRS occasions within a particular DRX cycle can be configured in a DCI format including PEI for the particular DRX cycle. A bitwidth of the bitfield may be determined based on the number of configured TRS resource sets in a cell, where each TRS resource set is mapped to one SSB index. The first/leftmost bit of the bitfield corresponds to TRS occasion/resource set index 0, the second bit corresponds to TRS resource set index 1, and so on. Value 0 in the bitmap indicates that the corresponding TRS resource set is not available while value 1 indicates that the corresponding TRS resource set is available. If UE does not detect a DCI format including TRS availability information, the UE assumes that TRS is not present on corresponding TRS occasions.

Alternatively, a DCI format carrying paging DCI of a current DRX cycle can include TRS availability information for a following DRX cycle. There are 6 bits reserved in paging DCI, 5 bits reserved in Short message, and additional reserved bits larger than 10 bits when the short message indicator is set to ’10’ (i.e. only short message is present in the DCI), as shown in Table 1 and Table 2. These reserved bits can be used to indicate availability of TRS. This will neither increase the overhead of network nor introduce new signal/channel. In case there is no paging DCI that can be used for the indication, we can define default behavior such that “UE assumes CSI-RS/TRS is unavailable if UE doesn’t receive explicit indication (paging DCI)”. As another option, SIB can configure multiple TRS configurations, and then use paging DCI to activate or to inform the availability of a certain configured TRS.

| Field (Item) | Bits |
| --- | --- |
| Short Message Indicator | 2 |
| Short Messages | 8 |
| Frequency domain resource assignment | Variable |
| Time domain resource assignment | 4 |
| VRB-to-PRB mapping | 1 |
| Modulation and coding scheme | 5 |
| TB Scaling | 2 |
| Reserved | 6 |

Table 1. DCI format 1_0 with CRC scrambled by P-RNTI in Rel-16

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring If set to 1: stop monitoring PDCCH occasions(s) for paging in this PO. |
| 4-8 | Not used in this release of the specification, and shall be ignored by UE if received. |

Table 2. Short Message indicator in NR Rel-16

Proposal 4: For L1 based signalling for the availability indication of TRS/CSI-RS at the configured occasion(s),

- Support Alt2 (L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion).

- PEI transmitted in a current DRX cycle can indicate TRS availability/unavailability information of configured TRS occasions within the current DRX cycle.

- Paging DCI of a current DRX cycle can include TRS availability information for a following DRX cycle.

In addition to L1 signal/channel based availability indication, gNB can configure a validity time interval for a TRS configuration. The TRS configuration may be unavailable after the validity time interval expires.

Proposal 5: gNB can configure a validity time interval for a TRS configuration. Upon expiry of the validity time, UE assumes that previous TRS configuration is unavailable.

Signaling overhead related to informing the availability can be reduced, if gNB can switch between one mode transmitting TRS on all of the configured TRS occasions and another mode dynamically transmitting or not transmitting on the configured TRS occasions, based on the number of UEs camping in a cell and/or an average paging rate in a Tracking Area or a RAN paging area. If the former mode is configured, UE may assume that TRS is transmitted on configured TRS occasions over one or multiple DRX/paging cycles. If the latter is configured, UE shall further check availability of TRS on configured TRS occasions. gNB may configure the former, if a large number of UEs camp on the cell and accordingly, need to transmit TRS on most of the configured TRS occasions.

Proposal 6: Support a TRS transmission mode that UE may assume that TRS are present on all configured TRS occasions, in order to reduce DCI signalling overhead.

# Conclusion

In summary, we have the following proposals for TRS occasion(s) for idle/inactive UEs:

Proposal 1: Support following methods to reduce the TRS configuration signalling overhead:

- Configure the parameters powerControlOffsetSS, scramblingID, and periodicityAndOffset, respectively, with same values for all TRS resources in a TRS resource set.

- A reference TRS resource has full configuration information. For other TRS resources, a subset of parameters is updated from the full configuration information of the reference TRS resource.

Proposal 2: QCL information for TRS configured for idle/inactive UEs is indicated per TRS resource set.

Proposal 3: A TRS resource set ID explicitly indicates an SSB index as a QCL source of the TRS resource set.

Proposal 4: For L1 based signalling for the availability indication of TRS/CSI-RS at the configured occasion(s),

- Support Alt2 (L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion).

- PEI transmitted in a current DRX cycle can indicate TRS availability/unavailability information of configured TRS occasions within the current DRX cycle.

- Paging DCI of a current DRX cycle can include TRS availability information for a following DRX cycle.

Proposal 5: gNB can configure a validity time interval for a TRS configuration. Upon expiry of the validity time, UE assumes that previous TRS configuration is unavailable.

Proposal 6: Support a TRS transmission mode that UE may assume that TRS are present on all configured TRS occasions, in order to reduce DCI signalling overhead.

# References

- “RAN1 Chairman’s Notes”, RAN1 106-e meeting, August 16th – 27th, 2021.