3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109984

Online, 11 – 19 October, 2021

Agenda Item:	8.12.2

Source: 	LG Electronics Inc.

Title: 	Mechanisms to improve Reliability of Broadcast/Multicast service

Document for:	Discussion and decision

# 1.	Introduction

The WID on NR Multicast and Broadcast Services was revised in RAN#88e [1].

One of the objectives led by RAN1 is to specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state:

- Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

  - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

- Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

In addition, RAN1 is involved in specification of RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

- Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

  - Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

In this contribution, we discuss mechanisms to improve reliability of Broadcast/Multicast service.

# 2.	PUCCH resource configuration

RAN1 previously agreed that for ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies. Currently, PUCCH-config for unicast is configured within UL BWP configuration. However, it is not clear whether PUCCH-config for multicast can be configured per CFR or per UE’s active UL BWP associated to the CFR.

We think that if PUCCH-config for multicast is used to configure UE specific PUCCH resources for ACK/NACK based HARQ-ACKs, it seems reasonable to configure PUCCH-config for multicast within UE’s active UL BWP associated to a CFR. Meanwhile, if PUCCH-config for multicast is used to configure group common PUCCH resources for NACK-only based HARQ-ACK, it seems reasonable to configure PUCCH-config for multicast within uplink common frequency resource shared by multiple UEs.

Proposal 1: PUCCH-config for multicast can be configured per CFR or per UE’s active UL BWP associated to the CFR.

Currently, PUCCH-Config for unicast supports only ACK/NACK based HARQ-ACKs for UE’s active BWP and cannot support NACK only based HARQ-ACK. Thus, assuming that group common PUCCH resources are used for NACK only based HARQ-ACK, if NACK only based HARQ-ACK is supported, it seems reasonable to configure NACK only based HARQ-ACK in PUCCH-config for mulicast within UL CFR, not in PUCCH-config for unicast for UE’s active BWP.

Proposal 2: Group common PUCCH resources for NACK only based HARQ-ACK are configured within UL CFR configured within UE’s active UL BWP.

- Group common PUCCH resources are only configured by PUCCH-config for multicast.

- UE specific PUCCH resources are configured on UE’s active BWP, as currently specified.

RAN1 agreed to support ACK/NACK based HARQ-ACK feedback for multicast for RRC_CONNECTED UEs receiving multicast. However, it is not clear how to individually allocate UE specific ACK/NACK based HARQ-ACK feedback for group common PDCCH/PDSCH. For example, considering PUCCH resource indicator and PDSCH-to-HARQ_feedback timing indicator in DCI Format 1_0 and 1_1, group common DCI may require multiple indicators for different UEs which seems not so feasible in case that a large number of UEs is receiving group common PDCCH/PDSCH.

We think that group common DCI can simply indicate a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK. But, different UEs in a same group could be configured to differently interpret the single indicators in DCI with individual dictionaries given by PUCCH-config. If different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config, different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI. In this way, each UE could be provided individual PUCCH resource even with the single indicators in group common DCI.

Proposal 3: For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

Proposal 4: For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured).

- Different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

When it comes to PTP, RAN1 previously agreed that for RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, retransmission(s) using PTP transmission is supported. The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB. The DCI is UE specific for PTP retransmission. The PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI have no ambiguity.

However, since RAN1 agreed that for ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies, we should clarify whether PUCCH-Config for multicast can be used to interpret the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI.

In our view, PTP retransmission and unicast transmission could have commonality in terms of allocation of PUCCH resources. Thus, we propose that for PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

Proposal 5: For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

In addition, we also think that HARQ-ACK to PTP retransmission can be treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook. For example, for separate codebooks for unicast and multicast are constructed, PTP retransmission could be treated as unicast.

Proposal 6: HARQ-ACK to PTP retransmission is treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook.

Meanwhile, it is unclear whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast. In our view, considering that PUCCH-Config for unicast can apply multicast HARQ-ACK, it seems better not to configure HP HARQ-ACK for multicast, in case HP HARQ-ACK is not configured for unicast.

Proposal 7: Clarify whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast.

# 3.	NACK only based HARQ-ACK

gNB can currently configure spatial relation for a PUCCH resource. Similarly, NACK only based PUCCH is expected to be related to a reference signal. For example, different PRBs and/or sequences for PUCCH can be related to different DL RS. If UEs in a group are receiving group common PDSCH based on different DL RS, UEs could also transmit PUCCH based on different DL RS. Thus, gNB may perform group common PDSCH retransmissions only based on a few RS considering spatial relation of PUCCH.

Proposal 8: Different group common PUCCH resources can be related to different DL RS e.g. in terms of symbol/slot and/or PRB and/or sequence for PUCCH.

If NACK only based HARQ-ACK is configured in PUCCH-config for multicast, RAN1 could further discuss whether ACK/NACK based HARQ-ACK can be also configured in the PUCCH config for multicast. gNB may configure a UE only with one of NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK in a PUCCH-config for multicast. Then, this UE may not be configured with PUCCH resources for group common PDSCHs in PUCCH-config for unicast. Or, even if NACK only based HARQ-ACK is configured in PUCCH-config for multicast, this UE may be also configured with PUCCH resources for ACK/NACK based HARQ-ACK to group common PDSCHs in PUCCH-config for unicast.

Alternatively, gNB can configure a UE with both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK in a PUCCH-config for multicast. If so, RAN1 should further discuss whether NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are can be jointly configured within a same PUCCH resource set or separately configured for different PUCCH resource sets.

Proposal 9: Select one of the following options to configure HARQ-ACK feedback scheme in PUCCH-config for multicast.

  - Option 1: Only one of NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in a PUCCH-config for multicast.

    - FFS: Whether/how to support ACK/NACK based HARQ-ACK if NACK only based HARQ-ACK is configured in PUCCH-config for multicast

    - FFS: Whether to support more than one PUCCH-config for multicast

  - Option 2: Both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in PUCCH-config for multicast.

    - FFS: Whether NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are can be jointly configured within a same PUCCH resource set or separately configured for different PUCCH resource sets.

RAN1 agreed to support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast. In PUCCH-config, PUCCH format 0 and format 1 are only allowed for a resource in a first PUCCH resource set. PUCCH format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. Thus, considering the current specification, if NACK only based HARQ-ACK is configured in PUCCH-config, it seems natural that only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

Proposal 10: Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

We think that both NACK-only based feedback to multicast PDSCH and ACK/NACK based feedback would be available for transmission in the same PUCCH slot. The ACK/NACK based feedback could correspond to either unicast HARQ-ACK or multicast HARQ-ACK to other multicast PDSCH. If such case occurs, NACK only HARQ-ACK can be changed to ACK/NACK based HARQ-ACK to be multiplexed with other ACK/NACK based HARQ-ACK for a same priority. This seems the simplest way except dropping one of them. Thus, we propose that when different PUCCHs for NACK only HARQ-ACK and ACK/NACK based HARQ-ACK are overlapped, NACK only HARQ-ACK is changed to ACK/NACK based HARQ-ACK to be multiplexed with other ACK/NACK based HARQ-ACK.

Proposal 11: When different PUCCHs for NACK only HARQ-ACK and ACK/NACK based HARQ-ACK are overlapped, NACK only HARQ-ACK is changed to ACK/NACK based HARQ-ACK to be multiplexed with other ACK/NACK based HARQ-ACK.

RAN1 previously discussed the following proposal from FL in RAN1#105e.

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives: - Alt1: Support multiplexing the HARQ-ACK bits. - Alt2: Support UE transmitting more than one PUCCHs in the same slot-based PUCCH slot. - Alt3: NACK-only bundling - Alt4: support sub-slot based PUCCH for NACK-only based feedback. - other alternatives are not precluded.

We think that when more than one NACK-only based feedback are available for transmission in the same PUCCH slot, the NACK-only based feedback can be changed to ACK/NACK based feedback, because multiplexing of ACK/NACK based feedback has been well developed since Rel-15. Benefits of the other alternatives are not clear, either. Especially, Alt2 and Alt4 may require enhanced UE capability which may be not what all UEs receiving multicast would support. Thus, we propose that when different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

Proposal 12: When different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

For unicast, UE procedure for multiplexing HARQ-ACK and SR with PUCCH format 0 and/or 1 has been specified in 38.213. RAN1 previously agreed that for ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI. Thus, we assume that ACK/NACK based HARQ-ACK for multicast can be multiplexed with SR for PUCCH format 0 and/or 1 as specified for unicast.

However, it is not clear how to multiplex NACK-only based HARQ-ACK and SR for PUCCH format 0 and/or 1. We propose to discuss multiplexing NACK-only based HARQ-ACK and SR for PUCCH format 0 and/or 1.

Proposal 13: Discuss UE procedure for multiplexing NACK-only based HARQ-ACK and SR for PUCCH Format 0 and/or 1

# 4.	Group common SPS

NOTE: Some proposals in this section are also shown in LG’s other contribution to AI 8.12.1.

Since multiple UEs receive group common SPS, it seems essential for gNB to ensure whether a particular UE in the group is receiving group common SPS activation/(re-)transmission/release. We think that HARQ-ACK for group common SPS will be helpful for gNB to ensure that all UEs in the group are following group common SPS. Especially we propose that all UEs in the group individually send UE specific confirmation to DCI indicating SPS activation or deactivation by using HARQ-ACK on UE specific PUCCH resource. The UE specific PUCCH resource can be allocated by DCI indicating SPS activation or deactivation.

Proposal 14: For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

- UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

In addition, reliability of a DCI indicating activation or release seems important for group common SPS. For example, if some of UEs in a same group misses an activation DCI, it seems not easy for the UEs to follow ongoing group common SPS transmissions. Thus, we propose to support repetition of a same DCI on multiple CORESETs with same or different TCI states for group common SPS.

Proposal 15: For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

When gNB activates group common SPS, a UE in the group may miss the activation DCI. If gNB did not receive confirmation from the UE or gNB did not receive HARQ-ACK to SPS PDSCH of a TB, gNB could schedule PTP transmission of the TB. In this case, initial HARQ transmission of the TB should be supported based on PTP transmission.

Proposal 16: For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

When gNB releases group common SPS, a UE in the group may miss the release DCI. If gNB did not receive confirmation from the UE, it is not clear how UE could finally release the group common SPS. Considering such erroneous case, it would be beneficial to introduce UE autonomous SPS release. It seems possible to use a UE’s individual timer based SPS release. However, considering misalignment among different UEs in the group, we should make sure that all UEs autonomously release group common SPS release at a same slot which could be pre-determined e.g. by RRC and/or DCI.

Proposal 17: After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

- The pre-determined time is determined by RRC and/or DCI.

For HARQ-ACK to group common SPS PDSCH, we could consider either group common PUCCH resource or UE specific PUCCH resource. Using group common PUCCH resource for group common SPS PDSCH seems beneficial because all UEs in the group send HARQ-ACK at the same time. Since there is no DCI for SPS PDSCH, group common PUCCH resources used for NACK only based HARQ-ACK can be semi-statically configured per SPS configuration for SPS PDSCH transmissions.

Proposal 18: For group common SPS, group common PUCCH resources used for NACK only based HARQ-ACK is semi-statically configured per SPS configuration for SPS PDSCH transmissions.

For SPS retransmission, DCI of which CRC is scrambled by G-CS-RNTI can allocate PRI/K1. Thus, it seems likely that for group common SPS retransmission, a PUCCH resource is allocated by DCI of which CRC is scrambled by G-CS-RNTI.

Proposal 19: For group common SPS retransmission, PUCCH resource is allocated by DCI of which CRC is scrambled by G-CS-RNTI.

SPS retransmission is dynamically scheduled. We think that either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK can be used for SPS PDSCH retransmission.

Proposal 20: Either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK is used for SPS PDSCH retransmission.

In RAN1#104bis-e, RAN1 agreed that for multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

In our view, if both unicast and multicast are scheduled by DCIs respectively, UE can determine a PUCCH resource based on the PRI of the last DCI scheduling either multicast or unicast, considering gNB can fully control multicast as well as unicast for an individual UE.

In addition, we could further discuss how to determine the PUCCH resource in case of dynamically scheduled multicast and dynamically scheduled multicast. We think that it is likely to determine the PUCCH resource based on the PRI indicated in the last DCI for multicast.

Proposal 21: For multiplexing HARQ-ACKs for dynamically scheduled multicast and dynamically scheduled unicast or for dynamically scheduled multicast PDSCHs only, UE determines a PUCCH resource based on the PRI of the last DCI scheduling either multicast or unicast.

In case of SPS PDSCH, PDSCH is not scheduled by DCI. Thus, if UE receives unicast/multicast SPS and dynamically scheduled unicast/multicast, it is not clear how we can determine PUCCH resource. We think that UE could determine a PUCCH resource based on unicast or the PRI of the last DCI. Alternatively, UE could determine a PUCCH resource based on a higher priority based on priority index.

Proposal 22: For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, for multicast SPS and dynamically scheduled unicast, or for dynamically scheduled multicast and multicast SPS, UE determines a PUCCH resource based on one of the following options:

- Option 1: The PRI of the last DCI with dynamic scheduling.

- Option 2: SPS (e.g. if SPS is unicast)

- Option 3: priority index

Furthermore, in case of unicast SPS PDSCH and multicast SPS PDSCH, neither unicast nor multicast are scheduled by DCI. In this case, it is not clear whether we can determine PUCCH resource based on the last DCI. We think that UE could determine a PUCCH resource based on SPS configuration index, priority index or unicast SPS configuration.

Proposal 23: For multiplexing HARQ-ACKs for multicast SPS and unicast SPS or for multicast SPS PDSCHs, UE determines a PUCCH resource based on one of the following options:

- Option 1: the lowest SPS configuration index

- Option 2: the highest priority of SPS between SPS configurations

- Option 3: unicast SPS configuration (or multicast SPS configuration)

RAN1 previously agreed that for slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured with pdsch-AggregationFactor, or UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table. Meanwhile, RAN1 also agreed to support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs. Thus, RAN1 can further clarify whether slot-level repetition can be also applied to group common SPS configuration. Considering that multiple UEs would receive group common SPS transmissions, it seems beneficial to provide those UEs in a group with multiple opportunities that the UEs can receive. In our view, both options agreed for group common PDSCH repetition can be also supported for group common SPS.

Proposal 24: For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

For group common SPS, it is unclear how group common PDSCH transmission can address different TCI states for different UEs in a same group, because group common SPS is not UE specific and different UEs may need to be configured with different TCI states. If repetition of group common SPS, we could associate different repetition of a same TB with different TCI states to allow different UEs to properly receive group common SPS. Or, gNB may provide separate SPS configurations to different UEs based on different TCI states even for the same multicast group of UEs. We propose to further discuss how to support different TCI states for group common SPS.

Proposal 25: Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

# 5.	Type-1 HARQ-ACK codebook

RAN1 made some agreements on Type-1 HARQ-ACK codebook as follows:

RAN1#105e Agreement: For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support - Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP). - FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4). - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed. RAN1#106e Agreement: For a UE configured with Type-1 HARQ-ACK codebook, - If UE is not configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for TDM-ed unicast and multicast. - If UE is configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for FDM-ed unicast and multicast.

RAN1 agreed that for Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. We think that when multiple serving cells are configured for a UE, it seems possible that only some of the serving cells are configured with CFRs. Thus, RAN1 could further clarify that ‘all serving cells for multicast’ in the previous agreement mean all serving cells where a CFR is configured and an interested service can be scheduled.

Proposal 26: For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells where a CFR is configured for an interested service for multicast

# 6.	Conclusion

In conclusion, we propose to discuss the following proposals for mechanisms to improve reliability of multicast services.

PUCCH resource configuration

Proposal 1: PUCCH-config for multicast can be configured per CFR or per UE’s active UL BWP associated to the CFR.

Proposal 2: Group common PUCCH resources for NACK only based HARQ-ACK are configured within UL CFR configured within UE’s active UL BWP.

- Group common PUCCH resources are only configured by PUCCH-config for multicast.

- UE specific PUCCH resources are configured on UE’s active BWP, as currently specified.

Proposal 3: For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

Proposal 4: For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured).

- Different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

Proposal 5: For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

Proposal 6: HARQ-ACK to PTP retransmission is treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook.

Proposal 7: Clarify whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast.

NACK only based HARQ-ACK

Proposal 8: Different group common PUCCH resources can be related to different DL RS e.g. in terms of symbol/slot and/or PRB and/or sequence for PUCCH.

Proposal 9: Select one of the following options to configure HARQ-ACK feedback scheme in PUCCH-config for multicast.

  - Option 1: Only one of NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in a PUCCH-config for multicast.

    - FFS: Whether/how to support ACK/NACK based HARQ-ACK if NACK only based HARQ-ACK is configured in PUCCH-config for multicast

    - FFS: Whether to support more than one PUCCH-config for multicast

  - Option 2: Both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in PUCCH-config for multicast.

    - FFS: Whether NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are can be jointly configured within a same PUCCH resource set or separately configured for different PUCCH resource sets.

- Proposal 10: Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

- Proposal 11: When different PUCCHs for NACK only HARQ-ACK and ACK/NACK based HARQ-ACK are overlapped, NACK only HARQ-ACK is changed to ACK/NACK based HARQ-ACK to be multiplexed with other ACK/NACK based HARQ-ACK.

- Proposal 12: When different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

- Proposal 13: Discuss UE procedure for multiplexing NACK-only based HARQ-ACK and SR for PUCCH Format 0 and/or 1

Group common SPS

Proposal 14: For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

- UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

Proposal 15: For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

Proposal 16: For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

Proposal 17: After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

- The pre-determined time is determined by RRC and/or DCI.

Proposal 18: For group common SPS, group common PUCCH resources used for NACK only based HARQ-ACK is semi-statically configured per SPS configuration for SPS PDSCH transmissions.

Proposal 19: For group common SPS retransmission, PUCCH resource is allocated by DCI of which CRC is scrambled by G-CS-RNTI.

Proposal 20: Either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK is used for SPS PDSCH retransmission.

Proposal 21: For multiplexing HARQ-ACKs for dynamically scheduled multicast and dynamically scheduled unicast or for dynamically scheduled multicast PDSCHs only, UE determines a PUCCH resource based on the PRI of the last DCI scheduling either multicast or unicast.

Proposal 22: For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, for multicast SPS and dynamically scheduled unicast, or for dynamically scheduled multicast and multicast SPS, UE determines a PUCCH resource based on one of the following options:

- Option 1: The PRI of the last DCI with dynamic scheduling.

- Option 2: SPS (e.g. if SPS is unicast)

- Option 3: priority index

Proposal 23: For multiplexing HARQ-ACKs for multicast SPS and unicast SPS or for multicast SPS PDSCHs, UE determines a PUCCH resource based on one of the following options:

- Option 1: the lowest SPS configuration index

- Option 2: the highest priority of SPS between SPS configurations

- Option 3: unicast SPS configuration (or multicast SPS configuration)

Proposal 24: For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

Proposal 25: Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

Proposal 26: For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells where a CFR is configured for an interested service for multicast

# Reference

[1] RP-201038	WID revision: NR Multicast and Broadcast Services,	Huawei, HiSilicon