3GPP TSG RAN WG1 #106bis-e	R1-2110003

e-Meeting, October 11th – 19th, 2021

Source:	Sharp

Title:	PUCCH coverage enhancement

Agenda Item:	8.8.2

Document for:	Discussion and Decision

Background

In this paper, we provide views on PUCCH coverage enhancement.

Dynamic PUCCH repetition factor indication

At RAN1#106-e meeting, the following agreements were made in AI 8.8.2 for dynamic PUCCH repetition factor indication [1].

Confirm the following working assumption Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication. - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor. - FFS: RRC signaling enhancement details Agreement - for a PUCCH resource, if both a new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication and the Rel-15/16 nrofSlots are configured, the new repetition parameter overrides nrofSlots. Agreement - In Rel-17, reuse the Rel-16 PUCCH repetition factors 2, 4, 8. - Do not support PUCCH repetition factor larger than 8 In Rel-17. Agreement Dynamic PUCCH repetition factor indication for SR or P/SP-CSI on PUCCH is not supported in Rel-17.

Regarding RRC parameters discussed at RAN1#post 106-e meeting, a new repetition parameter (e.g., PUCCH-nrofSlots-r17) should be configured per e.g., PUCCH-ResourceTBD-r17 as an additional information of PUCCH-Resource.

Proposal 1: A new repetition parameter (e.g., PUCCH-nrofSlots-r17) should be configured per e.g., PUCCH-ResourceTBD-r17 as an additional information of PUCCH-Resource.

In a case that both new repetition parameter and Rel-15/16 repetition parameter (i.e., nrofSlots) are configured, it was agreed that the new repetition parameter overrides nrofSlots. However, we think there are two options in a case that the new repetition parameter is not configured and nrofSlots is configured.

- Option 1: When the new repetition parameter is supported by the UE and not configured on the indicated PUCCH resource, the number of PUCCH repetitions (i.e., NPUCCHrepeat) is 1.

- Option 2: When the new repetition parameter is supported by the UE and not configured on the indicated PUCCH resource, NPUCCHrepeat is equal to nrofSlots or 1 (if nrofSlots is not provided).

In our view, Option 1 is more complex than Option 2. For example, Option 1 has to consider whether the new repetition parameter is supported because NPUCCHrepeat is nrofSlots or 1 in Rel-15/16. Furthermore, for Option 1, the UE has to be able to identify whether the new repetition parameter is not provided or not supported. Besides, in Rel-16 PUSCH repetition, if numerOfRepetitions is not provided, the number of repetitions K is equal to pusch-AggregationFactor or 1. Likewise, we think Option 2 is better.

Proposal 2: the number of repetitions NPUCCHrepeat for a PUCCH transmission corresponding to a PUCCH resource and a PUCCH format should be determined as:

    - if the new repetition parameter is configured on the PUCCH resource, NPUCCHrepeat is equal to the new repetition parameter;

    - elseif nrofSlots is configured on the PUCCH format, NPUCCHrepeat is equal to nrofSlots;

    - otherwise, NPUCCHrepeat=1.

If value set of new repetition parameter is {2, 4, 8}, nrofSlots should not be configured on PUCCH formats. This is because NPUCCHrepeat cannot be 1 due to nrofSlots with 2, 4 or 8 even if the new repetition parameter is not configured on a PUCCH resource.

DMRS bundling across PUCCH repetitions

  - Time domain window configuration

At RAN1#106-e meeting, the following agreement was made in AI 8.8.2.

Agreement - For DMRS bundling for PUCCH repetitions, RAN1 at least prioritize use cases 3 and 4a in R1-2104119.

In RAN1#106-e meeting, the following issue was remained:

- Enabling/disabling of DMRS bundling for PUCCH

  - Option 1: per UL BWP

  - Option 2: per PUCCH format

  - Option 3: per PUCCH resource

First, we don’t support Option 3 because the number of PUCCH resources for a given enabling/disabling is reduced and high flexibility is unnecessary for enabling/disabling.

Proposal 3: Enabling/disabling of DMRS bundling for PUCCH should be not configured “per PUCCH resource”.

In our view, the difference between Option 1 and Option 2 is whether finer granularity for enabling/disabling is needed or not. Option 2 has finer granularity than Option 1 while Option 1 has no critical problem and is simpler. For this reason, we are fine to support enabling/disabling of DMRS bundling for PUCCH is configured per UL BWP or PUCCH format.

Proposal 4: Enabling/disabling of DMRS bundling for PUCCH should be configured per UL BWP or PUCCH format.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: A new repetition parameter (e.g., PUCCH-nrofSlots-r17) should be configured per e.g., PUCCH-ResourceTBD-r17 as an additional information of PUCCH-Resource.

Proposal 2: the number of repetitions NPUCCHrepeat for a PUCCH transmission corresponding to a PUCCH resource and a PUCCH format should be determined as:

    - if the new repetition parameter is configured on the PUCCH resource, NPUCCHrepeat is equal to the new repetition parameter;

    - elseif nrofSlots is configured on the PUCCH format, NPUCCHrepeat is equal to nrofSlots;

    - otherwise, NPUCCHrepeat=1.

Proposal 3: Enabling/disabling of DMRS bundling for PUCCH should be not configured “per PUCCH resource”.

Proposal 4: Enabling/disabling of DMRS bundling for PUCCH should be configured per UL BWP or PUCCH format.

References

- RAN1 chairman’s note, RAN1#106 e-meeting, August 2021