3GPP TSG- RAN WG1 Meeting #106b-e                                                 R1-2110015

# e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.1.2.2

Source:	Apple Inc.

Title:	On Inter-Cell Multi-TRP Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #106 meeting, the following agreement on inter-cell multi-TRP operation have been achieved.

Agreement Introduce a new RRC indicator/signalling (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with, where the new indicator/signaling is not the exact PCI value - Detailed signalling design is up to RAN2 Agreement Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS : Which values to support other than 1. - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States Agreement - For inter-cell mTRP , one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex - FFS : The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP Agreement For a CSI-RS QCLed with a neighboring cell SSB, the CSI-RS EPRE is calculated based on powerControlOffsetSS and the SSB transmission power in the neighboring cell information.

In this contribution, we provide some discussion on inter-cell multi-TRP enhancement.

# Discussion

## Rate matching

In RAN1 #104, it was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI. Thus, the remaining issue is whether the cross-cell rate matching is needed. This should depend on whether UE uses advanced receiver for interference cancellation. Since the non-serving cell SSB may be from different beams and some may create strong interference to the UE. So, it is better to introduce a UE capability on whether the cross-cell rate matching is needed.

Proposal 1: Support to introduce a UE capability to report the following information

- Whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

## Number of PCIs

In RAN1 #106, the following is agreed on number of PCIs.

Agreement Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS : Which values to support other than 1. - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States

One open issue is whether to report a single value of X or more than 1 independent value of X with regard to different assumptions on SSB time domain position and periodicities. There are 3 cases in total:

- Case 1: SSBs from different cells are overlapped in time domain

- Case 2: SSBs from different cells are non-overlapped and multiplexed in the same slot

- Case 3: SSBs from different cells are non-overlapped and multiplexed in different slots

Case 1/2/3 could lead to different UE implementation effort. Case 1 may require UE to support simultaneous reception, case 2 may require UE to support fast beam sweeping since the SSBs may be multiplexed in a back-to-back manner, and case 3 could be much easier from SSB reception perspective without Rx beam sweeping and simultaneous reception related issues. Thus for different cases, the value of X can be different.

Proposal 2: Support to report 3 independent X values for the following cases:

- Case 1: SSBs from different cells are overlapped in time domain

- Case 2: SSBs from different cells are non-overlapped and multiplexed in the same slot

- Case 3: SSBs from different cells are non-overlapped and multiplexed in different slots

## Simultaneous reception of SSB + other signal

For SSB + PDSCH and SSB + CSI-RS for BM, the following restriction to ensure the QCL is defined in38.214.

If the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that the DM-RS and SS/PBCH block are quasi co-located with 'typeD', if 'typeD' is applicable.

If the UE is configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with 'QCL-TypeD' if 'QCL-TypeD' is applicable.

The QCL rule above should be applicable for signals associated with the same PCI. Further, it can also be extended to support SSB + other signals case.

Proposal 3: If SSB collides with DL signals associated with the same PCI, gNB should ensure the DL signals and SSB are QCLed with QCL-TypeD.

## Serving cell change problem

For inter-beam BM, it is required that the serving cell should not be changed, thus the non-UE dedicated signal should be QCLed indirectly with SSB from serving cell. For inter-cell mTRP, there is similar issues that the non-UE dedicated signal should not be QCLed with SSB from TRP with a different PCI.

Proposal 4: For inter-cell mTRP, the non-UE dedicated signal should be QCLed with SSB from serving cell indirectly to make sure there is no serving cell change.

# Conclusion

In this contribution, we discussed inter-cell multi-TRP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: Support to introduce a UE capability to report the following information

- Whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Proposal 2: Support to report 3 independent X values for the following cases:

- Case 1: SSBs from different cells are overlapped in time domain

- Case 2: SSBs from different cells are non-overlapped and multiplexed in the same slot

- Case 3: SSBs from different cells are non-overlapped and multiplexed in different slots

Proposal 3: If SSB collides with DL signals associated with the same PCI, gNB should ensure the DL signals and SSB are QCLed with QCL-TypeD.

Proposal 4: For inter-cell mTRP, the non-UE dedicated signal should be QCLed with SSB from serving cell indirectly to make sure there is no serving cell change.