3GPP TSG-RAN WG1 #106 bis-e		R1-2110024

e-Meeting, October 11th – 19th, 2021

Source: 	Apple Inc.

Title:                     Beam Management for New SCSs

Agenda item:	8.2.4

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 #104bis e-meeting, the following was agreed for beam management for new SCSs so as to extend current NR operation to 71GHz [1]:

Agreement: Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH Agreement: For timeDurationForQCL, beamSwitchTiming and beamReportTiming, - Following candidate values of FR2 are reused for 120 kHz: - timeDurationForQCL: 14 and 28 symbols - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols - beamReportTiming: 14, 28 and 56 symbols - For 480 kHz - Support at least the candidate values for 120 kHz scaled by 4x - FFS: Support for additional candidate value(s) - For 960 kHz - Support at least the candidate values for 120 kHz scaled by 8x - FFS: Support for additional candidate values(s) - FFS: UE capability signaling details - Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16). Agreement: For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI - FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

The following was further agreed in RAN1 106-e meeting [2]:

Agreement: For maxNumberRxTxBeamSwitchDL, - Support at least 2 and 4 as candidate values for 480 kHz - FFS: 7 - Support at least 2 as a candidate value for 960 kHz - FFS: Support for additional candidate value(s) including 4 Agreement: For maxNumberRxTxBeamSwitchDL, - For 480 kHz, support 7 as a candidate value for 480 kHz in addition to the agreed candidate values 2 and 4 - For 960 kHz, support one of the following alternatives - Alt-1: Support 1, 4 and [7] as candidate values for 960 kHz in addition to the agreed candidate values 2 - Alt-2: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2 - No additional candidate values are supported Agreement: For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming, - Support one of the following alternatives - Alt-1: No additional candidate values are supported for 120 kHz, 480 kHz and 960 kHz - Alt-2: 28 and 56 symbols are supported as additional candidate values for 480 kHz and 960 kHz, respectively - For UE capability signaling, UE reports one value of the candidate values in OFDM symbols per each SCS Agreement: - For additional beam switching time delay d of 120 kHz, support one of the following alternatives - Alt-1: 14 symbols - Alt-2: 28 symbols - FFS: value for 480 kHz Agreement: For the threshold values 48 or 48+ mentioned in Clauses 5.2.1.5.1 and 5.2.1.5.1a of 38.214, scale 48 to 4*48 for 480 kHz and 8*48 for 960 kHz. Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states - FFS: Details of multiple TCI state association with multiple PDSCHs Agreement: For the single TRP case, for multi-PDSCHs scheduled by a single DCI with a single DCI field ‘Transmission Configuration Indication’ that indicates a single TCI state (if the DCI field is present), - Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL - Case 1-1: tci-PresentInDCI enabled - Single QCL assumption based on the indicated codepoint of the single DCI field ‘Transmission Configuration Indication’ is applied for all scheduled PDSCHs - Case 1-2: tci-PresentInDCI not present - Single QCL assumption of the single scheduling DCI scheduled multi-PDSCHs is applied for all scheduled PDSCHs - Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - Down select one of the following alternatives - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs - FFS: Details of single QCL assumption - Alt 2: multiple QCL assumptions are applied - FFS: Details of multiple QCL assumptions - FFS: When some of PDSCHs are collided with semi-static UL symbols and then skipped - FFS: The multi-TRP case

In this contribution, we discuss the remaining issues of initial access channels, focusing on the perspectives below:

- Leftover timing associated with Beam Management.

- Potential Enhancements for BM operation.

# 2. Discussion

## 2.1 Timing associated with Beam Management

Table 1 summarizes the timing parameters related to BM and the value ranges based on the latest agreements made in RAN1 106 e-meeting.

Table 1: Rel-15/16 timing related to beam management

| Index | Parameter/IE | Definition | Value ranges for FR2-1 | New Values | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | timeDurationForQCL | minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing. | 120kHz: {14, 28} | 480kHz {56, 112}, FFS: 28 960kHz {112, 224}, FFS: 56 | Per SCS Per FS |
| 2 | beamSwitchTiming | The minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission | 120kHz: {sym14, sym28, sym48, sym224, sym336} | 480kHz {56, 112,192, 896,1344}, FFS: 28 960kHz {112,192, 384,1792,2688} FFS: 56 | Per SCS Per FS |
| 3 | Additional beam switching time delay ‘d’ | Additional delay for beam switching when different SCSs is used for PDCCH and A-CSI-RS. | 60kHz: {sym14} | 120kHz: FFS 14 vs. 28 480kHz: FFS | Per SCS Per FS |
| 4 | beamReportTiming | The number of OFDM symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report. | 120kHz: {sym14, sym28, sym56} | 480kHz {56, 112, 224} FFS: 28 960kHz {112, 224, 448} FFS: 56 | Per SCS Per band |
| 5 | maxNumberRxTxBeamSwitchDL | The number of Tx and Rx beam changes UE can perform on this band within a slot. | 120kHz: {n4, n7, n14} | 480kHz {2,4,7} 960kHz {2,4}, FFS: {1},{7} | Per SCS Per band |

In RAN1 104bis e-meeting, absolute timing defined for 120kHz in FR2-1 was agreed to be reused for 120kHz, 480kHz and 960kHz SCS in different cases, which include value of timeDurationForQCL, beamSwitchTiming and beamReportTiming. The value for additional beam switching time delay ‘d’ with 480/960 kHz SCS and the value of maxNumberRxTxBeamSwitchDL for 960kHz SCS remains FFS.

Similar as parameter of ‘timeDurationForQCL’ and ‘beamSwitchTiming’, the processing timeline of additional beam switching generally involves two parts, one is PDCCH processing and the other is associated beam adjustment e.g., intra-UE communication, beam selection, sub-array switching and phase-shift change, etc. Given the fact that the higher frequency band is sort of FR2-1 extension to beyond 52.6GHz, processing time for new SCSs is preferrable to allow reusing the existing FR2-1 hardware regardless of SCS. It is the reason that RAN1 agreed the following in the RAN1 #104 e-meeting as a general guideline for timeline related discussions [1]:

Agreement: - RAN1 use the absolute time duration for 120 kHz SCS as the upper bound for the discussion of UE processing timelines (not related to PDCCH monitoring) for 480 kHz and 960 kHz SCS for NR operation in 52.6 to 71 GHz - RAN1 strives to reduce the absolute time durations from the upper bound if feasible

Proposal 1: For additional beam switching time delay ‘d’, reuse the absolute time defined for 60kHz i.e., 28 symbols for 120kHz and 112 symbols for 480kHZ.

## 2.2 Potential Enhancements for BM operation

In RAN1 104 e-meeting, support of Multiple Beams for Multiple PDSCHs was discussed. It was agreed to further study the QCL assumption(s) the UE should apply for each PDSCH in multi-PDSCH scheduled by a single DCI as shown in FIG.2:

- Case 1: When all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL.

- Case 2: When some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL.

Figure 2: Identified use cases for potential beam management enhancement

In Rel-15/16, for Case 1 and 2, a UE assumes that the DM-RS ports of PDSCH of a serving cell are QCLed with the RS(s) in the CORESET associated with a monitored search space with the lowest CORESET ID in the latest slot within the active BWP of the serving cell are monitored by the UE. Consequently, QCL assumption for the TBs in Case 1 and TB1/TB2 in Case 2 maybe varied if the QCL assumption of CORESETs in the latest slot are different.

For 480kHz and 960kHz SCS, the slot duration becomes much shorter compared to SCSs supported for FR2 in earlier release. It motivates to introduce a new mechanism to allow a single default QCL assumption for TBs that have scheduling offset less than timeDurationForQCL. In RAN1 106-e meeting, two alternatives were identified as follows:

Agreement: - PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - Down select one of the following alternatives - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs - FFS: Details of single QCL assumption - Alt 2: multiple QCL assumptions are applied - FFS: Details of multiple QCL assumptions - FFS: When some of PDSCHs are collided with semi-static UL symbols and then skipped - FFS: The multi-TRP case

As already mentioned above, a key design principle for beam management with larger 480/960kHz SCSs is to as much as possible avoid beam switching times due to quite short slot duration. For this reason, Alt.1 is preferable compared to Alt.2. Regarding the single QCL, one possible way is to make it configurable by network based on the latest beam management report from UE. A new MAC CE or scheduling DCI adopted by Rel-17 FeMIMO can be further utilized to achieve a faster update of the default QCL assumption.

Proposal 2: Support a mechanism to allow a single QCL assumption at least for multi-PDSCH scheduled by a single DCI that have scheduling offset less than timeDurationForQCL.

One more relevant FFS aspect is how to handle the MOs within different QCL assumptions within a multi-PDSCH duration. It should be noted that it can happen in principle due to separate configurations of MOs and multi-PDSCH scheduling. As illustrated in FIG.3, different QCL assumptions between the latest CORESETs and PDSCHs result in additional gap(s) to perform beam switching, given the fact that the UE is still required to monitor PDCCHs in the CORESET. The spectrum efficiency loss caused by the gap depends on several factors, e.g., length of switching gap, the period of search space associated with CORESET and TCI configuration of the CORESET(s). Since all of this relevant information is known at the gNB side, it may be beneficial to consider some mechanisms to allow gNB enabling or disabling the PDCCH monitoring within the scheduled multiple PDSCH duration such that the switching gap can be dynamically adapted based on the relevant factors.

Figure 3: The switching gap in case different TCIs between default beam and CORESETs

Proposal 3: Consider a mechanism to enable/disable the PDCCH monitoring within the multi-PDSCH duration to improve resource efficiency.

# 3. Conclusion

In this contribution, we have presented our views on open issues related to beam management enhancement with new SCSs on above 52.6GHz frequency bands. Based on the discussions above, the following was proposed:

Proposal 1: For additional beam switching time delay ‘d’, reuse the absolute time defined for 60kHz i.e., 28 symbols for 120kHz and 112 symbols for 480kHZ.

Proposal 2: Support a mechanism to allow a single QCL assumption at least for multi-PDSCH scheduled by a single DCI that have scheduling offset less than timeDurationForQCL.

Proposal 3: Consider a mechanism to enable/disable the PDCCH monitoring within the multi-PDSCH duration to improve resource efficiency.

# References

- RAN1 104bis e-meeting Chairman notes

- RAN1 106 e-meeting Chairman notes